3GPP TSG-RAN WG1 Meeting #108-e	R1-2200952

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon

Title:	Remaining issue of initial access signals and channels for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss our views on the remaining issues of initial access including discovery burst transmission window (DBTW), Q value, and CORESET#0 configuration according agreements / summary in [1][2].

Discovery burst transmission window and Q value

Discovery burst transmission window length

Agreement (RAN1 #107-e)

For 480 and 960 kHz, supported DBTW lengths are:

- {1.25, 1, 0.75, 0.5, 0.25, 0.125, X} ms, where X = 0.0625 if Q=8 is supported and X is removed if Q=8 is not supported.

In RAN1#107-e, the following agreement on DBTW lengths for 480 and 960 kHz was reached.

In NRU Rel-16, if discoveryBurstWindowLength is not provided, UE assumes that the duration of the discovery burst transmission window is a half frame. Similarily, if discoveryBurstWindowLength is not provided in FR2-2, UE can assume 5ms as the default DBTW length for all supported SCS (120/480/960 kHz). UE behavior regarding SSB search remains unchanged as long as the default DBTW length includes all SSB candidate positions and there is no need to modify related spec.

Proposal 1: Support default DBTW length of 5ms if discoveryBurstWindowLength is not provided in FR2-2.

- No specification change is needed.

Agreement (RAN1 #106b-e):

For 120kHz SCS, for NSSBQCL values:

- If 2 bits are available in MIB for NSSBQCL, at least support {16, 32, 64}

- If 1 bit is available in MIB for NSSBQCL, support {32, 64}

  - FFS: methods to indicate more NSSBQCL values without increasing used number of bits, e.g., {16, 32, 64}

- Note: value NSSBQCL < 64 indicates DBTW enabled/supported and operation with shared spectrum.

- Note: For operation without shared spectrum channel access, a UE expects to be configured with NSSBQCL = 64. Use of NSSBQCL=64 in shared spectrum is not precluded.

- FFS: 1 bit or 2 bits used for NSSBQCL

Agreement (RAN1 #107-e):

- Support DBTW with 480 and 960 kHz SCS.

- For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame

- Working assumption: Use 2 bits for Q:

  - SubcarrierSpacingCommon

  - spare bit in MIB

- Send LS to RAN2 for confirming the use of the spare bit in MIB

  - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used

Agreement (RAN1 #107-e):

- Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz.

- Supported values of NSSBQCL: {16, 32, 64}

  - Note:

    - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled

    - UE is expected to be configured with NSSBQCL=64 in licensed operations

    - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

Agreement (RAN2 #116b-e):

- A1: RAN2 does not agree to using the spare bit in MIB for the signaling of FR2-2 QCL assumptions for SSB. Respond to RAN1 LS accordingly.

- A2: The legacy MIB is used for FR2-2 (i.e. we do not define new MIB for FR2-2).

- A3: Up to RAN1 how to resolve QCL configuration (no suggestions from RAN2). This need not be included in LS to RAN1.

QCL relations among SSB candidates

The agreements on NSSBQCL in RAN1#106b-e, 107-e and RAN2#116b-e are listed. In RAN2#116b-e, RAN2 does not agree to use the spare bit in MIB for the signaling of FR2-2 QCL assumptions for SSB. The legacy MIB is used for FR2-2 (i.e. RAN2 doesn’t define new MIB for FR2-2). Since only 1 bit is available in MIB for NSSBQCL, according to agreement in RAN1#106b-e, {32, 64} is supported to indicate NSSBQCL in MIB for 120 kHz. In RAN1#107-e, it was further agreed that DBTW is supported for 480 and 960 kHz SCS and, assuming that both SubcarrierSpacingCommon and spare bit in MIB are available to indicate NSSBQCL, NSSBQCL={16, 32, 64} was supported for all three numerologies. Given RAN2 decision to not to use the spare bit in MIB and aligned with the previous RAN1 agreements on supporting DBTW and using at least SubcarrierSpacingCommon to indicate NSSBQCL for all three numerologies, we propose to support indicating NSSBQCL ={32,64} in MIB using subCarrierSpacingCommon for 120, 480 and 960 kHz SCS. Therefore, the minimum indicated NSSBQCL would be 32 during initial access.

Proposal 2: Aligned with the previous RAN1 agreements, indication of NSSBQCL ={32,64} in MIB is supported using subCarrierSpacingCommon for 120, 480 and 960 kHz SCS

Note that, for operation with shared spectrum, indicating NSSBQCL =32 in MIB does not restrict the gNB to transmit 32 distinct SSB beams. For instance, while NSSBQCL =32 is indicated in MIB, gNB can still transmit only M distinct SSB beams where 32 modM=0 (for instance, M=16). In such a case, a UE that has detected the SSB with candidate index idwould monitor Type0-PDCCH in monitoring occasions corresponding to candidate SSB indexes ic where ic mod32= id mod32 and, further, may soft combine the PDSCH associated with Type0-PDCCH corresponding to such candidate SSB indexes id and ic.  Note that, as 32 modM=0, ic mod32= id mod32 implies that ic modM= id modM which, in turns, means that the detected SSB with candidate index id is actually QCL with the SSB with candidate index ic.

In NRU Rel-16, QCL relation between SSB candidate indexes are indicated not only in MIB using the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset but also in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon using SSB-PositionQCL-Relation IE.  Table 1 shows occurrences of SSB-PositionQCL-Relation IE along with the corresponding field names/descriptions in 38.331. Following observations can be made from Table 1:

- SSB-PositionQCL-Relation IE may be common among cells/SSBs on the indicated frequency (frequency specific) or applied to a specific cell/SSB (cell specific).

- If cell specific value is indicated, it overwrites the frequency specific value for the cell.

Moreover, according to 38.213, for operation with shared spectrum in both FR1 and FR2-2, in a serving cell “NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block”. Therefore, specification already supports overwriting the QCL relation obtained from the MIB using dedicated signaling.  Based on the above discussion, we propose the following

Proposal 3: For operations with shared spectrum, support indicating SSB-PositionQCL-Relation in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon with parameter values {16,32,64}.

- SSB-PositionQCL-Relation applicable to a serving cell overwrites NSSBQCL ={32,64} acquired from the MIB of that cell.

  - Note: Such overwriting mechanism is already supported in 38.213.

Table 1: Occurrences of SSB-PositionQCL-Relation IE in 38.331

| Parent IE | Field name | Description |
| --- | --- | --- |
| SIB2 | ssb-PositionQCL-Common-r16 | Frequency specific: QCL relation between SSB candidate indexes common for intra-frequency neighbor cells |
| SIB3 | ssb-PositionQCL-r16 | Cell specific: QCL relation between SSB candidate indexes for a specific intra-frequency neighbor cell. If provided, this overwrites the value provided by ssb-PositionQCL-Common-r16 in SIB2 for the indicated cell. |
| SIB4 | ssb-PositionQCL-Common-r16 | Frequency specific: QCL relation between SSBs common among inter-frequency neighbor cells on a specific frequency. |
| SIB4 | ssb-PositionQCL-r16 | Cell specific: QCL relation between SSB candidate indexes for a specific inter-frequency neighbor cell. If provided, this overwrites the value provided by ssb-PositionQCL-Common-r16 in SIB4 for the indicated cell. |
| MeasObjectNR | ssb-PositionQCL-Common-r16 | Frequency specific: QCL relation between SSB candidate indexes for all measured cells in this MeasObjectNR (the SSBs have the same frequency and SCS) |
| MeasObjectNR | ssb-PositionQCL-r16 | Cell specific: QCL relation between SSB candidate indexes for a specific measured cell as specified in this MeasObjectNR. If provided, this overwrites the value signaled by ssb-PositionQCL-Common-r16 in this MeasObjectNR. |
| ServingCellConfigCommon | ssb-PositionQCL-r16 | Cell specific: QCL relation between SSB candidate indexes for the serving cell configured in this ServingCellConfigCommon |

The note in the above RAN1 #107-e agreement, “UE is expected to be configured with NSSBQCL=64 in licensed operations” was not captured in the specification. We believe that it is important to specify that the UE is expected to be indicated with NSSBQCL=64 in a licensed band as, during initial access, UE may not know whether it is operating in a licensed band or an unlicensed band. Therefore, if UE is indicated with  NSSBQCL<64 in a licensed band, UE may assume that it operates in an unlicensed band and would monitor Type0-PDCCH associated with candidate SSB indexes ic where ic modNSSBQCL= id modNSSBQCL and idis the candidate SSB index of the detected SSB. This would needlessly increase UE monitoring effort during the initial access as, in a licensed band, these additional monitoring occasions would not carry a Type0-PDCCH or carry a Type0-PDCCH with a different QCL assumption than the detected SSB with candidate index id  and, hence, in general, are not detectable using the beam used for the detection of SSB with candidate index id. There were some comprehensive discussions on how to capture the UE assumption of NSSBQCL=64 for licensed band in 38.213. Some companies were concerned to adopt the note in the RAN1 #107-e agreement “as is” since it may imply that gNB should configure NSSBQCL through SSB-PositionQCL-Relation even in a licensed band deployment. One way to address this concern is to capture in 38.213 that, for operation without shared spectrum channel access in FR2-2, UE assumes NSSBQCL=64 and expects that the same value for NSSBQCL is indicated in a MIB provided by a SS/PBCH block. Another alternative is to not to define NSSBQCL for licensed band altogether and capture in 38.213 that, for operation without shared spectrum channel access in FR2-2, UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block.

Proposal 4: The Note “UE is expected to be configured with NSSBQCL=64 in licensed operations” in the agreement in RAN1#107-e is captured in 38.213 by one of the following alternatives:

- ALT 1:  “For operation without shared spectrum channel access in FR2-2, UE assumes NSSBQCL=64 and expects that the same value for NSSBQCL is indicated in a MIB provided by a SS/PBCH block”.

  - Adopt TP#1 in Section 4.1 of TS 38.213.

- ALT 2: “For operation without shared spectrum channel access in FR2-2, UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block.”

  - Adopt TP#1A in Section 4.1 of TS 38.213.

#### TP#1 for TS38.213

## ===========Start of TP#1 for TS 38.213 =========== ## 4.1 Cell search *** unchanged part omitted *** For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** unchanged part omitted *** For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. For operation without shared spectrum channel access in FR2-2, UE assumes NSSBQCL=64 and expects that the same value for NSSBQCL is indicated in a MIB provided by a SS/PBCH block. Table 4.1-2: Mapping subCarrierSpacingCommon to NSSBQCL for operation with and without shared spectrum channel access in FR2-2 subCarrierSpacingCommon; NSSBQCL. scs15or60; 32. scs30or120; 64. *** unchanged part omitted *** ===========End of TP#1 for TS 38.213 ===========

#### TP#1A for TS38.213

## ===========Start of TP#1A for TS 38.213 =========== ## 4.1 Cell search *** unchanged part omitted *** For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** unchanged part omitted *** For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. For operation without shared spectrum channel access in FR2-2, a UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block. Table 4.1-2: Mapping subCarrierSpacingCommon to NSSBQCL for operation with spectrum channel access in FR2-2 subCarrierSpacingCommon; NSSBQCL. scs15or60; 32. scs30or120; 64. *** unchanged part omitted *** ===========End of TP#1A for TS 38.213 ===========

CORESET#0/Type0-PDCCH common search space design

CORESET#0 configuration for 120 kHz SCS

Working assumption (RAN1 #106b-e) For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz, support multiplexing pattern 1 with 96 PRB CORESET#0, and {1, 2} symbol durations • Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design. Agreement (RAN4 #101b-e): Consider the different channelization for licensed band(s) and unlicensed band(s) - Fixed sync raster for unlicensed bands - Fixed scheme should not be constrained by IEEE channel raster - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster. - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs - Floating sync raster for licensed bands - Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity. - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands

RB offset values from CORESET#0 to SSB are not agreed yet for the set of agreed (Mux pattern, NRBCORESET, NsymbCORESET) for 120 kHz, 480 kHz, and 960 kHz in FR2-2. Although the exact values of supported RB offsets from CORESET#0 to SSB may be restricted by the synch raster and channel raster design in FR2-2, RAN1 can still discuss and agree on the design principles and the nominal values of these RB offsets. In our companion paper [3], we propose a solution to the channel raster and sync raster. Aligned with the above agreement in RAN4 101b-e, within the frequency range from 66GHz to 71GHz where licensed band operation is expected, we propose to use the floating channel raster similar as those in FR2-1 with the step size of 120kHz for 120kHz SCS. The sync raster with 3*17.28MHz gap is assumed according to the agreement in RAN4. It is observed that the RB offsets in Rel-15 for the supported (Mux pattern, NRBCORESET, NsymbCORESET) tuples can be inherited which ensures at least one sync raster point for each channel. For band n263 (57-71GHz) where shared spectrum operation is deployed, it is beneficial to adopt fixed channelization on the consecutive non-overlapped 100MHz channel grid as shown in Figure 1. It balances the requirement of interference mitigation among unlicensed deployments sharing same frequency band and the flexibility of deployment for different unlicensed band allocations in different regions. Two sync raster points per 100MHz channel are proposed to provide additional flexibility for SSB configuration. For CORESET#0 with 48 and 96 PRB with multiplexing pattern 1, it would be beneficial for gNB to transmit portion of discovery burst (e.g. SSB, Type0-PDCCH, RMSI PDSCH) with the same beam and as compact as possible in time and frequency domain to save LBT and beam switching overheads. Considering only frequency domain resource allocation type 1 is allowed for RMSI PDSCH, aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for RMSI PDSCH. Similar concept has been adopted in NR-U Rel-16. Based on the above discussion, Table 13-10A in 38.213 can be updated to Table 1 as follows:

Figure 1	Examples of channel raster for n263 with shared spectrum operation

Figure 2 Example of RB-level offset configuration for {SS/PBCH block, PDCCH}={120, 120} kHz when 96 RB CORESET0 is configured.

Table 2 Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 5 | 3 | 24 | 2 | 24 |
| 6 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 7 | 3 | 48 | 2 | 48 |
| 8 | 1 | 48 | 1 | 0 |
| 9 | 1 | 48 | 1 | 28 |
| 10 | 1 | 48 | 2 | 0 |
| 11 | 1 | 48 | 2 | 28 |
| 12 | 1 | 96 | 1 | 0 |
| 13 | 1 | 96 | 1 | 76 |
| 14 | 1 | 96 | 2 | 0 |
| 15 | 1 | 96 | 2 | 76 |

Proposal 5: Support the following CORESET#0 RB offsets for {SSB, CORESET#0} SCS={120, 120} kHz:

- For CORESET#0 with 24 RBs: The same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: The same as supported values in Table 13-8 of 38.213 in addition to RB offset values of [0] and [28] RBs for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets of [0] and [76] RBs for multiplexing pattern 1.

CORESET#0 configuration for 480 kHz and 960 kHz SCS

Working assumption (RAN1 #107-e) For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets), if additional entries are left, support multiplex pattern 3 with 24 PRB and 2 symbol duration, and multiplexing pattern 3 with 48 PRB and 2 symbol duration. Working assumption (RAN1 #107-e) For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets) and multiplex pattern 3 with 24 and 48 PRB and 2 symbol duration (with required RB offsets), if additional entries are left, support multiplexing pattern 1 with 96 PRB and 2 symbol duration - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design.

Concerns regarding RB offset design for {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz are even more important in 480/960 kHz as the beam switching delay may be too large to be absorbed in CP in 480/960 kHz SCS and, therefore, it is critical to have the possibility of transmitting SSB and its associated RMSI PDCCH/PDSCH without a need for a switching gap. Also, as discussed earlier, aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for PDSCH carrying SIB1. For multiplexing pattern 3 with 24 and 48 RBs, the corresponding RB offsets can be adopted from Table 13-8 of 38.213. In summary, considering the synch raster and channel raster design in FR2-2 in our companion paper [3], we propose the following:

Proposal 6: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

- For CORESET#0 with 24 RBs: The same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: The same as supported values in Table 13-8 of 38.213 in addition to RB offset values of [0] and [28] RBs for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets of [0] and [76] RBs for multiplexing pattern 1.

Figure 3 Example of RB-level offset configuration for {SS/PBCH block, PDCCH}={480, 480} kHz and {960, 960} kHz when 48 RB CORESET#0 is configured.

Based on the above discussion, Table 13-10B in 38.213 can be updated to Table 2 as follows:

Table 3   Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz and {960, 960} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 9 | 3 | 24 | 2 | 24 |
| 10 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 11 | 3 | 48 | 2 | 48 |
| 12 | 1 | 96 | 2 | 0 |
| 13 | 1 | 96 | 2 | 76 |

RRC Parameters

SSB-PositionQCL-Relation-r17

The following conclusion was reached in RAN1 #107b-e regarding the inclusion of SSB-PositionQCL-Relation-r17 to RRC parameter list. However, this IE was not included in the RRC parameter list [6].

Conclusion (RAN1 #107b-e): RRC parameters list to capture changes identified below: - Add the following note to the comment section of discoveryBurstWindowLength-r17 row in RRC parameter list - “Note: This parameter is to be included in both SIB1 and the common serving cell configuration parameters”. - Support adding “SSB-PositionQCL-Relation-r17” to RRC parameter list as both UE-specific and cell-specific parameter. - Inform RAN2 (by adding notes to RRC parameter list) that the either the value range of the information element SSB-PositionQCL-Relation-r16 needs to be extended, or a new Rel-17 IE need to be defined to allow configuration of Q = 16, 32, or 64 in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon for RRM measurements when operating with shared spectrum channel access in FR2-2.

We think SSB-PositionQCL-Relation-r17 should be included in the RRC parameter list as per the conclusion in RAN1 #107b-e.

Proposal 7: Include SSB-PositionQCL-Relation-r17 in the RRC parameter list as per the conclusion in RAN1 #107b-e.

Conclusions

In this paper, we discuss the issues related to initial access. The following proposals are made.

Proposal 1: Support default DBTW length of 5ms if discoveryBurstWindowLength is not provided in FR2-2.

- No specification change is needed.

Proposal 2: Aligned with the previous RAN1 agreements, indication of NSSBQCL ={32,64} in MIB is supported using subCarrierSpacingCommon for 120, 480 and 960 kHz SCS

Proposal 3: For operations with shared spectrum, support indicating SSB-PositionQCL-Relation in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon with parameter values {16,32,64}.

- SSB-PositionQCL-Relation applicable to a serving cell overwrites NSSBQCL ={32,64} acquired from the MIB of that cell.

  - Note: Such overwriting mechanism is already supported in 38.213.

Proposal 4: The Note “UE is expected to be configured with NSSBQCL=64 in licensed operations” in the agreement in RAN1#107-e is captured in 38.213 by one of the following alternatives:

- ALT 1:  “For operation without shared spectrum channel access in FR2-2, UE assumes NSSBQCL=64 and expects that the same value for NSSBQCL is indicated in a MIB provided by a SS/PBCH block”.

  - Adopt TP#1 in Section 4.1 of TS 38.213.

- ALT 2: “For operation without shared spectrum channel access in FR2-2, UE expects subCarrierSpacingCommon = ‘scs30or120’ from a MIB provided by a SS/PBCH block.”

  - Adopt TP#1A in Section 4.1 of TS 38.213.

Proposal 5: Support the following CORESET#0 RB offsets for {SSB, CORESET#0} SCS={120, 120} kHz:

- For CORESET#0 with 24 RBs: The same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: The same as supported values in Table 13-8 of 38.213 in addition to RB offset values of [0] and [28] RBs for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets of [0] and [76] RBs for multiplexing pattern 1.

Proposal 6: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

- For CORESET#0 with 24 RBs: The same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: The same as supported values in Table 13-8 of 38.213 in addition to RB offset values of [0] and [28] RBs for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets of [0] and [76] RBs for multiplexing pattern 1.

Proposal 7: Include SSB-PositionQCL-Relation-r17 in the RRC parameter list as per the conclusion in RAN1 #107b-e.

- References

- R1-2200690, “Summary #2 of email discussion on initial access aspect of NR extension up to 71GHz”, Moderator (Intel Corporation)

- Chair’s Notes RAN1#107b-e

- R1- 2202437, “Discussion on the channel raster and sync raster in FR2-2”, Huawei, HiSilicon

- ETSI EN 301 893.

- ETSI EN 302 567.

- R1-2200699, “Consolidated higher layers parameter list for Rel-17 NR”, Moderator (Ericsson)