3GPP TSG RAN WG1 Meeting #108-e                                                                                    R1-2200954

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	Remaining issues of PUCCH enhancement for 52-71GHz spectrum

Document for:	Discussion and Decision

# Introduction

The discussion on enhancing PUCCH format 0, 1, and 4 to multiple physical resource blocks (PRBs) is almost finished. In this contribution, potential CRs are proposed according to the agreements up to RAN1 107b-e meeting.

# Potential CRs for multi-RB PUCCH format 0/1/4

## Resource mapping for multi-RB PUCCH in TS 38.211

In Rel-15/16, PUCCH format 0/1/4 occupy one resource block. In Rel-17, multi-RB PUCCH format 0/1/4 are supported to increase the number of RBs under PSD limitation in shared spectrum operation. The descriptions of mapping to physical resources for PUCCH format 0/1/4 in TS 38.211 [1] are shown as follows. Note that the description for PUCCH format 0 is modified for Rel-17.

Clause 6.3.2.3.2 in TS 38.211 for PUCCH format 0 ##### 6.3.2.3.2 Mapping to physical resources The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with to resource elements k,lp,μ assigned for transmission according to clause 9.2.1 of [5, TS 38.213] in increasing order of first the index over the assigned physical resources spanning MRBPUCCH,0 resource blocks, and then the index l on antenna port p=2000.

Clause 6.3.2.4.2 in TS 38.211 for PUCCH format 1 ##### 6.3.2.4.2 Mapping to physical resources The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with to resource elements k,lp,μ which meet all of the following criteria: - they are in the resource blocks assigned for transmission according to clause 9.2.1 of [5, TS 38.213], - they are not used by the associated DM-RS The mapping to resource elements k,lp,μ not reserved for other purposes shall be in increasing order of first the index over the assigned physical resource block, and then the index on antenna port .

Clause 6.3.2.6.5 in TS 38.211 for PUCCH format 3 and 4 ##### 6.3.2.6.5 Mapping to physical resources The block of modulation symbols z0,…,z(NSFPUCCH,sMsymb-1) shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with z(0) to resource elements (k,l)p,μ which meet all of the following criteria: - they are in the resource blocks assigned for transmission, - they are not used by the associated DM-RS The mapping to resource elements (k,l)p,μ not reserved for other purposes shall be in increasing order of first the index over the assigned physical resource blocks according to clause 9.2.1 of [5, TS 38.213], and then the index on antenna port .

It is our understanding that PUCCH format 1 and 4 can be assigned to either one or more than one physical resource block. Hence the current descriptions that “over the assigned physical resource block” for PUCCH format 1 and “over the assigned physical resource blocks” for PUCCH format 4 are not accurate. The modification should be aligned with PUCCH format 0, which would not affect resource mapping for PUCCH format 3. As such, we propose the following TP#1:

Proposal 1: Support the following TP#1 in clause 6.3.2.4.2 and clause 6.3.2.6.5 of TS 38.211.

TP#1 for TS 38.211 Clause 6.3.2.4.2 and Clause 6.3.2.6.5 < Unchanged parts are omitted > ##### 6.3.2.4.2 Mapping to physical resources The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with to resource elements k,lp,μ which meet all of the following criteria: - they are in the resource blocks assigned for transmission according to clause 9.2.1 of [5, TS 38.213], - they are not used by the associated DM-RS The mapping to resource elements k,lp,μ not reserved for other purposes shall be in increasing order of first the index over the assigned physical resource blocks spanning MRBPUCCH,1 resource blocks, and then the index on antenna port . < Unchanged parts are omitted > ##### 6.3.2.6.5 Mapping to physical resources The block of modulation symbols z0,…,z(NSFPUCCH,sMsymb-1) shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with z(0) to resource elements (k,l)p,μ which meet all of the following criteria: - they are in the resource blocks assigned for transmission, - they are not used by the associated DM-RS The mapping to resource elements (k,l)p,μ not reserved for other purposes shall be in increasing order of first the index over the assigned physical resource blocks spanning MRBPUCCH,s resource blocks according to clause 9.2.1 of [5, TS 38.213], and then the index on antenna port . < Unchanged parts are omitted >

## RE calculation for multiplexing of coded UCI bits to PUCCH format 4 in TS 38.212

The Clause 6.3.1.6 in TS 38.212 [2] describes multiplexing of coded UCI bits to PUCCH. The following formula for RE calculation should be modified to support multi-RB PUCCH format 4.

Clause 6.3.1.6 in TS 38.212 for multiplexing of coded UCI bits to PUCCH For PUCCH format 3, set NUCIsymbol=12⋅NPRBPUCCH,3/NSFPUCCH,3 , where is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,3 is the spreading factor for PUCCH format 3 [4, TS 38.211]. For PUCCH format 4, set NUCIsymbol=12/NSFPUCCH,4, where is the spreading factor for PUCCH format 4.

The modification can be referred to the formula for PUCCH format 3, which is also included in the box above. Therefore, we propose the following TP#2:

Proposal 2: Support the following TP#2 in clause 6.3.1.6 of TS 38.212.

TP#2 for clause 6.3.1.6 of TS 38.212 < Unchanged parts are omitted > For PUCCH format 3, set NUCIsymbol=12⋅NPRBPUCCH,3/NSFPUCCH,3 , where is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,3 is the spreading factor for PUCCH format 3 [4, TS 38.211]. For PUCCH format 4, set NUCIsymbol=12⋅NPRBPUCCH,4/NSFPUCCH,4, where NPRBPUCCH,4 is the number of PRBs that is determined by the UE for PUCCH format 4 transmission according to Clause 9.2 of [5, TS 38.213], and is the spreading factor for PUCCH format 4. < Unchanged parts are omitted >

## Typo for PUCCH format 4 in TS 38.213

A typo for PUCCH format 4 should be fixed in Clause 9.2.1 of TS 38.213 [3] and we propose the following TP#3:

Proposal 3: Support the following TP#3 in clause 9.2.1 of TS 38.213.

TP#3 in clause 9.2.1 of TS 38.213 < Unchanged parts are omitted > If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. For PUCCH transmission in FR2-2, the PUCCH resource can also include a number of PRBs MRBPUCCH,4 provided by nrofPRBs; otherwise, MRBPUCCH,1MRBPUCCH,4=1. < Unchanged parts are omitted >

# Conclusions

Based on the discussion above, we arrive at the following proposal.

Proposal 1: Support the following TP#1 in clause 6.3.2.4.2 and clause 6.3.2.6.5 of TS 38.211.

TP#1 for TS 38.211 Clause 6.3.2.4.2 and Clause 6.3.2.6.5 < Unchanged parts are omitted > ##### 6.3.2.4.2 Mapping to physical resources The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with to resource elements k,lp,μ which meet all of the following criteria: - they are in the resource blocks assigned for transmission according to clause 9.2.1 of [5, TS 38.213], - they are not used by the associated DM-RS The mapping to resource elements k,lp,μ not reserved for other purposes shall be in increasing order of first the index over the assigned physical resource blocks spanning MRBPUCCH,1 resource blocks, and then the index on antenna port . < Unchanged parts are omitted > ##### 6.3.2.6.5 Mapping to physical resources The block of modulation symbols z0,…,z(NSFPUCCH,sMsymb-1) shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with z(0) to resource elements (k,l)p,μ which meet all of the following criteria: - they are in the resource blocks assigned for transmission, - they are not used by the associated DM-RS The mapping to resource elements (k,l)p,μ not reserved for other purposes shall be in increasing order of first the index over the assigned physical resource blocks spanning MRBPUCCH,s resource blocks according to clause 9.2.1 of [5, TS 38.213], and then the index on antenna port . < Unchanged parts are omitted >

Proposal 2: Support the following TP#2 in clause 6.3.1.6 of TS 38.212.

TP#2 for clause 6.3.1.6 of TS 38.212 < Unchanged parts are omitted > For PUCCH format 3, set NUCIsymbol=12⋅NPRBPUCCH,3/NSFPUCCH,3 , where is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,3 is the spreading factor for PUCCH format 3 [4, TS 38.211]. For PUCCH format 4, set NUCIsymbol=12⋅NPRBPUCCH,4/NSFPUCCH,4, where NPRBPUCCH,4 is the number of PRBs that is determined by the UE for PUCCH format 4 transmission according to Clause 9.2 of [5, TS 38.213], and is the spreading factor for PUCCH format 4. < Unchanged parts are omitted >

Proposal 3: Support the following TP#3 in clause 9.2.1 of TS 38.213.

TP#3 in clause 9.2.1 of TS 38.213 < Unchanged parts are omitted > If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. For PUCCH transmission in FR2-2, the PUCCH resource can also include a number of PRBs MRBPUCCH,4 provided by nrofPRBs; otherwise, MRBPUCCH,1MRBPUCCH,4=1. < Unchanged parts are omitted >

# References

- 3GPP TS 38.211-h00, “NR; Physical channels and modulation”

- 3GPP TS 38.212-h00, “NR; Multiplexing and channel coding”

- 3GPP TS 38.213-h00, “NR; Physical layer procedures for control”