3GPP TSG RAN WG1 Meeting #108-e	R1- 2200956

e-Meeting, 	February 21 – March 3, 2022

Agenda Item:	8.2.5

Source:	Huawei, HiSilicon

Title:	Remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum left after RAN1#107bis-e [1] [2], including multiple PDSCH/PUSCH scheduling and HARQ.

# DMRS enhancement

In RAN1#107bis-e meeting, there is a discussion about whether or not DMRS bundling in Rel-17 Coverage enhancement WI can be applied for 120 kHz SCS of FR2-2. Based on the views from many companies there is no consensus to support DMRS bundling across multiple PUSCH with different TB in FR2-2 in Rel-17. But the applicable of DMRS bundling for the same TB to FR2-2 with 120 kHz SCS is left for further discussion. In our view, coverage issue also exits in FR2-2 due to larger path loss than FR2-1 with the higher frequency range. Extending this feature to FR2-2 for 120kHz SCS is beneficial to enrich the usage scenario without additional standard effort.

Proposal 1: Support DMRS bundling FG developed in CovEnh WI for the same TB to FR2-2 with 120 kHz SCS.

# Enhancement on PDSCH and PUSCH

## Clarification on “scheduling PXSCH” in early agreements

In RAN1#107bis-e meeting, whether “scheduled PDSCH” or “scheduled PUSCH” in early agreement corresponding to valid SLIV or configured SLIV has been discussed for the following cases:

Case 1: For NDI/RV, are NDI/RV fields for invalid PXSCHs present in multi-PXSCH scheduling DCI? Case 2: For RV field, is the bit-width between 1 bit and 2 bits determined based on the number of configured SLIVs or valid SLIVs? Case 3: For CSI-request, is the number M determined based on the number of configured SLIVs or valid SLIVs? Case 4: For CBGTI field, is the presence of CBGTI field determined based on the number of configured SLIVs or valid SLIVs? Case 5: For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs? Case 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI. Case 7: In the case of multi-PDSCH scheduling via a single DCI with ‘tdmSchemeA’, cancel both of two repeated PDSCHs if at least one of repeated PDSCHs collides with semi-static UL symbols or determine the validity rule for each of repeated PDSCHs

For cases of 1/2/4, there are agreements that the NDI/RV and CBGTI fields in the DCI correspond to the configured SLIVs indicated by TDRA. The remaining cases are still under discussion.

For case 3, it is described in TS38.214 that “When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH”. If the “scheduled PUSCH” is translated as “configured SLIV”, gNB need to ensure that the (M-1)th configured SLIV indicated by TDRA is not collided with any pre-configured DL transmission. Otherwise, the CSI feedback cannot be transmitted. Thus, we support to interpret “scheduled PUSCH” as valid SLIV because it can maximize the probability of CSI feedback without introducing additional scheduling restriction.

For case 5, we support to interpret “scheduled PDSCH” corresponding to valid SLIV. The motivation to forbid Out-Of-Order scheduling is to reduce the number of parallel HARQ processing at receivers. If the PDSCH is not transmitted, the complexity on HARQ process does not exists. On the other hand, there is less restriction of scheduling from OOO issues when only valid SLIVs are counted. Some companies [2] expressed concerns that the timeline of scheduling in the spec need to be updated if valid SLIV is adopted.  In our view, the timeline of scheduling and restriction of scheduling are two different things. The timeline of scheduling is to guarantee the processing time and the restriction of scheduling is to guarantee the low complexity of HARQ processing. The timeline of scheduling can be shorter if valid SLIV is adopted, but it does not mean that it needs to be relaxed. When UE receive the scheduling signals, UE assumes the OOO case does not happen, so it is reasonable to regard the OOO scheduling is determined based on the valid SLIV. The existing timeline can still work if valid SLIV is adopted for OOO scheduling.

For case 6, we support to interpret “scheduled PDSCH” corresponding to valid SLIV. This is one of the OOO case in NRU and the consistent behavior as case 5 should be maintained.

Proposal 2: The “scheduled PUSCH” in case 3 (CSI request) and “scheduled PDSCH” in case 5 (OOO) and case 6 (NNK1) should correspond to valid SLIV.

For case 7, we think this case is not exactly about valid or configured SLIV. It is related to how to deal with the repetitions of PDSCHs transmitted by multi-TRP when one of the repetitions is overlapped with semi-static UL symbols. In our view, the motivation of configuration of ‘tdmSchemeA’ with single DCI is to guarantee the robustness of processing at receiver side, if one of the repetitions is not valid, the robustness may not be ensured. In Rel-16, the conflicting of any one of the repetitions of PDSCHs with semi-static UL symbols is treated as an error case. We suggest skip the whole HARQ process if any one of the repetitions of PDSCHs is overlapped with semi-static UL symbols to align with Rel-16 rules.

Proposal 3: In the case of multi-PDSCH scheduled by a single DCI and repetitionScheme is configured with 'tdmSchemeA', if one of the repetitions of a scheduled PDSCH collides with semi-static UL symbols, the corresponding SLIV is considered not valid.

## Time domain bundling for Type-2 HARQ codebook

Agreement: For multi-PDSCH scheduling with a single DCI: Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell. ….. The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method. For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured) …

In RAN1#107-e meeting, whether configured or valid SLIVs scheduled by single DCI are allocated to bundling groups is left for more discussion. In our view, the fixed HARQ bit allocation among configured SLIVs is simpler for UE implementation because it is independent of valid SLIV which might be dynamically changed. It is also aligned with the decision on HARQ-ACK bit ordering based on the configured SLIV position in the TDRA row when time domain bundling is disabled. So we propose to allocate PDSCH corresponding to the “configured” SLIVs in TDRA row to the bundling groups when time domain bundling is enabled for type-2 HARQ codebook.

Proposal 4: Support to allocate PDSCHs corresponding to configured SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI to the bundling groups.

## Out-of-ordering for DCI-to-Data

In RAN1#107bis-e meeting, there was some discussion about DCI-to-data OOO issues and the following proposal was discussed without agreement:

### Proposal #2.2-1 (DCI-to-data OOO): - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI.

For the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV is considered as an OOO scheduling or not needs further discussion. In our view, the motivation of DCI-to-data OOO scheduling restriction is reduce the number of parallel HARQ process number at receiver side. Considering the maximum number of multiple scheduling PXSCHs by single DCI is eight, if this OOO scheduling case is allowed, UE needs to deal with at most 16 PXSCHs at the same time which will increase the complexity of UE implementation and the processing timeline requirement may not be achieved. Based on the reason above, we suggest this OOO case is regarded as an OOO scheduling.

Proposal 5: UE is not expected to be scheduled with two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol and these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV.

## Calculation of nHARQ-ACK for PUCCH power control when UCI payload size is equal to or less than 11 bits

In RAN1#107bis-e meeting, there was some discussion about the calculation of nHARQ-ACK for PUCCH power control when UCI payload size is equal to or less than 11 bits, current spec needs updating when time domain bundling is enabled. For type-1 CB, it was agreed as following:

Agreement For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows. - For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH (which carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not) only associated with the last SLIV as received, to determine Nm,creceived.

Based on the above agreements, we proposed to adopt the following TP.

Proposal 6: For multi-PDSCH scheduling by single DCI and Type-1 HARQ-ACK codebook, consider the TP#1 in TS38.213 v17.0.0 for the calculation of nHARQ-ACK.

*** < Beginning of TP#1 for TS 38.213 v17.0.0> *** 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ============== Unchanged Text Omitted ====================== If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where - NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs - Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c - Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH. - if enableTimeDomainHARQ-Bundling is provided for a serving cell c, for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, a PDSCH only associated with the last SLIV configured in TDRA table is considered as received, a PDSCH other than the PDSCH associated with the last SLIV configured in TDRA table is considered as not received. - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH. ============== Unchanged Text Omitted ====================== *** < End of TP#1 for TS 38.213 v17.0.0> ***

For type-2 CB, when the time domain is enabled and the number of bundling groups is 1, the HARQ-ACK for multiple PDSCHs scheduled by single DCI will be bundled together and contained in the first HARQ-ACK sub-codebook, the calculation of nHARQ-ACK for the first HARQ-ACK sub-codebook in the current spec for TB based HARQ-ACK feedback could be reused.  If the UE is provided numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell c, or the UE is not provided numberOfHARQ-BundlingGroups but configured with pdsch-TimeDomainAllocationListForMultiPDSCH for a serving cell c, nHARQ-ACK  is the sum of nHARQ-ACK,TB  and nHARQ-ACK,multi where nHARQ-ACK,multi can be determined based on multi-PDSCH scheduling DCI. Based on the above discussion, we propose to adopt the following TP.

Proposal 7: For multi-PDSCH scheduled by single DCI Type-2 HARQ-ACK codebook, consider the TP#2 in TS38.213 v17.0.0 for the calculation of nHARQ-ACK.

*** < Beginning of TP#2 for TS 38.213 v17.0.0> *** 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel ============== Unchanged Text Omitted ====================== If a UE is not provided PDSCH-CodeBlockGroupTransmission and is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH for any serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions and does not support multiple PDSCH receptions, or for SPS PDSCH reception, or for a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, or a UE is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1 for any serving cells, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c+g=0G-1VDAI,mlast,gDL-c=0Ncells,gDL-1UDAI,c,gmodTD,gNTB,max,gDL+c=0Ncells,gDL-1m=0Mg-1Nm,c,greceived+NSPS,c,g where - NcellsDL is a number of serving cells where the UE is configured to receive unicast PDSCHs - Ncells,gDL is a number of serving cells where the UE is configured to receive multicast PDSCHs for a G-RNTI g or a G-CS-RNTI g - G is a total number of G-RNTIs or G-CS-RNTIs configured to the UE - M is the number of PDCCH monitoring occasions for unicast DCI formats - Mg is the number of PDCCH monitoring occasions for multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g - TD=2NC-DAIDL where NC-DAIDL the number of bits for the counter DAI field in unicast DCI formats - TD,g=2NC-DAI,gDL where NC-DAI,gDL the number of bits for the counter DAI field in multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, that the UE detects within the M PDCCH monitoring occasions. - if Ncells,gDL=1, VDAI,mlast,gDL is the value of the counter DAI in the last multicast DCI format with G-RNTI g, or G-CS-RNTI g, scheduling PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the Mg PDCCH monitoring occasions - if NcellsDL>1 or if Ncells,gDL>1 - if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion - if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg, for G-RNTI g or G-CS-RNTI g, PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the total DAI in the at least one DCI format that includes a total DAI field - VDAI,mlastDL=0 or VDAI,mlast,gDL=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c in any of the M or Mg PDCCH monitoring occasions, respectively. - UDAI,c or UDAI,c,g, for G-RNTI g or G-CS-RNTI g, is the total number of DCI formats scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the M or Mg PDCCH monitoring occasions, respectively, for serving cell c. UDAI,c=0 or UDAI,c,g=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for serving cell c in any of the M or Mg, respectively,PDCCH monitoring occasions. - NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1. - NTB,max,gDL=1. - if the UE is not provided numberOfHARQ-BundlingGroups for serving cell c, Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is the number of transport blocks the UE receives in a PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI formats that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c. If the UE is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1 for serving cell c, Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is the number of DCI formats that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided or not provided, or the number of DCI formats that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c - NSPS,c or NSPS,c,g, for G-RNTI g or G-CS-RNTI g, is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M or Mg PDCCH monitoring occasions, respectively. …… If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and - the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and - instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that the UE does not receive in a slot as described in clause 11.1. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits - The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable. - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook. - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook. If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi for obtaining a PUCCH transmission power, as described in clause 7.2.1, with nHARQ-ACK,multi=VDAI,mlastDL-c=0NcellsDL,multi-1UDAI,cmultimodTDNHARQ-ACKmax+c=0NcellsDL,multi-1m=0M-1Nm,creceived,multi where - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions - NcellsDL,multi is the number of serving cells each of which is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH in NcellsDL serving cells - UDAI,cmulti is the total number of DCI formats scheduling more than one PDSCH reception that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cmulti=0 if the UE does not detect any DCI format scheduling more than one PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions - Nm,creceived,multi is the number of transport blocks the UE receives in all PDSCHs scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCHs, excluding any PDSCH that the UE does not receive in a slot as described in clause 11.1, scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and the UE reports corresponding HARQ-ACK information in the PUCCH If a UE is provided numberOfHARQ-BundlingGroups for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. If a UE - is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for - SPS PDSCH reception, - any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and - PDSCH reception scheduled by a DCI format scheduling one PDSCH - PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells, - NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and - instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax HARQ-ACK information bits for serving cell c. - The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable. - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook. - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook. If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi for obtaining a PUCCH transmission power, as described in clause 7.2.1, with nHARQ-ACK,multi=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cmultimodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,multi where - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH reception for any serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, in any of the M PDCCH monitoring occasions - UDAI,cmulti is the total number of DCI formats scheduling more than one PDSCH reception that the UE detects within the M PDCCH monitoring occasions for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1. UDAI,cmulti=0 if the UE does not detect any DCI format scheduling more than one PDSCH reception for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, in any of the M PDCCH monitoring occasions - if harq-ACK-SpatialBundlingPUCCH is not provided, Nm,creceived,multi is the number of transport blocks the UE receives in all PDSCHs scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and not provided numberOfHARQ-BundlingGroups, or the number of bundling groups, each of which corresponds to an enabled transport block in a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, and contains at least a PDSCH that the UE receives in a slot as described in clause 11.1, and the UE reports corresponding HARQ-ACK information in the PUCCH - if harq-ACK-SpatialBundlingPUCCH is provided, Nm,creceived,multi is the number of PDSCHs, excluding any PDSCH that the UE does not receive in a slot as described in clause 11.1, scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and not provided numberOfHARQ-BundlingGroups, or the number of bundling groups, each of which contains at least a PDSCH that is scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, and the UE receives in a slot as described in clause 11.1, and the UE reports corresponding HARQ-ACK information in the PUCCH ============== Unchanged Text Omitted ====================== *** < End of TP#2 for TS 38.213 v17.0.0> ***

# Conclusions

We discussed the remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum including DMRS enhancement, multiple PDSCH/PUSCH scheduling and HARQ with the following observations and proposals.

Proposal 1: Support DMRS bundling FG developed in CovEnh WI for the same TB to FR2-2 with 120 kHz SCS.

Proposal 2: The “scheduled PUSCH” in case 3 (CSI request) and “scheduled PDSCH” in case 5 (OOO) and case 6 (NNK1) should correspond to valid SLIV.

Proposal 3: In the case of multi-PDSCH scheduled by a single DCI and repetitionScheme is configured with 'tdmSchemeA', if one of the repetitions of a scheduled PDSCH collides with semi-static UL symbols, the corresponding SLIV is considered not valid.

Proposal 4: Support to allocate PDSCHs corresponding to configured SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI to the bundling groups.

Proposal 5: UE is not expected to be scheduled with two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol and these two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV.

Proposal 6: For multi-PDSCH scheduling by single DCI and Type-1 HARQ-ACK codebook, consider the TP#1 in TS38.213 v17.0.0 for the calculation of nHARQ-ACK.

Proposal 7: For multi-PDSCH scheduled by single DCI Type-2 HARQ-ACK codebook, consider the TP#2 in TS38.213 v17.0.0 for the calculation of nHARQ-ACK.

# References

- R4-2200684, Discussion summary #1 of [107bis-e-R17-52-71GHz-05]	Moderator (vivo)

- R1-2200569, Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)