3GPP TSG RAN WG1 Meeting #108-e	                                       R1-2200962

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.16.3

Source:	Huawei, HiSilicon

Title:		     Rel-17 UE features for enhanced IIoT/URLLC

Document for:	Discussion and Decision

Introduction

After the RAN1#106-e meeting, the preliminary RAN1 UE features list for Rel-17 [1] was provided, and further updated in RAN1#107b-e [2]. In this paper, our views for Rel-17 UE features for enhanced IIoT/URLLC are given.

Discussions on Rel-17 UE features for enhanced IIoT/URLLC

UE feedback enhancements for HARQ-ACK

- FG 25-1

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- There is a typo “Idenfify” in the column of components.

- FG 25-2/25-3/25-3a

| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Comments:

- For reporting type, we suggest per UE for 25-2 while per FS for 25-3/3a, to align with the granularity of the prerequisites. For FG 25-3b, since FG 25-3 or FG 25-3a would be the pre-requisite, then it can be reported as per FS also similar as FG 25-3/3a.

- For prerequisite feature groups,

  - For FG 25-2, it looks ok to us to keep 4-23, since it should be straightforward that UEs supporting 25-2 for short format slot based repetition should be able to support slot based repetition for other formats. However, would be fine to remove it also, since there is no strong dependency it seems.

  - For FG 25-3, both FG 4-23 and 11-3 should be kept. In our understanding, sub-slot PUCCH repetitions would result in slot PUCCH repetitions in some cases also, thus 4-23 should be there.

  - For FG 25-3a, we agree that no need to add 30-5 from coverage as the prerequisite. Sub-slot based repetition can apply same mechanism that designed for slot based repetition in terms of dynamic number indication, but as to the UE capability no strong dependency.

  - For FG 25-3b, the prerequisite can be “FG 25-3 or FG 25-3a”. If only FG 25-3 is listed there, the question is whether hopping can be done in case of dynamic indication of the repetitions, in our understanding the hopping can be applicable either.

| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per FSUE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per FSUE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD25-3 or 25-3a | Yes | N/A |  | Per FSUE | No | No | N/A |  | Optional with capability signaling |

- FG 25-4/25-5/25-6/25-7

| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. The values higher than 1 can be applied to sub-slot based configuration only. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Comments:

- FG 25-4 & FG 25-5: Change “Per UE” to “Per band” to align with the granularity of the prerequisite.

- FG 25-6:

  - Change “Per UE” to “Per band” to align with the granularity of the prerequisite.

  - For component 5: 1) “type 3 or” should be kept. For a UE supporting FG 25-6, gNB can configure either type 3 HARQ-ACK codebook or enhanced type 3 HARQ-ACK codebook, thus the reported value for component 5 should be applied to both type 3 HARQ-ACK codebook and enhanced HARQ-ACK codebook type. Note that we don't think this will change the Rel-16 behaviour, since it is applied only when the UE reports the support of FG25-6. If UE doesn't support FG 25-6 but only FG 10-16, then still follow Rel-16 behaviour; 2) Regarding “The values higher than 1 can be applied to sub-slot based configuration only” in the note column, we don't think it is that critical since it should be common understanding that only 1 actual PUCCH transmission with HARQ-ACK will be transmitted per slot for slot-based PUCCH, therefore it is preferred to delete it for simplicity. However, if people have strong desire to keep it, we are fine also, though we may need some further modification considering the case of two simultaneous HARQ-ACK CB.

- FG 25-7: During #107b-e meeting, the following agreements are achieved:

Agreement: For one-shot HARQ-ACK re-transmission, the value range for HARQ re-tx offset is given by [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value] with an indication of 1 slot / sub-slot within that range. - FFS the fixed value of min_HARQ_retx_offset_value - FFS the fixed value of max_HARQ_retx_offset_value Agreement For one-shot HARQ-ACK re-transmission, - the minimum value for the HARQ re-tx offset min_HARQ_retx_offset_value is -7. - the maximum value for the HARQ re-tx offset max_HARQ_retx_offset_value is 24. - Note: UE capability reporting on the UE supported value of the minimum value and maximum value range for HARQ_retx_offset in the scope of [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value ] that can be indicated by the gNB for the UE can be further discussed in UE capabilities

  - For the candidate values for the minimum value for the HARQ re-tx offset, we propose to use {-7, -6, -5,-4,-3,-2,-1, 0, 1}. For UE reporting a value equal to or larger than 0 for minimum value for the HARQ re-tx offset, such UE is not supporting HARQ-ACK codebook re-transmission triggered before the initial PUCCH transmission slot.

  - For the candidate values for the maximum value for the HARQ re-tx offset, we propose 1 as the minimum value for the candidate value set and 24 as the maximum value for the candidate value set.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 3. Supported minimum value for the HARQ re-tx offset 4. Supported maximum value for the HARQ re-tx offset |  | Yes | N/A |  | Per UE | No | No | N/A | Candidate values for component 4 is: {-7, -6, -5,-4,-3,-2,-1, 0, 1}; Candidate values for component 5 is: {1, 2, …, 23, 24}. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 25-8

| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-8: Change “Per UE” to “Per FS” to align with the prerequisite.

- FG 25-9/25-10

| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A | Note: this feature applies to cells in the same TAG only | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A | Note: this feature applies to cells in the same TAG only | Optional with capability signaling |

Comments:

- Regarding to the FFS part of FG 25-9/25-10, we have following potential proposals during the discussion of last meeting.

High priority compromised proposal 6-1a: - FGs 25-9 and 25-10 are not split into multiple FGs - Add a note in FG 25-9/10 that if UE supporting FG 25-9/10 also supports both FGs 6-9 and 6-9a, the UE support the case of different numerologies between switchable carriers High priority alternative proposal 6-1a’: - FGs 25-9 and 25-10 are split into ones for the capability for same length (in physical time) of overlapping PUCCH slots/sub-slots and the others for different length (in physical time) of overlapping PUCCH slots/sub-slots between switchable carriers - Add a note in the split FGs 25-9/10: “If UE supporting this FG also supports both FGs 6-9 and 6-9a, the UE support the case of different numerologies between switchable carrier”

  - We are fine with proposal 6-1a. In our understanding, if UE already supports 6-9 and 6-9a and also reports to support PUCCH switching, then it should be able to support PUCCH switching between cells with different numerology. However, without this note, it would mean that if a UE report the supporting of FG 25-9/10, even the UE doesn’t support 6-9 and 6-9a, the UE has to support PUCCH switching between cells with different numerology, which to us is a big step. Therefore, we think the note is necessary.

  - We are fine with proposal 6-1a’ also. The main bullet together with the words in blue is ok for us. And for the note, if we don’t add it, it may cause the misunderstanding that a UE supporting FG 25-9/10 or its splitting FGs (based on proposal 6-1a’) will automatically support FG 6-9 and FG 6-9a, even the UE only wants to support the case with same numerology for PUCCH switching. Under proposal 6-1a’, for the FG for same length of overlapping PUCCH slots, it covers both the case of same numerology and different numerologies, without any clarification, it means UE supports PUCCH switching with different numerologies between switchable carriers, then it means UE needs to support different numerologies across NR carriers within the same PUCCH group as defined in FG 6-9 and FG 6-9a, which makes a UE only support same numerology not able to support PUCCH switching. Therefore, we think the note is also necessary for proposal 6-1a’.

Enhancements for unlicensed band URLLC/IIoT

- FG 25-12

| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-12: Add basic feature components agreed for a UE that can operate as an initiating device in the semi-static channel access mode as follows.

| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. Sensing to initiate a semi-static CO or transmit after a gap greater than 16us from any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL 4. Validating COT initiator assumption indicated in UL scheduling DCI | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Intra-UE Multiplexing/Prioritization

- FG 25-14/25-15

| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | FFS: Candidate value set for component 2: d3 = {0, 1, …, 2μ-1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. | Optional with capability signaling |

Comments:

- FG 25-14: Fine with the current FG 25-14, including cells in yellow.

- FG 25-15: The FFS part has been confirmed during last meeting, therefore “FFS:” should be removed.

- FG 25-16

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-16:

  - For FFS whether to separate capability for different UCI type, we are fine to remove the FFS bullet.

  - Remove [] for component 2) and component 3) according to the agreements.

  - Change 11-3 to one of {11-4, 11-4a}, as 11-4/11-4a is the UE capability of supporting two HARQ-ACK codebook with different priorities.

  - Delete 12-1 from the prerequisite feature group for FG 25-16. 12-1 is to define prioritization of overlapping channel/signals with two priority levels in physical layer, while 25-17 here is to define multiplexing of overlapping channel/signals with two priority levels in physical layer. There is no need to couple these two capabilities. It also aligns with #capability 1 UE as agreed #107-e meeting.

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | one of {11-4, 11-4a}11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 25-18

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-18:

  - Remove [at least] as it is already concluded not to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17 during #107-e meeting.

  - The terminology of “PUCCH/PUSCH” covers three cases: PUCCH + PUCCH, PUSCH + PUSCH, and PUSCH + PUSCH. And FG 25-18 only refers to the case of “PUCCH + PUSCH”. We propose to change the component as terminology of “PUCCH/PUSCH” to “PUCCH and PUSCH” to eliminate confusion.

  - In #107-e meeting, it has been concluded that simultaneous PUCCH/PUSCH transmission of same priority over different cells is not supported in Rel-17. We propose to add “of different priority” in the component.

  - Change “Per UE” to “Per BC”, since the capability for UE would dependent on the CA band combination.

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH PUCCH and PUSCH transmissions of different priority on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UEBC | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Propagation delay compensation enhancements

- FG 25-19/25-19a/25-20

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure |  | Yes | N/A |  | Per UE | no | no | N/A |  | Optional with capability signaling |

Comments:

- FG 25-19/25-19a:

  - Slightly prefer to delete FG 25-19 from the prerequisite for FG 25-19a, because for UEs that will support PRS there is no need to request it to support RTT-based PDC with TRS. Of course, since TRS is a mandatory feature, it would be reasonable to assume PDC with TRS and SRS should be supported also for a UE supporting PDC with PRS and SRS, therefore we are fine not removing FG 25-19 also.

  - For FG 25-19a, the following additional component should be added. Similar as that for positioning method, the maximum number of PRS resources in the set should be UE capability also.

- Max number of DL PRS Resources in the DL PRS Resource Set for PDC. Candidate Values = {1, 2, 4, 8, 16, 32, 64}.

Note: 16, 32, 64 are only applicable to FR2 bands.

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | 1. Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS 2. Max number of DL PRS Resources in DL PRS Resource Set for PDC Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |

- FG 25-20: Fine with the current including cells in yellow.

- References

R1-2108697, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1 #106-e

R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1 #107bis-e