3GPP TSG RAN WG1 Meeting #108-e	R1-2200963

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.11.1.1

Source:	Huawei, HiSilicon

Title:	Sidelink resource allocation to reduce power consumption

Document for:	Discussion and Decision

- Introduction

According to RAN#94-e decision, RAN1 is tasked to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022. In RAN1#107bis-e, a number of issues provided in RP-212880 (status report of WI: NR sidelink enhancement) were discussed, however, there are a few open issues remaining unsolved, which we address in this paper.

- Open issues

## Selection and reporting of subset of candidate resources within RX-UE's SL DRX active time

The following working assumption was made at RAN#107-e:

When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED)

Working Assumption (RAN1#106bis-e)

When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer:

- Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE

- Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE

  - FFS: Details on when the number of subsets of candidate resource is less than the threshold

  - FFS: The subset of candidate resource outside of the active time should consider each inactive time period

  - FFS: UE selection of resource selection window to overlap with indicated RX UE active time

  - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE

- Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

According to the LS from RAN2 [1]:

- TX UE shall select initial transmission resource only in the RX UE’s active time where SL DRX timers are running now or will be running in future (at least on-duration timer). Further details of active time can be considered later. FFS on spec impact.

The resource exclusion is based on RSRP measurement of PSCCH/PSSCH according to decoded SCI in the sensing window, and therefore a UE can avoid potential resource collision in the resource selection window subject to reservation of the SCI. However, any resources can be excluded regardless of whether the resource are within the indicated SL-DRX active time of the RX UE, i.e. there could be a case that all resources within the indicated SL-DRX active time of the RX UE are excluded according to current procedure.

Observation 1: Current sensing and resource selection procedures specified in TS 38.214 cannot ensure there is a subset of the candidate resources which is within the indicated active time of the RX UE.

RAN1 needs to design solutions to allow a UE to implement RAN2 agreements, otherwise, a revised work in RAN2 can further risk overall WI progress. To ensure that TX UE shall select initial transmission resource only in the RX UE’s active time, the subset determined at PHY layer has to be defined to ensure it includes the resources within the RX UE’s active time. This cannot be left to UE implementation, given that the specified sensing and resource exclusion procedure does not permit a UE to choose whatever resource it wants.

Proposal 1: RAN1 shall specify a solution to ensure a non-empty subset of candidate resources within the indicated SL DRX active time.

The non-empty subset can include resources for retransmission in addition to that for initial transmission, which is beneficial for the case that the RX UE does not decode the SCI of initial transmission successfully within indicated active time of RX UE (due to half-duplex or bad channel conditions). The RX UE would not know the location of retransmissions, and is unable to wake up during inactive time to decode subsequent retransmissions given that these retransmissions would not take place during the active time, which can results in failure of SL transmission. On the other hand, the more the resource included in the subset of the candidate resources, the more power saving gain due to SL-DRX can be achieved, because the RX UE does not need to wake up from the SL-DRX inactive time to receive retransmissions which can allow a UE to maximize Deep Sleep duration. As a result, it is beneficial for the subset of the candidate resources to include sufficient number of candidate resources (both for initial transmission and retransmissions) within the active time of the RX UE, e.g. the subset of the candidate resources should be no smaller than X∙Ntotal, where the value of X can be same as in step 7, and Ntotal  is the total number of candidate single-slot resources of the set SA within the SL DRX active time.

Observation 2: The introduction of threshold is necessary to define the non-empty subset to include sufficient number of candidate single-slot resources so as to meet RAN2 agreements.

For the 1st FFS in the Option 2, when the number of subsets of candidate resource is less than the threshold, it may be caused by the over exclusion of resource in step 7 based on the procedure specified in 8.1.4 of TS38.214. In order to determine a candidate resource set containing sufficient candidate resource within indicated active time of the RX UE, necessary modification for step 7 on the legacy resource selection procedure is needed.

- In step 7, the remaining candidate resource set has to satisfy the restriction of X∙Ntotal, which is guaranteed by raising RSRP threshold to re-implement step 4 to step 6. However, current step 7 does not take into account whether or not the remaining candidate resource set will include the resources within the indicated active time of the RX UE. It is possible that step 7 recovers few candidate resources within the indicated active time of RX UE, but there is still not sufficient available candidate resources left within the indicated active time of RX UE. Hence after step 7 if the remaining candidate resources within the indicated active time of the RX UE is smaller than the threshold X∙Ntotal, the UE restores those excluded candidate resources within Ntotal only in step 6 via RSRP thresholds increment after step 7, and the RSRP thresholds corresponds to candidate resources within inactive time of RX UE does not need to be increased.

For the 3rd FFS, the overlapping ratio of RSW (Resource Selection Window) and indicated active time of RX UE is the premise for a candidate resource set to include a candidate resource within indicated active time of the RX UE. Given that both the starting time of RSW (relative to triggering moment slot n) and the SL-DRX active time of RX UE are determined by MAC layer, the UE implementation can ensure there are overlapping slots between RSW and the SL-DRX active time. So there is no needs to further discuss how to guarantee the overlapping between RSW and active time of SL DRX.

Proposal 2: The reported subset of the candidate resources with the provided SL DRX active of RX UE shall satisfy a threshold.

- The same higher layer parameter (sl-TxPercentageList) is reused for the ratio / threshold, that is, X∙Ntotal number of candidate single-slot resources remaining within the SL DRX active time of the initialized set SA in Step 4) is to be met by using the RSRP threshold increment only for the SL DRX active time in Step 7, where the Ntotal is the total number of candidate single-slot resources of the set SA within the SL DRX active time.

  - The UE shall satisfy this new threshold in addition to the remaining minimum X∙Mtotal number of candidate single-slot resources threshold for the whole set SA in Step 7.

If full-sensing with SL DRX is configured, there is also possibility that over exclusion of resource happens after a UE performs the step 5, i.e. there are too many candidate resources within the indicated SL DRX active time of RX UE being excluded due to slots not monitored, the remaining available candidate resources can be very limited. In this case, applying step 7 does not help. Hence recovery is needed after step 5 if the remaining candidate resources within the indicated active time of the RX UE is smaller than the threshold X∙Ntotal. Specifically, restore all candidate resources within the indicated active time which are excluded due to step 5 operation. Putting back candidate resources within active time is to satisfy the restriction of X∙Ntotal, the excluded candidate resources within inactive time of RX UE does not need to be put back considering they are not reliable candidate resources.

## Random selection in pools with mixed RA schemes

At RAN#106-e, the following agreement was made:

For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration

In this section, we discuss the impact of resource collision when random resource selection is performed by a UE which does not perform sensing in a resource pool configured with mixed type of RA schemes.

### Insufficiency of option 12

Different to LTE-V, Rel-17 NR supports different types of RA schemes to be configured in the same resource pool including full sensing (both Rel-16 and Rel-17 can share the same pool), partial sensing and random resource selection, so as to improve resource utilization efficiency and improve overall performance given that NR supports re-valuation and pre-emption checking. However, given that random resource selection UE does not perform sensing, simply allowing sensing-based UE and random resource selection UE in the same resource pool without any conditions, nor collision control, would produce a too-high rate of resource selection collision, thus unacceptable PRR performance drop as per our simulations in the Appendix A-1.

Observation 3: Option 12 can result in uncontrolled number of random resource selection operation in the same resource pool with sensing-based operation, which significantly degrades PRR performance of sensing-based operation including both Rel-16 and Rel-17.

It is not always possible for a sensing-based UE to back off the reservation made by a random selection UE, because the random selection UE can pre-empt any reserved resources with any priority levels, and pre-emption checking performed by sensing-based UE is priority-based subject to mode 2 sensing and resource selection procedure, i.e. if a random selection (re-)transmissions (including reserved transmission) with lower priority (higher value) than that of sensing-based, the sensing-based UE would not re-select its reservations and the collision of resources cannot be avoided given that pre-emption checking is not performed when prioTX≤prioRX.

Observation 4 Pre-emption checking is priority-based and thus it is not always possible for a sensing-based UE to back off resource reserved by a random resource selection UE.

### Necessity of option 1

Option 1 allows only a certain number of random selection UEs to perform PSSCH transmission in the resource pool configured for mixed types of RA, to avoid significant PRR performance loss of the sensing-based UEs. Taking into account different QoS profiles of the NR sidelink services, it is more reasonable to allow such permissions on a priority basis, so that the power-constrained URLLC sidelink QoS would be guaranteed, whilst the performance of the full-sensing UE can still be resilient to those URLLC reduced-sensing based PSSCH transmissions. On the other hand, in our simulation, we consider the resource pool is already quite busy due to large number of full-sensing UEs with dense traffic. If the resource pool is less busy, with fewer full-sensing UEs transmitting, it may be possible that network can re-configure the priority threshold to allow additional power-constrained UEs associated with relatively smaller priority values to share transmission with full-sensing in the same pool for higher PRR performance. Relevant simulation can be found in the Appendix A-1.

In addition, option 1 has similar design of Rel-16 to set a priority threshold for pre-emption to control pre-emption operation in the resource pool. The introduction of option 1 is with same purpose, and is simple to be implemented to resolve the critical performance issue of option 12.

Observation 5: Option 1 has similar design principle of Rel-16 pre-emption where a priority threshold can be (pre-)configured, so as to control the number of random resource selection in the resource pool with mixed RA schemes, which benefits power-constrained high priority sidelink transmission in terms of reduced power consumption as well as improved reliability, whilst introducing limited and controllable impact on full-sensing UEs in the same resource pool.

### Way forward discussion

The latest proposal (as follows) proposed during RAN1#106bis-e discussion can be a good starting point to further discuss the issue at RAN1#108-e meeting.

Proposal 3-1 (IX): For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection,

- Option 1: a priority threshold value is can be (pre-)configured for the resource pool or a subset of resources, at or below the threshold value which random resource selection is allowed.

  - Note, lower value means higher priority

  - FFS: Remaining details for the RRC parameter (e.g., possible priority threshold values, and whether the priority threshold values can be based on different measured CBR)

  - FFS: whether to support a priority threshold value (pre-)configured for a subset of resources in a resource pool.

There are mainly three concerns:

- Which resource pool is used for random resource selection transmission of which priority is higher than the priority threshold

Resource pools, including which RA scheme(s) is allowed in the RP, are provided by network. Given that Rel-17 sidelink UE feature already supports Tx capabilities with partial sensing and random resource selection in addition to Rel-16 full sensing, network can well handle how to configure appropriate resource pool, including multiple resource pools (up to 8 Tx resource pool in Rel-16) for all UEs to operate sidelink in the resource pool(s) configured with appropriate RA schemes. For example, network can (pre-)configure a random selection only resource pool without need of priority threshold, where a UE can perform random resource selection if it does not meet the priority threshold (pre-)configured in a RP with mixed types of RA schemes. Network can also provide more than one RPs with mixed types of RA schemes, of each has different priority thresholds for UEs with PSSCH transmissions associated with different priorities to perform random selection accordingly. It also includes the possibility of operating in exceptional resource pool if here is no normal resource pool can be found (e.g. priority threshold is not met) to perform random resource selection, which can be discussed in RAN2 for details.

- Whether a subset of resources is essential

With a resource pool partitioned into subset of resources, there can be two cases in terms of whether high priority random selection can cross between partitioned resources: Case 1 (crossing between partitioned resources is not allowed, as shown in Figure 1) vs Case 2 (crossing between partitioned resources is allowed, as shown in Figure 2).

Figure 1. RP with partitioned resource (Case 1), where crossing between partitioned resources is not allowed.

The performance of Case 1 is same as that where there are two separate resource pools (FDMed) configured, and therefore is not needed.

Figure 2 RP with partitioned resource (Case 2), where crossing between partitioned resources is allowed.

In case 2, although crossing between partitioned resources does not impact full-sensing performance, high priority random resource selection can be affected by low priority random resource selection due to no sensing is performed. This reduces the PRR gain in case 1 where high priority random resource selection can benefit from the sensing performed by low priority full sensing UEs (as we shown in our simulation, see Figure 7). It may be no problem in case that congestion level of the system is low, i.e. collision probability is low, and therefore the high priority random resource selection can be performed in any partitioned resource with minor PRR loss, although this is an over-optimized solution.

- How to use exceptional resource pool

There can be multiple resource pools (pre-)configured, it is assumed that network can ensure that there is always a TX resource pool (pre-)configured for SL transmissions. In Rel-16, condition for a UE to perform random resource selection in an exceptional resource pool is defined in RAN2, thus it is more appropriate for RAN2 to decide whether such condition includes the case that a random resource selection UE with a TB transmission of which priority value larger than the (pre-)configured threshold in the exceptional resource pool

In summary, it is not a reasonable way forward to close an identified issue which can cause severe system performance degradation. Instead, the possible way forward may be that to allow network to control resources and value of priority threshold to handle different scenarios. For example, there could be a default configuration with a priority threshold for a single resource pool, and additionally, if necessary, the priority value threshold can be applied to a subset of resources in that resource pool.

Proposal 3: When UE performs random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection.

  - A priority threshold value is (pre-)configured for the resource pool, below or equal to which random resource selection is allowed in that partition. Note, lower value means higher priority.

  - The (pre-)configured priority threshold can be any of the 8 priority values.

  - For the sake of progress, we can agree, if necessary, to introduce additional configuration to allow such priority threshold to apply for the subset of resource in the resource pool to optimize this issue.

## Re-evaluation/pre-emption checking after random selection

Random resource selection is used to minimize the power consumption, however with revaluation/pre-emption checking after random resource selection, it will significantly increase the power consumption. If reliability is concerned, for a UE, it should use partial sensing, i.e. either PBPS + CPS, or CPS only, with revaluation/pre-emption checking. Thus, this feature is over-optimized and redundant compared to existing RA schemes on balancing the trade-off between power consumption and reliability.

Proposal 4: A UE does not perform re-evaluation/pre-emption checking after random resource selection when the UE is configured to perform random resource selection RA.

## Conditions to disable CPS for periodic transmission in resource (re)selection

Given that a resource pool cannot disable aperiodic transmission, if a UE does not perform CPS for resource selection including re-evaluation and pre-emption checking (i.e. PBPS only), the UE cannot detect aperiodic reservation regardless of the PSSCH transmission is periodic or aperiodic, and thus may end up with resource collision if there is only PBPS results, particular in case of heavy density of aperiodic traffic in the resource pool, and therefore PRR performance degradation as shown in our simulation results in Appendix A-2. Thus, the working assumption to allow M = 0 is not justified from performance perspective.

Observation 6: Resource pool cannot disable aperiodic transmission, thus without performing CPS, a UE cannot detect aperiodic reservation from other UEs, resulting in PRR performance degradation.

M is a logical value representing the number of consecutive logical slots before ty0, where ty0 is the first slot of the selected Y or Y’ candidate slots, i.e. n+TA is M logical slots earlier than slot ty0SL,  and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, however, Tproc,0SL+Tproc,1SL are physical values representing UE processing delay.  For example, in both cases as shown, M = 1, but Case 2 would be invalid given that n+TA is later than n+TB where no CPS result can be obtained.

Figure 3 M =1 for CPS in different cases

A safer value for minimum M can be 4, given that the minimum value of Tproc,0SL+Tproc,1SL is 4, which works for all cases. Although M can be redefined with respect to Tproc,0SL+Tproc,1SL, it seems an avoidable effort, and is unnecessary. It is reasonable that the value of M should not be too small due to performance concerns, where it is more likely that aperiodic reservations can be missed given that a SCI reserve up to 31 slots away.

Proposal 5: When a UE performs partial sensing, CPS should be always performed, whether Prsvp = 0 or Prsvp ≠ 0. The lower value range limit for contiguousSensingWindowPeriodic and contiguousSensingWindowAperiodic is 4.

## Resource pool segregation for periodically occurring resources

The introduction of PBPS and CPS are already sufficient to detect both periodic reservation and aperiodic reservation to meet the WI objective. In addition, resource pool segregation for periodically occurring resources can be achieved by network implementation. Given this WI is already delayed and RAN1#108-e is the last RAN1 meeting of this WI, such new design framework shall be avoided.

Proposal 6: Rel-17 NR sidelink does not support resource pool segregation for periodically occurring resources.

- Conclusions

This contribution has provided our view on resource allocation to reduce power consumption with observations and proposals as follows:

Observation 1: Current sensing and resource selection procedure specified in TS 38.214 cannot ensure there is a subset of the candidate resources which is within the indicated active time of the RX UE.

Proposal 1: RAN1 shall specify a solution to ensure a non-empty subset of candidate resources within the indicated SL DRX active time.

Observation 2: The introduction of threshold is necessary to define the non-empty subset to include sufficient number of candidate single-slot resources so as to meet RAN2 agreements.

Proposal 2: The reported subset of the candidate resources with the provided SL DRX active of RX UE shall satisfy a threshold.

- The same higher layer parameter (sl-TxPercentageList) is reused for the ratio / threshold, that is, X∙Ntotal number of candidate single-slot resources remaining within the SL DRX active time of the initialized set SA in Step 4) is to be met by using the RSRP threshold increment only for the SL DRX active time in Step 7, where the Ntotal is the total number of candidate single-slot resources of the set SA within the SL DRX active time.

  - The UE shall satisfy this new threshold in addition to the remaining minimum X∙Mtotal number of candidate single-slot resources threshold for the whole set SA in Step 7.

Observation 3: Option 12 can result in uncontrolled number of random resource selection operation in the same resource pool with sensing-based operation, which significantly degrades PRR performance of sensing-based operation including both Rel-16 and Rel-17.

Observation 4 Pre-emption checking is priority-based and thus it is not always possible for a sensing-based UE to back off resource reserved by a random resource selection UE.

Observation 5: Option 1 has similar design principle of Rel-16 pre-emption where a priority threshold can be (pre-)configured, so as to control the number of random resource selection in the resource pool with mixed RA schemes, which benefits power-constrained high priority sidelink transmission in terms of reduced power consumption as well as improved reliability, whilst introducing limited and controllable impact on full-sensing UEs in the same resource pool.

Proposal 3: When UE performs random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection.

  - A priority threshold value is (pre-)configured for the resource pool, below or equal to which random resource selection is allowed in that partition. Note, lower value means higher priority.

  - The (pre-)configured priority threshold can be any of the 8 priority values.

  - For the sake of progress, we can agree, if necessary, to introduce additional configuration to allow such priority threshold to apply for the subset of resource in the resource pool to optimize this issue.

Proposal 4: A UE does not perform re-evaluation/pre-emption checking after random resource selection when the UE is configured to perform random resource selection RA.

Observation 6: Resource pool cannot disable aperiodic transmission, thus without performing CPS, a UE cannot detect aperiodic reservation from other UEs, resulting in PRR performance degradation.

Proposal 5: When a UE performs partial sensing, CPS should be always performed, whether Prsvp = 0 or Prsvp ≠ 0. The lower value range limit for contiguousSensingWindowPeriodic and contiguousSensingWindowAperiodic is 4.

Proposal 6: Rel-17 NR sidelink does not support resource pool segregation for periodically occurring resources.

- References

- R1-2200007, “ Reply LS on SL resource selection with DRX”, RAN WG2, RAN WG2#116-e, 1st - 12th November, 2021

# Appendix: Simulation results and assumptions

## A-1 Random selection in pools with mixed RA schemes

We simulate the random resource selection and the full sensing RA, with following two cases:

- Case 1 - LTE-V baseline, i.e. configuration provides one TX resource pool for full-sensing V-UEs (for V2V only) and another for random resource selection P-UEs (for P2V only) respectively.

- Case 2 – All UEs (for P-UE & V-UE), regardless of type of RA, are in the same TX resource pool, i.e. configurations provides a single resource pool.

Figure 4. RP configurations for Case 1 and Case 2.

We simulate the random resource selection and the full sensing RA. The system-level results of average PRR, from the only full-sensing V-UEs perspective, is shown in Figure 4. We observe a clear PRR loss of Case 2 over Case 1 from V-UE’s point of view only in Figure 5, if admission of all the P-UEs is allowed to the same resource pool with V-UEs.

Figure 5. Average PRR from full-sensing V-UEs only perspective

We further simulate case 3

- Case 3 – Some of P-UEs select to transmit PSSCH in a TX resource pool configured for mixed full-sensing and reduced-sensing RA, and the remaining P-UEs remain in the dedicated reduced-sensing TX resource pool for PSSCH transmission.

We set different priority thresholds (e.g. priority threshold = 2, 3 and 7) for the resource pool configured for mixed types of RA. A reduced-sensing P-UE can select to transmit PSSCH in the resource pool, if the priority value of this PSSCH is smaller than the priority threshold; otherwise, it selects to transmit PSSCH in another resource pool configured for reduced-sensing RA. In the same way as for Cases 1 and 2, we use random selection RA for reduced-sensing RA, and from the only full-sensing V-UE perspective, where simulation assumption can found in the Appendix Table 1.

Figure 6. Average PRR from full-sensing V-UEs only perspective, when in a resource pool for mixed types of RA where random selection UEs are admitted if priority < priority threshold.

Based on these simulation results in Figure 6, for the case that only the highest priority (priority threshold = 2) reduced-sensing P2V PSSCH transmissions (i.e. priority = {1}) are selected to share with those full-sensing V2V PSSCH transmissions in the resource pool configured for mixed types of RA, the impact on the average PRR for those V-UEs performing full-sensing is negligible compared to the baseline (the PRR loss is small, even in longer communicating range). For priority threshold = 3 (i.e. allowing priority value = {1, 2}), impact on V2V transmissions is a bit higher at larger distance. However, if more reduced-sensing PSSCH transmissions are admitted, e.g. a priority threshold = 7 is configured (allowing priority value = {1, 2, 3, 4, 5, 6}), are selected to share with those full-sensing PSSCH transmissions, the performance of average PRR for the V-UEs performing full-sensing degrades with a clear margin, is much worse with increase over communicating distance. This is because a too-high priority threshold value (i.e. admitting too-much low priority traffic) leads to over-populating with reduced-sensing P-UEs and full-sensing UEs sharing the same resource pool, and excessive pre-emptions imposed on full-sensing UEs. For those full-sensing V-UEs with lower priorities than the reduced-sensing UEs are forced to reselect resources to avoid collisions. On the other hand, V-UE continues to re-select resources due to pre-emptions caused by P2V transmission, which further introduces delays. Given the limited resource in a selection window subject to PDB requirement, those full-sensing V-UEs have to increase the RSRP threshold to sustain a sufficient number of single-candidate resources for resource selection. This means those full-sensing V-UE have to (re-)select resources which may cause higher interference to PSSCH reception at targeted recipient UEs, and therefore degrades the PRR performance.

We then look at the PRR performance of random selection UEs only in the Case 3. For those reduced-sensing UEs with PSSCH transmissions in the resource pool configured for mixed types of RA, their PRR performance is higher than that if they were in a dedicated resource pool for reduced-sensing RA, as shown in Figure 7, where we use priority threshold = 2 which has almost no impact on full-sensing UEs performance.

Figure 7. Average PRR from random selection P-UEs only perspective

In this case, around 1% PRR gain is observed at distance 80m for Case 3. This is due to the sensing effort made by the full-sensing UEs, so that a full-sensing UE can re-select resources reserved/occupied by a random selection UE to reduce the collision probability. However, in the dedicated resource pool for random selection RA (case 1 baseline), given that no SCI is monitored by any UE, there is no back-off from the reserved resources to reduce collision.

Table 1: Basic simulation assumptions for mixed types of RA

| Parameter | Assumption |
| --- | --- |
| Frequency | 6 GHz |
| Simulation bandwidth | 40 MHz |
| RP configuration | Case 1: two RPs - one is configured for full-sensing RA, where 3 sub-CHs are configured; the other is configured for reduced-sensing RA only, where 2 sub-CHs are configured. In each RP, one sub-CH consists of 10 PRBs. Case 2: There is a single RP configured for mixed types of RA, where 5 sub-CHs are configured, each consists of 10 PRBs. Case 3: two RPs - one is configured for mixed types RA, where 3 sub-CHs are configured; the other is configured for reduced-sensing RA only, where 2 sub-CHs are configured. In each RP, one sub-CH consists of 10 PRBs. |
| Sub-carrier spacing | 60 kHz SCS |
| Resource allocation | V-UE: Full-sensing (Mode 2 in Rel-16); P-UE: Random resource selection in Rel-14 |
| Synchronization | Ideal time frequency synchronization |
| Link type | Direct vehicle-to-vehicle link & direct pedestrian-to-vehicle link. Interference is considered among P2V and V2V links for the RP that is configured for mixed types of RA. |
| Antenna model | TR 37.885 Option 1 |
| Traffic model | For P2V: Aperiodic: Packet size: uniformly random in the range between 200 bytes and 800 bytes with the quantization step of 200 bytes. Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms, and 100% vehicles generate packets. Priority value (value range = {1, 2, 3, 4, 5, 6, 7, 8}) per packet is uniformly distributed. For V2V: Aperiodic-1 of TR 37.885: Packet size: uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes. Inter-packet arrival time: 50ms + an exponential random variable with the mean of 50 ms, and 100% vehicles generate packets. Priority value (value range = {1, 2, 3, 4, 5, 6, 7, 8}) per packet is uniformly distributed. |
| Deployment and UE drop | Urban-A as defined in TR 37.885 for 500 V-UEs and 500 P-UEs. |
| Number of Tx/Rx antennas | 2Tx/4Rx |
| Cast type | Unicast (distance index: corresponding to 20m) |

## A-2 Conditions to disable CPS

We evaluate a scenario that a resource pool enables periodic reservation, all the V-UEs perform Rel-16 mode 2 resource allocation scheme with either periodic or aperiodic reservation (randomly picked), and a PU-E are perform partial sensing in the following two cases:

- Case 1 PBPS only: all P-UEs performs PBPS only

- Case 2 PBPS + CPS: all P-UEs performs both PBPS and CPS

Detailed simulation assumption can be found in Table 2 of Appendix.

Figure 8 P-UE’s PRR performance in PBPS+CPS case and PBPS only case

Figure 8 (the left figure shows 1:1 periodic and aperiodic mixed traffic, where there is around absolute 0.5% PRR drop at 100m, and the right figure shows 1:3 periodic and aperiodic mixed traffic, where there is around absolute 1% PRR drop at 100m ).

Table 2 Basic simulation assumptions for PBPS and CPS

| Parameter | Assumption |
| --- | --- |
| Frequency | 6 GHz |
| Simulation bandwidth | 40 MHz |
| RP configuration | One RP, where 5 sub-CHs are configured, each consists of 10 PRBs. |
| Sub-carrier spacing | 60 kHz SCS |
| Resource allocation | P-UE: Partial sensing V-UE: Full-sensing |
| Synchronization | Ideal time frequency synchronization |
| Link type | Direct vehicle-to-vehicle link & direct pedestrian-to-vehicle link. Interference is considered among P2V and V2V links. |
| Antenna model | TR 37.885 Option 1 |
| Traffic model | For both P2V and V2V: Packet size: uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes. For periodic traffic: Inter-packet arrival time (ms):100ms, latency: 100ms, and 100% UEs generate packets. For aperiodic traffic: Inter-packet arrival time: 25ms + an exponential random variable with the mean of 25 ms, latency: 25ms, and 100% UEs generate packets. |
| Deployment and UE drop | Urban-A as defined in TR 37.885 for 500 V-UEs and 500 P-UEs. |
| Number of Tx/Rx antennas | 2Tx/4Rx |
| Cast type | Unicast |