3GPP TSG RAN WG1 Meeting #108-e	R1-2200973

E-meeting, February 21 – March 3, 2022

Agenda Item:	5.1

Source:	Huawei, HiSilicon

Title:	Discussions on the remaining issues for UL Tx switching

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e, discussions regarding enhancements for UL Tx switching have achieved the agreement and the conclusion as cited below [1]:

Conclusion: - For Rel-17 Tx switching between Band A and Band B, no additional specification impact to support 1-port transmission via DCI format 0_1 for UL CA option 2. Agreement: - For a UE capable of 2Tx-2Tx switching and configured with UL Tx switching via uplinkTxSwitching, to differentiate the switching delay for 1Tx-2Tx switching from that for 2Tx-2Tx switching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode. - If 1Tx-2Tx mode is derived by the new RRC parameter, then there is on one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. RAN1 assume the uplink is configured with RRC parameter “carrier1” by RAN2. - The default value of the new RRC parameter is 1Tx-2Tx switching mode. - In a configured switching mode, the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode. - Note: This RRC parameter doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

# Discussion

## TP and RRC parameter list for the UL Tx switching

The latest editor draft CR (Mihai’s version v4r3) in RAN1#107-e was not endorsed unfortunately because different understanding on how to capture one RRC parameter. A TP in Appendix A.1 is proposed based on the latest editor draft CR and tries to resolve the issue by agreeing the following RRC parameter list together. The following RRC parameter list is to capture the agreement in RAN1#107-e and differentiate the switching delay for 1Tx-2Tx switching from that for 2Tx-2Tx switching.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_RF_FR1_enh-Core | Uplink Tx switching enhancements | ULTxSwitching-2T2T-Mode | new | For a UE capable of 2Tx-2Tx switching and configured with uplinkTxSwitching, indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode in which the switching gap duration for a triggered uplink switching [xx 38.214] is equal to the switching time capability value reported for the switching mode. If 1Tx-2Tx mode is derived by the RRC parameter, then there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. |  |  |  | UE-specific |

In order to capture the agreement in RAN1#107-e, the TP in Appendix A.1 adds the following sentence. Thus we can avoid the conflict between the new RRC parameter (2Tx-2Tx Switching Mode) and existing RRC parameters.

If [new Rel-17 RRC parameter to the 2Tx-2Tx Switching Mode] is not configured, then there is expected to be one uplink configured with uplinkTxSwitching for the UE, on which the maximum number of antenna ports among all configured periodic/aperiodic SRS and activated semi-persistent SRS resources should be 1 and non-codebook based transmission is not configured.

Because the MIMO layer is not in any RAN1 agreement, but the number of ports is. The UE behavior specified in TS 38.214 has nothing about 2-layer transmission. So the TP in Appendix A.1 does not contain the sentence about “maxNumberMIMO-LayersCB-PUSCH”.

In addition, the following description about the RRC parameter uplinkTxSwitchingdualULTxState for CA option 2 is inaccurate in latest editor draft CR. The “1-port” marked in yellow contains the Tx state “1T+1T” while “1-port” marked in yellow should only mean the Tx state “2T+0T” or “0T+2T”.

If the UE is configured with [1T with uplinkTxSwitchingdualULTxState], when the UE transmitted 1-port or 2-port transmission on one carrier on one band followed by no transmission on this carrier and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier.

Minor changes about the RRC parameter uplinkTxSwitchingdualULTxState for CA option 2 is proposed as below.

If the UE is configured with [1T with uplinkTxSwitchingdualULTxState], when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on this carrier and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier.

Proposal 1: Adopt the TP in Appendix A.1 for the UL Tx switching in TS 38.214 clause 6.1.6 and agree the following RRC parameter list.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_RF_FR1_enh-Core | Uplink Tx switching enhancements | ULTxSwitching-2T2T-Mode | new | For a UE capable of 2Tx-2Tx switching and configured with uplinkTxSwitching, indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode in which the switching gap duration for a triggered uplink switching [xx 38.214] is equal to the switching time capability value reported for the switching mode. If 1Tx-2Tx mode is derived by the RRC parameter, then there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. |  |  |  | UE-specific |

## Clarification on CA based SRS carrier switching

Regarding the CA based SRS carrier switching, clarification of the "prioritization rules" & "suspension" is needed. In RAN1#107-e, there was a discussion on SRS carrier switching under [107-e-NR-7.1CRs-04] [2]. During this discussion, the following conclusion was made.

Conclusion Regarding SRS carrier switching priority rules: - For Rel-16, it is concluded that no modification in specifications should be made to clarify the current UE behaviour or to introduce a new UE behaviour regarding SRS carrier switching priority rules. - For releases later than Rel-16, it is concluded to consider introducing a new UE capability for indicating simultaneous transmission while switching, and/or clarify the UE behaviour in the case of intra-band CA. - Note: If introduced, the new UE capability should always assume no simultaneous transmission while SRS carrier switching for the bands in the band combinations that are signalled to not support simultaneous transmission within BandCombinationList-UplinkTxSwitch.

According to the current specification in TS38.214 clause 6.2.1.3, the “suspending” only appears in the following paragraph.

#### 6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

In this paragraph, there is no description of prioritization rules. So, the “suspending” and the determination and application procedure of prioritization rules are two non-overlapping functions of SRS carrier switching. The specification related to the “suspending” function can be modified independently without impacting prioritization rules.

Let us make the following assumptions:

For UL Tx switching: CC1 and CC2

For SRS carrier switching:  CC2(switching from carrier) and CC3 (switching to carrier)..

According to the current specification in 38.214 clause 6.2.1.3, during SRS transmission on CC3, the UE would temporarily suspend the uplink transmission on CC2. According to the note in the conclusion in RAN1#107-e, during SRS transmission on CC3, if introduced, the new UE capability should always assume no simultaneous transmission on CC1. It means  the UE would also temporarily suspend the uplink transmission on CC1 during SRS transmission on CC3. So we suggest to adopt the TP in Appendix A.2.

Proposal 2: Adopt the TP in Appendix A.2 for uplink suspension of SRS carrier switching in TS 38.214 clause 6.2.1.3.

Note that there is a parallel discussion on SRS carrier switching, which is focused on how to determine the applicability of the prioritization rules. In addition to the conclusion in RAN1#107-e [2], there are other achieved agreements summarized as below [3]:

Agreement The prioritization rules of SRS carrier switching apply to at least the source CC. - FFS : Whether the specification needs to be updated or not Agreement Down select one from the following two options in RAN1#106-e meeting to determine which UL CCs other than the source CC should be used for SRS carrier switching priority rules: - Option 1: The UL CCs in the same band as the source CC - Option 2: The UL CCs can be any carriers which result in uplink transmissions beyond the UE ’s indicated uplink carrier aggregation capability. - Companies should indicate how “UE’s indicated uplink CA capability” is derived based on current ASN.1 signaling.

Essentially, the key point to determine the applicability of the prioritization rules is whether any UE RF hardware is shared or not between component carriers. When SRS carrier switching is triggered, the “switch to” carrier borrows RF hardware from the “switch to” carrier, thus the prioritization rules should be applied to the “switch from” carrier (source carrier). Similarly, in case of UL Tx switching, not only the source carrier but also the other carrier configured as a paired carrier of the source carrier should be also taken into account for the prioritization rules.

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

Besides, as we can see from the current version of spec, a UE will determine whether to apply the prioritization rules or not based on the “uplink carrier aggregation capability”. It was realized that how UE derives which carrier should be taken into account for the prioritization rules is not clear enough. The LTE-like definition is suggested here by introducing the carrier set, then UE will judge whether an overlapping uplink transmission is beyond “uplink carrier aggregation capability” or not on each carrier in the carrier set:

define the UL CC set S(d)= {s0(d)… sN-1(d)} as the set of carriers of serving cells that meet all the following conditions, where d is the target CC, and s0(d) is the source CC. - {s0(d)… sN-1(d)} are in the same band as s0(d). - {s0(d)… sN-1(d)} are in the same TAG as s0(d).

Based on this structure, we feel add “configured with UL Tx switching” into the condition of UL carrier set is an appropriate fix of the function of prioritization rules. In conclusion, we suggest to adopt the TP2 in Appendix.

Proposal 3: Adopt the TP in Appendix A.3 for prioritization rules of SRS carrier switching in TS 38.214 clause 6.2.1.3.

## Back-to-back switching with SRS carrier switching

In UL Tx switching, considering UE complexity of frequent UE RF retuning, the max number of switching for a UE is restricted to be one switching per one slot. If a UE is configured with SRS carrier switching and UL Tx switching, there may be too many switching occurrences in a slot. For example, as shown in Figure 1 and 2, a UE is configured with UL Tx switching between CC1 and CC2 and is configured with SRS carrier switching between CC2 and CC3. CC2 is the switching-from carrier and CC3 is the switching-to carrier. If the UE is to transmit on CC1 after the SRS transmission on CC3, the UE may firstly switch to CC2 and then switch to CC1 (Figure 1), resulting in two Tx switchings in a short period. Alternatively, in this scenario, the UE can directly switch to CC1 after the SRS transmission on CC3 (Figure 2). According to current spec, a UE has support of a switching between CC1 and CC3 as long as the UE reports supports of both UL Tx switching and SRS carrier switching. No new capabilities are required to indicate the direct switching between CC1 and CC3.

Thus, UE can avoid unnecessary frequent Tx switching. For this, the DCI that schedules the uplink transmission succeeding a SRS carrier switching should be received by the UE early enough so that a sufficient UE processing time to prepare and handle such Tx switching is guaranteed.

Figure 1 two Tx switching after the SRS transmission on CC3

Figure 2 one Tx switching after the SRS transmission on CC3

In other words, a cut-off time to schedule the succeeding uplink transmission is defined as that the time interval between the last symbol of PDCCH scheduling the succeeding uplink transmission and the first symbol of SRS transmission on CC3 should be at least N2 symbols and an additional time duration (for example, the RF retuning time).

This method not only releases UE burden from too frequent RF retunings but also allows a gNB to schedule the succeeding slot so that no UL throughput loss for network operation, the cost is the DCI should be received by the UE earlier. With the help of earlier arrival of scheduling DCI, if a UE prefer to implement two switchings/RF retunings in this case, then it is still up to UE to do it. But it provides the availability to avoid frequent RF retunings. For example, if the switching time from CC3 to CC1 is the sum of two reported switching gaps, UE can choose the best implementation, either direct switching or two-step switching.

Certainly, if the succeeding uplink transmission is too much later from the SRS transmission on CC3, there is no frequent Tx switching in a short period. Similar to the agreed restriction of one switching per one slot for UL Tx switching, the short period to define a too frequent Tx switching is also one slot. In other words, as shown in Figure 3, the scheduling cut-off time is applicable to a uplink transmission succeeding a SRS carrier switching occurrence only if the time interval between the last symbol of the RF retuning time after SRS transmission and the first symbol of the succeeding uplink transmission is less than or equal to 13 symbols.

Figure 3 PDCCH schedules the next transmission after the SRS transmission on CC3

Proposal 4: For a UE configured with UL Tx switching on two uplinks and configured with SRS carrier switching for a third uplink, if a uplink transmission is scheduled after a SRS carrier switching occurrence and the time interval between the first symbol of the uplink transmission and the last symbol of SRS transmission is less than or equal to an interval of 13 symbols plus the RF retuning time required by SRS carrier switching, then the last symbol of PDCCH scheduling the uplink transmission should be no later than at symbol L, where the time interval between symbol L and the first symbol of SRS transmission is larger than  N2 symbols plus the RF retuning time.

- In case of different SCS between the uplink transmission and the SRS transmission, the 13 symbols are with respect to the smaller SCS.

# Conclusion

In this contribution, analysis of the issues of introducing UL Tx switching for Rel-17 are provided. The following proposals are given:

Proposal 1: Adopt the TP in Appendix A.1 for the UL Tx switching in TS 38.214 clause 6.1.6 and agree the following RRC parameter list.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_RF_FR1_enh-Core | Uplink Tx switching enhancements | ULTxSwitching-2T2T-Mode | new | For a UE capable of 2Tx-2Tx switching and configured with uplinkTxSwitching, indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode in which the switching gap duration for a triggered uplink switching [xx 38.214] is equal to the switching time capability value reported for the switching mode. If 1Tx-2Tx mode is derived by the RRC parameter, then there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. |  |  |  | UE-specific |

Proposal 2: Adopt the TP in Appendix A.2 for uplink suspension of SRS carrier switching in TS 38.214 clause 6.2.1.3.

Proposal 3: Adopt the TP in Appendix A.3 for prioritization rules of SRS carrier switching in TS 38.214 clause 6.2.1.3.

Proposal 4: For a UE configured with UL Tx switching on two uplinks and configured with SRS carrier switching for a third uplink, if a uplink transmission is scheduled after a SRS carrier switching occurrence and the time interval between the first symbol of the uplink transmission and the last symbol of SRS transmission is less than or equal to an interval of 13 symbols plus the RF retuning time required by SRS carrier switching, then the last symbol of PDCCH scheduling the uplink transmission should be no later than at symbol L, where the time interval between symbol L and the first symbol of SRS transmission is larger than  N2 symbols plus the RF retuning time.

- In case of different SCS between the uplink transmission and the SRS transmission, the 13 symbols are with respect to the smaller SCS.

# References

R1-2112826, “[107-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #107-e, November 11th – 19th, 2021.

R1-2112817, “Summary of [107-e-NR-7.1CRs-04] Issue#4: Discussion on SRS carrier switching”, Moderator (Huawei), 3GPP RAN1 #107-e, November 11th – 19th, 2021.

R1-2106100, “Summary of [105-e-NR-7.1CRs-12] Issue#26 SRS carrier switching”, Moderator (ZTE), 3GPP RAN1 #105-e, May 10th – 27th, 2021.

# Appendix

# TP for the UL Tx switching in 38.214 clause 6.1.6

<Unchanged parts are omitted – 38.214> ### 6.1.6 Uplink switching The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod2T2T if [new Rel-17 RRC parameter for the 2Tx-2Tx Switching mode] is configured, and uplinkTxSwitchingPeriod otherwise: - If a UE indicated a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination - Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or - Configured with uplink carrier aggregation, or - Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink. the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively. If [new Rel-17 RRC parameter to the 2Tx-2Tx Switching Mode] is not configured, then there is expected to be one uplink configured with uplinkTxSwitching for the UE, on which the maximum number of antenna ports among all configured periodic/aperiodic SRS and activated semi-persistent SRS resources should be 1 and non-codebook based transmission is not configured. If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap. #### 6.1.6.1 Uplink switching for EN-DC For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), if the UE is configured with uplink switching with parameter uplinkTxSwitching, - for the UE configured with switchedUL by the parameter uplinkTxSwitchingOption, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - when the UE is to transmit an NR uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - when the UE is to transmit an E-UTRA uplink that takes place after an NR uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - the UE is not expected to transmit simultaneously on the NR uplink and the E-UTRA uplink. If the UE is scheduled or configured to transmit any NR uplink transmission overlapping with an E-UTRA uplink transmission, the NR uplink transmission is dropped, - for the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - when the UE is to transmit an NR two-port uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - when the UE is to transmit an E-UTRA uplink that takes place after an NR two-port uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - the UE is not expected to transmit simultaneously a two- port transmission on the NR uplink and the E-UTRA uplink. - in all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. - when the UE is configured with tdm-PatternConfig or by tdm-PatternConfig2 - for the E-UTRA subframes designated as uplink by the configuration, the UE assumes the operation state in which one-port E-UTRA uplink can be transmitted. - for the E-UTRA subframes other than the ones designated as uplink by the configuration, the UE assumes the operation state in which two-port NR uplink can be transmitted. #### 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - [If [new Rel-17 RRC parameter for the 2Tx-2Tx Switching mode] is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of of NTx1-Tx2 on any of the carriers.] - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on a carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', - when the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - If the UE is configured with [1T with uplinkTxSwitchingdualULTxState], when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on this carrier and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. #### 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. <Unchanged parts are omitted – 38.214>

# TP for uplink suspension of SRS carrier switching in 38.214 clause 6.2.1.3

<Unchanged parts are omitted – 38.214> #### 6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2, and also the uplink transmission on carrier c3 if the UE is configured with uplinkTxSwitching-r16 for uplink switching between uplink carrier c2 and c3. <Unchanged parts are omitted – 38.214>

# TP for prioritization rules of SRS carrier switching in 38.214 clause 6.2.1.3

<Unchanged parts are omitted – 38.214> #### 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell d with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sd={s0d…sN-1(d)} as the set of carriers of serving cells that each carrier meets one of the following conditions: - si(d) is in the same band as s0(d), or s0(d) and si(d) are both configured with uplinkTxSwitching-r16. - si(d) is in the same TAG as s0(d). where 1≤i≤N-1. <Unchanged parts are omitted – 38.214> For an SRS transmission starting in symbol Nc1d of carrier dc1 and a conflicting transmission in carrier sc2i(d) starting in symbol Nsc2i, where 1≤i≤N-1, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and NdNc1 is at least N2 symbols and an additional time duration TSRSCS, and the time interval between the last symbol of PDCCH and NsiNc2 is at least N2 symbols; and - semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1d, and considered active at least N2 symbols before Nsic2. where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across c1d, si(d)c2 and their corresponding scheduling cells. The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier and transmission of a physical signal/channel over a carrier of a serving cell in set Sd: - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sd happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sd happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sd configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in the set Sd whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. <Unchanged parts are omitted – 38.214>