3GPP TSG-RAN WG1 Meeting #108-e	R1-2200979

e-Meeting, February 21– March 3, 2022

Agenda Item:	8.16.15

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for LTE based 5G terrestrial broadcast

Document for:	Discussion and Decision

Introduction

After RAN1#107-e discussion, the RAN1 UE features for Rel-17 LTE is updated in [1]. This contribution provides our views for Rel-17 LTE_terr_bcast_bands_part1 UE features.

UE feature related discussion

The UE feature for LTE_terr_bcast_bands_part1 is updated as follows after RAN1#107-e meeting:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3. LTE_terr_bcast_bands_part1 | 3-1 | Support of new channel bandwidth for PMCH | [TBD: whether separate components are needed for different bandwidths] | Support of dedicated MBMS cells | Yes |  | UE cannot receive MBMS in the corresponding MBSFN area | Per band | N/A | N/A |  | Optional with capability signaling |

The remaining issue is whether separate components are needed for different bandwidths.

As agreed the supported system bandwidth indicated in MIB is set to 5 MHz or 3 MHz, for which PMCH can be configured with 6/7/8 MHz.

From the perspective of UE implementation capability, a single UE capability supporting flexible bandwidth of 6/7/8 MHz is sufficient. The component of the feature could be written to support the feature of flexible bandwidth for PMCH with system bandwidth limited to 5 or 3 MHz. However, there is worry from other company that UE that is targeting a single region shall implement the bandwidths for the other regions with the corresponding potential different RF requirements if a common capability is defined. With this regard taken into account, separate UE capabilities corresponding to different bandwidth is also acceptable.

Proposal:

- The component of the feature can be written as:

- Support flexible PMCH bandwidth (6/7/8MHz) allocation for the system bandwidth indicated in MIB set to 5 MHz or 3 MHz.

- Alternatively, define separate UE capabilities corresponding to different bandwidth (6/7/8 MHz).

- References

R1-2112900, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#107-e, November 11-19, 2021.