3GPP TSG RAN WG1 Meeting #108-e	R1-2200982

eMeeting, Feb. 21 – Mar. 3, 2022

Agenda Item:	8.11.1.1

Source:	FUTUREWEI

Title:	Power consumption reduction for sidelink resource allocation

Document for:	Discussion and decision

# Introduction

At RAN#86, a WI on sidelink enhancements was agreed for Rel-17 [1], modified in [2], and further updated in [3]. In this WI, there is an objective on resource allocation enhancements to reduce power consumption:

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

Since the Rel-17 SL WI normative work was not finished in RAN1#107-e, the scope was discussed in RAN#94-e. It was agreed in RAN#94-e that RAN1 is to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022, and all RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e.

In the status report of Rel-17 SL WI for RAN#94-e [4], a list of remaining open issues on physical layer aspects on resource allocation to reduce UE’s power consumption was summarized and copied below.

  - Finalization of pre-emption/re-evaluation checking for aperiodic transmission

  - Finalization of selection/report of candidate resources in which at least its subset is within RX UE's active time

  - Finalization of SL CBR measurement in partial sensing

  - CPS monitoring window for aperiodic transmission when UE performs at least CPS in a Tx pool

  - T1 of RSW when UE performs only CPS in a Tx pool with periodic reservation for another TB disabled

  - Sensing and SL CBR measurement during its SL DRX inactive time

  - Re-evaluation and pre-emption checking after random selection

  - Resource pool segregation for periodically occurring resources

  - Random resource selection in pools with mixed RA schemes

  - Conditions in which CPS can be disabled in resource (re)selection

These open issues were discussed in RAN1#107bis-e. A set of agreements was reached and listed in the Appendix.

In this contribution, we continue the discussions on the remaining open issues but not yet resolved or discussed in RAN1#107bis-e.

# Discussions

## Pre-emption/re-evaluation checking after random resource selection

In RAN1#103-e, it was agreed that re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception). However, a UE that is capable of sensing can still perform random resource selection for further power saving or due to lack of sensing results. One remaining issue on re-evaluation and pre-emption is whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing.

If UE performs random resource selection for further power saving, forcing UE to perform re-evaluation and pre-emption will diminish the power saving advantage of random resource selection. Therefore, if UE decides to choose random resource selection over any form of partial sensing and power consumption is main concern of UE, UE should not perform re-evaluation and pre-emption checking.

Since the configured CPS sensing window for periodic and minimum CPS window size for aperiodic traffic can be very small, if there is a concern on the performance, UE can perform sensing on some limited CPS sensing slots as they are still useful for re-evaluation and pre-emption. In this case, re-emption/re-evaluation checking after random resource selection is not applicable then.

Based on above discussions, we do not support any form of forcing UE performing re-evaluation/pre-emption checking after random resource selection, even in a certain pre-configured resource pool.

In some scenarios, UE decides to choose random resource selection not just for further power saving purpose. For example, if a UE already has some sensing results, but due to a small remaining PDB which leads to insufficient candidate resource slots or sensing window size (below a threshold), the UE may choose random selection. As agreed in RAN1#107-e, in CPS for aperiodic traffic, when the minimum M slots for CPS cannot be guaranteed, the following two options are supported,

- Option A, the UE ensures the Y’min criterion is fulfilled

- Option B: UE performs random resource selection

And when the UE performs Option A or Option B is up to UE implementation. So, the UE may choose option B for random resource selection. Some companies argued that in this case, re-evaluation and pre-emption is necessary to reduce the collisions. However, again, if a UE has a concern on the potential collisions, the UE should choose option A to obtain some sensing results, even on the slots that are less than that of the minimum sensing window, which may still have a certain performance improvement. When UE performs re-evaluation and pre-emption, the sensing results on the short sensing window for the initial resource selection are still useful. If a UE chooses option B due to a very short PDB, performing re-evaluation/pre-emption may not be very helpful. Since during the discussions in RAN1#107-e and RAN1#107bis-e, some companies think it may be beneficial to do re-evaluation and pre-emption checking by continuing sensing even a UE chooses option B. To have a certain flexibility, we think it is ok to leave re-evaluation and pre-emption after random resource selection to UE implementation.

So based on the discussions above, our preference is to not support it in this case. However, if there is a very strong desire, we could also accept to leave it to implementation on top of Option B.

Proposal 1: For a UE capable of sensing, do not support OR up to UE implementation to perform re-evaluation and pre-emption checking (if enabled sl-PreemptionEnable is provided) after random resource selection.

## Sidelink DRX

### Sensing during SL DRX inactive time: full sensing case

SL DRX is also a mechanism for SL power saving.  A UE transmits and receives SL signals only when DRX is on. One open issue is on UE performing partial sensing during the DRX off period. If the DRX on/off setting and partial sensing are independent from each other, not performing partial sensing during the DRX off period will impact the resource selection performance significantly. In RAN#107bis-e, the partial sensing in SL DRX inactive time was discussed and an agreement was reached as shown in Appendix and copied below.

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

We now discuss the remaining FFS on the sensing in SL DRX inactive time for the case when full sensing is configured in the UE by a higher layer. First, UE enables SL DRX for power saving. When a UE configured with full sensing, it then impacts the power saving purpose if UE performs the sensing on all slots in SL DRX inactive time. However, not requiring UE performs sensing in SL DRX may impact the performance, particularly when the resource selection is triggered right after SL DRX off time. For better balancing the PRR and power saving performance, we prefer to apply the same rule as in the agreement for full sensing case, provided that the UE supports partial sensing. One issue is that when UE is configured for full sensing, UE may not have partial sensing configurations for PBPS and CPS. Then for periodic based sensing or sensing for periodic traffic, for a given Preserve in sl-ResourceReservePeriodList, UE performs the default sensing occasion (the most recent sensing occasion) for any slot within RSW. For aperiodic traffic, similar to CPS, UE performs minimum M slots before n+T1. However, since configuration of minimum M slots may not be available when UE is configured with full sensing. In this case, UE performs sensing from 31 logical slots before n+T1 to slot n-1 in SL DRX inactive time

Proposal 2: When a UE is configured with full sensing, the UE performs SL reception of PSCCH and RSRP measurement on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration,

- When it is enabled and if UE supports partial sensing,

  - for a given Preserve in sl-ResourceReservePeriodList, UE monitors only on the default (most recent) periodic sensing occasion for any slot with in RSW.

  - in addition, UE performs sensing from 31 logical slots earlier than slot n+T1 to slot n-1 .

- When it is disabled or if UE does not support partial sensing, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time.

### Selection/report of candidate resources in which at least its subset is within RX UE's active time

When the DRX active time of Rx-UE is provided from high layer, the following was agreed in RAN1#107-e.

Agreement from RAN1#107-e: When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE

There are several FFS’s to be discussed. For the first FFS, when the number of subsets of candidate resource is less than the threshold, theoretically, it is sufficient to have one candidate resource included in SA, which can be included as one addition criterion in Step 7 of the procedure for forming resource candidate set in 38.214 to terminate the procedure.

Several alternative options were proposed in RAN1#107bis-e to solve the issue when the number of candidate resource in the Rx DRX active time is less than the threshold. One of predominant option (option 1) is to set another threshold of X*Ntotal on the number of resources in SA in the active time together with the original threshold on all available resources in SA, where Ntotal is the number of RSW slots in the active time. Since Ntotal can be chosen any number which could be as small as 1, a preconfigured minimum number of slots in DRX active time, N, was proposed as an enhancement along with the new threshold. However, with a preconfigured N, it is possible that the available number of slots is less than N. A further fix or enhancement is needed.

Since RSW is set by UE implementation, as long as there is 1 slot of RSW in the DRX active time, with the existing RSRP threshold increasing, there will be 1 candidate resource in SA in Rx DRX active time. Therefore, we do not need a preconfigured N. The new threshold on the number of candidate resources in active time can be simply as 1. For enhancement, in order to have a flexibility for MAC selecting a resource and UE forming SA, we propose to specify the additional threshold on the number of subsets of candidate resource in RX UE active time as a fraction of the original threshold, i.e., a*X*Mtotal, where a can be preconfigured. It is seen that with a = Ntotal/Mtotal, the threshold becomes X*Ntotal, which can be a default value of a. Since the threshold a*X*Mtotal changes with the number of Mtotal, if the number of available active slots is small, UE can choose a small T2 to have a small Mtotal and the threshold can still be satisfied without addition fix.

Proposal 3: When PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE, an additional criterion is included in Step 7 of the procedure for forming resource candidate set in 38.214,

- Alt 1: the number of subsets of candidate resource in RX UE active time is at least 1.

- Alt 2: the number of subsets of candidate resource in RX UE active time is no less than a*X*Mtotal, and

  - the factor a by default is Ntotal/Mtotal or preconfigured with other values, where Ntotal is the number of slots within the Rx DRX active time.

## CPS window for aperiodic traffic and CPS disabling

For CPS window for aperiodic traffic, the following has been agreed in RAN1#107bis-e.

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - For minimum size M of the CPS monitoring window [n+TA, n+TB]: - By default, M is 31 unless (pre-)configured with another value - The range of (pre-)configured M is from 0 (working assumption) to 30

The lower bound of minimum size M for CPS window 0 is still working assumption to be confirmed. The value depends on whether CPS can be disabled or not.

For a UE that is capable of partial sensing, in general, it is not preferred to disable CPS in resource (re)selection as minimum CPS window size can be small, which does not consume much power. However, it is better to keep it as an alternative option for additional power savings.

The condition to disable CPS and switch to random resource selection can be specified. Based on past CBR measurement, the UE can detect whether the system load is high or low. When the system load is low, there is no resource, or a small number of resources occupied by other nearby UEs. Under this condition, the UE can disable CPS and switch to random resource selection to reduce power consumption. The conditions can then be set by a threshold on CBR. To enable CPS, a predefined timer can be activated once the UE switches to random resource selection.

Another condition is for low power levels (e.g., battery levels). In such a case, when the battery is about to drain out or is below a certain level, to reduce power consumption, it is beneficial that UE disables CPS and perform random resource selection. For low power level UEs, they may also be configured to perform random resource selection all the time and not to switch between partial sensing and random resource selection.

Proposal 4: UE can disable CPS when measured CBR is below a threshold. A timer or counter can be preconfigured to switch back to partial sensing.

If CPS can be disabled, the lower bound of M can be set to M=0. However, if CPS cannot be disabled, some other non-zero value should be used.

As in the agreement for CPS, the minimum number of M slots for CPS defined for aperiodic traffic includes the processing time, Tproc,0 and Tproc,1.  With a small M value, e.g, M=1, it is possible that the sensing window [n+TA, n+TB] becomes zero size, which is equivalent to CPS disabling. Therefore, to ensure that there is at least 1 logical slot for the CPS sensing, the lower bound should be the number of logical slots within Tproc,0SL+Tproc,1SL  plus one, which is then SCS dependent. Therefore, it is inappropriate to set M=1. If a single value is preferred for the lower bound of M, since for 15 kHz SCS, the number of physical slots in Tproc,0SL+Tproc,1SL is 4, we can set lower bound of M=5. Then for most cases, we have at least one slot. For other cases, but for other cases when there are more logical slots within the processing time Tproc,0SL+Tproc,1SL  , additional requirement is needed, i.e., UE at least includes one logical slot before ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing.

Proposal 5: The lower bound of minimum M slots can be set as 0 if CPS can be disabled, or 5 if CPS cannot be disabled.

- In addition, UE should include at least the latest logical slot before ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing if the configured M does not ensure one logical slot before ty0SL-(Tproc,0SL+Tproc,1SL)

## Random resource selection in pools with mixed RA schemes

In RAN1#106-e, three options to resolve the issue of low priority transmissions in mixed RA (and one option to do nothing) were agreed, namely:

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration.

In RAN1#106bis-e, when these options were discussed, option 1 received majority support. Option 1 was then discussed with several updates. However, no conclusion was reached in RAN1#106b.  The discussions were continued in RAN1#107bis-e. Again, option 1 received predominant supports but several companies preferred to include the case that the priority is applied to a subset of resources in the pool, i.e., pool partitioning. In the latest proposal copied below, the pool partitioning was included as an FFS option.  However, there was no consensus on this proposal in the end as some companies think that the partition is unnecessary, and it can be achieved by configuring separate pools.

Proposal 9 (VI): - When UE performs random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, - A priority threshold value is (pre-)configured for one of N partitions of the resource pool, below or equal to which random resource selection is allowed in that partition. Note, lower value means higher priority. The (pre-)configured priority threshold can be any of the 8 priority values. - Decide whether N=1 or N=2 in RAN1#108e - Note, how to capture the random selection UE behavior to transmit a TB with the priority value larger than the (pre-)configured threshold in the exceptional resource pool is up to RAN2 discussion.

Again, as the problem is important (and the configuration can be set such that the priority is not applied on a pool), we can accept Option 1 without partitioning. However, below we explain more as to why being able to apply the priority threshold to a subset of resources is beneficial.

First, the overlap with a Rel-16 pool could happen even for a Rel-17 pool that only has random resource selection. It is then much beneficial to apply a threshold to the overlapped part in the pool for random resource selection. On the other hand, if there is an overlap between two Rel-17 resource pools with different thresholds, some specification or rules are needed when there is a resource pool overlap. For example, if a UE is configured with two resource pools assigned with different priority thresholds, we may specify that the lower priority threshold (the higher priority threshold value) of the two pools is applied to the overlapped resource set.

Second, the low-priority random selection may not get its data transmitted in time if its priority is lower (higher priority value) than the configured priority threshold. Some low priority data may have long delay. It is better to allow some low priority transmissions in each pool. Therefore, a subset of resources in the pool without any restrictions is beneficial for this case.

Configuring separate pools instead of applying the pool partition in a specific way can partially solve some issue, e.g., with separate pools in FDM, the latency issue can be addressed. However, it does not solve the pool overlapping issues and it will also introduce new issues. First, with separate pool configurations, the number of pools increases. We have maximum number of 8Tx pools.  Also due to many-to-one mapping between Tx and Rx pool, number of the combinations of Tx pools (to form a Rx pool) also increases.  There questionable that current setting of the maximum number of Tx pools or Rx pools is sufficient. Second, with the pool partition, the full/partial sensing UE and the RRS with priority values access the resource pool with full flexibility. With separate pools, the UE has to do a cross-pool scheduling for resource allocation to utilize resources in both pools. Currently, retransmissions are not allowed in different pools. In addition, with FDM, the number of subchannels in a resource pool becomes smaller which limits maximum number of TB size.

Based on above discussions, we can see clearly that applying the priority threshold to a subset of resources is beneficial.

Proposal 6: When UE performs random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection,

- A priority threshold value is (pre-)configured for the resource pool or a subset of resource in the resource pool, below or equal to which random resource selection is allowed. Note, lower value means higher priority. The (pre-)configured priority threshold can be any of the 8 priority values.

  - When there is an overlap between two Rel-17 pools with different priority threshold values, the smallest value is applied on the overlapped resources

  - When there is an overlap between a Rel-16 pool and a Rel-17 pool, use the priority threshold for the pool also just with random resource selection

## Update of resource exclusion in Step 6

In RAN1#107bis-e, several companies proposed to update the resource exclusion in Step 6 of 8.1.4 in TS38.214, which was briefly discussed. The views were diverse, and the discussion of the issue was postponed. The concern was on the procedure step 6 c) which is copied below.

6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: ….. c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

The procedure was specified based on full sensing which was before the trigger slot n. The sensing slots for PBPS and CPS may be located after slot n. Several companies proposed to update the parameters, particularly on Tscal (consequently, Q) and  t'n'SL for partial sensing.  A proposal was initialized in RAN1#107bis-e as follows.

In Step 6 c) of TS38.214 Section 8.1.4, adopt the following changes: - t'n'SL=ty0SL-(Tproc,0SL+Tproc,1SL) if slot ty0SL-(Tproc,0SL+Tproc,1SL) belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL’ is the first slot after slot ty0SL-(Tproc,0SL+Tproc,1SL) belonging to the set t'0SL,t'1SL,...,t'T'max-1SL. - Down-select between: - Alt. 1: Tscal=n+T2-(ty0SL-(Tproc,0SL+Tproc,1SL)) converted to milliseconds - Alt. 2: Tscal=tyLSL-(ty0SL-(Tproc,0SL+Tproc,1SL)) converted to milliseconds, where slot tyLSL is the last slot of the Y or Y’ candidate slots

The procedure is to check whether the periodic reservations from other UEs on slots t'm+q×Prsvp_RX'SL,  have any resource overlap with the candidate resources Rx,y+j×Prsvp_TX' for the periodic transmissions for the sensing UE (Tx UE). The number of periods Q is however determined by Tscal which is converted from T2 (determined by sensing UE) in second. The resource on t'm+q×Prsvp_RX'SL,  is limited to the RSW of the sensing UE. Therefore, setting an appropriate Q value is just to find the resource conflict within the RSW for efficient check. With a larger Q, the unnecessary slots t'm+q×Prsvp_RX'SL,  are included for resource overlap check but will not affect the outcome as Rx,y+j×Prsvp_TX' is always within the candidate resources in RSW. On the other hand, the step 6 c) is also used for re-evaluation and pre-emption where the sensing is performed after resource selection triggering slot n. Therefore, there is no issue to use existing definition of Tscal and Q. Similarly for the reference slot t'n'SL. Then, it is unnecessary to update the definition of these two parameters for resource exclusion in Step 6 for partial sensing.

Proposal 7: Update of Tscal and t'n'SL definitions for resource exclusion in Step 6 is unnecessary for partial sensing.

## RRC Parameters

For PBPS and CPS for periodic traffic, the following has been agreed on the CPS sensing.

Agreement in RAN1#106bis-e: When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value.

The range of configured RRC parameter (contiguousSensingWindowPeriodic) on M for CPS window was discussed in RAN1#107-e and RAN1#107bis-e. While the upper bound 30 has been agreed, there was no consensus on the lower bound.

Since sensing ends at n+TB which is Tproc,0SL+Tproc,1SL slots earlier than ty0SL,  the smallest value of M should be large enough to cover physical slots Tproc,0SL+Tproc,1SL i.e., 4, 6, 11, and 21 slots for 15, 30, 60, and 120 kHz SCS, respectively, to ensure the UE has enough time for processing sensing results and resource selection, and also has at least one logical slot for CPS sensing. If logical slots have 1:1 mapping to consecutive physical slot, then the smallest value of M should be one slot more than Tproc,0SL+Tproc,1SL to ensure UE has enough processing time and has at least 1 sensing slot. If logical slots do not have 1:1 mapping to physical slots, the number of logical slots over Tproc,0SL+Tproc,1SL consecutive physical slots is generally larger than 1 logical slot and also SCS dependent. Therefore, it is better to have the minimum value of M to be SCS dependent. Denote M’proc,mSL as the number of logical slots spanning on Tproc,0SL+Tproc,1SL consecutive physical slots where M’proc,mSL is SCS dependent. The lowest value of M can be specified as M’proc,mSL+Y where Y≥ 1 can be a fixed value for all SCS’s. Similar to the minimum CPS size for aperiodic traffic discussed before, we can have a unified lower bound, e.g., 5 logical slots. However, we still need to fix the issue that for some cases that there are no resources available when there is no slot for sensing due to a larger number of slots within the processing time. Different from the CPS for aperiodic where the minimum number of slots is defined, the M is configured exact CPS window for periodic traffic.  In this case, we specify an exact number of logical slots for CPS sensing, e.g., 1 slot.

Proposal 8: For RRC parameter contiguousSensingWindowPeriodic related to the CPS window configuration,

- Alt. 1: the lowest value of M is SCS dependent.

- Alt. 2: the lowest value of M is 5.

  - When there is no logical slot available for CPS sensing before ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing for the configured M, include the latest logical slot before  ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing

# Conclusion

The remaining open issues for sidelink power saving were discussed. We propose the following:

Proposal 1: For a UE capable of sensing, do not support OR up to UE implementation to perform re-evaluation and pre-emption checking (if enabled sl-PreemptionEnable is provided) after random resource selection.

Proposal 2: When a UE is configured with full sensing, the UE performs SL reception of PSCCH and RSRP measurement on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration,

- When it is enabled and if UE supports partial sensing,

  - for a given Preserve in sl-ResourceReservePeriodList, UE monitors only on the default (most recent) periodic sensing occasion for any slot with in RSW.

  - in addition, UE performs sensing from 31 logical slots earlier than slot n+T1 to slot n-1.

- When it is disabled or if UE does not support partial sensing, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time.

Proposal 3: When PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE, an additional criterion is included in Step 7 of the procedure for forming resource candidate set in 38.214,

- Alt 1: the number of subsets of candidate resource in RX UE active time is at least 1.

- Alt 2: the number of subsets of candidate resource in RX UE active time is no less than a*X*Mtotal, and

  - the factor a by default is Ntotal/Mtotal or preconfigured with other values, where Ntotal is the number of slots within the Rx DRX active time.

Proposal 4: UE can disable CPS when measured CBR is below a threshold. A timer or counter can be preconfigured to switch back to partial sensing.

Proposal 5: the lower bound of minimum M slots can be set as 0 if CPS can be disabled, or 5  if CPS cannot be disabled.

- In addition, UE should include at least the latest logical slot before ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing if the configured M does not ensure one logical slot before ty0SL-(Tproc,0SL+Tproc,1SL)

Proposal 6: When UE performs random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection,

- A priority threshold value is (pre-)configured for the resource pool or a subset of resource in the resource pool, below or equal to which random resource selection is allowed. Note, lower value means higher priority. The (pre-)configured priority threshold can be any of the 8 priority values.

  - When there is an overlap between two Rel-17 pools with different priority threshold values, the smallest value is applied on the overlapped resources

  - When there is an overlap between a Rel-16 pool and a Rel-17 pool, use the priority threshold for the pool also just with random resource selection

Proposal 7: Update of Tscal and t'n'SL definitions for resource exclusion in Step 6 is unnecessary for partial sensing.

Proposal 8: For RRC parameter contiguousSensingWindowPeriodic related to the CPS window configuration,

- Alt. 1: the lowest value of M is SCS dependent.

- Alt. 2: the lowest value of M is 5.

  - When there is no logical slot available for CPS sensing before ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing for the configured M, include the latest logical slot before  ty0SL-(Tproc,0SL+Tproc,1SL) for CPS sensing

# References

RP-193257, “NR sidelink enhancement”, RAN#86, Dec. 09-12, 2019.

RP-201385, “WID revision: NR sidelink enhancement”, RAN#88-e, June 29-July 3, 2020.

RP-202846, “WID revision: NR sidelink enhancement”, RAN#90-e, Dec. 7-11, 2020.

RP-212880, “Status report of WI: NR Sidelink enhancement”, RAN1, Dec. 6-17, 2021.

# Appendix

The agreements from RAN1#107bis-e are listed below.

Agreement

When UE is configured to perform partial sensing by a UE higher layer (including when SL DRX is configured), SL RSSI is measured in slots where the UE performs partial sensing and PSCCH/PSSCH reception over the SL CBR measurement window defined in Rel-16. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured.

- If the number of SL RSSI measurement slots is below a (pre-)configured threshold, a (pre-) configured SL CBR value is used.

Agreement

When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n,

- The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots.

  - tyiSL is the first candidate slot after slot n+T3.

- UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool

  - It is up to UE implementation

- UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL.

  - FFS: When the minimum M slots for CPS cannot be guaranteed,

- All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

- For minimum size M of the CPS monitoring window [n+TA, n+TB]:

  - By default, M is 31 unless (pre-)configured with another value

  - The range of (pre-)configured M is from 0 (working assumption) to 30

Agreement

When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- T1 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4.

  - No update to specification is necessary due to this agreement

- Note: The selected Y’ slots do not overlap with the sensing window

Agreement

Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer.

- When it is enabled,

  - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion.

  - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS.

- Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time.

- Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17.

FFS the case when full sensing is configured in the UE by a higher layer