3GPP TSG RAN WG1 Meeting #108-e	R1-2200984

e-Meeting, Feb. 21 – Mar. 3, 2022

Agenda Item:	8.16.11

Source:	FUTUREWEI

Title:	UE features for NR sidelink enhancement

Document for:	Discussion and decision

# Introduction

At RAN#86, a WI on sidelink enhancements was agreed for Rel-17 [1], modified in [2], and updated in [3]. In the latest WID [3], the objectives on resource allocation enhancements to enhance reliability include:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

  - Note: RAN2 work will start after RAN#89.

As the Rel-17 is to be completed soon, the discussions on Rel-17 UE features started in RAN1#106bis-e. For the Rel-17 sidelink, the UE feature discussions started from the preliminary RAN1 UE features for Rel-17 NR were provided in [4] including the UE features for Rel-17 sidelink based on the WI sidelink enhancements.

At RAN1#106bis-e and RAN1#107-e, five agreements were reached, and two working assumptions were made to specify several Rel-17 SL UE features. At RAN1#107bis-e, the discussions on UE features continued, concluded with three agreements, one working assumption, and two conclusions, which are listed in the Appendix.

In this contribution, we provide comments on the detailed components and/or prerequisites for the agreed and potential UE features enhanced sidelink. Based on the discussions, we provide our proposed updates on the UE features for Rel-17 sidelink enhancements.

# Discussions

In the preliminary RAN1 UE features for Rel-17 NR summarized in [4], five FGs were listed as follows.

- 32-1	[Receiving NR sidelink of PSCCH/PSSCH/PSFCH/S-SSB]

- 32-2	[Receiving NR sidelink of PSFCH/S-SSB only]

- 32-3	Transmitting NR sidelink mode 2 with full sensing

- 32-4	Transmitting NR sidelink mode 2 with partial sensing

- 32-5	Inter-UE coordination in NR sidelink mode 2

The original table with the details for the NR sidelink enhancement in [4] is included as Table 1 in APP-2 of Appendix.

In RAN1#106bis-e and RAN1#107-e, several agreements and working assumptions were reached. Several features were specified with details to be determined. In particular, the following UE features are agreed as Rx/Tx capabilities.

- 32-2: Receiving NR sidelink of PSFCH/S-SSB

- 32-4: Transmitting NR sidelink mode 2 with partial sensing.

- 32-4a: Transmitting NR sidelink mode 2 with random resource selection (RRS).

In addition, two features for inter-UE coordination were agreed, i.e., one for scheme 1 and one for scheme 2, listed as follows.

- 32-5a: Inter-UE coordination scheme 1 in NR sidelink mode 2

- 32-5b: Inter-UE coordination scheme 2 in NR sidelink mode 2.

In RAN1#107bis-e, the following details on FGs 32-4 and 32-4a for transmit capability were agreed.

- For 32-4 partial sensing, since companies agreed that the full sensing is not a prerequisite of partial sensing, the components of Rel-16 FG 15-3 were included in FG 32-4 as its components except component 4 for full sensing. The notes from FG 15-3 were also included in FG 32-4 with the note on RRS in exceptional pool to be further discussed.

- Similarly for 32-4a RRS, the components of Rel-16 FG 15-3 were included in FG 32-4 as its components except component 4 full sensing. The notes from FG 15-3 were also included in FG 32-4 with the note on RRS in exceptional pool to be further discussed.

For both FGs above, the additional components are FFS and the prerequisite is to be determined.

In addition, the FG 32-5b on inter UE coordination scheme 2 is further split to two FGs, i.e., for scheme 2 transmit capability and for scheme 2 transmit capability, given as

- 32-5b-1: Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2

- 32-5b-2: Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2

It is also concluded that the UE without NR SL reception is supported in Rel-17, which corresponds to the Type A UE in the standard discussions.

For FG 32-2 Receiving NR sidelink of PSFCH/S-SSB, there was no consensus on whether to further split the reception capability of PSFCH and S-SSB. Eventually, it is concluded that the corresponding FFS is removed and the whole FG is highlighted as an unstable one.

Based on existing agreements and conclusions, we now continue to discuss the Rel-17 SL UE features.

### Type A UE and Synchronization Feature

As aforementioned, in RAN1#107bis-e, it was concluded that the UE without NR SL reception is supported in Rel-17. However, it is meaningless if such UE does not support any other Rel-17 UE features. The purpose of the conclusion is to support Type A UE which is only for Rel-17 SL transmissions. Therefore, a Type A UE should support a transmit feature which is clearly FG 32-4a RRS. In order to support transmissions in SL, such a UE has to support certain synchronization in order to transmit on the SL frame structure.

In RAN1#107bis-e, a new synchronization FG 32-4b was proposed and discussed but no consensus was achieved. In the FL proposal, the components introduced in FG 32-4b are the components for the Rel-16 synchronization FG 15-4 except that synchronization to a synchRef UE is removed as the UE-A does not have the capability of receiving S-SSB for synchronization. However, there were some questions on some components.

First, the gNB is included as one synchronization source. Since UE-A does not have capability of reception of S-SSB, it is not clear that it has the capability of reception of the synchronization signal from gNB. For this issue, we think that during the discussions we only agreed that Type A UE is not capable of performing reception of any SL signals and channels. We did not discuss whether UE-A is capable of reception of gNB signals. Although the Rel-17 SL WI focus on mode 2, it does not mean that it is only for out-of-coverage case. We prefer that Type-A UE has the capability of sync with gNB. Therefore, we prefer to include the sync with gNB case in FG 32-4b.

The other question is whether to include the transmission feature component in the synchronization FG 32-4b. Since this is the synchronization FG which is not viewed as either a reception capability or transmit capability, following the Rel-16 synchronization FG 15-4, we should include the components of the transmit features. In particular, the capability of transmitting S-SSB should be included as GNSS sync is supported so that the UE can be a syncRef UE.

Proposal 1: Support adding an FG for synchronization sources for NR sidelink transmission FG 32-4b

- Include components of Rel-16 FG 15-4 as components of FG 32-4b (except the sync to the syncRef UE) including the component of transmitting S-SSB.

Regarding the prerequisite, we can follow FG 15-4 and include at least one of Rel-17 FGs, i.e., FG 32-4a RRS and FG 32-4 partial sensing, and related Rel-16 FGs, i.e., FG 15-1, FG 15-2, and FG 15-3.

Proposal 2: The prerequisite of Rel-17 sync FG 32-4b is at least one of 15-1, 15-2, 15-3, 32-4, 32-4a.

The details of FG32-4b is included in Table 3 in APP-3 Updated FG Table for Rel-17 SL Enhancement in [6]  and Proposed updates

### Type B UE and FG 32-2 for reception of S-SSB and PSFCH

For the Type B UE feature, i.e., 32-2 receiving NR sidelink of PSFCH/S-SSB only, an FG 32-2 was agreed in RAN1#107-e with FFS on whether further split the reception of PSFCH and S-SSB and FFS on the components, as shown in Table 2, i.e., the updated table in [6]. However, in RAN1#107bis-e due to diverse view on whether further split the FG, no consensus can be made. It was concluded that the whole FG is highlighted as unstable with the FFS on the further split removed.

As discussed in our previous contributions [7][8], since the UE reception types are used as the reference for evaluation and designing of SL power saving features in R17, there were no agreements to support these UE features. Therefore, we may not need to specify UE features for all these SL reception types. Also based on the WID in [3], the objective for power saving is to introduce sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. Further, it is included in the WID that enhancements introduced in Rel-17 should be based on the functionalities specified in Rel-16. The WID indicates that Rel-16 NR sidelink resource allocation mode 2 is the prerequisite of the power saving features for Rel-17 sidelink. Therefore, it is not necessary to define FGs for the three UE types. If there is no consensus, we prefer to drop FG 32-2.

If FG 32-2 for Type B UE reception is kept as most companies want it, we do not support further split of the capability of PSFCH and S-SSB as we did not have any discussions, agreement, or evaluation in RAN1 SL group on such new type of UEs. This is also aligned with the conclusion from last meeting for no consensus in RAN1.

Proposal 3: Prefer not to include the FG 32-2.

- If FG32-2 is agreed, do not support further split the capabilities for PSFCH and S-SSB receptions as different FGs

If FG 32-2 is kept, here are the discussions on the related Rel-16 UE features as the pre-requisites or the components of this new feature.

15-1: For UE SL reception FG 15-1, all are not supported except component 8 as SCS and CP still needs to be reported for PSFCH reception.

15-3: All transmission features in FG 15-3 are not needed as Type B UE feature is for receiving capability only.

15-4: Synchronization is needed for SL transmission. S-SSB reception is supported in this feature. Therefore, FG 15-4 is supported.

15-5: Since type B UE does not support PSCCH reception, it cannot measure or process CBR measurements. As agreed in RAN1#107-e, when no SL CBR measurement result is available, a (pre-)configured SL CBR value is used. Congestion control based on CBR and CR is still supported. However, since there is no CBR measurement, there is no need to consider the congestion control feature support here.  Instead, it can be considered for FGs of Tx capabilities.

15-11: This FG is needed as PSFCH reception is supported in this new feature

15-23: This FG cannot be supported as UE is unable to receive the RSRP report from Rx UE for open loop power control.

Based on above discussions, in summary, component 8 of 15-1 is included as a component of FG 32-2 and FGs 15-4 and 15-11 are included as prerequisites of FGs 32-2. Since FG 32-2 is defined as a receive capability, as companies preferred in RAN1#107bis-e, we are fine to include only the Rx related components of 15-4 and 15-11 as components of FG 32-2.

Proposal 4: The components of FG 32-2 (if it is specified) include component 8 of 15-1 and Rx related components of FGs 15-4 and 15-11.

Since this is a Rx capability and the synchronization is self-contained, there is no FG included as the prerequisite.

The detailed FG 32-2 with proposed updates is included as in Table 4 of APP-3.

### FG 32-4a Transmitting NR sidelink mode 2 with RRS

For FG 32-4a RRS feature, since it is a transmitter capability, all UE receiving FGs are not applicable. Only transmitting FG 15-3 and FG 15-5 for congestion control are applied.

15-3: Since Type A/B UEs can support random resource selection but not full sensing, to avoid the conflict with features of type A/B UEs, component 4 for full sensing capability in FG 15-3 should be excluded.

15-5: Since a type A/B UE does not support PSCCH reception, it cannot measure or process CBR measurements. Therefore, component 1 is not supported. As agreed in RAN1#107-e, for random resource selection, when no SL CBR measurement result is available, a (pre-)configured SL CBR value is used. Congestion control based on CBR and CR is still supported. Component 2 can be updated as UE can adjust its radio parameters based on a pre-configured CBR value and CRlimit. For component 3, type A/B UE can only processes CR.

As shown in APP-1, in the updated FG 32-4a agreed in RAN1#107bis-4, the components of FG 15-3 were included in FG 32-4 except component 4 for full sensing. However, the components for congestion control were not included, which we propose to include as the components of FG 32-4a.

Proposal 5: Include congestion control components as in FG 15-5 to FG 32-4a RRS with necessary updated based on Rel-17 agreements.

For FG 32-4a RRS, no prerequisite is necessary. Therefore, the prerequisite of FG 32-4a is none.

In addition, if FG 32-4a is not supported, a UE does not support random resource selection in a shared resource pool. However, a UE may still support random resource selection in the exception pool if UE supports Rel-16 basic feature.  Correspondingly, the column “Consequence if the feature is not supported by the UE” for FG 32-4a is updated in Table 5.

Proposal 6: For FG 32-4a RRS, in the column of “Consequence if the feature is not supported by the UE”, the description should be updated as

- UE does not support transmission according to the Rel-17 random resource selection and resource allocation

In the agreed FG 32-4a RRS from RAN1#107bis-e, the note on the Random selection according to Rel-16 in the exceptional pool is to be confirmed. One issue of the note is that the meaning of “according to Rel-16” is not clear. Since there is no RRS FG specified and UE may only support Rel-17 RRS and basic sync FG, but not support any Rel-16 features, how to access exceptional pool according to Rel-16 procedures is not well defined. However, from PHY point of view, there is no problem for a UE capable of Rel-17 RRS to access the exceptional pool with RRS. The issue may be an upper layer problem. Therefore, if there is no issue from upper layer, UE can support RRS according to Rel-16 in the exceptional pool. Another way is that we can specify that the exceptional pool in Rel-16 is supported in Rel-17, we can simply remove the wording “according to Rel-16”.

Proposal 7: For the note on Random selection in FG 32-4a RRS,

- Alt. 1 confirm the note on random selection according to Rel-16 in the exceptional pool if there no issue in the upper layer procedure.

- Alt. 2 remove “according to Rel-16” if the exception pool in Rel-16 is supported in Rel-17.

The updated FG 32-4a with our proposals is in Table 5 of APP-3.

### FG 32-4 Transmitting NR sidelink mode 2 with Partial Sensing

For the agreed FG 32-4 partial sensing feature, as discussed in RAN1#107bis-e, full sensing is not prerequisite of the partial sensing. Then in the updated FG 32-4 agreed in RAN1#107bis-e, the components of Rel-16 FG 15-3 except full sensing component 4 were included.

First, as agreed, RRS is supported in partial sensing as one of the options when minimum M slots for CPS cannot be guaranteed. Therefore, FG 32-4a RRS should be a prerequisite of FG 32-4 partial sensing. In general, the reception capability FG 15-1 should be the prerequisite of FG 32-4. Also, since congestion control and CBR measurement is supported for partial sensing, FG 15-5 is a prerequisite FG for 32-4.

Proposal 8: Add FG 32-4a, FG 15-1, and FG 15-5 as the prerequisite of FG 32-4 partial sensing.

Regarding the note on the Random selection in FG 32-4, similar to that in FG 32-4a, Proposal 7 is applicable.

In FGs 15-3, 32-4 and 32-4a for SL transmissions, as in the components, up to B sidelink processes are supported. One issue arises that if UE reports more than one FGs of these three, how many processes should UE support it. We prefer that in this case, the total number of processes is B instead of sum of each value B in all reported FGs. A note can be added in both FG 32-4 and 32-4a.

Proposal 9: Add the following note to FGs 32-4 and 32-4a

- If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B.

### UE Features for Inter-UE Coordination

For FG 32-5a inter UE coordination scheme 1, we do not need to separate the Tx vs Rx for inter UE coordination features. We can see that it may be possible a Scheme 1 Rx UE only uses RX coordination information without performing sensing and does not try to help others by TX. However, a Scheme 1 Rx UE needs to detect the PSCCH/PSSCH to receive coordination information as MAC-CE is always used for coordination transmission (only coordination information in SCI is Rx optional). And even for the RX only UE, it also may need to send the coordination request or some info via RRC signaling to UE-A which is related to Tx functionality.  Similarly for Scheme 1 Tx UE, i.e., UE-A, UE-A needs the reception capability of PSCCH for sensing to generate coordination information, i.e., again Tx and Rx mixed. Therefore, there is no need to split the Tx and Rx features of Scheme 1.

For preferred vs non-preferred, many unified designs, e.g., container, content format, were agreed. We do not think it is necessary to split them. Similarly for request vs condition- based coordination, further splitting the FG is unnecessary. We have agreed RRC parameters. These combinations can be controlled or enabled/disabled by RRC parameters. Therefore, we do not support further splitting of FG 32-5a unless there is an agreement on some UE feature which is subject to UE capability.

Proposal 10: Do not support further split of FG 32-5a unless there is an agreement on some UE feature which is subject to UE capability.

For both inter-UE coordination features, it is not necessary that a UE supporting inter-UE coordination must support the partial sensing feature or vice-versa.  For inter-UE coordination, the mechanism of partial sensing is not suitable for Scheme 1 UE to generate coordination information and Scheme 2 Tx UE to detect the potential/expected conflict. Therefore, full-sensing FG 15-3 should be the pre-requisite of FG 32-5a and FG 32-5b-1. For inter-UE coordination, synchronization from a syncRef UE is beneficial. Therefore, inter UE coordination should support Rel-16 FG 15-4 for synchronization. Supporting both Tx and Rx PSFCH features is necessary for Scheme 1 and Scheme 2 Tx UEs. Then FG 15-11 should also be supported for both FG 32-5a and FG 32-5b-1. Other Rel-16 mode 2 basic features such as FG 15-5 congestion control and 15-23 FG power control should also be supported for Scheme 1 UEs as they transmit and receive PSSCH for coordination information and data. For scheme 2 Tx UE, it is not necessary to support these two features.

Since some Rel-16 FGs are prerequisites of another Rel-16 FG, e.g., the receive capability FG 15-1 is the prerequisite of FG 15-3, an issue arises whether such prerequisite structure should be inherited or not. If not, FG 15-1 should then be listed as the prerequisite for FG 32-5a and FG 32-5b-1. Although not inheriting Rel-16 prerequisite provides larger flexibility in defining the prerequisite of Rel-17 FGs, we do not see the need based on the current agreement. Inheriting Rel-16 prerequisite may also facilitate the implementation of Rel-17 SL if Rel-16 SL is implemented.

Proposal 11: Support inheriting the prerequisite FGs of a Rel-16 FG as the requisite of a Rel-17 FG if the Rel-16 FG is listed as the prerequisite of the Rel-17 FG.

Proposal 12: Add FGs 15-3, 15-4, 15-5, 15-11, 15-23 as the prerequisite of FGs 32-5a.

Proposal 13: Add FGs 15-3, 15-4, and 15-11 as the prerequisite of FG 32-5b-1.

Regarding FG 32-5b-2 Scheme 2 Rx feature, if FG 32-2 is defined, it is possible that Scheme 2 Rx UE may only be capable of S-SSB and PSFCH, but not capable of any type of sensing. Then FG 32-2 should be the requisite of FG 32-5b-2 instead of Rel-16 FG 15-3. Although FG 32-2 includes synchronization, it only includes the Rx related component. For inter UE coordination Scheme 2, it is also important that Scheme 2 Rx UE can be syncRef UE so that Scheme 2 Tx UE can synchronize with it. Therefore, full synchronization FG 15-4 should be the prerequisite of FG 32-5b-2.

If FG 32-2 is not agreed, the Rel-16 FG 15-11 should be included as the prerequisite of FG 32-5b-2.

Proposal 14: Add at least one of FG 32-2 (if specified) or FG 15-11 as the prerequisite of FG 32-5b-2.

The updated FGs 32-5a and 32-5b-1/32-5b-2 are included in Table 7 and Table 8, respectively, of APP-3.

For inter-UE coordination scheme 2, PSFCH format 0 transmission and reception are supported for reporting and receiving the conflict report as coordination information, respectively. Since a non-destination of a TB from UE-B can be UE-A, UE-A needs to transmit more PSFCHs than a Rel-16 UE for HARQ-ACK purpose only. Also, since UE-B needs to receive a PSFCH for the conflict report without transmitting a PSSCH, UE-B needs to receive more PSFCHs than a Rel-16 UE for HARQ-ACK only. Therefore, the number of simultaneous PSFCH receptions and/or transmissions shall be increased. For simplicity on specification support, it would be preferred to report them separately for inter-UE coordination and HARQ-ACK. However, in RAN1#107bis-e, companies preferred to jointly report of the PSFCH transmission and reception. In this case, we propose to set a minimum number of PSFCHs for Scheme 2 conflict indication, i.e., the joint report in FG 32-5b-1 for PSFCH Tx and in FG 32-5b-3 for PSFCH Rx should be greater than corresponding PSFCH HARQ-ACK Tx in FG 15-11 and PSFCH HARQ-ACK Rx in FG 32-2 (if specified) or FG 15-11. The minimum is then dependent on the granularity of the number of simultaneous PSFCH transmissions and receptions.

Proposal 15: Support additional PSFCH simultaneous transmission and reception for inter-UE coordination scheme 2 and report them separately from that for HARQ-ACK.

Proposal 16: If joint report of PSFCH simultaneous transmission and reception for coordination Scheme 2 and for HARQ-ACK is supported, the number of simultaneous PSFCH transmission and reception supported in Scheme 2 FGs should be greater than the one reported in FGs for HARQ-ACK.

- If UE reports more than one FGs of 15-11, and 32-5b-1, the reported value M in FG 32-5b-2, is the total number, which should be greater than the one in FG 15-11.

- If UE reports more than one FGs of 15-11, 32-2 (if specified), and 32-5b-2, the reported value N for simultaneous PSCFH receptions in FG 32-5b-2, is the total number, which should be greater than the one in FG 15-11 or FG 32-2.

In RAN1#107bis-e, we have the following agreement on the container of inter-UE coordination information.

Agreement The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

Since 2nd stage SCI is supported as the container for inter-UE coordination in Scheme 1, and 2nd stage SCI is UE Rx optional, a new FG should be specified to support the reception of 2nd stage SCI for Scheme 1 inter-UE coordination information. FG 32-6 was introduced in RAN1#107bis-e for this capability, which is then supported.

In addition, UE who support this FG should support inter-UE coordination Scheme 1. Therefore, FG 32-5a should be the prerequisite of this new FG.

Proposal 17: Support a new FG, i.e., FG 32-6, for reception of Scheme 1 inter-UE coordination information over 2nd stage SCI.

- FG 32-5a for inter-UE coordination Scheme 1 should be prerequisite of FG 32-6.

In RAN1#107bis-e, we have the following agreement on the container for the explicit request transmissions.

Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

Similar to the case for coordination information transmissions, the 2nd stage SCI is supported as the container for transmission of the explicit request, which is UE Rx optional. A Rel-17 FG should be specified to include such capability of reception of 2nd stage SCI for the explicit request. As shown in above agreement, a single format SCI 2-C is used for inter-UE coordination information and request. Therefore, the capabilities of reception of the coordination information and the explicit request should be supported in one FG.

Proposal 18: Support for reception of an explicit request and coordination information in Scheme 1 inter-UE coordination over 2nd SCI should be included in the same FG, i.e., FG 32-6.

In RAN1#107-e, we have the following working assumption on the additional criteria for Condition 2-A-1 of inter-UE coordination Scheme 2.

Working Assumption A resource pool level (pre-)configuration can enable one of the following options: - Option 1: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI: - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI: - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively - Option 4: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource. - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s). - Support of Option 4 is subject to UE capability - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

If this working assumption is confirmed, option 4, the SIR based criteria is supported. However, the support of option 4 is subject to UE capability as UE-A needs to perform two RSRP measurements and find the difference between these two measurements, which is new behavior on UE SL sensing. A new FG is needed to specify this feature.

Proposal 19: Support a new FG, i.e., FG 32-7, for determination of expected conflict in Scheme 2 based on RSRP difference.

# Conclusion

The new UE features for Rel-17 sidelink enhancement were discussed. We observe and propose the following:

Proposal 1: Support adding an FG for synchronization sources for NR sidelink transmission FG 32-4b

- Include components of Rel-16 FG 15-4 as components of FG 32-4b (except the sync to the syncRef UE) including the component of transmitting S-SSB.

Proposal 2: The prerequisite of Rel-17 sync FG 32-4b is at least one of 15-1, 15-2, 15-3, 32-4, 32-4a.

Proposal 3: Prefer not to include the FG 32-2.

- If FG32-2 is agreed, do not support further split the capabilities for PSFCH and S-SSB receptions as different FGs

Proposal 4: The components of FG 32-2 (if it is specified) include component 8 of 15-1 and Rx related components of FGs 15-4 and 15-11.

Proposal 5: Include congestion control components as in FG 15-5 to FG 32-4a RRS with necessary updated based on Rel-17 agreements.

Proposal 6: For FG 32-4a RRS, in the column of “Consequence if the feature is not supported by the UE”, the description should be updated as

- UE does not support transmission according to the Rel-17 random resource selection and resource allocation

Proposal 7: For the note on Random selection in FG 32-4a RRS,

- Alt. 1 confirm the note on random selection according to Rel-16 in the exceptional pool if there no issue in the upper layer procedure.

- Alt. 2 remove “according to Rel-16” if the exception pool in Rel-16 is supported in Rel-17.

Proposal 8: Add FG 32-4a, FG 15-1, and FG 15-5 as the prerequisite of FG 32-4 partial sensing.

Proposal 9: Add the following note to FGs 32-4 and 32-4a

- If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B.

Proposal 10: Do not support further split of FG 32-5a unless there is an agreement on some UE feature which is subject to UE capability.

Proposal 11: Support inheriting the prerequisite FGs of a Rel-16 FG as the requisite of a Rel-17 FG if the Rel-16 FG is listed as the prerequisite of the Rel-17 FG.

Proposal 12: Add FGs 15-3, 15-4, 15-5, 15-11, 15-23 as the prerequisite of FGs 32-5a.

Proposal 13: Add FGs 15-3, 15-4, and 15-11 as the prerequisite of FG 32-5b-1.

Proposal 14: Add at least one of FG 32-2 (if specified) or FG 15-11 as the prerequisite of FG 32-5b-2.

Proposal 15: Support additional PSFCH simultaneous transmission and reception for inter-UE coordination scheme 2 and report them separately from that for HARQ-ACK.

Proposal 16: If joint report of PSFCH simultaneous transmission and reception for coordination Scheme 2 and for HARQ-ACK is supported, the number of simultaneous PSFCH transmission and reception supported in Scheme 2 FGs should be greater than the one reported in FGs for HARQ-ACK.

- If UE reports more than one FGs of 15-11, and 32-5b-1, the reported value M in FG 32-5b-2, is the total number, which should be greater than the one in FG 15-11.

- If UE reports more than one FGs of 15-11, 32-2 (if specified), and 32-5b-2, the reported value N for simultaneous PSCFH receptions in FG 32-5b-2, is the total number, which should be greater than the one in FG 15-11 or FG 32-2.

Proposal 17: Support a new FG, i.e., FG 32-6, for reception of Scheme 1 inter-UE coordination information over 2nd stage SCI.

- FG 32-5a for inter-UE coordination Scheme 1 should be prerequisite of FG 32-6.

Proposal 18: Support for reception of an explicit request and coordination information in Scheme 1 inter-UE coordination over 2nd SCI should be included in the same FG, i.e., FG 32-6.

Proposal 19: Support a new FG, i.e., FG 32-7, for determination of expected conflict in Scheme 2 based on RSRP difference.

# References

RP-193257, “NR sidelink enhancement”, RAN#86, Dec. 09-12, 2019.

RP-201385, “WID revision: NR sidelink enhancement”, RAN#88-e, June 29-July 3, 2020.

RP-202846, “WID revision: NR sidelink enhancement”, RAN#90-e, Dec. 7-11, 2020.

R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, AT&T, NTT DOCOMO Inc., RAN#106-e, Aug. 2021.

R1-2108426, “Updated RAN1 UE features list for Rel-16 NR after RAN1#106-e”, AT&T, NTT DOCOMO Inc., RAN#106-e, Aug. 2021.

R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, AT&T, NTT DOCOMO Inc.

R1-2108835, “UE features for NR sidelink enhancement”, Futurewei, RAN#106bis-e, Oct. 2021.

R1-2110888, “UE features for NR sidelink enhancement”, Futurewei, RAN#107-e, Nov. 2021.

R1-2200023, “UE features for NR sidelink enhancement”, Futurewei, RAN#107bis-e, Jan. 2022.

# Appendix

## Agreements in RAN1

The agreements and the working assumption from RAN1#106bis-e are listed below.

Working assumption

- Necessary Rel-16 FGs are handled, via pre-requisites of Rel-17 FGs for sidelink enhancements

  - Necessary components in Rel-16 FGs are added to Rel-17 FG components for sidelink enhancements if an entire Rel-16 FG cannot be included as pre-requisites

  - FFS whether/how to address the case where pre-requisites or additional components cannot address the issue for FG 15-x supported by Rel-17 UEs

Agreement

- FG 32-4 is updated as follows

  - FFS whether capability for synchronization as components or prerequisite or another FG

  - FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a

| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 24) UE can perform periodic-based partial sensing and resource allocation operation. 35) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 32-4a is updated as follows

  - FFS whether capability for synchronization as components or prerequisite or another FG

  - FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a

| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 32-5b is split into multiple FGs; One for Tx capability and the other for Rx capability as follows

| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. FFS whether/how to split FG 32-5b into multiple FGs | 32-5b-2, TBD | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

Conclusion

- There is no consensus in RAN1 on “FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs”

  - Remove the FFS in FG 32-2, highlight FG 32-2

Conclusion

- UE without NR SL reception is supported in Rel-17

## Original Proposed FG Table for Rel-17 SL Enhancement in [4]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by NR Uu or preconfiguration. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [32-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [32-1], [32-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [32-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |

Table 1 Initial proposed UE features for Rel-17 sidelink enhancement in [4].

## Updated FG Table for Rel-17 SL Enhancement in [6]  and Proposed updates based on latest agreements.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b | Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. FFS whether/how to split FG 32-5b into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Table 2 Updated UE features for Rel-17 sidelink enhancement in [6].

| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit NR sidelink based on the synchronization to an eNB. 6) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 7) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 8) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a FFS whether other components will be included | At least one of 15-1,15-2, 15-3, 32-4, 32-4a | Yes | No |  | Per band | N.A. | N.A. | N.A. | This FG can only be reported as a pre-requisite to FG 32-4a Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 3 Proposed FG 32-4b for synchronization sources for NR sidelink transmission.

| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. 2) UE can receive using the subcarrier spacing and CP length defined for a given band in RAN4 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 7) UE can receive up to N PSFCH(s) resources in a slot. FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 2 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 4/5/6 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-2 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-2 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-2 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4 Proposed updated FG 32-2 for rceiving NR sidelink of PSFCH/S-SSB.

| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu 8) UE can adjust its radio parameters based on pre-configured CBR value and CRlimit (FG 15-5 component 2 with update) 9) UE can process CR within the time it indicates (FG 15-5 component 3 with update) FFS whether any other components should be added | [TBD] None | [Yes] | [No] | UE does not support transmission according to the Rel-17 random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 5 Proposed updated FG 32-4a for random resource selection.

| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 24) UE can perform periodic-based partial sensing and resource allocation operation. 35) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] 32-4a, 15-1, 15-5 | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 6 Proposed updated FG 32-4 for partial sensing.

| 32. NR_SL_enh | 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] 15-3, 15-4, 15-5, 15-11, 15-23 | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 7 Proposed updated FG 32-5a for inter UE coordination Scheme 1.

| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. FFS whether/how to split FG 32-5b into multiple FGs | 32-5b-2, TBD15-3, 15-4, 15-11, | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] at least one of 32-2 or 15-11 | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

Table 8 Proposed updated FGs 32-5b-1 and 32-5b-2 for inter UE coordination Scheme 2.

## Latest updated FG Table for Rel-16 NR SL V2X in [5]

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-1 | Receiving NR sidelink | 1) UE can receive NR PSCCH/PSSCH. Up to a total of A sidelink HARQ processes across all links are supported. 2) UE can receive X PSCCH in a slot. 3) UE can attempt to decode Y= NRB non-overlapping RBs per slot 4) UE supports reception of PSSCH according to the 64QAM MCS table 5) UE supports PT-RS reception in FR2. 8) UE can receive using the subcarrier spacing and CP length defined for a given band in RAN4 10) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 12) UE can receive using 30 kHz subcarrier spacing with normal CP in FR1, 120 kHz subcarrier spacing with normal CP FR2 | None | Yes | Yes |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: NRB is the number of RBs defined per channel bandwidth by RAN4 in 38.101-1 Table 5.3.2-1 for FR1 and 38.101-2 Table 5.3.2.-1 for FR2 Note: Component 8 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 12 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-1 candidate value set: {16, 24, 32, 48, 64} Component-2 candidate value set: {floor (NRB /10 RBs), 2*floor (NRB /10 RBs)} Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-8 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to 8 configured grants can be configured for a UE. Up to C sidelink HARQ processes are supported including those for configured grants 2) UE can transmit PSSCH according to the normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2 on the same carrier as sidelink. 6) UE can transmit using the subcarrier spacing and CP length it reports. 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 9) Support downlink pathloss based open loop power control 11) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1 |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Candidate values for C are {8,16} Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: For Component 6, if a band is not indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1, the reported numerology shall be the same for sidelink and uplink. Component (9) is only required to be supported in a band not indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E.1-1 In a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1, the UE supports at least 30 kHz with normal CP in FR1, and at least 120 kHz with normal CP in FR2 | Optional with capability signalling For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. |
| 15-3 | Transmitting NR sidelink mode 2 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform mode 2 sensing and resource allocation operations 6) UE can transmit using the subcarrier spacing and CP length it reports for FG 15-1 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu | 15-1 | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 This is the basic FG for NR sidelink Candidate values for B are {8,16} Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 4 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 6 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates | 15-1 and at least one of 15-2 and 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for NR sidelink Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-6 | Short-term time-scale TDM for in-device coexistence | - Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 UE supports LTE V2X sidelink in the band combination | No | No | When LTE V2X sidelink operates in the same band, UE supports TDM for in-device coexistence only when resource pool of NR sidelink does not overlap with resource pool of LTE sidelink in time domain. UE does not support subframe boundary alignment for in-device coexistence when the bands in the band combination are different | per band combination | N.A. | N.A. | N.A. |  | Optional without capability signalling |
| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu by DCI format 3_1 for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3 | UE supports LTE V2X sidelink | Yes | No |  | Per band | N.A. | N.A. | N.A. | Component-2 candidate value set: {0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms } | Optional with capability signalling |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation | UE supports LTE V2X sidelink | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH according to the 256QAM MCS table | At least one of 15-2, 15-3 | Yes | Yes | UE does not support transmission according to the 256QAM MCS table | Per band | N.A. | N.A. | N.A. | Note: RAN4 to decide support for 256QAM transmission in an FR | Optional with capability signalling |
| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive up to N PSFCH(s) resources in a slot. 3) UE can transmit up to M PSFCH(s) resources in a slot | At least one of 15-1, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} Candidate values for M are {4, 8, 16} | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-12 | Low-spectral efficiency 64QAM MCS table | 1) UE can transmit and receive PSSCH according to the low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2, 15-3 | Yes | Yes | UE does not support transmission/reception according to the low spectral-efficiency 64QAM MCS table | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-14 | Sidelink CSI report | 1) UE can transmit and receive sidelink CSI-RS with up to P antenna port(s). 2) UE supports RI and CQI feedback on sidelink. | 15-1 and at least one of 15-2 and 15-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Note: Component 1 candidate values are P = {1,2} Note: When P=1, UE reports RI=1 Note: P=2 is optional | Mandatory with capability signalling for UEs supporting NR sidelink |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (in different bands) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | 15-14 with P=2 | No | No | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. | RAN1 does not see a need for the gNB to know if the feature is supported but would like to leave final decision to RAN2 | Optional without capability signalling |
| 15-19 | Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | 15-1 | No | Yes | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. | RAN1 does not see a need for the gNB to know if the feature is supported but would like to leave final decision to RAN2 | Optional with capability signalling |
| 15-22 | Support of fewer than 14 consecutive sidelink symbols in a slot | - UE additionally supports transmission/reception of SL slot configured with 7, 8, 9, 10, 11, 12, 13 consecutive symbols and all the corresponding DMRS patterns | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports SL only in a SL slot configured with 14 consecutive symbols. | Per band | N.A. | N/A | N.A. |  | Optional with capability signalling |
| 15-23 | Support of open loop SL power control and RSRP report | - Support sidelink pathloss based open loop power control and RSRP report in case of unicast | 15-1 and at least one of 15-2 and 15-3 | Yes | Yes |  | Per band | N.A. | N/A | N.A. | This is the basic FG for NR sidelink | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| 15-24 | Simultaneous reception of downlink and sidelink | UE supports simultaneous reception of NR downlink and NR sidelink in a band combination for which the UE indicated simultaneous sidelink and downlink support in a band combination. | 15-1 | Yes | No |  | Per feature set | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 | FG 15-2 | Yes | No |  | Per FS | N.A. | N.A. | N.A. | If the UE indicates support for FG 15-2 in a band indicated with only the PC5 interface in Table 5.2E.1-1 of 38.101-1, the UE must indicate that FG 15-25 is supported for a band combination with that band. | Optional with capability signalling |

Table 9 Rel-16 UE sidelink features.