3GPP TSG RAN WG1 #108	R1-2201027

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	SPS PDSCH activation and PUCCH resource selection for the 1st SPS PDSCH

Document for:		Discussion and Decision

# Introduction

During RAN1#107 and RAN1#107bis the question of whether the 1st SPS PDSCH transmitted in response of the SPS PDSCH activation should be considered as an SPS-PDSCH (PDSCH without a corresponding PDCCH), or a “regular” dynamic-grant-scheduled PDSCH that has a corresponding PDCCH [4, 5, 6].

From the PDSCH reception perspective the question is merely philosophical as there is no difference to that, but it makes a difference to how the HARQ-ACK corresponding to that 1st PDSCH after the SPS activation is sent.

- Does the UE transmit the HARQ-ACK on a PUCCH resource configured for the SPS-PDSCH by the RRC in n1PUCCH-AN of the SPS-Config, or

- Does the UE transmit the HARQ-ACK on a PUCCH resource indicted by the SPS-PDSCH activation DCI’s PUCCH Resource Indicator as if the PDSCH was a ”regular” dynamically granted PDSCH?

# Discussion

## Perceived TS 38.213 ambiguity

The TS 38.213 definition was said to leave some ambiguity on what the correct interpretation should be, when it refers to an  “SPS-PDSCH as PDSCH reception without a corresponding PDCCH“, which could be interpreted so that the activation DCI is on a PDCCH corresponding to the 1st SPS-PDSCH.

Further the TS 38.213 subclause 9.2.3 has the following statement:

For a SPS PDSCH reception ending in slot , the UE transmits the PUCCH in slot where is provided by the PDSCH-to-HARQ_feedback timing indicator field in DCI format 1_0 or, if present, in DCI format 1_1 activating the SPS PDSCH reception. If the UE detects a DCI format 1_1 that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in slot , the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot where is provided by dl-DataToUL-ACK.

This can be read so that the activation DCI is used to derive the PUCCH related information. The reading is correct, but it is important to recognize that the PDSCH-to-HARQ-ACK_feedback timing indicator field in the activation DCI is applied after each SPS-PDSCH to determine the time location of the HARQ-ACK of the PDSCH. That is, this field in the activation DCI is stored and used for every instance of the SPS-PDSCH, it is not something applied to the 1st SPS-PDSCH only. Thus it bears no relation to the PUCCH resource selection question at hand.

The PUCCH resource selection part of the subclause 9.2.3 defines the PUCCH resource selection for PDSCH reception without a corresponding PDCCH, i.e. for SPS-PDSCH, but it does not explicitly say whether the 1st PDSCH after the activation DCI is considered as a PDSCH that has a corresponding PDCCH, or not.

If a UE transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

Observation 1: This part of TS 38.213 is not clear whether the 1st PDSCH after receiving the activation DCI should be considered as having a corresponding PDCCH, or not having a corresponding PDCCH, and thus whether the PUCCH resource n1PUCCH-AN in the SPS-Config, or the PUCCH resource indicated by the activation DCI should be used to convey the HARQ-ACK of the 1st SPS-PDSCH.

In Rel-16, the SPS operation has been further extended to multiple SPS configurations and using SPS-PUCCH-AN-List. From the Rel-16 description using SPS-PUCCH-AN-List it seems to be more clear that all SPS PDSCH receptions are to utilize the configured SPS PUCCH resources using SPS-PUCCH-AN-List and not the ones configured in PUCCH-ResourceSet:

If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN. …. If the UE is provided SPS-PUCCH-AN-List and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions and SR, if any, the UE determines a PUCCH resource to be - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the first entry in sps-PUCCH-AN-List if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the second entry in sps-PUCCH-AN-List, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize obtained from the second entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the third entry in sps-PUCCH-AN-List, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize obtained from the third entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or - a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the fourth entry in sps-PUCCH-AN-List, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706.

Observation 2: TS 38.213 in Rel-16 is clear that any SPS PDSCH (including the 1st SPS PDSCH after receiving the activation DCI) is to utilize the configured SPS resource list SPS-PUCCH-AN-List.

## MAC definition

The MAC specification defines the reception of the DCI scheduling a PDSCH as well as triggering SPS-PDSCH reception in subclause 5.3.1 of TS38.321. The critical difference in the two procedures is that with SPS-PDSCH there is no indication of DL assignment delivered to the higher layers, i.e. the activation DCI does not “schedule” the first SPS-PDSCH, but it initializes the SPS reception and the 1st SPS-PDSCH is already processed as any other SPS-PDSCH.

Selected parts of TS38.321 subclause 5.3.1 with the dynamic grant and SPS-PDSCH activation part side-by-side

| ### 5.3.1 DL Assignment reception Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information. When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell: |  |
| --- | --- |
| < Reception of a “regular” dynamic grant DCI > | < Reception of SPS-activation DCI > |
| 1> if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI: 2> if this is the first downlink assignment for this Temporary C-RNTI: 3> consider the NDI to have been toggled. 2> if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment: 3> consider the NDI to have been toggled regardless of the value of the NDI. 2> indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity. | 1> else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI: … 2> if the NDI in the received HARQ information is 0: … 3> else if PDCCH content indicates SPS activation: 4> store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment; 4> initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1; |
| For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall: 1> if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell: 2> instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity; 2> set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration; 2> consider the NDI bit for the corresponding HARQ process to have been toggled; 2> indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity. |  |

Observation 3: According to the MAC specification 5.3.1, the SPS-activation DCI does not lead to indication of “the presence of a downlink assignment” there is no related delivery of the associated   HARQ information to the HARQ entity. In other words, the SPS-activation DCI does not schedule the 1st SPS-PDSCH, it just activates the SPS-PDSCH operation.

Observation 4: According to the MAC specification subclause 5.3.1, the SPS-PDSCH reception after receiving the activation DCI follows the SPS-PDSCH configuration.

Proposal 1: based on the above two observations, conclude that the 1st SPS-PDSCH after receiving the activation DCI is considered as SPS-PDSCH, and the PUCCH handling follows the SPS-Config. The PUCCH-related fields in the SPS-PDSCH activation DCI are ignored.

Proposal 2: Agree to the following clarification to TS 38.213 v15.14.0 and v16.8.0. A corresponding draft CR to Rel-15 is provided in [7]:

Rel-15: If a UE transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN. A PDCCH carrying a DL SPS activation is not considered to correspond to any of the SPS PDSCHs. Rel-16: If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN. A PDCCH carrying a DL SPS activation is not considered to correspond to any of the SPS PDSCHs.

# Conclusion

Observation 1: This part of TS 38.213 is not clear whether the 1st PDSCH after receiving the activation DCI should be considered as having a corresponding PDCCH, or not having a corresponding PDCCH, and thus whether the PUCCH resource n1PUCCH-AN in the SPS-Config, or the PUCCH resource indicated by the activation DCI should be used to convey the HARQ-ACK of the 1st SPS-PDSCH.

Observation 2: TS 38.213 in Rel-16 is clear that any SPS PDSCH (including the 1st SPS PDSCH after receiving the activation DCI) is to utilize the configured SPS resource list SPS-PUCCH-AN-List.

Observation 3: According to the MAC specification 5.3.1, the SPS-activation DCI does not lead to indication of “the presence of a downlink assignment” there is no related delivery of the associated   HARQ information to the HARQ entity. In other words, the SPS-activation DCI does not schedule the 1st SPS-PDSCH, it just activates the SPS-PDSCH operation.

Observation 4: According to the MAC specification subclause 5.3.1, the SPS-PDSCH reception after receiving the activation DCI follows the SPS-PDSCH configuration.

According to the current specifications, the 1st SPS-PDSCH after receiving the SPS activation DCI is considered as SPS-PDSCH, and the related HARQ-ACK transmission follows the SPS-config. However, RAN1 specifications leave this somewhat up for interpretation and a clarification might be beneficial.

Proposal 1: based on the above two observations, conclude that the 1st SPS-PDSCH after receiving the activation DCI is considered as SPS-PDSCH, and the PUCCH handling follows the SPS-Config. The PUCCH-related fields in the SPS-PDSCH activation DCI are ignored.

Proposal 2: Agree to the following clarification to TS 38.213 v15.14.0 and v16.8.0. A corresponding draft CR to Rel-15 is provided in [7]:

Rel-15: If a UE transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN. A PDCCH carrying a DL SPS activation is not considered to correspond to any of the SPS PDSCHs. Rel-16: If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN. A PDCCH carrying a DL SPS activation is not considered to correspond to any of the SPS PDSCHs.

# References

- 3GPP TS 38.213 NR; Physical layer procedures for control

- 3GPP TS 38.321 NR; Medium Access Control (MAC) protocol specification, V15.12.0

- 3GPP TS 38.331 NR; Radio Resource Control (RRC) protocol specification, V15.13.0

- R1-2112700		FL summary #3 of PUCCH coverage enhancement, Moderator (Qualcomm)

- R1-2210774		FL summary #3 of PUCCH coverage enhancement, Moderator (Qualcomm)

- R1-2200776		Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT, Moderator (Nokia)

- R1-2201028		Draft 38.213 CR on SPS PDSCH activation and PUCCH resource selection for the 1st SPS PDSCH, Nokia, Nokia Shanghai Bell

# Annex A : SPS-config in TS38.331 V15.13.0

SPS-Config ::= 		SEQUENCE {

periodicity 			ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes 	INTEGER (1..8),

n1PUCCH-AN 				PUCCH-ResourceId 														OPTIONAL, -- Need M

mcs-Table 				ENUMERATED {qam64LowSE} 											OPTIONAL, -- Need S

...

}

n1PUCCH-AN

HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

# Annex B : SPS in TS38.321 V15.12.0

### 5.3.1	DL Assignment reception

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE:	CURRENT_slot refers to the slot index of the first transmission occasion of a bundle of configured downlink assignment.

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

[ clauses not copied ]

### 5.8.1	Downlink

Semi-Persistent Scheduling (SPS) is configured by RRC for a Serving Cell per BWP. Activation and deactivation of the DL SPS are independent among the Serving Cells.

For the DL SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

RRC configures the following parameters when SPS is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	nrofHARQ-Processes: the number of configured HARQ processes for SPS;

-	periodicity: periodicity of configured downlink assignment for SPS.

When SPS is released by upper layers, all the corresponding configurations shall be released.

After a downlink assignment is configured for SPS, the MAC entity shall consider sequentially that the Nth downlink assignment occurs in the slot for which:

(numberOfSlotsPerFrame × SFN + slot number in the frame) =
[(numberOfSlotsPerFrame × SFNstart time + slotstart time) + N × periodicity × numberOfSlotsPerFrame / 10] modulo (1024 × numberOfSlotsPerFrame)

where SFNstart time and slotstart time are the SFN and slot, respectively, of the first transmission of PDSCH where the configured downlink assignment was (re-)initialised.