3GPP TSG RAN WG1 #108-e			R1-2201034

e-Meeting, February 21th – March 3rd, 2022

Agenda Item:	8.2.3

Source:	InterDigital, Inc.

Title:	Remaining issues for enhanced PUCCH formats 0/1/4

Document for:	Discussion and Decision

# Introduction

In RAN1#107bis-e [1], the following agreements on enhancements for PUCCH formats 0/1/4 were made:

Conclusion - For DMRS of enhanced (multi-RB) PF4, do not support Type-1 low PAPR sequences if pi2BPSK is configured for the PUCCH resource. - No change to 38.211 Section 6.4.1.3.3.1 is needed

In this contribution, we discuss remaining issues and associated standards impacts on PUCCH enhancements in 52.6 – 71GHz.

# Discussions

Potential coverage imbalance issues

Some companies proposed handling of a potential coverage imbalance issues for PUCCH formats 2 and 3. While number of RBs of PUCCH formats 2 and 3 varies dynamically based on payload, number of RBs for multi-RB PUCCH format 4 does not vary and payload is limited to a maximum of 115 bits. Therefore, if the PUCCH payload is larger than 115 bits and configured to use PUCCH formats 2 and 3, coverage may not be identical to PUCCH format 4 due to an actual number of RBs possibly less than a configured value. Following proposals were provided to resolve the potential coverage imbalance issues.

- Redesign the number of RBs for PUCCH format 4

  - One solution to resolve the power imbalance issue is to revisit number of RBs for PUCCH format 4 (e.g., supporting dynamic variation based on the payload). If number of RBs for PUCCH format 4 is revisited, all related aspects (e.g., rate matching) should be revisited as well. Given the limited time, it is preferred to keep the current design.

- Fix the number of RBs for PUCCH formats 2 and 3

  - Another proposal is to fix the number of RBs for PUCCH formats 2 and 3. However, as clearly identified in WID [2], this WI only discusses enhancement for PUCCH formats 0, 1 and 4 not PUCCH formats 2 and 3. Given Rel-17 is in maintenance stage, it is preferred to discuss the enhancement in the future releases if the enhancement for PUCCH formats 2 and 3 is needed.

In addition, although the coverage for PUCCH formats 2 and 3 may not be optimized, we believe that such issues can be handled by gNB configuration with the flexibility given by the current specification.

Observation 1: Redesigning the number of RBs for PUCCH format 4 requires additional discussion of all related aspects should be revisited as well.

Observation 2: Enhancement on PUCCH formats 2 and 3 is out of scope for NR 52-71.

Observation 3: Potential coverage imbalance issue can be handled by gNB configuration with the flexibility given by the current specification.

Proposal 1: It is preferred to keep the current design for PUCCH formats in NR 52-71.

# Summary

In this contribution, we discussed remaining issues for PUCCH enhancements of NR in 52.6 – 71 GHz for PUCCH formats 0/1/4. From the discussions, we made the following observations and proposal:

Observation 1: Redesigning the number of RBs for PUCCH format 4 requires additional discussion of all related aspects should be revisited as well.

Observation 2: Enhancement on PUCCH formats 2 and 3 is out of scope for NR 52-71.

Observation 3: Potential coverage imbalance issue can be handled by gNB configuration with the flexibility given by the current specification.

Proposal 1: It is preferred to keep the current design for PUCCH formats in NR 52-71.

# References

- “Final Report of 3GPP TSG RAN WG1 #107bis-e”, 3GPP RAN1 Meeting #107bis-e, e-Meeting, January 17th – 25th, 2022,

- RP-212637, “Revised WID:  Extending current NR operation to 71 GHz”.