3GPP TSG RAN WG1 #108-e			R1-2201036

e-Meeting, February 21th – March 3rd, 2022

Agenda Item:	8.2.4

Source:	Moderator (InterDigital, Inc.)

Title:	Discussion Summary #1 for Beam Management for new SCSs

Document for:	Discussion and Decision

# Introduction

In this contribution, we summarize all issues discussed on beam management and timings associated with beam-based operation for new SCSs to support NR from 52.6 GHz to 71 GHz in RAN#108-e.

# Multiple QCL Assumptions for Multiple PDSCHs/PUSCHs

## Multiple QCL assumptions based on timeDurationForQCL

### Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [Huawei/HiSi, 1] | Proposal 2: Support the following TP in clause 5.1.5 of TS 38.214 (Proposal 2.1-e in [2]) < Unchanged parts are omitted > When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH. < Unchanged parts are omitted >. |
| ###### [FUTUREWEI, 2] | Proposal 1: Adopt Proposal 2.1-e to address the issue of activation of the set of TCI states across PDSCH with offsets greater than or equal to timeDurationForQCL in single DCI multi-PDSCH scheduling. If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH. |
| ###### [InterDigital, 3] | Observation 6: As multi-PDSCH transmission is a general transmission method in FR2-2 and application time of MAC CE based TCI state activation, it is desired to have a flexible indication timing while the application could be done when multi-PDSCH transmission is finalized. Proposal 3: Support TP#2 for 38.214. Text proposal 2 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***. |
| ###### [vivo, 4] | - Updating activated TCI states or unified beam within a span for multi-PDSCH scheduling is allowed. - Adopt the following TP in TS 38.214. If the a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the slot with the each scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH.. |
| ###### [ZTE/Sanechips, 7] | Proposal 1: For multi-PDSCH scheduling by a single DCI, adopt either of TP#1-1 or TP#1-2 in TS 38.214. - the indicated TCI state should be based on the activated TCI states in the first slot with all the scheduled PDSCHs. - same gNB restriction on MAC-CE updates as for multi-slot PDSCH scheduling should be applied. ### TP#1-1 for TS 38.214 ============================== Start of TP#1-1 for TS 38.214 =============================== 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted *** ============================== End of TP#1-1 for TS 38.214 =============================== ### TP#1-2 for TS 38.214 ============================== Start of TP#1-2 for TS 38.214 =============================== 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted *** ============================== End of TP#1-2 for TS 38.214 =============================== |
| ###### [NTT DOCOMO, 8] | Proposal 2: When the UE is configured with a multi-slot PDSCH or the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH (i.e., to adapt the following TP). -------------------------- Start of Text Proposal for TS 38.214 -------------------------- <Unchanged parts omitted> ### 5.1.5 Antenna ports quasi co-location … If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and the corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. … <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 --------------------------. |
| ###### [Nokia/NSB, 10] | Proposal 1: support the indicated TCI states should be based on the activated TCI states in the slot with the scheduled PDSCH. Adopt the following CR. ================================= Start of the text ptoposal ================================= 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH . *** Unchanged text omitted *** ================================= End of the text proposal ================================= |
| ###### [Intel, 11] | Proposal 4: When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCHs. |
| ###### [Ericsson, 12] | Adopt TP#1 which captures the QCL assumption for each PDSCH with offset ≥ timeDurationForQCL when multi-PDSCH scheduling with a single DCI is configured and the TCI field in DCI is present. Reason for changes For multi-PDSCH scheduling, prior agreements on the QCL assumption for each PDSCH with offset ≥ timeDurationForQCL are not fully captured for the case when tci-PresentInDCI is configured. Summary of changes Captures QCL assumption for each PDSCH with offset ≥ timeDurationForQCL when tci-PresentInDCI is configured Specs/Sections impacted 38.214 Section 5.1.5 Consequences if not approved Incomplete specification of UE behavior. ----------------------------------------- Text Proposal (TP#1) for 38.214, Section 5.1.5 ---------------------------------------- *** Unchanged text omitted *** If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted *** ----------------------------------------------------------- End Text Proposal ----------------------------------------------------------- |
| ###### [Apple, 13] | Proposal 1: Reuse the Rel-16 TCI determination rule defined for multi-slot PDSCH - Not delete the sentence of ‘and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH’ for multi-slot PDSCH scheduling for FR2-2. |
| ###### [Xiaomi, 14] | TP#1 for TS 38.214 Clause 5.1.5 ============================= Unchanged part omitted ============================ Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH(s) is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', - the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s), and the time offset between the reception of the DL DCI and a PDSCH is less than the threshold timeDurationForQCL, the same QCL assumption is applied to this scheduled PDSCHs. - If a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets, - the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same value of coresetPoolIndex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s), and the time offset between the reception of the DL DCI and a PDSCH is less than the threshold timeDurationForQCL, the same QCL information is applied to this scheduled PDSCH. - If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. If the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s), in which case the UE shall not expect to be configured with higher layer parameter repetitionNumber, and the time offset between the reception of the DL DCI and a PDSCH is less than the threshold timeDurationForQCL, the same QCL parameters are applied to this PDSCH. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). ============================= Unchanged part omitted ============================ TP#2 for TS 38.214 Clause 5.1.5 ============================= Unchanged part omitted ============================ If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s), and the time offset between the reception of the DL DCI and a PDSCH is equal to or greater than a threshold timeDurationForQCL, if applicable, the QCL assumption used for the PDCCH transmission is applied to this PDSCH. If the PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s), and the time offset between the reception of the DL DCI and a PDSCH is equal to or greater than a threshold timeDurationForQCL, if applicable, the indicated TCI state applies to this PDSCH. ============================= Unchanged part omitted ============================ TP#3 for TS 38.214 Clause 5.1.2.3 ============================= Unchanged part omitted ============================ For a UE configured by the higher layer parameter repetitionScheme set to 'fdmSchemeA' or 'fdmSchemeB', and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', - If is determined as "wideband", the first nPRB2 PRBs are assigned to the first TCI state and the remaining nPRB2 PRBs are assigned to the second TCI state, where nPRB is the total number of allocated PRBs for the UE. - If is determined as one of the values among {2, 4}, even PRGs within the allocated frequency domain resources are assigned to the first TCI state and odd PRGs within the allocated frequency domain resources are assigned to the second TCI state. - If the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s) over multiple slots, the mapping between TCI state and the PRBs is the same in each slots. - The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. ============================= Unchanged part omitted ============================ TP#4 for TS 38.214 Clause 5.1.2.1 ============================= Unchanged part omitted ============================ When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA' and indicated DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and the DCI schedules one or more PDSCH(s) over multiple slots, the same mapping method between TCI state and the PDSCH transmission occasion is applied in each slots. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. The UE expects the PDSCH mapping type indicated by DCI field 'Time domain resource assignment' to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. ============================= Unchanged part omitted ============================ TP#5 for TS 38.214 Clause 9.1.4 ============================= Unchanged part omitted ============================ When a UE is configured by the higher layer parameter repetitionNumber in PDSCH-TimeDomainResourceAllocation, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, in which case UE shall not expect to be configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)'. ============================= Unchanged part omitted ============================ |
| ###### [Samsung, 15] | Proposal 2: QCL assumption of multi-PDSCH scheduling for FR 2-2 can be treated as that of a multi PDSCH (Proposal 2.1-b) - Adopt TP#1 for TS 38.214. ## TP#1 for TS 38.214 ============================== Start of TP #1 for TS 38.214 ================================== 5.1.5 Antenna ports quasi co-location =============================== Unchanged Text Omitted =================================== If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH or the slot including the first scheduled PDSCH, respectively, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a the corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. =============================== Unchanged Text Omitted =================================== ============================== End of TP #1 for TS 38.214 ================================== |
| ###### [MediaTek, 16] | Proposal 2: When the UE is configured with multi-PDSCH scheduling, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCHs. |
| ###### [LGE, 18] | Proposal #3: The following two alternative text proposals in TS 38.214 Section 5.1.5 can be considered for updating activated TCI states and Alt 1 is preferred to reduce UE’s implementation complexity. - Alt 1: When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot corresponding to with the first scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. - Alt 2: When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH.. |

### Summary of views

In RAN1#107bis-e, RAN1 reached a consensus to support TCI state activation for multi-PDSCH scheduled by a single DCI based on activated TCI states for a firstly scheduled PDSCH. However, there were different views on whether to keep “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH”. Given the situation, RAN1 endorsed the following text proposal only for editorial issues.

The TP below for section 5.1.5 of TS38.214v17.0.0 is endorsed

============================== Start of TP for TS 38.214 ===============================

5.1.5 Antenna ports quasi co-location

*** Unchanged text omitted ***

When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a the corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

If the PDCCH carrying the scheduling DCI is received on one component carrier, and a the PDSCH scheduled by that DCI is on another component carrier:

-    The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-    When the UE is configured with enableDefaultBeamForCCS, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, or if the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

*** Unchanged text omitted ***

============================== End of TP for TS 38.214 ===============================

The companies are expected to continue the discussion on whether to include “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH” or not and the following is a summary of the companies’ views.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | TPs for TCI state activation for multi-PDSCH | TP#1: ZTE/Sanechips, NTT Docomo, Apple, Xiaomi, MediaTek, LGE (with modification) 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***. TP#2: Huawei/HiSi, FUTUREWEI, Nokia/NSB, Intel, Ericsson, LGE < Unchanged parts are omitted > When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH. < Unchanged parts are omitted >. |

### [Closed] 1st round discussion

#### Observation 2.1-a

According to the summary, 7 companies are supporting the text proposal #1 with the existing specification for a multi-shot PDSCH in Rel-16. On the other hand, 8 companies prefer to introduce a specification support without “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH”. As there’s no clear majority view, the moderator suggests to continue the discussion.

| Company | Input |
| --- | --- |
| OPPO | From our understanding, RAN1 does not have any agreement to allow gNB to change the activated TCI state in the middle of the multi-PDSCH scheduling. If this understanding is correct, the R16 baseline should be reused. Moreover, we don’t see a clear motivation to revert this baseline. This is because either TP#1 or TP#2 will give a same result. The only difference is TP#2 implies that a MAC-CE configuring activated TCI state change in middle of multi-PDSCH scheduling is allowed but the activation time will be delayed. However, this enhancement does not have a clear benefit. Thus, it should be further clarified what the benefit is over the baseline? At this moment, we think TP#1 follows well the baseline concept and we support TP#1. |
| Intel | As we understood the discussion last meeting, it converged to the UE assumption that the indicated TCI states are based on the activated TCI states for the first PDSCH from multi-slot PDSCH transmission. Therefore, we support TP#2 which additionally includes that the same activated TCI states are expected by the UE across slots with multi-PDSCH transmission. To us, the latter wording better describes UE behaviour. Otherwise, if the activated TCI states could be not the same, what whould be the expected UE behaviour? |
| LG Electronics | We prefer TP#1 but can be acceptable to TP#2 if majority supports it. @ Intel, For both TP#1 and TP#2, UE determines indicated TCI state based on the first PDSCH. The only difference is that for TP#1, gNB cannot signal MAC-CE such that activated TCI states are changed in the middle of multiple PDSCHs scheduled by a DCI, while for TP#2 gNB can do. |
| Samsung | Our preference is TP#1. We think that if TP#2 is supported, it may increase the UE complexity. Since the UE may need to remember deactivated TCI state after MAC-CE activation applied, and may require unnecessary beam change due to the different codepoint according to MAC-CE activation. |
| vivo | As we discussed in our contribution [4], unified beam feature has been inteoduced in Rel-17 MIMO. We think that needs to be discussed here whether UE can be scheduled multi-PDSCH by a single DCI together with unified beam behavior. In other words, whether unified beam validation are the same across the slots with the scheduled PDSCHs should be discussed. In our view, the span of multi-PDSCH scheduling, which is defined from the beginning of the first scheduled PDSCH to the end of the last scheduled PDSCH can be much longer than that of a multi-slot PDSCH, which is defined from the beginning of the first PDSCH repetition to the end of the last PDSCH repetition, due to that multi-PDSCH scheduling supports multiple PDSCHs scheduled in consecutive or non-consecutive slots. If the activated TCI states informed by MAC CE or the validation timing of unified beam would be prohibited to appears within a long span for multi-PDSCH scheduling, it is too restrictive for gNB scheduling. In our view, updating activated TCI states or unified beam within a span of multi-PDSCH scheduling should be allowed. Now if we have to follow the rule of multi-slot scheduling, the following two cases (multi-CC scheduling or unified beam) need to be discussed: - In case of multi-CA scheduling, simultaneousTCI-UpdateList indicates TCI States Activation/Deactivation for UE-specific PDSCH MAC CE applied for these CCs simultaneously. An example of single slot PDSCH scheduling on CC1 and multi-PDSCH scheduling on CC2 is illustrated in Figure 1. Figure 1. multi-CC scheduling with simultaneousTCI-UpdateList If QCL assumption for PDSCH2~4 on CC2 follows activated TCI sates in the slot of PDSCH1 but the indicated TCI states of single-slot PDSCH scheduled on CC1 follows new activated TCI states informed by MAC CE (reusing the rule of Rel-16), then there is confusion that which beam for simultaneous reception for two CCs. In that sense, “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH” as in TP#1 is needed if following the rule of multi-slot scheduling to avoid the case illustrated in Figure 1. - In case of unified beam as llustrated in Figure 2, when the UE is configured with unified beam feature and the unified beam is updated from the second slot, which is within the span corresponding to the single DCI. Note that unified beam applies to other channels or RSs including SRS, PUSCH, PDCCH and CSI-RS for the UE allocated in the span. Assuming multi-slot PDSCH rule about QCL assumption would be adopted where the indicated TCI state is based on the activated TCI states in the first slot where the first scheduled PDSCH locates, we need to discuss whether other channels and RSs starting after the first slot follows the old or new unified beam. If the QCL assumptions for all of other channels and RSs starting after the first slot than the multiple PDSCHs scheduled by the DCI would follow the old unified beam used for the first scheduled PDSCH, the rule of unified beam introduced in Rel-17 that unified beam is determined only by DL DCI format and applied from a determined time point uniformly would be violated. In contrast, if the QCL assumptions for all of other channels and RSs starting after the first slot than the multiple PDSCHs scheduled by the DCI would follow the new unified beam, the UE would be required to apply two different unified beams at the same time, one of which is for the multiple PDSCHs scheduled by the DCI, and the other is for other channels and RSs than the multiple PDSHs, again the rule of unified beam in Rel-17 would be violated. Note that TP#1 with or without the sentence “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH” does not solve the issue illustrated in Figure 2. Figure 2 unified beam behaviour In summary, adopting multi-slot PDSCH rule for multi-PDSCH scheduling will conflict with the rule of activated TCI states applied in multi-CCs case in Rel-16 and/or unified beam rule in Rel-17. On the contrary, by treating 'each PDSCH' of multiple PDSCHs scheduled by a single DCI as an independent single slot PDSCH, the legacy rule in the above two cases can be applied directly without any more effort for discussion. |
| Nokia/NSB | We don’t think this issue is controversial. We don’t prefer TP#1 because the last sentence is only applicable for PDSCH repetition across slots. We think “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH” is assuming PDCCH repetition. For multi-PDSCH, no PDSCH is spanning multiple slots. So, if companies to support TP#1 want to clarify this UE assumption, we can update TP#2 as follows. Once agreed with the first PDSCH as the reference, there is no way to indicate different active TCI state by single DCI. When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH, UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCHs |
| Apple | We still prefer TP#1 to keep the same TCI state rule as what is defined for Rel-16 multi-slot PDSCH to avoid unnecessary complexity of multi-PDSCH scheduling. On the combination of Rel-17 BM features with multi-PDSCH scheduling operation, we do not see clear show stopper. Instead, assuming both featues are kept without modification, the FIG1/FIG2 from vivo can be avoided by deferring the first PDSCH# after the time instance of Activation MAC CE (FIG1) or renewed unified beam (FIG2). In our view, these examples exactly demonstate the necessity to avoid TCI state/Pools update in the middle of multi-PDSCH transmission. |
| ZTE, Sanechips | We think there is no difference between multi-PDSCH scheduling and multi-slot PDSCH scheduling in terms of the activated TCI state restriction, and thus no reason to abandon the Rel-16 rule to restrict the gNB behavior on updating the activated TCI states in Rel-17 FR2-2. We prefer TP#1 to define the same activated TCI states restriction for above two scheduling types. But we are also open to TP#2 if majority companies support it. Another difference between TP#1 and TP#2 is that TP#2 limits "...and not configured with a multi-slot PDSCH". We don’t understand why does TP#2 have this restriction while TP#1 does not? |
| Intel | We support the text proposed by Nokia/NSB. |
| Qualcomm | Support TP1. Also, we think this issue is non-critical, i.e. gNB implementation can fully avoid such corner case. |
| MediaTek | We support TP1. As pointed out by many companies, we fail to see why the restriction of same activated TCI states across the scheduling can only apply to multi-slot PDSCH. The only difference is multi-PDSCHs can across more slots but why would gNB schedule multiple PDSCHs across many slots and change activated TCI states in the middle? If gNB prefer to change the activated TCI states in the middle of scheduled PDSCHs with large gaps, wouldn’t it be more reasonable to use different DCIs for PDSCH scheduling before and after the change of activated TCI states? Moreover, what if the updated activated TCI states doesn’t included the TCI state used to receive the multi-PDSCHs? What will be the UE behavior? |
| DOCOMO | We prefer TP #1, but TP #2 is also fine. |
| Ericsson | We support TP#2. The sentence "UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH" artificially restricts gNB behavior with no impact on UE behavior since the UE still assumes the TCI states that are active in the first slot. We don't think activating new TCI states during the time span of a multi-PDSCH scheduling affects UE complexity in any significant way, especially since the UE doesn't uses these states until later. We also agree with vivo: In our view, updating activated TCI states or unified beam within a span of multi-PDSCH scheduling should be allowed. Do companies who support Rel-17 "unified" TCI state indication also propose to disallow updates of the unifed beam during the time span of a multi-PDSCH scheduling? So far there is nothing in the spec that would disallow this. We think there should be some consistency for multi-PDSCH, meaning TCI states can be activated during the time span of a multi-PDSCH scheduling both for "conventional" TCI state indication, and Rel-17 "unified" TCI state indication. |
| FW | While we agree that TP#1 is not broken, our preference is TP #2. As we noted in our contribution, the span of multi-PDSCH can be significant and disallowing update of activated TCI states within this span seems unnecessarily restrictive. |
| Xiaomi | Support TP#1. It is because the best beam between UE and gNB is assumed to be the same from the beginning of the first scheduled PDSCH to the end of the last scheduled PDSCH that single TCI state/SRI for the multi-PDSCHs is supported. Then, changing the activated TCI states within the span of multi-PDSCHs is not necessary. |
| CATT | Support TP#1. This give more flexibility to gNB scheduling. |

#### 1st round discussion summary

As shown in the summary below, 10 companies supported TP#1 while 9 companies supported TP#2. The reason that the companies support TP#1 is that they failed to observe any reason to change Rel-16 restriction. On the other hand, the companies supporting TP#2 argued that the span of multi-PDSCH scheduling, which is defined from the beginning of the first scheduled PDSCH to the end of the last scheduled PDSCH can be much longer than that of a multi-slot PDSCH. If the activated TCI states informed by MAC CE or the validation timing of unified beam would be prohibited to appears within a long span for multi-PDSCH scheduling, it is too restrictive for gNB scheduling.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | TPs for TCI state activation for multi-PDSCH | TP#1: ZTE/Sanechips, NTT Docomo, Apple, Xiaomi, MediaTek, LGE (with modification), OPPO, Samsung, Qualcomm (10 companies) 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***. TP#2: Huawei/HiSi, FUTUREWEI, Nokia/NSB, Intel, Ericsson, LGE, vivo (9 companies) Acceptable: LGE, Docomo < Unchanged parts are omitted > When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH. < Unchanged parts are omitted >. TP#3 (by Nokia/NSB): Nokia/NSB, Intel < Unchanged parts are omitted > When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH, UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCHs < Unchanged parts are omitted >. |

### [Closed] 2nd round discussion

As companies’ views are almost evenly divided, the moderator believes that we need more email discussion and GTW session, if possible, on whether to adopt TP#1 or TP#2. Companies which didn’t express their views are requested to express their views as soon as possible. In addition, based on the inputs from vivo and Samsung, Proposal 2.1-c is proposed for Rel-17 unified TCI framework. Please share your views on this proposal.

#### Proposal 2.1-a (TP#1 from 1st round discussion)

5.1.5 Antenna ports quasi co-location

*** Unchanged text omitted ***

When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

#### Proposal 2.1-b (TP#2 from 1st round discussion)

5.1.5 Antenna ports quasi co-location

*** Unchanged text omitted ***

When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH.

*** Unchanged text omitted ***

| Company | Input |
| --- | --- |
| Huawei, HiSilicon | Our first preference is Proposal 2.1-b. However, we can be flexible and accept Proposal 2.1-a as, essentially, in both proposals the UE behavior is the same (at least with respect to the used/indicated TCI states for Rel-17 multi-PDSCH). Proposal 2.1-a unnecessarily restricts gNB not to update the activated set of TCI states during multi-PDSCH span. |
| Qualcomm | Support 2.1-a. Perfer to reuse legacy rule/implementation. Also, the issue is non-critical and can be avoided by gNB. Suggest not to spend much time on this issue in maintenance. |
| CATT | We prefer proposal 2.1-b for its flexibility. |
| vivo | As we stated before, we actually prefer to treat each PDSCH in a multi-PDSCH scheduling as an independent single slot PDSCH. Although adopting multi-slot PDSCH rule for multi-PDSCH scheduling is not our first preference, for the sake of progress, we can accept the majority view and live with either proposal 2.1-b or 2.1-a. |
| Samsung | Our preference is Proposal 2.1-a for reusing existing rules. However, for the sake of progress, if majority of companies prefer Proposal 2.1-b, we can live with that. |
| Apple | We support proposal 2.1-a to reuse the Rel-16 multi-slot PDSCH rule. On the other hand, if proposal 2.1-c below would be agreed, we can also consider proposa 2.1-b as anyway the active TCI state pool maybe updated in the middle of span if Rel-17 unified TCI framework is enabled and UE has to implement TCI update in the middle of multi-PDSCH scheduling. |
| Ericsson | Please see our comments below regarding Proposal 2.1-c and how it relates to Proposal 2.1-b. |
| DOCOMO | Either Proposal 2.1-a or Proposal 2.1-b is fine for us. We understand one of the the motivations of Proposal 2.1-b is consistency on whether TCI states can be updated within multi-PDSCH span for "conventional" TCI state indication, and Rel-17 "unified" TCI state indication. If Proposal 2.1-c is adopted (i.e. updating activated TCI states or unified beam within a span for multi-PDSCH scheduling is allowed for Rel-17 unified TCI framework), then Proposal 2.1-b should be adopted. Proposal 2.1-a is also fine to us, i.e. keeping same rule for multi-slot PDSCH and multi-PDSCH, as pointed out by Qualcomm that gNB can avoid such updating case. |
| LG Electronics | Our first preference is Proposal 2.1-a considering UE implementation complexity. But we can accept Proposal 2.1-b as a compromise. |
| ZTE, Sanechips | We tend to support proposal 2.1-a. Besides, we are a little confused about why “......and not configured with a multi-slot PDSCH” is not limited in proposal 2.1-a. In our view, to avoid multi-PUSCH and multi-slot PUSCH are configured simultaneously, such limitation should also be added in the proposal 2.1-a |
| Lenovo | We slightly prefer proposal 2.1-a, also fine with proposal 2.1-b |
| Nokia/NSB | Support proposal 2-1b. We have concern on the yellow highlighted part is not aligned with multi-PDSCH scheduling. . When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. |
| Intel | We still prefer TP#3, i.e., Proposal 2.1-b with the following additional text “UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCHs.” |
| Moderator | Proposal 2.1-a: ZTE/Sanechips, Apple, Xiaomi, MediaTek, LGE, OPPO, Samsung, Qualcomm, Lenovo/MotM (11 companies) Acceptable: Huawei/HiSi, vivo, DOCOMO Proposal 2.1-b: Huawei/HiSi, FUTUREWEI, Nokia/NSB, Ericsson, LGE, CATT (8 companies) Acceptable: LGE, Docomo, vivo, Samsung, DOCOMO, Lenovo/MotM TP#3: Intel < Unchanged parts are omitted > When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher payer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot corresponding to the first scheduled PDSCH, UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCHs < Unchanged parts are omitted >. |
| Samsung | Thank you for updated summary. We are fine with Proposal 2.1-a and Proposal 2.1-c with one clarification for Proposal 2.1-a We think another “(s)” should be added for consistency as follow: #### Proposal 2.1-a (TP#1 from 1st round discussion) 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci- PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted *** |
| Ericsson | We support the conclusion in Proposal 2.1-c. We still don’t understand why companies are okay with unified TCI framework to update the TCI states during the time span of a multi-PDSCH scheduling, but it’s not okay to do so for multi-PDSCH using Rel-16 TCI framework; this seems quite inconsistent. But anyway, for the sake of progress we can accept the direction of Proposal 2.1-a. However, the wording of Proposal 2.1a seems more tailored to multi-slot PDSCH. How about the following which uses the correct wording from Proposal 2.1-b but adds back the sentence about the UE expecting the activated TCI states to remain the same across slots? In this way legacy Rel-16 sentence for multi-slot PDSCH remains unchanged, and the behavior for multi-PDSCH scheduling is properly written. If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. |
| LGE | Regarding Steve’s modification, we are OK with that direction. But additional change seems necessary because UE have a common behavior regardless of whether multi-slot PDSCH and/or multi-PDSCH scheduling is configured. If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. |
| Futurewei | The conclusion in Proposal 2.1-c is fine by us. We also share the concern expressed by Ericsson on the inconsistency between 2.1-a and 2.1-c. That said, the modified version of 2.1.b proposed by Ericsson below which enforces the activated TCI states to be the same across the slots corresponding to the scheduled PDSCHs, is acceptable to us. |
| Moderator | @All Please check the updated Proposal 2.1-a from Ericsson and LGE and express your concern as soon as possible only if it’s serious. #### Updated Proposal 2.1-a (from Ericsson and LGE) If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. |
| Intel | The updated Proposal 2.1-a from Ericsson and LGE is fine with Intel. |
| Nokia/NSB | Regarding to Rel-17 unified TCI framework, the basic framework of TCI activation has consistency with Rel-15/16. Only way to defining TCI states and indication of active TCI states are enhanced. The proposed text doesn’t say about these aspects, so there is no confliction on this. Regarding to the new added text, I am even fine with the current text, prefer slight update below. (yellow highlight) The deleted part is related to repetition scheme. Also, it is better to clarify that same for all PDSCHs by a DL DCI. Also, I am still unclear the reason to delete the part “and not configured with a multi-slot PDSCH”. Because there is a specification to explain the restriction of configuring both schemes, but good to have it for clarity. But, we are still fine with this for the progress. . When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI. |
| Xiaomi | Thanks for your good effort. To make a progress, we can be flexible to support that updating activated TCI states within a span for multi-PDSCH scheduling is allowed or not allowed. However, the behavior should be consistent for Rel-15/16 TCI framework and Rel-17 TCI framework. |
| LGE | We are fine with Nokia’s modification and let me clarify the reason of removing “and not configured with a multi-slot PDSCH”. According to agreements so far, we can have three types of configurations for multi-slot PDSCH and multi-PDSCH scheduling. - Type 1: Multi-slot PDSCH is configured and multi-PDSCH scheduling is not configured - Type 2: Multi-slot PDSCH is not configured and multi-PDSCH scheduling is configured - Type 3: Both of multi-slot PDSCH and multi-PDSCH scheduling are configured In original Proposal 2.1-a, the corresponding sentence was applicable to type 2 UE but not to type 3 UE. However, in updated Proposal 2.1-a, by adding “and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI”, the sentence can be applied to type 3 UE as well for multi-PDSCH scheduling DCI, since UE shall expect the same activated TCI states across slots or PDSCHs irrespective of whether multi-slot PDSCH or multi-PDSCH scheduling is configured. |
| vivo | Thanks for the discussion. We have some clarification questions to the updated proposal 2.1-a. 1. Does “the first scheduled PDSCH” mean the first scheduled PDSCH by a single DCI regardless or the first scheduled PDSCH whose time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL? 2. In “and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI.”, does “the scheduled PDSCHs by a DL DCI” mean all the scheduled PDSCHs by a single DCI or only scheduled PDSCHs whose time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL? Our understanding is that we are talking about only scheduled PDSCH with offset >= timeDurationForQCL in all the discussion through multiple meetings, but the above wording seems indicating regardless of timeDurationForQCL. |
| Moderator | This is my understanding on your questions. - Yes. - All the scheduled PDSCHs by a single DCI. Our understanding is that we are talking about only scheduled PDSCH with offset >= timeDurationForQCL in all the discussion through multiple meetings, but the above wording seems indicating regardless of timeDurationForQCL. - This discussion has been triggered from the CR discussion in RAN1#107bis-e. Please note that this is to discuss which TCI states are activated in the scheduled multi-PDSCHs no which TCI states are applied in the scheduled multi-PDSCHs. |
| vivo | It says “the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI”. Based on your reply, that means the activated TCI states for all PDSCHs scheduled by a single DCI should be the same even for those scheduled PDSCH(s) whose time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than a threshold timeDurationForQCL. On your comment “Please note that this is to discuss which TCI states are activated in the scheduled multi-PDSCHs no which TCI states are applied in the scheduled multi-PDSCHs”. You seems indicating the applied TCI states for PDSCHs could be different from the activated TCI states, could you please clarify? |
| Moderator | Rel-15/16 DCI based TCI state indication mechanism is using 2 steps. - MAC CE based TCI state activation/deactivation (e.g., TCI state 1, 2, 3 and 4) - DCI based TCI state indication among the activated TCI states (e.g., TCI state 2 among the activated TCI states) The issue is to discuss whether 1 can be different among the scheduled PDSCHs and the proposal is to make the activated TCI states same based on the first PDSCH. The applied TCI state for each PDSCH can be different based on timeDurationForQCL. In short, my answer to your question “You seems indicating the applied TCI states for PDSCHs could be different from the activated TCI states, could you please clarify?” is the applied TCI states can be different for each PDSCH, but the applied TCI states should be part of the activated TCI states. |
| Huawei/HiSi | First, we are a bit confused with your following answer to Huaming : In short, my answer to your question “You seems indicating the applied TCI states for PDSCHs could be different from the activated TCI states, could you please clarify?” is the applied TCI states can be different for each PDSCH, but the applied TCI states should be part of the activated TCI states.. In the context of Rel-15/16 (which is at least the original concern of 2.1-a and 2.1-b), our understanding is that, for all PDSCHs of a multi-PDSCH whose offsets are larger than timeDurationForQCL, the applied TCI state (or a pair of applied TCI states in the case of multi-TRP) is the same. It is the TCI state (or a pair of TCI states in the case of multi-TRP) indicated by the 'Transmission Configuration Indication' field in the scheduling DCI from the set of TCI states that are active in the slot that carries the first PDSCH with offset larger than timeDurationForQCL. Second, Regarding 2.1-a, 2.1-b, we fail to see a strong technical reason to keep optimizing the TP. Either of 2.1-a or 2.1-b would adequately work in our view As such, we can support only one of the following: - First choice: Original Proposal 2.1-b in Section 2.1.4.2 of FL summary. - Second choice: Original Proposal 2.1-a in Section 2.1.4.1 of FL summary. - Third choice: Updated Proposal 2.1-a (from Ericsson and LGE) provided earlier in this Email thread (although we fail to see the difference of the Updated Proposal 2.1-a and the Original Proposal 2.1-a in Section 2.1.4.1 with respect to the UE behaviour or even gNB behaviour. Third, Regarding Conclusion in Section 2.1.4.3, given a lot of discussions around the conclusion and the fact that this conclusion does not have a spec impact, we do not see any reason to support it at this time. |
| LGE | Thanks for the discussion. Following the discussion, I tend to agree with the moderator. Let me take an example: - A DL DCI schedules 4 PDSCHs at slot #n/n+1/n+2/n+3. - Slot #n/n+1 are within timeDurationForQCL while slot #n+2/n+3 are outside of timeDurationForQCL. In this case, - Q1: Is “the first scheduled PDSCH” in Proposal 2.1-a slot #n or slot #n+2? - Q2: Are “scheduled PDSCHs by a DL DCI” in Proposal 2.1-a slot #n/n+1/n+2/n+3 or slot #n+2/n+3? What I have thought is, - Q1: Slot #n - Q2: Slot #n/n+1/n+2/n+3 But it seems that companies have different understanding… |
| Ericsson | Thank-you Seonwook for constructing this example for discussion: Let me take an example: - A DL DCI schedules 4 PDSCHs at slot #n/n+1/n+2/n+3. - Slot #n/n+1 are within timeDurationForQCL while slot #n+2/n+3 are outside of timeDurationForQCL. In this case, - Q1: Is “the first scheduled PDSCH” in Proposal 2.1-a slot #n or slot #n+2? - Q2: Are “scheduled PDSCHs by a DL DCI” in Proposal 2.1-a slot #n/n+1/n+2/n+3 or slot #n+2/n+3? What I have thought is, - Q1: Slot #n - Q2: Slot #n/n+1/n+2/n+3 I thought we were all on the same page on this previously, and clearly if there is not common understanding on this fundamental point, it is going to be hard to agree on a TP. Our understanding all along is aligned with Seonwook’s: Q1: Slot #n Q2: Slot #n/n+1/n+2/n+3 One last point (editorial) about the following change suggested by Nokia: “and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI” I think the following is more accurate: “and the UE shall expect the activated TCI state(s) are the same for all PDSCHs scheduled by the same DL DCI” |
| Moderator | @Keyvan First: Yes, if all PDSCHs are after timeDurationForQCL, then same TCI states should applied to all PDSCHs. However, I never mentioned that all PDSCHs are after timeDurationForQCL. Therefore, if there’s any PDSCHs before timeDurationForQCL, the applied TCI states should be different. In addition, the applied TCI states can be different based on whether tci-PresentInDCI as well. That’s why I said that the applied TCI states can be different for each PDSCH. Second: I am just trying to find a compromise that we can reach a consensus. I am fine with any text proposal if the TP works properly and we can reach a consensus. Third: I also think that it may be early to agree on something in this meeting regarding Rel-17 unified TCI framework. Anyway, let’s continue the discussion. @Seonwook, Steve I can’t understand why this is still an issue. Clearly, my answer to the questions is Q1: Slot #n Q2: Slot #n/n+1/n+2/n+3. @All I am not sure that we can reach any consensus before the end of this meeting, but, let’s continue the email discussion. I will try to update my FL summary soon. |

#### Proposal 2.1-c (For Rel-17 unified TCI framework)

Conclusion

For multi-PDSCH scheduling with Rel-17 unified TCI framework in FR2-2, updating activated TCI states or unified beam within a span for multi-PDSCH scheduling is allowed.

| Company | Input |
| --- | --- |
| Huawei, Hisilicon | We have some questions for clarification: First, we don’t understand what “For multi-PDSCH scheduling with Rel-17 unified TCI framework in FR2-2” means. Do we have a “multi-PDSCH scheduling without Rel-17 unified TCI framework in FR2-2”? Can we remove this from the proposal altogether? Moderator: Rel-17 unified TCI framework is totally different with Rel-15/16 beam indication procedure. Please check Samsung’s explanation in the below. Second, is the concern updating A) the activated TCI state by gNB within the span of multi-PDSCH or B) applying a new indicated TCI state by UE within the span of multi-PDSCH? Moderator: According to my understanding on the discussion so far, the majority of companies do not prefer B). Therefore, the concern is about A). If the intention is A), then it is being discussed in 2.1.4.1 and 2.1.4.2. If the intention is B) we have already agreed that all PDSCHs after timeDurationForQCL use the same indicated TCI state(s) that is based on the activated TCI states in the first slot with the scheduled PDSCH. Moderator: This is a new Rel-17 procedure based on BeamAppTime_r17 not timeDurationForQCL. Or is the intention is that the applied TCI state that is indicated using Rel-17 unified TCI mechanism can change within the multi-PDSCH span while, based on the earlier agreement, the applied TCI state that is indicated using Rel-16 TCI mechanism cannot change within the multi-PDSCH span? Moderator: The intention is that the activated TCI states which are indicated using Rel-17 MAC CE can change the activated TCI states or not within the multi-PDSCH span not about the indication. We undertand that this proposal is the outcome of the discussion of [Samsung, 15] in 2.2.3.1 but the proposal in that section does not not entirely clear for us either. |
| Qualcomm | For Proposal 2.1-c, we believe it is aligned with current spec, i.e. beam application time can happen between 2 PDSCHs scheduled by same DCI and new beam will be applied to PDSCH(s) after the application time. We can have conclusion, since no spec impact to our understanding. Otherwise, please share the TP. Moderator: I agree with you. I will make this as a conclusion. |
| vivo | Our understanding of this proposal is about whether Rel-17 unified TCI framework can be enabled together with multi-PDSCH scheduled by a single DCI, and if so, what’s the UE bahaviour. Moderator: As mentioned multiple times, my understanding is that no Rel-17 beam management is prohibited in FR2-2 based on the work scope. As we don’t have any agreement to prohibit unified TCI framework in FR2-2, in my understanding, Rel-17 unified TCI framework can be enabled together with multi-PDSCH scheduled by a single DCI. It’s not clear to us whether proposal 2.1-a or proposal 2.1-b (if either agreed) specifies UE bahaviour when Rel-17 TCI framework is configured. If it’s allowed to change TCI state or unified beam during the span of multi-PDSCH, we think a clear UE bahaviour should be specified. Moderator: Proposal 2.1-a or proposal 2.1-b is for Rel-15/16 TCI behavior and Proposal 2.1-c is for Rel-17 unified TCI framework. |
| Samsung | In Rel-17 TCI framework, MAC-CE and DCI can indicate Rel-17 TCI state. - For Rel-17 MAC-CE based beam indication and activation follows timeline of Rel-15/16 MAC-CE application. Our view is that MAC-CE based beam indication and activation for multi-PDSCH scheduling in Rel-17 TCI framework may depend on proposal 2.1-a or proposal 2.1-b. - For Rel-17 DCI based beam indication, TCI state is applied starting from the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH.Note that the first slot can be withn a span for multi-PDSCH scheduling. So our understanding is that the spec already allows to change TCI state within a span for multi-PDSCH scheduling. We believe that simple approach can be if both Rel-17 TCI framework and multi-PDSCH are configured, updating unified beam within a span is allowed. Based on the above discussion, we support the proposal. |
| Apple | As commented above, we are open to the FL proposal. |
| Ericsson | For multi-PDSCH scheduling, we think it makes sense to try to have unified behavior on beam indication/activation/application between Rel-17 unified TCI framework and Rel-16 TCI framework. So in that sense, we think Proposal 2.1-b is more closely aligned to Proposal 2.1-c. While we still support Proposal 2.1b, if we really can't achieve consensus between Proposal 2.1a and 2.1b, maybe it would be better to take a step back and revisit the agreement that is forcing us to find a solution to support a single TCI state for all PDSCHs with offset >= timeDurationForQCL. Thus, we would treat all PDSCHs with offset >= timeDurationForQCL as individual PDSCHs as suggested by vivo. Then, if the TCI states are updated within the time span of these PDSCHs, the UE applies them. This would seem to align with Rel-17 unifed TCI framework as well. Regarding Proposal 2.1-c, we are supportive of the general idea, but we think it would be better to discuss an actual TP to see what the exact spec impact is. Moderator: After checking the updated 38.214, I don’t think we need any actual specification change for Proposal 2.1-c. So, I will make it as a conclusion rather than an agreement. |
| DOCOMO | We are fine with the proposal2.1-c. We also share Qualcomm’s point. At this moment, proposal 2.1-c should be “proposed conclusion”, not “proposal”. We find no specification impact by the text. Moderator: After checking the updated 38.214, I don’t think we need any actual specification change for Proposal 2.1-c. So, I will make it as a conclusion rather than an agreement. |
| LG Electronics | We have the similar understanding on Rel-17 FeMIMO feature with Samsung. However, it should be first clarified in Rel-17 FeMIMO session how Rel-17 TCI framework operates with multi-slot PDSCH. In detail, Rel-17 TCI can be updated even in the middle of the span of multi-slot PDSCHs? Moderator: As mentioned multiple times, my understanding is that no Rel-17 beam management is prohibited in FR2-2 based on the work scope. As we don’t have any agreement to prohibit unified TCI framework in FR2-2, in my understanding, Rel-17 unified TCI framework can be enabled together with multi-PDSCH scheduled by a single DCI. Regarding the commonality, we think UE behavior for multi-slot PDSCH can be similar with that for multi-PDSCH scheduling. In that sense, Proposal 2.1-a/b can be the separate issue from Proposal 2.1-c. |
| ZTE, Sanechips | We think that we should focus on proposal 2.1-a/b first and discuss proposal 2.1-c. At present, we are open to proposal 2.1-c Moderator: As there’s not much time to discuss, I would like to discuss both issues simultaneously. |
| Lenovo | We are open to proposal 2.1-c, however, we agree with Ericsson to discuss an actual TP releated to this proposal. Moderator: After checking the updated 38.214, I don’t think we need any actual specification change for Proposal 2.1-c. So, I will make it as a conclusion rather than an agreement. |
| Nokia/NSB | Fine in principle. Agree with E///, we can have actual TP on this. Moderator: After checking the updated 38.214, I don’t think we need any actual specification change for Proposal 2.1-c. So, I will make it as a conclusion rather than an agreement. |
| Intel | Fine in principle and would like to look at the TP if the spec is impacted. Moderator: After checking the updated 38.214, I don’t think we need any actual specification change for Proposal 2.1-c. So, I will make it as a conclusion rather than an agreement. |
| Samsung | We are fine with Proposal 2.1-a and Proposal 2.1-c with one clarification for Proposal 2.1-a |
| Ericsson | We support the conclusion in Proposal 2.1-c. |
| LGE | As to Proposal 2.1-c, we have a question for better understanding. What is the UE behaviour for multi-slot PDSCH reception with Rel-17 unified TCI framework? Even in that case, is it allowed to update activated TCI states or unified beam within a span for multi-slot PDSCH? If YES, we are fine with the proposed conclusion. |
| Futurewei | The conclusion in Proposal 2.1-c is fine by us. We also share the concern expressed by Ericsson on the inconsistency between 2.1-a and 2.1-c. That said, the modified version of 2.1.b proposed by Ericsson below which enforces the activated TCI states to be the same across the slots corresponding to the scheduled PDSCHs, is acceptable to us. |
| Moderator | Actually, I provided my response in the updated FL summary. I copied my response in the below for your convenience. Moderator: As mentioned multiple times, my understanding is that no Rel-17 beam management is prohibited in FR2-2 based on the work scope. As we don’t have any agreement to prohibit unified TCI framework in FR2-2, in my understanding, Rel-17 unified TCI framework can be enabled together with multi-PDSCH scheduled by a single DCI. |
| LGE | I’m not against the combination of Rel-17 TCI framework with FR2-2. Rather, I’m asking Rel-17 feMIMO behavior especially if UE is allowed to update Rel-17 TCI state in the middle of a span of slot-aggregated PDSCH, which should have been clarified in Rel-17 FR1/2-1. If it is allowed, we are fine with Proposal 2.1-c which is in-line with slot-aggregated PDSCH with Rel-17 TCI framework. |
| CATT | Regarding the issue of Rel-17 beam management , it is true that there is no agreement to prohibit it, but that does not mean It is automatically supported. It has been the practice that support of any feature of another WI is only decided in a case by case manner. |
| Qualcomm | Also, suggest to replace unified beam to the indicated unified TCI. To our understanding, the indicated TCI is selected from one of the activated TCI(s). Conclusion For multi-PDSCH scheduling with Rel-17 unified TCI framework in FR2-2, updating activated TCI states or unified beam the indicated unified TCI within a span for multi-PDSCH scheduling is allowed. To Seonwook/Shupeng, no restriction to forbid update of the indicated unified TCI within slot-aggregated PDSCHs. So my understanding is that it is allowed. You can check the spec editor’s understanding. |
| Moderator | @Seonwook, Shupeng As clearly capture in the below, what we can discuss is whether particular feature should be excluded for 52.6-71GHz. If there’s no explicit proposal and corresponding agreement to exclude multi-PDSCH for Rel-17 beam management, there’s no restriction in my understanding. - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided @Yan My original intention is to handle MAC CE based TCI state activation issue for Rel-17 unified TCI framework, but I also agree that whether indication is allowed or not should be further discussed. @All Based on the discussions with Seonwook, Shupeng and Yan, I feel that we need further discussion for Proposal 2.1-c. |
| Nokia/NSB | Regarding to Rel-17 unified TCI framework, the basic framework of TCI activation has consistency with Rel-15/16. Only way to defining TCI states and indication of active TCI states are enhanced. The proposed text doesn’t say about these aspects, so there is no confliction on this. |

#### 2nd round discussion summary

For Proposal 2.1-a and Proposal 2.1-b, the moderator tried to approve Proposal 2.1-a as more companies are supporting Proposal 2.1-a than Proposal 2.1-b. However, some companies suggested wording updates, but other companies didn’t agree with the updates.

For Proposal 2.1-c, whether Rel-17 unified TCI framework is supported with multi-PDSCH scheduled by a single DCI. In the moderator’s understanding, if there’s no explicit proposal and corresponding agreement to exclude multi-PDSCH for Rel-17 beam management, there’s no restriction to support any features.

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

    - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

Given the situation, RAN1 failed to reach any consensus during 2nd round discussion. Please continue the discussion in 3rd round discussion.

### [Closed] 3rd round discussion

For TCI state activation issue with Rel-15/16 TCI states, the moderator excluded Proposal 2.1-b and provided Proposal 2.1-a, Proposal 2.1-d (proposed by Ericsson and updated by LGE), and Proposal 2.1-e (updated by Nokia/NSB).

For multi-PDSCH scheduled by a single DCI in Rel-17 unified TCI framework, the moderator provided some questions. Please provide your view.

#### Proposal 2.1-a (TP#1 from 1st round discussion)

5.1.5 Antenna ports quasi co-location

*** Unchanged text omitted ***

When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

#### Proposal 2.1-d (Suggested by Ericsson and updated by LGE)

*** Unchanged text omitted ***

When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

#### Proposal 2.1-e (Updated by Nokia/NSB)

*** Unchanged text omitted ***

When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

#### Proposal 2.1-f (Another update by Ericsson)

*** Unchanged text omitted ***

When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17] and not configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI state(s) in the slot corresponding to the first scheduled PDSCH, and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled for all PDSCHs scheduled by a the same DL DCI. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

| Company | Input |
| --- | --- |
| Ericsson | Comment #1 I'm starting to wonder if a safer route might be to agree on a TP at least for the case of Rel-15/16 beam management, i.e., for the case when the UE is not configured with tci-StateId_r17. This would allow more time for companies to check the implications when for the case of Rel-17 beam management, i.e., the case when the UE is configured with tci-StateId_r17. Please see Comment #3 below. Comment #2 At least for the case when the UE is not configured with tci-StateId_r17, we can accept Proposal #2.1-e, but we think it could be improved with the following editorial update: and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled for all PDSCHs scheduled by a the same DL DCI. Comment #3 We wanted to check for what is the common understanding on the following for the case when the UE is configured with with tci-StateId_r17., i.e., Rel-17 beam management. In the email discussion on the reflector, Seonwook provided the following example: Let me take an example: - A DL DCI schedules 4 PDSCHs at slot #n/n+1/n+2/n+3. - Slot #n/n+1 are within timeDurationForQCL while slot #n+2/n+3 are outside of timeDurationForQCL. and the following statement from Huaming was confirmed by several companies as common understanding, at least for Rel-15/16 beam management PDSCH#1 &2 in slot #n/+1 will use default beam and PDSCH#3 &4 in slot n+2/n+3 will use the indicated TCI state based on the activated TCI state list of PDSCH#1 in slot #n; and the activated TCI state list will not change between slot#n/n+1/n+2/n+3. While we’re establishing common understanding, I’d like to check how the rule “and the UE shall expect the activated TCI state(s) are the same across the slots corresponding to the scheduled PDSCHs by a DL DCI” might affect Rel-17 beam management for the case when the DL DCI schedules multi-PDSCH and BeamAppTime_r17 is such that the UE should apply a new unified TCI within the time span of the multiple scheduled PDSCHs. The most recent version of 38.214 Section 5.1.5 contains the following w.r.t. Rel-17 beam management When the UE would transmit the last symbol of a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication and without DL assignment, or corresponding to the PDSCH scheduling by the DCI carrying the TCI -State indication, and if the indicated TCI-State is different from the previously indicated one, the indicated [TCI-State] with [tci-StateId_r17] should be applied starting from the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH. Using Seonwook's example, let’s further assume that prior to the DL DCI scheduling the 4 PDSCHs, the UE received another DL DCI that indicates a new TCI state that is different than the previously indicated TCI state (i.e., unified TCI state is updated), and that “the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH” corresponding to this DCI is slot n+4 of Seonwook's example. Which behavior will apply for slots n+3 and n+4 if the UE is configured with tci-StateId_r17, i.e., REl-17 beam management? - Alt-1: In both slots n+3 and n+4 the UE applies the TCI state(s) that are active in slot #n and applies the new TCI state(s) that are indicated by the previous DCI to a later multi-PDSCH scheduling. - Alt-2: In slot n+3 the UE applies the TCI state(s) that are active in slot #n. In slot #n+4 the UE applies the new TCI state(s) that are indicated by the previous DCI. If there is not common understanding on the behavior, then it would be wise to restrict the agreed TP to the case when the UE is not configured with tci-StateId_r17. |
| Intel | Among the three proposals above, our first preference is Proposal 2.1-e and our second preference is Proposal 2.1-d. We also could accept the updated Proposal 2.1-e with modification provided by Ericsson in their Comment #2. |
| LG Electronics | We prefer updated Proposal 2.1-e with modification provided by Ericsson. |
| Samsung | We are generally fine with the direction of proposals However, we have difficulty to understand the difference between Proposal 2.1-d and 2.1-e. Although it is explained in the reflector that the intention of deleted part is related to repetition scheme, we didn’t quit get it. |
| vivo | As we stated on the reflector, if the common understanding of the group is the following UE behavior using Seonwook’e example, we are fine with the direction of these proposals. PDSCH#1 &2 in slot #n/+1 will use default beam and PDSCH#3 &4 in slot n+2/n+3 will use the indicated TCI state based on the activated TCI state list of PDSCH#1 in slot #n; and the activated TCI state list will not change between slot#n/n+1/n+2/n+3. Our preference is Proposal 2.1-f. On Ericsson’s question in their comment 3 regarding what’s the UE bahaviour when UE is configured with tci_StateId_r17, taking the example in Ericsson’s comment 3, our understanding is the following (Alt-2 in Ericsson’s comment 3): In slot n+3, the UE applies the indicated TCI state(s) that are based on the activated TCI state list in slot #n. In slot #n+4, the UE applies the new TCI state(s) that are indicated by the DCI without DL assignment. |
| Huawei, HiSilicon | After revieweing all the discussions on the reflector and here, our views regarding a few discussed issues are provided below: - Regarding Seonwook/Huaming following example: Example I Scenario: - A DL DCI schedules 4 PDSCHs at slot #n/n+1/n+2/n+3. - Slot #n/n+1 are within timeDurationForQCL while slot #n+2/n+3 are outside of timeDurationForQCL. Suggested behavior: For applied TCI state: PDSCH#1 &2 in slot #n/n+1 will use default beam and PDSCH#3 &4 in slot n+2/n+3 will use the indicated TCI state based on the activated TCI state list of PDSCH#1 in slot #n; For activated TCI state: and the activated TCI state list will not change between slot#n/n+1/n+2/n+3.. We agree with the above suggested behavior for both applied TCI state and activated TCI state at least for the scenario that TCI update/indication is provided using Rel-15/16 mechansim and Not using Rel-17 mechanism. Further, - We believe that the suggested behavior for applied TCI state at least for the scenario that TCI update/indication is provided using Rel-15/16 mechansim and Not using Rel-17 mechanism is already agreed in previous meetings and supporting any other behavior would require reverting previous agreements (which we don’t support such a revertion). - Regarding Steve concern about implication of tci-StateId_r17 configuration. Although we have sympathy with this concern, we do not think the possible implication of Rel-17 TCI indication should be discussed in the TP that is agreed in this Section because: - The original TP 2.1-a and TP 2.1-b and the updated TP 2.1-d, TP 2.1-e, and TP 2.1-f aim to address the activated TCI state behavior for multi-PDSCH based on Rel-15/16 mechansim. They don’t aim to address the applied TCI state. We think it is better to keep the scope of the TP in this section only limited to its original intention of addressing the activated TCI state behavior based on Rel-15/16 mechansim without mentioning anything about Rel-17 mechansim. - The paragraph that TP 2.1-a, TP 2.1-d, TP 2.1-e, and TP 2.1-f are concerned about also addresses the activated TCI state behavior for multi-slot PDSCH. However, to our understanding, the activated TCI state behavior for multi-slot PDSCH has not changed based on Rel-17 mechansim (this paragraph has not changed in Rel-17 spec). At this point, we don’t see a strong reason to modify this paragraph by bringing up the Rel-17 TCI indication mechanism for multi-PDSCH. - Having said that above, depending on the outcome of the discussion in Section 2.1.5.5, we are open to discuss further TP to address the possible implication of Rel-17 TCI indication mechanism on this paragraph. - Regarding the supported TP among TP 2.1-a, TP 2.1-d, TP 2.1-e, and TP 2.1-f: We are still struggling to understand what is the technical difference between any two of the above TPs. To us, the are exactly the same. All TPs support exactly the same suggested behavior for the applied and indicated TCI state in Example I. We wonder if proponent companies of TP 2.1-d, TP 2.1-e, and TP 2.1-f can clearly explain to us how their proposed TPs support a different behavior regarding the activated or indicated TCI states than in TP 2.1-a. As such, we support TP 2.1-a which is the simplest TP. As a second choice, we can also accept TP 2.1-d which, to us, is only a wordier version of TP 2.1-a. |
| Qualcomm | Support Proposal 2.1-e. |
| Ericsson | Support Proposal 2.1-d and 2.1-f. |
| CATT | With the email discussion we think “not configured with [tci-StateId_r17]” is needed since we would prefer the TP not have restriction impact on the TCI state update during the multi-pdsch scheduling. |
| Intel | Support either Proposal 2.1-e or Proposal 2.1-f |
| Xiaomi | We prefer proposal 2.1-a and 2.1-d. We agree with Huawei that this is about the activated TCI state, not the applied TCI state. Well, we also have concern about the problem rised by Ericsson and we are open to further discuss it, but not here. The original purpose is to decide whether updating activated TCI states within a span for multi-PDSCH scheduling is allowed or not allowed. |
| ZTE, Sanechips | Support Proposal 2.1-e. |

#### Discussion for multi-PDSCHs with Rel-17 unified TCI framework

Q1. In the moderator’s view, if there’s no explicit proposal and corresponding agreement to exclude multi-PDSCH for Rel-17 beam management, there’s no restriction to support any features.

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

    - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

Do you agree with the moderator’s view?

| Company | Input |
| --- | --- |
| Intel | Agree with moderator’s view. |
| LG Electronics | Agree |
| Samsung | Agree |
| vivo | Agree |
| Qualcomm | Agree |
| Ericsson | Agree with the moderator's view. The default is that Rel-17 beam management is supported for FR2-2. The combination of the two features would not be supported only if there is (1) a technical issue with the spec for the combination of features that is not resolved during maintenance, or (2) there is an explicit agreement not to support the combination. |
| Xiaomi | Agree |
| ZTE, Sanechips | Agree |

Q2. Do you think that multi-PDSCH scheduled by a single DCI should be supported in Rel-17 unified TCI framework?

| Company | Input |
| --- | --- |
| Intel | Yes |
| LG Electronics | Yes, we don’t need to unnecessarily prohibit the combination of multi-PDSCH scheduling DCI with Rel-17 feature. |
| Samsung | Yes |
| vivo | Yes |
| Qualcomm | Yes |
| Ericsson | Yes |
| Xiaomi | Yes |
| ZTE, Sanechips | Yes |

Q3. Which option do you prefer for TCI state activation?

Opt1. Updating activated TCI states within a span for multi-PDSCH scheduling is allowed.

Opt2. Updating activated TCI states within a span for multi-PDSCH scheduling is not allowed.

Note: The current consensus for Rel-15/16 TCI state activation is aligned with Opt2.

| Company | Input |
| --- | --- |
| Intel | At a first glance, we prefer Opt.2. But we also agree with Ericsson’s Comment #1 from Section 2.1.5 of the present FL Summary document that some more time may be needed for companies to assess all aspects around Rel-17 unified TCI framework and multi-PDSCH joint operation. So, we’re open for discussing Opt.1 as well. |
| LG Electronics | We are treating two issues in this section. - Issue 1: Update of activated TCI states via MAC-CE (whose mechanism is common for R16 and R17, that is, nothing is changed in Rel-17) - Issue 2: Update of indicated TCI states after beam application time (which is newly introduced in R17) In our view, Q3 falls into Issue 1. Then, we prefer Opt2. As Rel-17 specification doesn’t change UE behavior for multi-slot PDSCH (i.e., activated TCI state list will not change across slots corresponding to scheduled multi-slot PDSCH), the common rule for multi-PDSCH scheduling is also preferred, even if tci-StateId_r17 is configured. |
| Samsung | Since updating of activated TCI states via MAC-CE are not changed in Rel-17,Opt2 would be correct, if we agree Proposal 2.1-a, d or e. |
| vivo | If we wee to agree any Proposal 2.1-a, d, e, or f, updating activated TCI states is not allowed given Rel-17 still use MAC-CE for update. |
| Qualcomm | Support Opt2, which is implied by all Proposal 2.1-a,d,e,f that under discussion. |
| Ericsson | We prefer Opt1 If any of Proposals 2.1-a/e/e/f is agreed, then it would imply Opt 2 since there is no distinction in the text proposal on whether or not the UE is configured with tci-StateId_r17. However before agreeing on any of these TPs, I wanted to establish what is common understanding between Alt-1 and Alt-2 in our Comment #3 in Section 2.1.5.4 of this FL summary. We observe that if some companies have the understanding of Alt-2, then Proposals 2.1 a/e/d/f would conflict with that behavior, i.e., it would not allow an indicated update (by DCI) to a unfied TCI state within the time span of a multi-PDSCH scheduling. |
| Xiaomi | Support Opt2. |
| ZTE, Sanechips | Prefer Opt 2 in order to align with Rel-15/16 and Proposal 2.1-a/d/e/f. |

Q4. If Opt1 is preferred, do you think that additional specification support is needed?

Note: In the moderator’s view, after having another look, there’s no restriction for now. However, if we agree any TP among Proposal 2.1-a, 2.1-d or 2.1-e, then the restriction applies to Rel-17 unified TCI framework as well i.e., adopting the TP applies Opt2 for Rel-17 unified TCI framework as well.

| Company | Input |
| --- | --- |
| Ericsson | It depends. We prefer Opt1, and we observe that if companies want consistent behavior between Rel-16 and Rel-17 beam management for the case of multi-PDSCH scheduling, then it seems that the following text proposal would allow that, and no additional spec work would be needed. With this TP, it means activated TCI states can be updated during the time span of a multi-PDSCH scheduling. It also means that that if a previous DL DCI (with our without DL assignment) updates the unfied TCI state and that update is to take effect within the time span of multi-PDSCH scheduling, then it would be applied to the PDSCH in the slot for which the update is to take effect. TP which would allow Opt1 behavior: If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state(s) should be based on the activated TCI state(s) in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. |

Q5. Which option do you prefer for TCI state indication?

Opt3. Updating indicated TCI states within a span for multi-PDSCH scheduling is allowed.

Opt4. Updating indicated TCI states within a span for multi-PDSCH scheduling is not allowed.

Note: The current specification support for Rel-15/16 TCI state indication is aligned with Opt3.

Note2: A new TCI state can be indicated by a DCI without PDSCH scheduling in Rel-17 unified TCI framework. In addition, a new beam is applied after beam application time from a beam indication DCI. In such cases, actual beam application timing can be a middle of a span for multi-PDSCH scheduling.

| Company | Input |
| --- | --- |
| Intel | We prefer Opt.3. But we also open for discussion Opt.4. |
| LG Electronics | Before determining one between Opt3 and Opt4, we would like to clarify what UE behavior for multi-slot PDSCH scheduling with Rel-17 TCI framework is assumed in Rel-17 FeMIMO. We can follow that behavior as well for multi-PDSCH scheduling case. |
| Samsung | We prefer Opt3. |
| vivo | Same as LG, we prefer to clarify UE behabiour in MIMO seesion on multi-slot PDSCH as we think a consistent UE bahaviour is preferred given we’re following multi-slot PDSCH rule for multi-PDSCH scheduling. Our understanding is that current specification is aligned with Opt3. |
| Qualcomm | Support Opt3 as for FR2-1. Unified solution for both FR2-1 and FR2-2 is preferred |
| Ericsson | We prefer Opt3 As we point out in Q4, if the common view is Opt3, then if any of Proposals 2.1-a/e/e/f is agreed, Opt3 would not allow a DCI indication to update unified TCI state during the time span of a multi-PDSCH scheduling (i.e., Opt 3), since the all 4 proposals say that for the PDSCHs with offset >= timeDurationForQCL, the UE applies the TCI state that is active in slot corresponding to the first scheduled PDSCH. |
| ZTE, Sanechips | We would like to follow the same mechanism as for multi-slot PDSCH scheduling + Rel-17 unified TCI framework. We can suggest Rel-17 FeMIMO to deal with these two scheduling types with Rel-17 TCI framework in a unified way. If we have to make a decision here, we prefer Opt3. |

#### Proposal 2.1-a (TP#1 from 1st round discussion)

5.1.5 Antenna ports quasi co-location

*** Unchanged text omitted ***

When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated 
TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the 
UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the 
indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), 
and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the 
UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is 
not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-
PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving 
cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between 
the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal 
to the threshold timeDurationForQCL.

*** Unchanged text omitted ***

#### 3rd round discussion summary

After extensive email discussions, most of companies prefer to decide whether the applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH or not in the next meeting. Given the situation, Huawei proposed the following proposal.

Proposal: Support TP#1.

- RAN1 can continue discussion to down-select between the following Alt1 and  Alt2. If Alt 1 is agreed, TP#1 is updated accordingly.

- Alt1) The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH

- Alt2) The applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH

The proposal is updated by LGE and the moderator and the updated proposal is provided in Proposal 2.1-g. During the email discussion, no concern has been raised so far.

### [High Priority] Proposal 2.1-g (clean version based on the email discussion and for the GTW session)

Support TP#1.

- RAN1 continues discussion to down-select between the following Alt1 and Alt2.

- Alt1) The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH

- Alt2) The applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH

TP#1

5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***

## Others

### Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [OPPO, 5] | Observation : single value offset configuration in tdmSchemeA case may restrict multiple SLIV configuration for multiple PDSCH scheduling. Proposal : R17 introduces multi-value offset configuration for M-TRP with tdmSchemeA. |
| ###### [Intel, 11] | Proposal 5: RAN1 should clarify whether repetition scheme ‘tdmSchemeA’ is supported for multi-TRP case of multi-PDSCH operation. |
| ###### [Samsung, 15] | Proposal 3: To support Rel-17 BM for FR2-2: - Support applying the indicated TCI state with tci-StateID_r17, if the indicated TCI state is different from the previously indicated one during the multi-PDSCH scheduled by a single DCI |
| ###### [LGE, 18] | Proposal #4: When TDM scheme A is applied for multi-PDSCH scheduling with multi-TRP scenario, StartingSymbolOffsetK symbols between the last symbol of TB#n and the first symbol of TB#n+1 should be guaranteed. |

### Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.2.1 | Application of offset for M-TRP with tdmSchemeA. | Single value offset configuration in tdmSchemeA case may restrict multiple SLIV configuration for multiple PDSCH scheduling. R17 introduces multi-value offset configuration for M-TRP with tdmSchemeA: OPPO When TDM scheme A is applied for multi-PDSCH scheduling with multi-TRP scenario, StartingSymbolOffsetK symbols between the last symbol of TB#n and the first symbol of TB#n+1 should be guaranteed: LGE |
| 2.2.2 | Whether ‘tdmSchemeA’ is supported for multi-TRP case of multi-PDSCH operation. | There is no any explicit agreement made by RAN1 that excludes this scheme except maybe the aforementioned agreement from RAN1 #104. In order to avoid any misunderstanding, it proposed to explicitly clarify where the repetition scheme ‘tdmSchemeA’ is supported for multi-TRP case of multi-PDSCH operation or not: Intel |
| 2.2.3 | Application of Rel-17 TCI state | Support applying the indicated TCI state with tci-StateID_r17, if the indicated TCI state is different from the previously indicated one during the multi-PDSCH scheduled by a single DCI: Samsung |

### [Closed] 1st round discussion

#### Observation 2.2-a

Please share your views on the following proposals

[OPPO, 5]

Proposal : R17 introduces multi-value offset configuration for M-TRP with tdmSchemeA.

| Company | Input |
| --- | --- |
| Intel | First of all, we prefer to converge to the common understanding on whether tdmSchemeA is supported or not for multi-PDSCH operation in multi-TRP case. |
| LG Electronics | We don’t understand why multiple offset values are needed. From our understanding, the offset is related to beam switching time based on UE capability so it cannot be changed per slot. |
| Samsung | Introducing multi-value offset configuration for M-TRP with tdmSchemeA might be a time consuming task, since motivation and the details are not clear yet. So we would prefer not to discuss due to limited time. |
| vivo | In Rel-16, same value of offset is configured by RRC for each intra-slot based PDSCH repetition scheduled by a corresponding DCI and no multi-value offset is supported. In Rel-17, our understanding is that S-DCI based PDSCH scheduling only aims to reduce the overhead of DCI, rather than enhancing the flexible scheduling pattern for TDMschameA. We don’t support this proposal in Rel-17 at this maintenance stage. |
| Nokia/NSB | We don’t think this is essential. To Intel, tdmSchemeA is similar as fdmSchemeA/B. only tdmSchemeB is restricted which includes repeitition in TDRA table. |
| Apple | The target use cases should be clarified first for multi-value offset. In our undersanding, there are two TDRAs within a single slot for tdmSchemeA and therefore a single offset value is sufficient to determine the location of 2nd PDSCH. This can be reused for multi-PDSCH scheduling. |
| Qualcomm | We believe single value can work. |
| DOCOMO | We think multi-offset value issue can be deprioritized, as reasons mentioned by LGE, Samsung, vivo. |
| Ericsson | Same view as DOCOMO |
| Moderator | This discussion is closed as many companies prefer not to further discuss this issue. |

[Intel, 11]

Proposal 5: RAN1 should clarify whether repetition scheme ‘tdmSchemeA’ is supported for multi-TRP case of multi-PDSCH operation.

| Company | Input |
| --- | --- |
| Moderator | According to discussion so far, the intention of the agreed text proposal was to prohibit ‘repetitionNumber’ with multi-PDSCH scheduled by a single DCI. Having said that, in the moderator’s understanding, ‘tdmSchemeA’ is supported multi-PDSCH operation with multi-TRP. |
| OPPO | We understand that tdmSchemeA is supported for multi-PDSCH operation with M-TRP. If this needs to be clarified in the spec, we are open for discussion. |
| Intel | In RAN1 #104 in the beginning of the WI on NR extension up to 71GHz, RAN1 group made the following agreement regarding multi-PDSCH operation: Agreement: - The followings will not be considered in this WI. - Single DCI to schedule both PDSCH(s) and PUSCH(s) - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots) As ‘tdmSchemeA’ assumes a kind of repetition, we wold like to clarify whether it is supported for multi-PDSCH or not. |
| LG Electronics | We share the same view with the moderator and OPPO. @ Intel, Our understanding is that prior agreement is mainly for single TRP case and this was already clarified by RAN1 reflector during the discussion under [106bis-e-NR-52-71GHz-04] thread. |
| Samsung | Agree with Moderator’s view |
| vivo | We have the same understanding as with moderator |
| Nokia/NSB | Agree with moderator tdmSchemeA is similar as fdmSchemeA/B. only tdmSchemeB is restricted which includes repeitition in TDRA table. |
| Qualcomm | Same understanding as Moderator. Only tdmSchemeA is agreed, but not the slot-based repetition |
| DOCOMO | We are fine with moderator’s understanding. |
| FW | We share same view as moderator |
| Xiaomi | Agree with moderator’s view. Clarification about prohibiting ‘repetitionNumber’, not ‘tdmSchemeA’, is needed. |
| Moderator | This discussion is closed as many companies share their understanding that ‘tdmSchemeA’ is supported for multi-PDSCH operation with multi-TRP. |

[Samsung, 15]

Proposal 3: To support Rel-17 BM for FR2-2:

- Support applying the indicated TCI state with tci-StateID_r17, if the indicated TCI state is different from the previously indicated one during the multi-PDSCH scheduled by a single DCI

| Company | Input |
| --- | --- |
| Moderator | For application of Rel-17 TCI state, as mentioned in the previous meeting, Rel-17 beam management is supported for FR2-2 unless there is an explicit agreement to exclude a feature according to the WID. Having said that, I don’t think we need to explicitly agree that Rel-17 BM is supported in FR2-2. |
| OPPO | We are not sure understanding the proposal. But from the proposal, it seems to suggest a new naming of the parameter. |
| Intel | Agree with moderator’s assessment. |
| LG Electronics | From our understanding, this issue is similar to the case for changing activated TCI states via MAC-CE (which is being discussed in Section 2.1), i.e., whether to allow changing TCI state (indicated by DCI) in the middle of multiple PDSCHs scheduled by a DCI. If this is the case, we prefer to follow the same principle with MAC-CE signaling. |
| Samsung | We think our Tdoc is misunderstood. We agree with Moderator that explicit agreement for Rel-17 BM in FR 2-2 is not needed. Our intention is to clarify whether single QCL assumption is maintained when the indicated Rel-17 TCI state, which is different from the previously indicated one, is applied during the multi-PDSCH scheduled by a single DCI. |
| vivo | This issue seems similar with activated TCI states informed by MAC CE during multi-PDSCH scheduling, which can be discussed in section 2.1 together. |
| Nokia/NSB | We agree with Moderator’s view. |
| Apple | As commented in section 2.1, a consistent rule should be applied for both Rel-16 and Rel-17 BM, i.e., TCI states pool is not updated during multi-PDSCH scheduling, which is applied for both MAC-CE based or DCI-based update. |
| Qualcomm | Same view as FL, no need for such discussion |
| DOCOMO | In our understanding, this proposal is related with observation 2.1-A. Same principle should be adopted for whether allowing to update activated TCI satate in the middle of multi-PDSCH scheduling. |
| Xiaomi | According to Samsung’s clarification, this should be discussed in 2-1. |
| Moderator | Thanks for the clarification from Samsung. Now, relevant proposal is added in Proposal 2.1-c. Please continue the discussion in 2.1.4.3. |

[LGE, 18]

Proposal #4: When TDM scheme A is applied for multi-PDSCH scheduling with multi-TRP scenario, StartingSymbolOffsetK symbols between the last symbol of TB#n and the first symbol of TB#n+1 should be guaranteed.

| Company | Input |
| --- | --- |
| OPPO | From our understanding, for TDM Scheme A, the parameter StartingSymbolOffsetK has nothing to do with the TB#n and TB#n+1. The offset K is used to determine the PDSCH from the second TRP corresponding to the same TB of the PDSCH from the first TRP. |
| Intel | First of all, we prefer to converge to the common understanding on whether tdmSchemeA is supported or not for multi-PDSCH operation in multi-TRP case. |
| LG Electronics | As OPPO pointed out, StartingSymbolOffsetK was introduced to provide a gap for a UE to change from the first TRP to the second TRP, for the same TB. However, since we introduced intra-slot repetition for multi-PDSCH scheduling (different from Rel-16), we should consider the additional case of beam switching time between the second TRP (for TB#n) and the first TRP (for TB#n+1). We think the UE would require the same amount of gap to change from the second TRP (for TB#n) to the first TRP (for TB#n+1) as well. Our proposal is to handle that case. For example, as shown in our Tdoc (R1-2202337), UE can be scheduled with 2 PDSCHs (by a single DCI) with TDM scheme A but the gap between the last symbol of the second PDSCH transmission occasion (TO) of TB#1 and the first symbol of the first PDSCH TO of TB#2 may be less than K (=StartingSymbolOffsetK) symbols. In this case, UE may not be able to receive TB#2 from the first TRP due to the lack of beam switching time. Therefore, we should clarify what is the expected UE behavior if StartingSymbolOffsetK symbols between the last symbol of TB#n and the first symbol of TB#n+1 cannot be guaranteed. |
| Samsung | In our view, the gap between different TB can be guaranteed by gNB implementation. |
| Vivo | In legacy specification, if two S-DCIs schedule respectively two single-slot PDSCH transmissions and the two PDSCHs are allocated in two adjacent slots, there is no gap restriction between the last symbol of TB#n and the first symbol of TB#n+1. For multi-PDSCH scheduling, same rule should be reused. So, we think no enhancement is needed. |
| Nokia/NSB | We don’t support the proposal. |
| Apple | First, it should be clarified that the scenario is for single DCI or multi-DCI multi-TRP. If it is single-DCI based multi-PDSCH scheduling with multi-TRP, our view is that the issue raised by LGe needs to be clarified and addressed. We are open to discuss whether anyting needs to do for the multi-DCI multi-PDSCH scheduling in multi-TPR case. |
| Qualcomm | No need for Proposal 4. No such restriction for legacy scheduling, which has similar issue |
| DOCOMO | We think it’s clear it can be guaranteed by gNb implementation. |
| Xiaomi | Fine with proposal #4. If the gap is not needed, why is the StartingSymbolOffsetK necessary? In addition, samsung’s view is acceptable for us. |
| Moderator | As many companies believe that this issue can be handled by gNB implementation and do not support the proposal, this issue is closed. |

#### 1st round discussion summary

For most of issues, many companies believe that there’s no need to further discuss the issues except Rel-17 unified TCI activation/deactivation issue proposed by Samsung. This discussion is closed now and please continue discussion on Rel-17 unified TCI activation/deactivation in 2.1.4.3.

# Parameters Associated with Beam-based Operation

## Parameters associated with Rel-16 beam management

### Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [Huawei/HiSi, 1] | Proposal 1: UL-DL or DL-UL w/o spatial domain filter change should be counted as a beam switch in maxNumberRxTxBeamSwitchDL for both FR2-1 and FR2-2. |
| ###### [ZTE/Sanechips, 7] | Proposal 3: We propose that additional beam switching time delay d = 112 symbols as the basic UE capability and d = 56 symbols as the optional UE capability. If the UE does not report its capability, d = 112 symbols can be used as default UE capability. |
| ###### [MediaTek, 16] | Proposal 1: For maxNumberRxTxBeamSwitchDL, the feasibility of candidate value {7} for 480kHz and candidate values {4,7} for 960kHz should be further discussed based on the RAN4 decision on the beam switching time for 480kHz and 960kHz. |

### Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.1.1 | Additional beam switching time delay d | We propose that additional beam switching time delay d = 112 symbols as the basic UE capability and d = 56 symbols as the optional UE capability. If the UE does not report its capability, d = 112 symbols can be used as default UE capability: ZTE/Sanechips |
| 3.1.2 | Definition change of maxNumberRxTxBeamSwitchDL | UL-DL or DL-UL w/o spatial domain filter change should be counted as a beam switch in maxNumberRxTxBeamSwitchDL for both FR2-1 and FR2-2: Huawei/HiSi |
| 3.1.3 | Feasibility of candidate values for maxNumberRxTxBeamSwitchDL for 480kHz and 960 kHz | For maxNumberRxTxBeamSwitchDL, the feasibility of candidate value {7} for 480kHz and candidate values {4,7} for 960kHz should be further discussed based on the RAN4 decision on the beam switching time for 480kHz and 960kHz: MediaTek |

### [Closed] 1st round discussion

#### Observation 3.1-a

Please share your views on the following proposals

[Huawei/HiSi, 1]

Proposal 1:  UL-DL or DL-UL w/o spatial domain filter change should be counted as a beam switch in maxNumberRxTxBeamSwitchDL for both FR2-1 and FR2-2.

| Company | Input |
| --- | --- |
| Intel | Don’t support. May be a separate parameter would be better to introduce in order for accounting UL-DL or DL-UL switches rather than update the existing maxNumberRxTxBeamSwitchDL with the established meaning. |
| LG Electronics | Do not support. As in Rel-15, how to count the number of beam switches is up to UE implementation. |
| Samsung | In our view, the beam switching refers to switch spatial filter. So we don’t think UL-DL or DL-UL w/o spatial domain filter can be counted as a beam switch. |
| vivo | We are open to discuss this issue. We also think whether the definition of maxNumberRxTxBeamSwitchDL is per UE or per panel needed to be clarified. For example, if UE support value ‘1’ for 960kHz, and UE supports simultaneous reception or transmission with two panels, can the scheme of SDM or FDM-a or FDM-b based PDSCH transmission for M-TRP be supported? |
| Nokia/NSB | Similar view as Samsung. |
| Qualcomm | Support, direction change should also count as beam change and should not happen too frequently |
| DOCOMO | We share similar view as Samsung. |
| Ericsson | Same view as Samsung |
| FW | We agree with Qualcomm view that direction change should also be included. |
| Xiaomi | Fine with this proposal 1. The definition of maxNumberRxTxBeamSwitchDL is the number of Tx and Rx beam changes UE can perform on this band within a slot. From our understanding, UL-DL or DL-UL w/o spatial domain filter change should be counted as a beam switch, otherwise, the definition should be the number of Tx or Rx beam changes UE can perform within a slot. If most of us think the definition of maxNumberRxTxBeamSwitchDL in Rel-15/16 is vague, we are fine with this proposal to make it clear. |
| Moderator | Many company believes that UL-DL or DL-UL without spatial domain filter change should not be counted as a beam switch. Given the situation, this discussion is now closed. |

[ZTE/Sanehips, 7]

Proposal 3: We propose that additional beam switching time delay d = 112 symbols as the basic UE capability and d = 56 symbols as the optional UE capability. If the UE does not report its capability, d = 112 symbols can be used as default UE capability.

| Company | Input |
| --- | --- |
| Intel | Support |
| LG Electronics | This seems overlapped with an issue which are being discussed under 8.16.2 UE feature agenda item. |
| Samsung | We think the proposal is aligned with the intention of the agreement that introduces UE capability signalling which indicates 56 symbols or 112 symbols. |
| vivo | We think this may be more appropriate for UE feature session. |
| Nokia/NSB | Up to UE feature discussion. |
| Qualcomm | Suggest to discuss in UE feature |
| DOCOMO | Fine with the proposal, based on previous agreement. |
| Xiaomi | Support. |
| Moderator | It seems that many companies prefer to discuss this issue in UE feature discussion. Having said that, please continue the discussion in UE feature session and now this discussion is closed. |

[MediaTek, 16]

Proposal 1: For maxNumberRxTxBeamSwitchDL, the feasibility of candidate value {7} for 480kHz and candidate values {4,7} for 960kHz should be further discussed based on the RAN4 decision on the beam switching time for 480kHz and 960kHz.

| Company | Input |
| --- | --- |
| Intel | Don’t support. Our understanding is that this issue was closed in the previous RAN1 meeting. So we don’t prefer to come back to them. |
| LG Electronics | Do not support, and we share the view with Intel. |
| vivo | We think this capability means the maximum number of beam changes supported by UE. If there is Rx-Tx or Tx-Rx transition in one slot, the actual number of beam change would be reduced due to 7.015us guard for transition switching. It’s not clear to us why more discussion on the already agreed values is needed. |
| Nokia/NSB | No need for further discussion in RAN1. |
| ZTE, Sanechips | Do not support it. We should not discuss this proposal anymore as the range of candidate values is flexible enough and no new agreements to support reverting the previous agreements. |
| DOCOMO | We share same view as Intel/LGE that the discussion was closed with following agreements. Agreement: For maxNumberRxTxBeamSwitchDL, - For 480 kHz, support 7 as a candidate value for 480 kHz in addition to the agreed candidate values 2 and 4 - For 960 kHz, support one of the following alternatives - Alt-1: Support 1, 4 and [7] as candidate values for 960 kHz in addition to the agreed candidate values 2 - Alt-2: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2 - No additional candidate values are supported Agreement: For maxNumberRxTxBeamSwitchDL, support 1, 4 and 7 as candidate values for 960 kHz in addition to the agreed candidate value 2. - Note: this is Alt-1 from the RAN1#106 agreement. |
| Xiaomi | Not support. |
| Moderator | As many companies believe that the discussion is already closed based on the previous agreements, this discussion is closed now. |

#### 1st round discussion summary

As many companies believe that no further discussion is needed or the issue should be discussed in UE feature session, this discussion is closed now.

## Minimum guard period Y between two SRS resources of an SRS resource set for antenna switching

### Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [InterDigital, 3] | Observation 1: The simple extrapolation of the number of symbols required for antenna switching gap with SRS resource set transition will lead to 8 and 16 blanked symbols respectively for 480 and 960KHz SCS if we use the absolute time duration for low SCSs. Observation 2: Supporting low values of antenna switching gap would bring efficient NR operation in 52-71GHz. Proposal 1: Introduce a UE capability signaling to indicate one of [2, 8] for 480 kHz and [2, 16] for 960 kHz, where 8 and 16 are mandatory values for 480 kHz and 960 kHz, respectively. |
| ###### [Nokia/NSB, 10] | Proposal 2: Support following value of minimum guard period Y between two SRS resources of an SRS resource set for antenna switching for 480 kHz and 960 kHz - Y=3 for 480kHz, Y=5 for 960kHz Proposal 3: Consider the introduction of Y values for advanced UE capability with shorter transient time according to RAN4 on-going discussion. |
| ###### [Intel, 11] | Proposal 1: The minimal guard period between SRS resources for antenna switching for SCS 480 kHz and 960 kHz is Y=2. Proposal 2: Depending on the availability of RAN4 feedback on antenna switching time requirements for a UE, introduce an optional UE capability for guard period of SRS antenna switching. When this capability is not reported, the minimal value of Y=2 is assumed for the guard period for antenna switching between SRS resources with SCS 480 kHz and 960 kHz. Otherwise, the value indicated in the capability signalling is used. |
| ###### [Qualcomm, 17] | Proposal 2: Support that the number of guard symbols between two SRS resources for antenna switching is 8 for 480 kHz, and 16 for 960 kHz. |

### Summary of views

In RAN1#107bis-e, an LS is sent to RAN4 to ask a discussion for a minimum guard period between two SRS resources of an SRS resource set for antenna switching for 480kHz and 960kHz.

R1-2200783		[Draft] LS on a minimum guard period between two SRS resources for antenna switching	InterDigital

Final LS is endorsed in R1-2200796, changing “kindly” to “respectfully”.

Based on the above LS, the following companies’ views are observed.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.3.1 | Values of a minimum guard period Y between two SRS resources of an SRS resource set for antenna switching | - 2 symbols for both 480 kHz and 960 kHz: Intel - 3 symbols for 480 kHz and 5 symbols for 960 kHz: Nokia/NSB - UE capability: InterDigital (2/8 for 480kHz and 2/16 for 960 kHz), Nokia/NSB(if advanced UE agreed in RAN4), Intel |

### [Closed] 1st round discussion

#### Observation 3.2-a

Based on the agreed LS, the moderator suggests to hold the discussion until RAN4’s response. Please indicate your preference if you have any issues on this suggestion.

| Company | Input |
| --- | --- |
| Intel | We could hold this discussion once more |
| LG Electronics | Agree with the moderator’s view that we can hold the discussion until RAN4 respond to RAN1’s LS. |
| Samsung | Agree with the moderator’s suggestion |
| vivo | Dual plolarization antenna ports formed by anolog beamforming per panel is a typical UE implementation, and two antenna ports are usually transmitted simultaneously, especially for FR2-2. In our view, we think UE vendors would not have some unusual panel configuration just to support some atypical configuration of antenna switching capability for beyond 52.6GHz. We propose antenna switching is not supported for FR2-2 in Rel-17. |
| Nokia/NSB | Agree with the moderator’s suggestion. We think the basic feature should be supported without UE capability. Because RAN4 is discussion to reduce transient time, we can further consider advanced UE with shorter gap. |
| Apple | We support moderator proposal. Our understanding is that the issue was transferred to RAN4 by sending LS. The exact same question was actually asked in GTW session in last meeting regarding what is the next step of RAN1 on this issue. It was explicitly clarified by Vice-Chair that the issue was left to RAN4. |
| ZTE, Sanechips | Agree with the moderator’s suggestion |
| Qualcomm | Agree with moderator |
| DOCOMO | Agree with moderator’s suggestion. |
| Ericsson | Agree with moderator's suggestion – wait for RAN4. |
| FW | Agree with moderator |
| Sony | Agree with the moderator’s suggestion |
| Xiaomi | Agree with moderator’s suggestion. |

#### 1st round discussion summary

As many companies share the moderator’s suggestion to hold the discussion, this discussion is closed now.

## Beam switching gap and scheduling restrictions for higher SCSs

### Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [Huawei/HiSi, 1] | Proposal 3: Regarding beam switch time, support the following - UE is not expected to be scheduled/configured with a signal/channel on one symbol before to and one symbol after of another signal/channel if the signals/channels have two different QCL-D assumptions. |
| ###### [FUTUREWEI, 2] | Proposal 2: Regarding beam switch time, support the following - Support a UE capability signalling for beam switch time. - UE is not expected to receive adjacent symbols with different QCL-D assumptions when the indicated beam switch time of UE is larger than X ns. - X can depend on the CP time of the operating SCS (eg X =[60] ns for 960 kHz and X=[120] ns for 480 kHz SCS). Proposal 3: Analogous to the overlapping PDCCH/PDSCH and PDCCH/PDCCH with different QCL-TypeD situation in FR2-1, in FR2-2 a precedence relation is necessary for UEs incapable of adjacent symbol reception with beam switching. |
| ###### [InterDigital, 3] | Observation 3: For UE capability signaling for Rx and Tx beam switching time with/without scheduling restriction, further discussions are essential for the specification support. Especially, various corner cases should be considered for the scheduling restriction. Observation 4: Given the limited discussion time in the maintenance phase, it is preferred to support a minimized and essential specification support. Observation 5: Based on the reported value, beam switching time gap can be handled by gNB implementation. Proposal 2: Introduce a UE capability signaling for Rx and Tx beam switching without any scheduling restriction. |
| ###### [CATT, 6] | Proposal 3: Support a UE capability signalling for beam switch time X (ns). The value is same for SCS=480/960 kHz, and is same for uplink/downlink transmission. Proposal 4: The UE does not expect to receive adjacent DL signals/channels which have different QCL-typeD assumptions within the reported beam switching time, at least for the following DL signals/channels: - PDCCH and PDCCH - PDSCH and PDCCH - PDSCH and PDSCH Observation 2: In order to guarantee the reception performance of PDSCH, additional beam switching gap need to be reserved between PDSCHs for the following conditions: one is when the beam switching time exceeds the CP length; the other is when the QCL-D assumptions are different for adjacent PDSCHs. Proposal 5: For PDSCH, the beam switching gap can be implemented by gNB scheduling or by defining extra gap symbol (s) between PDSCHs for the following scenarios: - Beam switching time exceeds the CP length - QCL-D assumptions are different for adjacent PDSCHs. Proposal 6: For beam switching between PDCCH monitoring occasions in different CORESETs that have different QCL-assumptions for 480/960 kHz, the same mechanism in R15/R16 can be reused with the definition of “overlapping” redefined to include the cases where the gap between PDCCH monitoring occasions in multiple CORESETs is less than the UE reported beam switching time. |
| ###### [ZTE/Sanechips, 7] | Proposal 2: Introduce a UE capability for the UE reporting the beam switching time it can support. |
| ###### [NTT DOCOMO, 8] | Proposal 1: - Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively. - Number of symbols for the beam switching time is up to UE feature discussion, and possibly RAN4 progress. - The beam switching time should be considered for two adjacent signals/channels with different QCL Type-D assumptions. |
| ###### [Intel, 11] | Proposal 3: Support UE capability signaling for beam switching time. The signaling may indicate a UE needs at least 1 symbol gap for both 480 kHz and 960 kHz. |
| ###### [Apple, 13] | Proposal 2: Define beam switching gap for the following cases with new SCSs based on beam switching time requirement replied from RAN4 (e.g.,1 symbol): - Rx beam switching between DL signals/channels with different QCL Type-D source RSs. - Tx beam switching time between UL signals/channels with different spatial relation RSs e.g., PUCCHs/PUSCHs/SRSs. |
| ###### [Samsung, 15] | Proposal 1: Reserve one symbol for beam switching gap when using 480 kHz and 960 kHz SCS at least for the two signals/channels with different QCL Type-D assumptions |
| ###### [Qualcomm, 17] | Proposal 1: Support UE capability for the beam switch time gap, required between two signals in each of the following cases. - Case 1: - Any two signal that have different specified QCL-TypeD assumptions - Case 2: - One signal has specified QCL-TypeD assumption, while the other is SSB or CSI-RS without specified QCL-TypeD assumption - Case 3: - The two signals are two CSI-RS resources without specified QCL-TypeD assumption - Case 4: - The two signals are two CSI-RS resources with same specified QCL-TypeD assumption in a resource set with higher layer parameter Repetition set to ‘ON’ - Case 5: - Any two signals that have different specified spatial relations - Case 6: - One signal has specified spatial relation assumption, while the other is SRS without specified spatial relation - Case 7: - The two signals are two SRS resources without specified spatial relation - Note: - The above “specified” means the beam indication is either explicitly provided by gNB or implicitly determined by spec. |
| ###### [LGE, 18] | Proposal #1: Support a UE capability for Rx and Tx beam switching time between adjacent DL signals/channels and adjacent UL signals/channels, respectively. Proposal #2: Define UE behavior (e.g., drop CG-PUSCH or drop DG-PUSCH depending on PUSCH cancellation time) when CG-PUSCH and DG-PUSCH transmissions are consecutively assigned and the reported beam switching time between CG-PUSCH and DG-PUSCH is not guaranteed. |

### Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.4.1 | Introduction of UE capability reporting on UE beam switching time with or without scheduling restriction | UE capability without scheduling restriction: InterDigital, ZTE/Sanechips, Intel - [InterDigital]: Given the limited discussion time in the maintenance phase, it is preferred to support a minimized and essential specification support. UE capability with scheduling restriction: Huawei/HiSi (one symbol before and one symbol after), FUTUREWEI (if the switching time>X ns), CATT, Docomo, Qualcomm, NEC, LGE, - [Huawei/HiSi]: UE is not expected to be scheduled/configured with a signal/channel on one symbol before to and one symbol after of another signal/channel if the signals/channels have two different QCL-D assumptions. - [CATT]: The UE does not expect to receive adjacent DL signals/channels which have different QCL-typeD assumptions within the reported beam switching time, at least for the following DL signals/channels |
| 3.4.2 | Scenarios for scheduling restriction | Huawei/HiSi: between signals/channels with different QCL Type-D assumptions CATT: PDCCH/PDCCH, PDSCH/PDCCH, PDSCH/PDSCH with different QCL Type-D Qualcomm: - Any two signal that have different specified QCL-TypeD assumptions - One signal has specified QCL-TypeD assumption, while the other is SSB or CSI-RS without specified QCL-TypeD assumption - The two signals are two CSI-RS resources without specified QCL-TypeD assumption - The two signals are two CSI-RS resources with same specified QCL-TypeD assumption in a resource set with higher layer parameter Repetition set to ‘ON’ - Any two signals that have different specified spatial relations - One signal has specified spatial relation assumption, while the other is SRS without specified spatial relation - The two signals are two SRS resources without specified spatial relation Apple: Rx beam switching between DL signals/channels with different QCL Type-D assumption and Tx beam switching time between UL signals/channels with different spatial relation info. |

### [Closed] 1st round discussion

#### Observation 3.3-a

According to the discussion in RAN1#107bis-e, the majority of the companies preferred to hold the discussion until RAN4’s response on UE beam switching time. As RAN1 does not receive RAN4’s response yet, the moderator suggests to hold the discussion until RAN4’s feedback. Please indicate your preference if there’s any issues on this suggestion.

| Company | Input |
| --- | --- |
| Intel | We could hold this discussion once more |
| LG Electronics | Agree with the moderator’s view that we could hold the discussion. |
| Samsung | Agree with the moderator’s suggestion |
| vivo | Agree with moderator’s suggestion |
| Nokia/NSB | Agree with the moderator’s suggestion. Assume UE beam switching delay discussed in RAN4 are 59~200ns, we think at most one symbol is needed. We prefer not to define UE capability, So, support either to agree 0 or 1 symbol gap between RSs with different QCL-typeD/Spatial relation. |
| Apple | Agree with moderator. |
| ZTE, Sanechips | Agree with moderator’s suggestion |
| Qualcomm | Fine with moderator’s suggestion |
| DOCOMO | Agree with moderator’s suggestion. |
| Ericsson | Agree with moderator's suggestion |
| FW | OK with Moderator’s suggestion |
| Sony | Agree with the moderator’s suggestion |
| Xiaomi | Support. |

#### 1st round discussion summary

As many companies share the moderator’s suggestion to hold the discussion, this discussion is closed now.

# Others

## Observations and Proposals from Contributions

| ###### Company | ###### Observations and Proposals from Contributions |
| --- | --- |
| ###### [CATT, 6] | Observation 1: When UE cannot measure the periodic CSI-RS at the scheduled transmission instance for beam management due to LBT failure, gNB could transmit aperiodic CSI-RS as the alternative measurement. Proposal 1: Aperiodic CSI-RS could be used as the alternative solution of missed L1 RSRP measurement of periodic CSI-RS due to LBT failure with minimized specification impact. Proposal 2: The beam management frame work should be reused for NR operation in 52.6-71 GHz and the beam management and CSI-RS configuration for beam management need to be enhanced. Proposal 7: In initial access, the beam adaptation for Msg3 and Msg4 transmission can be based on the valid beam report from UE which is extended from Rel-16. |
| ###### [Sony, 9] | - : Support aperiodic CSI-RS for beam failure detection (BFD) and candidate beam determination (CBD) at least for unlicensed band operation. - : Study and specify if needed single DCI scheduled multiple aperiodic CSI-RS and/or aperiodic SRS across multiple slots. - : Beam alignment during initial access procedure should be considered for NR above 52.6 GHz. |

## Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 5.1 | Beam reporting/alignment during initial access procedure | Yes: Sony, CATT |
| 5.4 | RS enhancement | Yes: Sony No: CATT |

## [Closed] 1st round discussion

### Observation 4-a

For RS enhancement, the moderator suggest not to further discuss the issue based on the following conclusion in RAN1#107bis-e.

Conclusion

From RAN1 perspective, there is no consensus to enhance beam management for shared spectrum operation in Rel-17.

Please provide your views on the following proposals on whether to introduce beam alignment during initial access.

[CATT, 6]

Proposal 7:  In initial access, the beam adaptation for Msg3 and Msg4 transmission can be based on the valid beam report from UE which is extended from Rel-16.

[Sony, 9]

Proposal 3	: Beam alignment during initial access procedure should be considered for NR above 52.6 GHz.

| Company | Input |
| --- | --- |
| Intel | Support the suggestion from moderator not discuss further any RS enhancement to beam management |
| LG Electronics | Changing initial access procedure may have a large impact on UE implementation and specification, so it is preferred not to discuss those issues in maintenance phase. |
| Samsung | Agree with the moderator’s suggestion not to discuss further RS enhancement. |
| vivo | Agree with moderator’s suggestion |
| Nokia/NSB | Agree with moderator’s suggestion. |
| Apple | Agree with moderator’s suggestion |
| ZTE, Sanechips | Agree with moderator’s suggestion |
| Ericsson | Agree with moderator's suggestion on RS enhancement On the other proposals – these are outside the scope of maintenance. |
| Sony | Although we think some further discussion on RS enhancements and Beam alignment are needed, we can accept the conclusion for the maintenance phase and hope some comeback in future release. |
| CATT | Considering the time scope of the WI we are fine with this conclusion. |

### 1st round discussion summary

As many companies share their views with the moderator’s suggestion, this discussion is closed now.

# References

- R1-2200955 Remaining issues of beam management enhancement for 52-71GHz spectrum	 Huawei, HiSilicon,

- R1-2200989 Remaining issues in Beam Management for Beyond 52.6GHz	FUTUREWEI,

- R1-2201035 Remaining issues for beam management for new SCSs	InterDigital, Inc.,

- R1-2201087 Remaining issues on beam management for new SCSs for NR operation from 52.6GHz to 71GHz	vivo,

- R1-2201268 Discussion on remaining issue for beam management for new SCSs	OPPO,

- R1-2201353 Remaining issues on beam management for new SCSs for up to 71GHz operation	CATT,

- R1-2201391 Remaining issues on the beam management for 52.6 to 71GHz	ZTE, Sanechips,

- R1-2201472 Remaining issues on Beam based operation for new SCSs for NR in FR2-2	NTT DOCOMO, INC.,

- R1-2201577 Remaining issues on beam management enhancement for NR Beyond 52.6GHz	Sony,

- R1-2201664 Beam Management Aspects	Nokia, Nokia Shanghai Bell,

- R1-2201690 Discussion on Beam management aspects for extending NR up to 71 GHz	Intel Corporation,

- R1-2201738 Beam Management for New SCSs	Ericsson,

- R1-2201766 Beam Management for New SCSs	Apple,

- R1-2201942 Maintenance on beam management for new SCSs	Xiaomi,

- R1-2202006 Maintenance on beam management for new SCSs for NR from 52.6 GHz to 71 GHz 	Samsung,

- R1-2202073 Remaining discussion on beam management for 52.6-71 GHz NR operation	MediaTek Inc.,

- R1-2202131 Beam managment for new SCS	Qualcomm Incorporated,

- R1-2202337 Enhancements for beam management to support NR above 52.6 GHz	LG Electronics.