3GPP TSG RAN WG1 Meeting #108-e		R1-2201053

e-Meeting, February 21st – March 3rd , 2022

Title:	                    [Draft] Reply LS on SRS for Multi-RTT positioning

Response to:	R4-2202680

Release:	Rel-17

Work Item:	         NR_pos_enh

Source:	vivo[RAN1]

To:	RAN4

Cc:	RAN2, RAN3

Contact person:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN1 thanks RAN4 for the LS and would like to provide the response as the following.

RAN4 kindly requests RAN1 to confirm whether Rel-15 SRS are also applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement, and if so, from which release.

RAN1 has reached the following agreements regarding only Rel-16 SRS is used for UE Rx-Tx time difference measurement for Multi-RTT positioning (RAN1#98) and Rel-15 SRS used for positioning is transparent to the UE (RAN1#105-e and RAN1#106-e).

Agreement (RAN1#98): Inform RAN2 of the following: - It is RAN1 understanding that discussion on mapping of positioning techniques to reference signals and measurements is out of RAN1 scope and therefore RAN1 does not intend to define such mapping. - It is RAN1 understanding that RAN2 will define signalling that can support any RAT dependent positioning technique including hybrid RAT dependent positioning solutions - RAN1 agreed on the tables below that provide mapping between reference signals and measurements based on the latest status of RAN1 discussion - The positioning techniques are indicated for information purpose as examples and do not intend to influence higher layer signalling to be developed by RAN2 - Note: Table will be modified in the future if additional reference signals for UE/gNB measurements are agreed DL/UL Reference Signals; UE Measurements; To facilitate support of the following positioning techniques. Rel.16 DL PRS; DL RSTD; DL-TDOA. Rel.16 DL PRS; DL PRS RSRP; DL-TDOA, DL-AoD, Multi-RTT. Rel.16 DL PRS / Rel.16 SRS for positioning; UE Rx-Tx time difference; Multi-RTT. Rel. 15 SSB / CSI-RS for RRM; SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM); E-CID. Agreement(RAN1#105-e): NR supports gNB reporting of the first arrival path UL-AOA/ZOA measurement per SRS for positioning resource and SRS for MIMO resource - Note: The use of SRS for MIMO resource is transparent to the UE Agreement(RAN1#106-e): Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF - FFS: Reporting of RSRP for the first arrival path - Note: The use of SRS for MIMO resource is transparent to the UE - FFS: Reporting of gNB Rx-Tx

In addition, as described in TS38.214, the Rx-Tx time difference measurements only corresponds to a single Rel-16 SRS (SRS resource for positioning). So, RAN1 would like to confirm, in release 17 and previous releases, RAN1 has not supported the use of Rel-15 SRS for UE Rx-Tx time difference measurement.

TS 38.214 h.00 5.1.6.5 The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers.

Furthermore, for Rel-17 NR_pos_enh, RAN1 also concluded that Tx TEG enhancement is only for  Rel-16 SRS and not applicable for Rel-16 SRS for UL TDOA positioning and Multi-RTT Positioning.

Agreement Confirm and modify the working assumption with the following modifications: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - Note: For mitigating UE Tx timing errors when both UL-TDOA and Multi-RTT, or UL-TDOA and DL-TDOA are used, the UE should provide the association information of UL SRS resources for positioning with Tx TEGs, subject to UE capability (in the bullets above): - to the serving gNB if a request to provide the association information is received from the gNB - to the LMF if a request to provide the association information is received from the LMF.

Finally, considering that RAN1 has finished Rel-17 work on NR_pos_enh, therefore, it may not be possible to study this topic, at leat in this release.

2	Actions

To RAN4

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into consideration in their future work.

3	Dates of next TSG RAN 1 meetings

TSG-RAN WG1 Meeting #109-e   16 – 27 May. 2022                                Electronic Meeting

TSG-RAN WG1 Meeting #110      22 – 26 August. 2022                            Toulouse, FR