3GPP TSG RAN WG1 #108-e 		R1-2201057

e-Meeting, February 21st – March 3rd, 2022

Title:	[Draft] Reply LS on NR-U channel information and procedures

Response to:	R3-216042

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	vivo [RAN1]

To:	RAN3

Cc:	RAN2

# Contact Person:

# Name:	                 	Gen Li

# E-mail Address:	reagan.li@vivo.com

# 1. Overall Description:

RAN1 would like to thank RAN3 for the LS R3-216042 on NR-U channel information and procedures. RAN1 discussed the questions in the LS, and would like to provide the following answers:

Q1: How should an NR-U channel be represented?

Answer from RAN1:

From RAN1 perspective, according to TS 37.213 Section 4.0, NR-U channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum.

Q2: According to current specifications, is an NG-RAN node supposed to sense the NR-U channel even when no data needs to be transmitted or is channel sensing performed only when the NG-RAN node needs to exchange traffic over the NR-U channel?

Answer from RAN1:

According to TS 37.213 Section 4.1, the NG-RAN node may apply LBT in order to perform transmissions. It is not specified in 3GPP specifications whether the NG-RAN node can sense the NR-U channel even when no data are available for transmission.

Q3: How is the ED threshold configured in RAN node?

Answer from RAN1:

The ED threshold in RAN node is determined according to TS 37.213 Section 4.1.5 where it is a function of maximum RAN node output power, NRU channel bandwidth and etc.

Q4: What is the ED threshold granularity (per channel, per cell, per UE…)?

Answer from RAN1:

Since the ED threshold is calculated as a function of NRU channel bandwidth, ED threshold granularity is per channel.

# 2. Actions:

# To RAN3

# ACTION: RAN1 respectfully ask RAN3 to take the above information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #109,	May 16, 2022– May 27, 2022

TSG RAN WG1 Meeting #110,       August 22, 2022–August 26, 2022