3GPP TSG RAN WG1 #108-e		        R1-2201077

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Maintenance on Rel-16 NR positioning

Agenda Item:	7.2.8

Document for:	Discussion & Decision

Introduction

In this contribution, we present our views on remaining issues for Rel-16 NR positioning.

Descriptions of dl-PRS-ID

In section 5.1.6.5 of TS38.214 [1], a description related to dl-PRS-ID is below.

The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource.

However, we don’t think a dl-PRS-ID (or a TRP) can be always associated with multiple DL PRS resource sets, as the number of DL PRS resource sets of a TRP can be 1 in some cases. This issue is resolved in TS37.355 [2] by using the following descriptions marked in yellow.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

Therefore, for the description in 38.214, it is better to replace ‘is’ with ‘can be’.

Thus, we propose that:

Proposal 1:

- Adopt the following modification into TS38.214 section 5.1.6.5 regarding dl-PRS-ID.

TS 38.214, section 5.1.6.5 < Unchanged parts are omitted > The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is can be associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. < Unchanged parts are omitted >

Descriptions of dl-PRS-CombSizeN-AndReOffset

In TS38.214, a description related to dl-PRS-CombSizeN-AndReOffset is below.

A DL PRS resource is defined by: … - dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. …

We found that the description of this parameter in TS38.214 is not aligned with the description in 37.355 and incomplete, since this parameter should not only specifie REOffset, but also CombSizeN.

NR-DL-PRS-Resource-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, dl-PRS-SequenceID-r16 INTEGER (0.. 4095), dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { n2-r16 INTEGER (0..1), n4-r16 INTEGER (0..3), n6-r16 INTEGER (0..5), n12-r16 INTEGER (0..11), ... }, dl-PRS-ResourceSlotOffset-r16 INTEGER (0..nrMaxResourceOffsetValue-1-r16), dl-PRS-ResourceSymbolOffset-r16 INTEGER (0..12), dl-PRS-QCL-Info-r16 DL-PRS-QCL-Info-r16 OPTIONAL, --Need ON ... }

dl-PRS-CombSizeN-AndReOffset This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource and the Resource Element (RE) offset in the frequency domain for the first symbol in a DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size. The relative RE offsets of following symbols are defined relative to the RE Offset in the frequency domain of the first symbol in the DL-PRS Resource according to TS 38.211 [41]. The comb size configuration should be aligned with the comb size configuration for the frequency layer.

Therefore, description related to CombSizeN should be added.

Thus, we propose that:

Proposal 2:

- Adopt the following modification into TS38.214 section 5.1.6.5 regarding dl-PRS-CombSizeN-AndReOffset.

TS 38.214, section 5.1.6.5 < Unchanged parts are omitted > A DL PRS resource is defined by: … - dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL-PRS Resource and the starting RE offset of the first symbol within athe DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. … < Unchanged parts are omitted >

Conclusion

In this contribution, we present our views on remaining issues for Rel-16 NR positioning with the following proposal:

Proposal 1:

- Adopt the following modification into TS38.214 section 5.1.6.5 regarding dl-PRS-ID.

TS 38.214, section 5.1.6.5 < Unchanged parts are omitted > The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is can be associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. < Unchanged parts are omitted >

Proposal 2:

- Adopt the following modification into TS38.214 section 5.1.6.5 regarding dl-PRS-CombSizeN-AndReOffset.

TS 38.214, section 5.1.6.5 < Unchanged parts are omitted > A DL PRS resource is defined by: … - dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL-PRS Resource and the starting RE offset of the first symbol within athe DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. … < Unchanged parts are omitted >

- References

- 3GPP TS38.214, V16.8.0, 2021.12

- 3GPP TS37.355, V16.7.0, 2021.12

- Appendix-Draft CR DL PRS parameter alignment with TS37.355

3GPP TSG RAN WG1 #108-e	R1-22xxxxx

e-Meeting, February 21st – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | V16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on DL PRS parameter alignment with TS37.355 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS38.214, the description of dl-PRS-ID ‘The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets’ is not correct as sometimes 1 DL PRS resource set of a dl-PRS-ID is allowed, which is reflected in TS37.355 with the wording ‘This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP’ In addition, the description of parameter dl-PRS-CombSizeN-AndReOffset in TS38.214 only specifies REOffset, which is not aligned with the description in 37.355 and incomplete, since this parameter should not only specify REOffset, but also CombSizeN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.The description of dl-PRS-ID is changed according to TS37355. 2.The description of dl-PRS-CombSizeN-AndReOffset is changed according to TS37355 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descriptions of parameter ‘dl-PRS-ID’ and ‘dl-PRS-CombSizeN-AndReOffset’ are not correct and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.6.5	PRS reception procedure

========================= Unchanged parts =========================

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer.

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource.

A positioning frequency layer is configured by NR-DL-PRS-PositioningFrequencyLayer, consists of one or more DL PRS resource sets and it is defined by:

-	dl-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing. The supported values of dl-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211], excluding the value of 240kHz.

-	dl-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-CyclicPrefix. The supported values of dl-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	dl-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resources sets belonging to the same DL PRS positioning frequency layer have a common Point A.

The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it can be associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource.

The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID.

-	If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell;

-	Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell.

If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell.

If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell.

A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by:

-	nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration.

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively.

-	dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor.

-	dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceID within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap.

-	dl-PRS-MutingOption1 and dl-PRS-MutingOption2 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If dl-PRS-MutingOption1 is configured, each bit in the bitmap of dl-PRS-MutingOption1 corresponds to a configurable number provided by higher layer parameter dl-prs-MutingBitRepetitionFactor of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If dl-PRS-MutingOption2 is configured each bit in the bitmap of dl-PRS-MutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor. Both dl-PRS-MutingOption1 and dl-PRS-MutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211].

-	NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo.

-	dl-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set.

-	dl-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-CombSizeN.

-	dl-PRS-ResourceBandwidth defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of dl-PRS-ResourceBandwidth.

-	dl-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-StartPRB.

-	dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211].

A DL PRS resource is defined by:

-	nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL-PRS Resource and the starting RE offset of the first symbol within the DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211].

-	dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset.

-	dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource.

-	dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS associated with the same dl-PRS-ID, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell.

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

========================= Unchanged parts =========================