3GPP TSG RAN WG1 #108-e			                   R1- 2201095

e-Meeting, February 21st – March 3rd , 2022

Source:	vivo

Title:	Maintenance on enhancements for DL-AoD method

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In RAN# 91e meeting, a revised WID on NR Positioning Enhancements was approved [1]. From the angle-based positioning perspective, the WI includes the following objective:

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3] - UL AoA for network-based positioning solutions. - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

In this contribution, we present our views on DL-AoD enhancement.

AoD enhancement with the first path

AoD enhancement with timing information

Agreement(RAN1#104b-e): - For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path - Option 1: Information corresponds to PRS-RSRP of the first arriving path - Option 2: Information corresponds to the angle of departure of the first arriving path - Option 3: Information corresponds to the arrival time of the first path - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path - FFS: Reporting of additional path to the first arriving path. - FFS: Measurement definition details - FFS: additional assistance data to support these enhancements - FFS: how the “first path” is selected among PRS resources in a PRS resource set - Note 1: Supporting multiple options as well as none of the options above is not precluded.

In this section, we only want to discuss option 3 besides option 1 in Rel-17 AoD enhancement. We acknowledge option 3 may be beneficial to identify the same path from multiple resources if multiple path RSRP(s) are reported for a resource when there is no time drift and UE moving. But, only the first path RSRP in Rel-17 is introduced for DL-AoD, how to ensure the path power from multiple resources is from to a path can be resolved by UE implementation that UE knows the arrival time. Otherwise, it’s hard to believe the judgment on the LMF side based on the timing report from UE can be more accurate than UE. In another word, if the timing is changed due to timing drift, the UE has more information to decide whether the path is the same or not. So, we don’t think reporting timing information is necessary for Rel-17 AoD positioning.

In addition, based on the definition and characteristics of AoD positioning, AoD positioning is to make use of the measured DL-PRS-RSRP for positioning. We prefer to retain this feature that is not affected by timing information.

And we would like to note that hybrid positioning has been supported for DL-TDOA and multi-RTT to support RSRPP reporting. We don’t think hybrid positioning is also needed for DL-AoD.

Agreement - Support the LMF to request DL PRS-RSRPP together with timing measurement as part of DL-TDOA and multi-RTT reporting enhancements - Note: This applies to the first path and also to additional paths. - Support the LMF to request UL SRS-RSRPP together with timing measurement as part of UL-TDOA and multi-RTT reporting enhancements - Note: This applies to the first path and also to additional paths.

So, we propose

- Only support first path RSRP reporting in DL-AoD positioning, and reporting multipath RSRP(s) are not introduced for DL-AoD in Rel-17.

- Reporting timing information is not introduced for DL-AoD in Rel-17.

Path RSRPP

Proposal 1.4b For DL PRS RSRPP, if receiver diversity is in use by the UE, the one or both of the following options are supported by the UE to LMF according to a UE capability: - Option 1 (RX diversity for the first path DL PRS RSRPP) - The same RX branch(es) as applied for the first path DL PRS RSRPP measurements are used for the additional paths DL PRS RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS RSRPP measurements, then reported DL PRS RSRPP value for the first path shall not be lower than the corresponding DL PRS RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for DL PRS RSRP) - The same RX branch(es) as applied for DL PRS RSRP measurements are used for DL PRS RSRPP measurements (i.e., the first and additional paths DL PRS RSRPP if those are provided) RAN4 Agreements: - Same Rx branches as applied for PRS-RSRP measurement are used for path PRS-RSRP measurement - Update the RSRPP definition to reflect that it is the path power on RE level - PRS-RSRPP measurement requirements in Rel-17 are defined for first path (i=1). - Note: RAN4 understanding that PRS-RSRPP definition is generic and applicable to all paths (i≥ 1). - Sent following LS reply to RAN1 containing above agreements: - R4-2202681, LS reply on definition of DL PRS path RSRP, Nokia RAN4 LS 2202775 The PRS-RSRPP can be reported in ways below : - RAN4 understands that PRS-RSRPP should be defined as the path RSRP per RE, in the same way PRS-RSRP is defined as power per RE. RAN4 respectfully asks RAN1 to confirm if this understanding is correct. - PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. - When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting. FG 27-2-1 [Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource]

There is an FFS note in the FG 27-2-1[2], which is about the associate PRS resource between PRS-RSRP and PRS-RSRPP. Before making a decision, we first analyze the impact. In our view, the relationship between PRS- RSRP and PRS-RSRPP may impact the following two aspects.

- Aspect 1: Rx diversity between PRS-RSRP and PRS-RSRPP

- Aspect 2: Normalization (relative to PRS-RSRP or PRS-RSRPP)

If the PRS-RSRP is reported for the PRS resource, the Rx diversity of PRS-RSRPP is deterministic and the same as the PRS-RSRP measurement based on the RAN1 and RAN4 agreement. Thus, UE does not need to compare multiple Rx diversity for Path RSRP measurement. In addition, differential reporting for PRS-RSRPP can be reported as the difference in dB with respect to the PRS-RSRP since it has been supported. In this stage, the protocol modification of RAN2 is also minimal as the following yellow highlighted. But, in this case, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource.

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15								OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15						OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126),

nr-DL-PRS-RSRPP-ResultDiff-r17		INTEGER (0..30),                       OPTIONAL,

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

nr-DL-AoD-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16	OPTIONAL,

...

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..30),

nr-DL-PRS-RSRPP-ResultDiff-r17		INTEGER (0..30),                       OPTIONAL,

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

...

}

Otherwise, if the PRS resource of path RSRP reporting is different from that of PRS-RSRP, the Rx branch selection is independent of PRS-RSRP measurement. UE should perform Rx branch selection by RSRPP measurement, and the normalization cannot be relative to PRS-RSRP since PRS-RSRP may not be reported. And then the new reporting IE (ie, nr-DL-AoD-RSRPP-MeasList-r16) and resource associations are required to be introduced as following yellow highlighted.

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

nr-DL-AoD-RSRPP-MeasList-r16			NR-DL-AoD-RSRPP-MeasList-r16,

...

}

In short, we acknowledge Rx branch selection by PRS-RSRPP directly may be beneficial for RSRPP-based positioning. But, considering the complexity of signaling and UE measurement, we slightly prefer to report PRS-RSRP is the prerequisite of the first path RSRPP of PRS resource, and then the Rx branches and normalization can refer to PRS-RSRP.

Agreement For reporting of DL PRS RSRPP and PRS RSRP in UE-A DL-AOD - The maximum number of DL PRS RSRPP M is a UE capability and its candidate values include {2,4,8,16,24}. - The capabilities for DL PRS RSRPP (M value) and DL PRS RSRP (N values) are such that M is less than or equal to N

Furthermore, the candidate value of 13-5 includes the {1,2,3,4,5,6,7,8} and the candidate value of 27-2-2 includes the {16, 24} so that the N can be {1,2,3,4,5,6,7,8, 16, 24}. However, M only includes [2,4,8,16,24]. So, in some cases, the path RSRP capability may be larger than PRS RSRP capability. However, the above agreement requires that M is less than or equal to N. Therefore, the current candidate value is not enough, at least 1 should be added to the candidate value to satisfy the agreement condition.

Based on the above analysis, we propose.

- PRS-RSRP reporting is the prerequisite of  the first path of the PRS resource:

  - Normalization: differential PRS-RSRPP reporting is reported as the difference in dB with respect to PRS-RSRP.

  - Same Rx branches as applied for PRS-RSRP measurement are used for PRS-RSRPP measurement.

- Value {1} should be added to the candidate value of the maximum number of PRS-RSRPP.

TRP Beam/antenna information

In RAN1#106-e [3] meeting and RAN2# 116-bis meeting [4] , the following agreement has been reached.

Agreement From the RAN1 perspective, for the TRP beam/antenna information to be optionally provided by the LMF to the UE for UE-based DL-AoD: - The LMF provides the quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - Note: the peak power per angle is not provided - Note: up to RAN3 to decide how the TRP beam information is provided to the LMF for both UE-assisted and UE-based - Send an LS to RAN2/RAN3 to decide on the signaling details RAN2 Agreements Proposal 2.1-1: enhance LPP assistance data signalling to allow UE to request and LMF to provide TRP beam/antenna information.

In our view, the angle range can be the maximum angle space of the TRP considering the case of beam shape for antenna elements separation d bigger than half-wavelength. In that case, it has multiple beams for one resource [5] [6], and then is difficult to restrict the multiple boresights in a fixed window. So, we propose the angle range can be [-90, 90] for omnidirectional antenna in horizontal and vertical directions.

Figure 1 Radiation pattern with antenna elements separation equals to two wavelength

In addition, for angle granularity, we evaluate the performance gain for different angle granularities (such as 0.5, 1, 2, 4 degrees) in Figure 2. Based on the evaluation, it is observed that the 0.5 and 1 degree can achieve sub-meter positioning accuracy. And the performance will be improved with the granularity reduction. So, in our view, to balance the performance and overhead, multiple granularities can be supported, and 0.5 and/or 1 degree granularity should be supported at least for high accuracy positioning,

Figure 2 The positioning performance with different angle granularity

Lastly, the maximum number of angles per TRP can be {360, 180, 90, 45} for different angle granularities (0.5, 1, 2, 4 degrees).

So, we propose:

- Support the following angle range, the maximum number of angles per TRP for relative Power/Angle response

  - [-90, 90] for omnidirectional antenna in horizontal and vertical direction.

        - 0 degree is represented as the boresight angle of the TRP.

  - The maximum number of angles per TRP can be {360, 180, 90, 45} for omnidirectional antenna in the horizontal and vertical direction.

Meanwhile, the quantization accuracy of relative power can refer to the reporting range of differential PRS-RSRP as defined from -30 dB to 0 dB with 1 dB resolution in TS 38.133.

- Support the quantization accuracy of relative power refers to the reporting range of differential PRS-RSRP as defined from -30 dB to 0 dB with 1 dB resolution in TS 38.133.

Subset measurement reporting

Agreement For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) reporting, the LMF may indicate in the assistance data (AD), one or both the following: - option 1: subject to UE capability, for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting: - a UE may include the requested PRS measurement for the subset of the PRS in the DL-AoD additional measurements if the requested PRS measurement of the associated PRS is reported - The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. - UE may report PRS measurements only for the subset of PRS resources. - Note: The subset associated with a PRS resource can be in a same or different PRS resource set than the PRS resource - option 2: subject to UE capability, for each PRS resource, the boresight direction information. - Note: Either case does not imply any restriction on UE measurement - FFS: prioritization of the PRS resources and resource subsets to be measured

Based on the above agreement in the RAN1#107-e meeting [8], the UE may include the requested PRS measurement for the subset of the PRS in the DL-AoD additional measurements if the requested PRS measurement of the associated PRS is reported.

Considering only enhancing the assistance data can not make sure the subset will be requested or reported, we prefer to add a new request signaling from LMF at least, so that UE can be requested to report the associated subset measurement results. Meanwhile, the following component 3 in FG 27-20 is needed since only enhancing the assistance data can not make sure the subset will be reported.

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. [2. Supported resource set relationship for the target PRS resource and the associated subset: {sameSet, DifferentSet, sameOrDifferentSet}] [3. Support associated subset measurement reporting] |
| --- | --- | --- | --- |

- Add a new request signaling at least from LMF to UE for subset measurement reporting.

- Component 3 in FG 27-20 should be supported.

Expect AOD/AOA and uncertainty

Agreement For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported - Indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s) is signaled by the LMF to the UE - The type of expected angle and uncertainty can be requested by the UE, between the following options - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE Agreement: - Uncertainty range for expected UL AoA/ZoA is defined as follows - Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) - φAOA - expected azimuth angle of arrival, ΔφAOA – uncertainty range for expected azimuth angle of arrival - Expected zenith angle of arrival as (θAOA - ΔθAOA/2, θAOA + ΔθAOA/2) - θAOA - expected zenith angle of arrival, ΔθAOA – uncertainty range for expected zenith angle of arrival - Select one of the following coordinate system alternatives for signaling UL AoA/ZoA assistance information - Alt.1: Only GCS is supported for AoA/ZoA assistance information indication - Alt.2: Both GCS and LCS are supported for AoA/ZoA assistance information indication - FFS: Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range) Agreement: Granularity of 0.1 degrees is applied for the expected AoA (φAOA), expected ZoA (θZOA ) and the corresponding uncertainty values

The granularity and reference angle of expected DL-AoD/ZoD value and uncertainty

In the previous RAN1 meeting, the granularity of 0.1 degrees is applied for the expected AoA (φAOA) in UL AoA. But for DL-AoD positioning, we wonder such a fine-granularity angle window can be used by UE since the window is only used to select PRS resource or Rx beam, and the window easily be changed as the UE moves. Therefore, in our view, the granularity can extend to 1 degree at least considering the function, accuracy of expected AoD for moving UE, and the signaling overhead.

- The granularity can extend to 1 degree at least for the expected angle and uncertainty of AoD positioning.

In addition, the definition of reference angle can be in both the global coordinate system (GCS) and the local coordinate system (LCS) in UL AoA. In the GCS, the reference direction of AoA is the geographical North and is positive in a counter-clockwise direction, while in the LCS, the reference direction of AoA is x-axis of LCS and positive in a counter-clockwise direction. The reference of ZoA is also given in the following.

| Definition | UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE. |
| --- | --- |

But, the LCS of UE is difficult to track, so we prefer only GCS is supported for expected DL-AoD/ZoD value and uncertainty in DL-AoD positioning

- Only GCS is supported for reference angle for expected angle and uncertainty of DL-AoD positioning.

TRP vs ARP

TS 38.305 3.1 Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality.

TS 37.355 6.4.3 For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set. DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE { dl-PRS-ResourceSetARP-r16 RelativeLocation-r16 OPTIONAL, -- Need OP dl-PRS-Resource-ARP-List-r16 SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OF DL-PRS-Resource-ARP-Element-r16 OPTIONAL, -- Need OP ... } DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE { dl-PRS-Resource-ARP-location-r16 RelativeLocation-r16 OPTIONAL, -- Need OP ... }

In RAN1#104e meeting agreement, TRP is in the bracket. It is a consensus that [TRP] should represent the same location entity. The discussion point is on whether should be TRP or ARP (which is a collocated set of PRS resources).

In the current specification, there are two entities that related to the same location. One is TRP, in TS 38.305, which is defined as a set of geographically co-located antennas. Another is ARP, which has not been discussed in RAN1 and defined in TS 38.305, but is used in TS 37.355 where the ARP is defined as a sub-element of TRP. That is, for each TRP, the multiple ARP location can be provided for associated PRS Resource ID per PRS Resource Set. The typical Exemplary of ARP is shown in Figure 3.

Figure 3 Exemplary of ARP and associated of PRS resources

Introducing ARP may be beneficial for high accuracy positioning since the antenna spacing is 10cm level. But for expected AoD, the benefit of introducing expected AoD per ARP is difficult to understand since it is a coarse estimation and with a search window.

In addition, in Rel-16, the expected RSTD is provided to the UE for each TRP. And we don’t see the reason for the expected AoD provided per ARP to be different with the expected RSTD. It is noted there is no parameter is related to ARP in NR-DL-PRS-AssistanceData. That is, ARP is only used in UE-B assistance data. The architecture of NR-DL-PRS-AssistanceData may need to be revised if we support providing expected AoD for each ARP. So we prefer expected AoD is provided to the UE for each TRP.

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-DL-PRS-SFN0-Offset-r16 NR-DL-PRS-SFN0-Offset-r16, nr-DL-PRS-ExpectedRSTD-r16 INTEGER (-3841..3841), nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 INTEGER (0..246), nr-DL-PRS-Info-r16 NR-DL-PRS-Info-r16, ... }

- Expected DL-AoD is provided to the UE for each TRP instead of ARP.

Conclusion

In this contribution, we discuss DL-AoD enhancements with the following proposals:

- Only support first path RSRP reporting in DL-AoD positioning, and reporting multipath RSRP(s) are not introduced for DL-AoD in Rel-17.

- Reporting timing information is not introduced for DL-AoD in Rel-17.

- PRS-RSRP reporting is the prerequisite of  the first path of the PRS resource:

  - Normalization: differential PRS-RSRPP reporting is reported as the difference in dB with respect to PRS-RSRP.

  - Same Rx branches as applied for PRS-RSRP measurement are used for PRS-RSRPP measurement.

- Value {1} should be added to the candidate value of the maximum number of PRS-RSRPP.

- Support the following angle range, the maximum number of angles per TRP for relative Power/Angle response

  - [-90, 90] for omnidirectional antenna in horizontal and vertical direction

        - 0 degree is represented as the boresight angle of the TRP.

  - The maximum number of angles per TRP can be {360, 180, 90, 45} for omnidirectional antenna in the horizontal and vertical direction

- Support the quantization accuracy of relative power refers to the reporting range of differential PRS-RSRP as defined from -30 dB to 0 dB with 1 dB resolution in TS 38.133.

- Add a new request signaling at least from LMF to UE for subset measurement reporting.

- Component 3 in FG 27-20 should be supported.

- The granularity can extend to 1 degree at least for the expected angle and uncertainty of AoD positioning.

- Only GCS is supported for reference angle for expected angle and uncertainty of DL-AoD positioning.

- Expected DL-AoD is provided to the UE for each TRP instead of ARP.

- References

- 3GPP RP-210903, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, March 16 - 26, 2021

- R1-2200781 LS on updated Rel-17 RAN1 UE features list for NR, RAN1, AT&T, NTT DOCOMO

- Chairman’s notes for 3GPP TSG RAN WG1 106 e-Meeting, August 16th – 27th, 2021

- Chairman’s notes for 3GPP TSG RAN WG2 116-bis-e-Meeting, January 17th – 25th, 2022

- R1-2107591 Remaining Details of NR Positioning UL-AoA Enhancements, Intel

- R1-2100237 Enhancement for UL AoA positioning, Huawei, HiSilicon

- Chairman’s notes for 3GPP TSG RAN WG1 106 bis e-Meeting, October 11th – 19th, 2021

- Chairman’s notes for 3GPP TSG RAN WG1 107 e-Meeting, Nov 11th – 19th, 2021