3GPP TSG RAN WG1 #108-e			                     R1-2201098

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Maintenance on inactive state positioning and on-demand PRS

Agenda Item:	8.5.6

Document for:	Discussion and Decision

Introduction

At the RAN#91e meeting, the WID on NR Positioning Enhancements RP-210903[1] was approved including the following objective related to inactive positioning and on-demand PRS:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state - Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3] - UE-initiated request of on-demand DL PRS transmission; - LMF (network)-initiated request of on-demand DL PRS transmission;

In this contribution, we will present our views on inactive positioning and on-demand PRS.

Positioning in inactive state

DL positioning in inactive state

In RAN1#107-e[2], the following agreements were achieved. In this section, DL positioning in inactive state is discussed.

Agreement For UE in RRC_INACTIVE state can support DL PRS processing outside and inside of the initial DL BWP: - For DL PRS processing outside of the initial DL BWP, the SCS, CP type of DL PRS can be the same or different as for the initial DL BWP - For DL PRS processing inside of the initial DL BWP, the SCS, CP type of DL PRS is the same as for the initial DL BWP. - Potential impact of retuning time and expected RSTD assistance information on DL PRS reception performance is up to RAN4 - UE capability(ies) will be defined for DL PRS processing in RRC_INACTIVE state - details are FFS - Send an LS to RAN4 on agreed by RAN1 UE behavior for reception of DL PRS in RRC INACTIVE state

PRS collision with other DL signals/channels

In inactive state, in addition to measuring the PRS, the UE would also perform cell search and initial access related procedures. Therefore, collisions between the PRS and these signals are also a problem. Due to no measurement gap is configured for PRS measurement in inactive state, hence, when PRS and other DL signals (e.g. SSB, SIB1, COREST0, MSG2/MSGB, paging, DL SDT, etc.) collide, how the UE processes the PRS needs to be considered. It is concluded that in RRC_INACTIVE state, reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT), which is also captured in TS38.214 [3] as the following.

The UE in RRC_INACTIVE mode is expected to prioritize the reception of any other DL signal than the reception of DL PRS.

However, what is ‘prioritize the reception of any other DL signal than the reception of DL PRS’? It is still not clear the behaviour of UE processing PRS, e.g., whether UE can process PRS and other DL signals/channels simultaneously or UE is not expected to process PRS when conflict occurs. From our point of view, as in inactive state, UE is in a low power state with limited capability, it is better for UE to not process PRS and other DL signals/channels simultaneously. Therefore, considering reception of DL PRS has lower priority, when time domain overlapping between PRS and other DL signals/channels occurs, UE is not expected to process PRS.

In addition, we also need to clarify the conflicts (or collision) in DL PRS and other DL signals/channels reception by UE. There may be 2 cases as following.

- Case 1: PRSs are within initial DL BWP and have the same SCS as initial BWP. In this case, the time domain occupation of PRS is determined by PRS symbol or slot (up to UE symbol/slot level buffering capability) occupancy considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty. For example, in the following figure, N symbols/slots are occupied by PRS measurement. Therefore, up to UE buffering capability type (symbol/slot level buffering capability), in inactive state, when time domain overlapping between PRS and other DL signals/channels occurs, UE is not expected to process PRS in the overlapping symbol or slot.

Figure1 Conflicts in DL PRS and other DL signals/channels (case1)

- Case 2: PRS are allocated in different BW and/or have the same/different SCS as initial DL BWP. Different with case1, in case 2, the switch time (e.g. RF retuning time) before/after PRS measurement is needed, which is similar to the switch time (0.5ms for FR1, 0.25ms for FR2) for MG mechanism. For example, if UE is expected to measurement PRS in case 2, UE needs the switch time to switch from the initial DL BWP to the PRS measurement. During the switch time, UE is not able to process PRS and receive other DL signals/ channels. Therefore, in this case, UE is not expected to process PRS within a time duration, which includes the occupation time of other DL signals/channels and a gap (0.5ms for FR1, 0.25ms for FR2) before/after the DL signals/channels. For example, in the following figure, UE is not expected to process PRS in slot/symbol j and j+N+1, as slot/symbol j and j+N+1 are overlapping with other DL signals/channels and a gap (or switch time).

Figure2 Conflicts in DL PRS and other DL signals/channels (case2)

In addition, if certain PRS symbols/slots are dropped, it will affect the UE to obtain complete positioning measurement results and RAN4 measurement period requirement. From our perspective, it is better for RAN4 to consider the impact of measurement period requirement when conflicts in DL PRS and other DL signals/channels occur.

Therefore, we propose

- In inactive state, when time domain overlapping between PRS and other DL signals/channels occurs, UE is not expected to process PRS, including

- When PRSs are within initial DL BWP and have the same SCS as initial BWP, UE is not expected to process PRS in the symbols/slots which are overlapping with other DL signals/channels.

- When PRSs are allocated in different BW and/or have the same/different SCS as initial DL BWP, UE is not expected to process PRS in the symbols/slots which are overlapping with other signals/channels and the gap (0.5ms or 0.25ms before/after other signals/channels).

- Note: The time domain occupation of PRS is determined by PRS symbol/slot occupancy considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty.

Applicablity of PRS processing window in inactive state

In the LS[4] as the following, RAN4 kindly asks RAN1 the applicability of PRS processing window in inactive state.

Based on previous RAN1 LS, RAN1 has introduced a PRS processing window to support PRS measurements outside measurement gaps. But when RAN4 is working on the PRS measurement requirements in RRC_INACTIVE state, it is unclear whether the PRS processing window can be applied in RRC_INACTIVE state, which may have impact on the measurement requirements. So RAN4 would like to ask RAN1 for the clarification on the following question: Q1: Whether the PRS processing window defined for PRS measurements outside measurement gaps can be also applied for PRS measurements in RRC_INACTIVE state? RAN4 kindly asks RAN1 to provide feedback on the question above.

We believe that the PRS Processing window does not need to be applied for PRS measurement in inactive state with the following reasons.

- Firstly, PRS processing window is introduced for latency reduction feature. For example, when UE measures PRS within a PRS processing window, if it determines that PRS processing with higher priority than other signals/channels, it can concentrate on processing PRS, while interrupting the reception of other signals/channels. Meanwhile, the PRS processing window is configured/activated by the serving gNB, so that the gNB can flexibly coordinate other DL signals /channels scheduling to minimize confict with PRS processing. However, in inactive state, PRS reception has lower priority than other DL signals/channels, which means that PRS processing does not affect the reception of other signals/channels. Therefore, in inactive state, there is no need to introduce a PRS processing window to coordinate PRS processing with the reception of other signals/channels.

- Then, in inactive state, there is no MG and it is up to UE implementation to measure PRS outside of the initial DL BWP. Similarly, regarding PRS measurement inside of the intial DL BWP if PRS matches the initial DL BWP, it is also natural to measure PRS up to UE implementation. For example, in inactive state, the serving gNB does not need to determine whether to use the MG or the PRS processing window for PRS measurement and there would be no signalling for MG/PRS processing window configuration/activation in inactive state

Therefore, we propose

- PRS processing window is not supported in inactive state.

PRS measurement and inactive DRX

RAN4 has discussed the positioning measurement delay requirement in inactive state, and the following conclusion regarding inactive DRX was achieved based on the LS[5].

RAN4 would like to inform RAN2 and RAN3 about the following agreements that it has reached regarding UE requirements for NR positioning measurements performed while the UE is in RRC_INACTIVE state. Agreement - DRX cycle should be considered in the positioning measurement delay requirements in RRC_INACTIVE state.. RAN4 also observes that DRX cycle may be different in different cells. RAN4 is further discussing the impact of DRX cycle on positioning measurement delay requirements under cell reselection in RRC_INACTIVE state.

It can be seen that DRX cycle is considered in the positioning measurement delay requirement, e.g. DRX cycle is used to calculate available PRS period via LCM(DRX cycle, PRS period). Considering that the value of the DRX cycle is relatively large, such as a typical value of 1280ms, this will cause a large delay in the PRS measurement in inactive state. Then, for some positioning services with specific latency requirements, the influence of inactive DRX cycle on the latency should be considered. For example, if the inactive DRX cycle very likely causes the UE to fail to meet the latency requirement(e.g. response time), it is necessary to avoid the UE from entering the inactive state as much as possible or set looser latency requirement; and if the impact of the inactive DRX cycle on the latency is not enough to affect the latency requirement, there is no need to intervene in the state of the UE.

Specifically, the LMF may obtain inactive DRX configurations(e.g. DRX cycle, etc.) from cells in advance; then, considering these DRX configurations and other factors (e.g. QoS requirement, UE capabilities, etc.), it may indicate some assistance information to the serving gNB (e.g. assistance information to keep UE in connected state for positioning), or set a looser latency requirement. It is also noted that, DRX cycle may be different in different cells. Therefore, the cells that provide DRX configuration can be not only the serving cell, but neighboring cells around the UE which may be reselected in inactive state, such as cells in the RNA area.

- Support the LMF to request inactive DRX configurations (e.g. DRX cycle, etc.) from the cells including UE serving cell and neighboring cells that may be reselected.

UL positioning in inactive state

In RAN1#107-e, the following agreements were achieved. In this section, remaining issues for SRS transmission in inactive state are discussed.

Agreement - For spatial relation of SRS for positioning by RRC_INACTIVE UEs, - Validity criteria for pathloss measurement (OLPC) is reused to determine validity of spatial relation for configured RS - If the UE determines that the UE is not able to meet the above validity criteria for spatial relation then the UE stops transmission of SRS resource for positioning - Note: the RS for spatial relation is a periodic or semi-persistent RS Agreement For OLPC of SRS for positioning by RRC_INACTIVE UEs, the following UE behaviour is used - If the UE determines that it is not able to accurately measure pathloss for pathloss reference RS, UE stops transmission on corresponding SRS resource set for positioning Agreement - The following options are supported for SRS for positioning transmission by RRC_INACTIVE UEs: - Option 1: - Subject to UE capability (which is a prerequisite for option 2), a UE may be configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP. - Option 2: - Subject to UE capability, a UE may be configured with an SRS for Positioning where the following parameters are additionally configured for the transmission of the SRS for Positioning during the RRC_INACTIVE state: frequency location and bandwidth, SCS, CP length. - The UE shall not transmit the SRS for Positioning when it is expected to perform UL transmissions in the initial UL BWP in RRC_INACTIVE state. - RAN1 assumes that - SRS for positioning for UEs in RRC_INACTIVE state is configured using the SRS-PosResourceSet IE - Send LS to RAN2 to define signaling for SRS for positioning configuration for RRC_INACTIVE UEs

SRS beam sweeping enabling in inactive state

Regarding SRS spatial relation that supported in inactive state, there may be two ways that can be applied to SRS transmission.

- One way is to configure SRS spatial relation with another RS, such as SSB or PRS. However, due to spatial relationships are not continuously maintained as in connected state, this way needs to consider the problem of spatial relationship failure (that is, the validation of spatial relation agreed) caused by UE movement. And if the UE determines that the UE is not able to meet the above validity criteria for spatial relation then the UE stops transmission of SRS resource for positioning. This mechanism not only interrupts SRS transmission more frequently, but requires additional measurements by the UE resulting in greater power consumption and complexity in inactive state.

- Another way is to enable SRS beam sweeping in inactive state. In this way, there is no need to consider maintaining the spatial relationship between the UE and gNBs, correspondingly, the validation issues of spatial relation RS no longer exists, which makes SRS beam sweeping especially suitable for inactive state. For example, the reliability of SRS transmission in inactive state will be increased since the problem of spatial relationship failure that causes SRS transmission to stop will not exist; the additional measurement for validation determination will be no longer needed, which is beneficial to power consumption and complexity.

Therefore, enabling SRS beam sweeping in inactive state should be supported.

However, based on current specification as following, when there is no ‘spatialRelationInfoPos’ included in the inactive SRS configuration, it is still up to UE to use fixed beam or beam sweeping. Therefore, additional indicator in SRS configuration to enable SRS beam sweeping in inactive state is needed.

If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources.

- If enabling SRS beam sweeping is enabled in inactive state, the following benefit will be obtained

- the reliability of SRS transmission in inactive state will be increased since the problem of spatial relationship failure that causes SRS transmission to stop will not exist

- the additional measurement for validation determination will be no longer needed, which is beneficial to power consumption and complexity.

- Support to enable SRS beam sweeping in inactive state.

- Additional indicator in SRS configuration to enable SRS beam sweeping in inactive state is needed.

- Adopt the following TP to enable SRS beam sweeping in inactive state.

TS 38.214, section 6.2.1.4 < Unchanged parts are omitted > If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources. If the UE is not configured with the higher layer parameter spatialRelationInfoPos but configured with the higher layer parameter ‘srsBeamSweeping’ in RRC_INACTIVE state SRS configuration, it is expected to use a different spatial domain transmission filter across multiple SRS resources in RRC_INACTIVE. < Unchanged parts are omitted >

On-demand DL PRS

In RAN1#106-bis and RAN1#107-e meeting, the following agreement regarding on-demand PRS were reached.

Agreement: - The following list of parameters is supported for UE-initiated and LMF initiated on-demand DL PRS request - Start/end time of DL PRS transmission - DL PRS resource repetition factor - Number of DL PRS resource symbols per DL PRS resource - DL-PRS CombSizeN - Number of DL PRS frequency layers - ON/OFF indicator (for LMF initiated request only) - FFS values for requested on-demand DL PRS parameters and whether parameters are resource-specific, TRP-specific, or PFL-specific

Agreement - From RAN1 perspective, for UE-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signalled - per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - per FR - Number of DL PRS frequency layers - per UE - Start/end time of DL PRS transmission - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource per resource set per positioning frequency layer per FR - UE recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - UE requests to provide the QCL information in the assistance data

Parameters of on-demand PRS

Remaining on-demand DL PRS parameters

In last RAN1 e-meeting, some remaining parameters for on-demand PRS were discussed but not reach an agreement. In this section, we discuss the remaining parameters need to be supported for UE-initiated and LMF-initiated on-demand DL PRS requests.

UE-initiated on-demand PRS

For UE-initiated on-demand PRS, except the already agreed parameters, the following parameters may be also considered.

Number of TRPs

- From the perspective of increasing accuracy, the number of TRPs is also a parameter for improving accuracy. For example, different numbers of TRPs for positioning in different LoS conditions will lead to different performances. So, UE can indicate the preferred TRP number to increase accuracy.

- In addition, from the perspective of power saving, by reducing the number of TRPs to be measured, the power saving gain is obtained. Therefore, when power saving is needed, UE can also indicate preferred number of TRPs to LMF.

- For UE-initiated on-demand PRS, as UE does not know specific TRP information, it is more reasonable for UE to request the number of TRPs rather than specific TRP indicator information.

Beam related information (beam directions)

- Although it has been agreed that DL PRS QCL information can be requested for on-demand DL-PRS, however, it is not always enough for beam information. One case is that there is no suitable existing signal for QCL to indicate the accurate beam information. Another case is the beam range indicated by QCL information is too coarse and finer granularity of beam information is needed. Therefore, except for the QCL information, we think beam related information should also be included in the on-demand DL-PRS request. At least beam directions can be considered to further provide accurate beam information.

- For beam directions, it can be an accurate angle (like a boresight angle) or an expected angle range (like expected AoD). From the perspective of UE request and gNB configuration, we think it is more reasonable to define the beam direction as a certain angle range. Firstly, usually UE request for on-demand PRS cannot know its accurate location, thus may not estimate the expected angle so precisely, which may cause a beam angle error. Secondly when gNB receive the accurate angle, it cannot configure this angle perfectly, and it will cause an angle error again. But a certain angle range request will reduce the error of angle estimation on UE side and configuration on gNB side.

- If the UE can obtain beam directions of gNBs (e.g. UE-based DL method), based on the previous measurement, it can indicate a more accurate beam directions/range of on-demand PRS resources (just like expected AoD/uncertain) to assist LMF/gNB to turn on/off certain PRS resources to improve network/UE efficiency.

LMF-initiated on-demand PRS

For LMF-initiated on-demand PRS, except the already agreed parameters, the following parameters may be also considered.

Indicator of TRPs (including TRP ID, list of TRP IDs, etc.)

- Unlike the UE-initiated on-demand PRS, LMF has more detailed information about each TRP, especially when LoS/NLoS indicators are reported to LMF, LMF can request for specific TRPs which may be mostly under LoS condition. Therefore, the indicator of TRPs will be more reasonable than number of TRPs for LMF-initiated on-demand DL-PRS request.

Indicator of frequency layers (including frequency layer ID, list of frequency layer IDs, etc.) or DL PRS PointA

- Unlike UE initiated on-demand PRS, we believe the indicator of frequency layers or DL-PRS PointA information is useful for LMF-initiated on-demand PRS request. LMF has more information of the channel condition, including LoS/NLoS, multipaths and power condition, thus can request for a better frequency domain location by indicator of frequency layers or DL-PRS PointA information.

Beam related information (including beam directions, beamwidth, beam granularity, etc.)

- The reason for requesting beam related information for LMF-initiated on-demand DL-PRS is similar as UE-initiated method, which can improve positioning accuracy by indicating the accurate beam information when QCL information is not available or suitable, or indicating the finer granularity of beam information when QCL information is too coarse.

- In addition, to further increase network/UE efficiency, we think LMF can request the gNB to switch on/off PRS resources within a certain direction range. This is similar to already supported expected AoA/uncertain, but expected AoA/uncertain is used to assist gNB reception, and the beam direction here is to assist gNB transmission (just like expected AoD/uncertain).

DL PRS Muting Option 1/2

- For DL PRS muting pattern, it is related to network deployment and interference level of TRPs, we think it is better for LMF to determine from a broader perspective.

- Support the following list of parameters for UE-initiated on-demand DL PRS request

  - Number of TRPs

  - Beam related information

    - Beam related information should be an expected angle range

- Support the following list of parameters for LMF-initiated on-demand DL PRS request

  - Indicator of TRPs

  - Indicator of frequency layers or DL PRS PointA information

  - Beam related information

  - DL PRS Muting Option 1/2

The granularity of on-demand DL-PRS parameters

In this section, we discuss the remaining issues for the granularity of on-demand DL-PRS parameters.

Beam related information

- As we have discussed in Section 3.1.1, the beam related information should be an angle range, which is associated with one on-demand DL-PRS resource. The motivation of beam related information is to provide beam information when there is no suitable existing signal for QCL or QCL information is too coarse. Therefore, we think the request for beam related information can be similar with QCL information. Considering the flexibility of beam information, we prefer the granularity is per resource.

- Two options for indication of beam related information, either

  - Option 1: per resource per resource set per positioning frequency layer per FR

    - UE recommends a beam information

  - Option 2: per resource per resource set per positioning frequency layer per FR

    - UE requests to provide the beam information in the assistance data

ON/OFF indicator of the on-demand PRS

- In our view, the interpretations of ON/OFF request at least include switching ON/OFF certain granularity of PRS configuration, such as per frequency layer, per TRP, per resource set or per resource. For example, when certain TRPs or PRS resources are no longer hearable, a TRP-level or resource-level OFF request/recommend to LMF is needed; then LMF can determine whether to switch ON/OFF these TRPs or PRS resources according to the comprehensive needs of the UEs in the entire region, which may help for the network efficiency and UE power saving. In addition, as the UE moves, in order to ensure the accuracy of positioning, the TRPs or PRS resources that was previously turned off can be requested to be switched on again. Or, the on-demand configurations for certain TRPs or PRS resources are no longer needed based on UE’s demand, an ‘OFF’ indicator to LMF can be requested, and then LMF can determine whether to switch ON/OFF this on-demand PRS configuration. Similarly, when the on-demand configurations of these TRPs or PRS resources are needed, an ‘ON’ indicator to LMF can also be requested.

- Furthermore, for certain granularity of PRS configuration (frequency layer/TRP/PRS resource set/PRS resource) mentioned above, there seems to be two kinds of interpretation for ‘ON/OFF’ indicator. One interpretation of ‘ON/OFF’ indicator is: fully enabling or disabling. For example: for a PRS resource, ‘OFF’ means that a TRP stops transmission of the PRS resource; and ‘ON’ means that a TRP starts the transmission of the ‘PRS resource’. Or for a TRP, ‘OFF’ means that stop PRS transmission of the TRP; and ‘ON’ means that start PRS transmission of the TRP. Another interpretation is: enabling or disabling the feature of ‘on-demand PRS’. For a PRS resource, ‘ON’ means that enable the on-demand feature of the PRS resource, such as change the bandwidth/period of the PRS resource; while ‘OFF’ means that disable the on-demand feature of the PRS resource, such as fallback to regular configuration of the PRS resource. Or for a TRP, ‘ON’ means that enable the on-demand feature of the TRP, such as enable on-demand PRS configuration of the TRP; while ‘OFF’ means that fallback to regular PRS configuration. We believe that these two interpretations are reasonable, and how to support or integrate them can be further studied.

- Support two options for indication of beam related information, either

  - Option 1: per resource per resource set per positioning frequency layer per FR

    - UE recommends a beam information

  - Option 2: per resource per resource set per positioning frequency layer per FR

    - UE requests to provide the beam information in the assistance data

- Support the ON/OFF indicator of the on-demand PRS in the following granularity: per frequency layer, per TRP, per resource set and per resource.

Explicit parameters request of on-demand PRS

In last RAN2 e-meeting, it has been agreed for UE to select one of predefined PRS configurations to request, which is associated with an identifier. And whether UE can request a configuration with explicit parameters or parameters outside the pre-configuration is still under discussion.

Agreement: The network can signal predefined PRS configurations to the UE and the UE can select one to request. FFS if the UE can request a configuration with different parameters and exactly which parameters are flexible. Agreements: Proposal 2: Define a new LPP assistance data IE which can contain a set of possible on-demand DL-PRS configurations, where each on-demand DL-PRS configuration has an associated identifier.

As far as we are concerned, the explicit parameters, at least for UE-initiated on-demand request, should be supported. Compared with predefined on-demand DL PRS, the explicit parameters request has its own advantages.

For downlink positioning, UE have the knowledge of most measurement information, which can help UE to request for on-demand DL-PRS parameters more accurate. For example, UE may measure some DL-PRS resource with different transmission beams and obtain measurement information such as RSRP, through these measurements UE can obtain the AoD or expected beam directions. But for LMF, not all the measured RSRPs are reported to LMF, and the reported quantized RSRP may also introduce performance loss. Thus, the UE can obtain a more accurate expected AoD or expected beam directions of gNBs than LMF.

Therefore, we suggest to support the request of explicit parameters at least for UE-initiated on-demand DL-PRS.

- Support the request of explicit parameters at least for UE-initiated on-demand DL-PRS.

Other issues of on-demand PRS

Interference caused by on-demand PRS to regular UEs

Figure 3 On-demand PRS scenario for different UEs with/without on-demand PRS request (1)

The interference of on-demand PRS are mainly caused by the different configurations. For example, in a certain area, TRPs can transmit the on-demand PRS with a period of 40ms as the low latency demand of some UEs (such as UE1 in Figure 3). However, other regular UEs (such as UE2 in Figure 3) adjacent to this area without on-demand PRS request still expects to receive regular PRS with a period of 80ms. Therefore, the additional instance of small period of on-demand PRS will cause interference of these regular UEs. The interference case is shown in the following Figure 4. In addition to PRS period, similarly, other changed PRS configurations (such as PRS repetition, PRS bandwidth, etc.) by on-demand request will also cause interference to adjacent regular UEs.

Figure 4 Interference caused by on-demand PRS

- Interference caused by on-demand PRS to regular UEs should be considered and solved by RAN1.

To solve the interference problem, one option is to switch off certain PRS resources pointing to regular UEs (such as switching off PRS3 and PRS4 in orange of Figure 3). It is not to completely switch off PRS3/4, but to disable potential configuration change of PRS3/4 and allow PRS3/4 to transmit based on the regular PRS configuration. For example, for PRS1 and PRS2, the TRP can transmit them with period of 40ms; while for PRS3 and PRS4, the TRP still transmit them with 80ms. Furthermore, the LMF can simply indicate resource level muting pattern to UE1 showing which PRS resources’ on-demand feature are switched off.

However, for on-demand UEs and regular UEs which are difficult to distinguish by beam direction, the above option cannot work, as shown in Figure 5. In this case, the LMF may indicate the on-demand PRS configurations of on-demand UE to regular UE, to enable interference cancellation by regular UE itself. In addition, the LMF can inform on-demand PRS configurations to the serving gNB of regular UE, then the serving gNB can schedule other RS/data transmission at right RE locations avoiding potentially disturbing by on-demand PRS.

Figure 5 On-demand PRS scenario for different UEs with/without on-demand PRS request (2)

Therefore, to solve the interference problem, we propose

- To solve the interference caused by on-demand PRS to regular UEs, support switching off certain PRS resources for regular UEs.

- PRS resource level muting can be considered.

- Note: It is not to completely switch off the PRS resources, but to allow the transmission of these PRS resources based on regular PRS configuration

- To solve the interference caused by on-demand PRS to regular UEs, support indicating on-demand PRS configuration to regular UEs and corresponding serving gNB.

On-demand measurement gap

In current specification, when the UE is expected to measure the PRS resource outside the active DL BWP it may request a measurement gap via higher layer parameter NR-PRS-MeasurementInfoList. If on-demand PRS is introduced, the corresponding on-demand measurement gap also need to be introduced.

For example, UE is configured by on-demand PRS with a small periodicity for small latency requirement, but the other PRS configuration remain unchanged, in this case the UE will not require a new measurement gap but still measure and process PRS according to the previous measurement gap configuration, thus failing to achieve the purpose of ‘on-demand’.

To support on-demand measurement gap, both the UE and LMF request procedure should be considered. Based on the progress of AI8.5.4, we believe that for on-demand measurement gap, LMF request procedure can be supported at least for LMF-initiated on-demand PRS.

- Support to introduce on-demand measurement gap for on-demand PRS in Rel-17.

  - LMF requests on-demand measurement gap should be supported.

The on-demand measurement gap can also be configured along with the on-demand DL PRS to further reduce the latency. For example, when the UE/LMF request an on-demand DL PRS, the on-demand measurement gap is also requested. Or after the gNB receives an on-demand DL PRS request, then gNB configures the on-demand measurement gap.

- The on-demand measurement gap can be requested and configured along with the on-demand DL PRS.

  - The on-demand measurement gap can be requested along with the request of on-demand DL PRS.

  - The on-demand measurement gap can be configured after gNB receives the request of on-demand DL PRS.

Procedure of on-demand PRS

When on-demand PRS is introduced, whether the procedure of on-demand DL PRS is different from the existing procedure should be considered. For example, when on-demand DL PRS and regular PRS are both existed, it is unclear how UE measures the on-demand DL PRS and normal PRS, e.g. will the UE perform measurement on the PRS of these two configurations, or only perform measurement on the on-demand PRS? In our opinion, one way is to measure the PRS according to the priority information. Therefore, the priority of on-demand DL PRS and regular PRS should be studied.

- The priority of on-demand DL PRS and normal PRS should be considered.

Conclusion

In this contribution, we discuss issues for inactive positioning and on-demand PRS with the following observation and proposals.

- If enabling SRS beam sweeping is enabled in inactive state, the following benefit will be obtained

- the reliability of SRS transmission in inactive state will be increased since the problem of spatial relationship failure that causes SRS transmission to stop will not exist

- the additional measurement for validation determination will be no longer needed, which is beneficial to power consumption and complexity.

- In inactive state, when time domain overlapping between PRS and other DL signals/channels occurs, UE is not expected to process PRS, including

- When PRSs are within initial DL BWP and have the same SCS as initial BWP, UE is not expected to process PRS in the symbols/slots which are overlapping with other DL signals/channels.

- When PRSs are allocated in different BW and/or have the same/different SCS as initial DL BWP, UE is not expected to process PRS in the symbols/slots which are overlapping with other signals/channels and the gap (0.5ms or 0.25ms before/after other signals/channels).

- Note: The time domain occupation of PRS is determined by PRS symbol/slot occupancy considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty.

- PRS processing window is not supported in inactive state.

- Support the LMF to request inactive DRX configurations (e.g. DRX cycle, etc.) from the cells including UE serving cell and neighboring cells that may be reselected.

- Support to enable SRS beam sweeping in inactive state.

- Additional indicator in SRS configuration to enable SRS beam sweeping in inactive state is needed.

- Adopt the following TP to enable SRS beam sweeping in inactive state.

TS 38.214, section 6.2.1.4 < Unchanged parts are omitted > If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources. If the UE is not configured with the higher layer parameter spatialRelationInfoPos but configured with the higher layer parameter ‘srsBeamSweeping’ in RRC_INACTIVE state SRS configuration, it is expected to use a different spatial domain transmission filter across multiple SRS resources in RRC_INACTIVE. < Unchanged parts are omitted >

- Support the following list of parameters for UE-initiated on-demand DL PRS request

  - Number of TRPs

  - Beam related information

    - Beam related information should be an expected angle range

- Support the following list of parameters for LMF-initiated on-demand DL PRS request

  - Indicator of TRPs

  - Indicator of frequency layers or DL PRS PointA information

  - Beam related information

  - DL PRS Muting Option 1/2

- Support two options for indication of beam related information, either

  - Option 1: per resource per resource set per positioning frequency layer per FR

    - UE recommends a beam information

  - Option 2: per resource per resource set per positioning frequency layer per FR

    - UE requests to provide the beam information in the assistance data

- Support the ON/OFF indicator of the on-demand PRS in the following granularity: per frequency layer, per TRP, per resource set and per resource.

- Support the request of explicit parameters at least for UE-initiated on-demand DL-PRS.

- Interference caused by on-demand PRS to regular UEs should be considered and solved by RAN1.

- To solve the interference caused by on-demand PRS to regular UEs, support switching off certain PRS resources for regular UEs.

- PRS resource level muting can be considered.

- Note: It is not to completely switch off the PRS resources, but to allow the transmission of these PRS resources based on regular PRS configuration

- To solve the interference caused by on-demand PRS to regular UEs, support indicating on-demand PRS configuration to regular UEs and corresponding serving gNB.

- Support to introduce on-demand measurement gap for on-demand PRS in Rel-17.

  - LMF requests on-demand measurement gap should be supported.

- The on-demand measurement gap can be requested and configured along with the on-demand DL PRS.

  - The on-demand measurement gap can be requested along with the request of on-demand DL PRS.

  - The on-demand measurement gap can be configured after gNB receives the request of on-demand DL PRS.

- The priority of on-demand DL PRS and normal PRS should be considered.

- References

- RP-210903-(rev.fr.RP-210897)-clean, Revised WID on NR Positioning Enhancements

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 107, e-Meeting, November 11th – 19th, 2021

- 3GPP TS38.214, V17.0.0, 2021.12

- R4-2202687, LS on the applicability of PPW

- R4-2202686, LS on DRX cycle used in PRS measurement in RRC_INACTIVE state