3GPP TSG RAN WG1 #108-e		R1-2201105

e-Meeting, February 21st - March 3rd, 2022

Source:	vivo

Title:	Remaining issues on PUSCH TB processing over multiple slots

Agenda Item:	8.8.1.2

Document for:	Discussion and Decision

- Introduction

In last RAN1 meeting, most work for TB processing over multiple slots (TBoMS) was completed. We will further discuss the remaining aspects for TBoMS in this contribution.

- Determination of starting bit for each slot of TBoMS

In RAN1#107bis, how to capture the mechanisms for starting bits determination in each slot of a TBoMS is discussed, and the discussions focus on the following aspects [2].

- Aspect 1: The definition of G and E for TBoMS

Two alternatives are provided for definition of number of coded bits available in a slot, as follows.

Alt 1. G is redefined as the total number of coded bits available for transmission of the transport block in a slot Alt 2. A new variable H is introduced, only for TBoMS, defined as the total number of coded bits available for transmission of the transport block in a slot

These two alternatives only have differences on how the spec. is written for the same UE behavior. Alt.1 may lead to less spec impacts compared to Alt.2, due to new variables and changes on equations can be avoided. Hence Alt.1 is slightly preferred.

Observation 1: Alt.1, i.e., G is redefined as the total number of coded bits available for transmission of the transport block in a slot, may lead to less spec impact.

- Aspect 2: The value of G and E for TBoMS

The second aspect is about whether the number of bits selected for interleaving for each slot is determined considering UCI multiplexing.

Interpretation 1. The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing. Interpretation 2. The starting index of circular buffer is determined assuming no UCI multiplexing, and the number of bits being selected in bit selection (value E) is determined assuming no UCI multiplexing.

Both of these two interpretations can work although they may lead to different number of transmitted coded bits and symbols. As is known in slot based PUSCH, the number of bits selected for interleaving is reduced due to number of bits for UCI multiplexing. Hence, interpretation 1 is more aligned with implementation of the slot based PUSCH in current spec.

Observation 2: Interpretation 1, i.e., the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing, matches current implementation logic for UCI multiplexing slot based PUSCH better.

- Aspect 3: Handling of filler bits in TBoMS

The third aspect is whether number of filler bits are considered in starting bit determination, and the following directions are provided.

Direction 1. Filler bits are considered to pre-determine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots. Direction 2. Filler bits are not considered to pre-determine the index of the starting bit for each allocated slot for TBoMS and overlap between bit sequences transmitted over consecutive slots is allowed.

For  both Directions, the position of filler bits after encoding can be predetermined, hence starting bit for each slot can be determined before transmission of the first slot of TBoMS. Direction 1 can make sure there are no overlapping coded bits between coded bits over consecutive slots, which is aligned with the previous agreed option C. Though overlapping coded bits may exists across consecutive slots with Dirction 2, from performance perspective, these two directions are not much different. Hence, either direction is acceptable.

Observation 3: Either direction on handling filler bits can work, and does not make much difference in decoding performance.

According to the observations above, we have the following proposal for capturing option C in spec.

Proposal 1: For capturing Option-C for determination of starting bit for each slot of TBoMS, following Alt/interpretation/direction is preferred.

- Alt 1: G is redefined as the total number of coded bits available for transmission of the transport block in a slot;

- Interpretation 1: The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing.

- On handing filler bits in TBoMS, either Direction 1 or Direction 2 is fine.

- Conclusion

In this contribution, we discussed the potential issues for PUSCH with TB processing over multiple slots. Based on the discussion in previous sections, we have the following observations and proposals:

Observation 1: Alt.1, i.e., G is redefined as the total number of coded bits available for transmission of the transport block in a slot, may lead to less spec impact.

Observation 2: Interpretation 1, i.e., the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing, matches current implementation logic for UCI multiplexing slot based PUSCH better.

Observation 3: Either direction on handling filler bits can work, and does not make much difference in decoding performance.

Proposal 1: For capturing Option-C for determination of starting bit for each slot of TBoMS, following Alt/interpretation/direction is preferred.

- Alt 1: G is redefined as the total number of coded bits available for transmission of the transport block in a slot;

- Interpretation 1: The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing.

- On handing filler bits in TBoMS, either Direction 1 or Direction 2 is fine.

- References

- Chairman’s notes, RAN1#107.

- R1-2200752, Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)