3GPP TSG RAN WG1 #108-e 						      				       R1- 2201116

E-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Remaining issues on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agenda Item:	8.12.3

Document for:  Discussion and Decision

Introduction

RAN1 work on Rel-17 NR multicast and broadcast services has been announced complete in previous meeting. In this contribution, we provide our considerations on maintenance issues for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, including broadcast common frequency resource (CFR) and MCCH/MTCH search space.

Discussion

Broadcast CFR

Currently, there are duplicated descriptions on broadcast CFR in 38.213 [1] by using cfr-Config-MCCH-MTCH and cfr-Config-Broadcast for CFR configuration, the duplication should be removed. The potential modification is provided below:

18	Multicast Broadcast Services

< Unchanged parts are omitted >

A UE can be configured by cfr-Config-MCCH-MTCH an MBS frequency resource for PDCCH and PDSCH receptions providing MCCH and MTCH [12, TS 38.331]; otherwise, the MBS frequency resource is same as for the CORESET with index 0 that is associated with the Type0-PDCCH CSS set for PDCCH and PDSCH receptions providing MCCH and MTCH. A UE monitors PDCCH for scheduling PDSCH receptions for MCCH or MTCH as described in clause 10.1.

In clauses referring to a higher layer parameter value provided by PDCCH-ConfigCommon or PDSCH-ConfigCommon, when applicable a corresponding higher layer parameter value for MCCH/MTCH PDCCH receptions or PDSCH receptions, respectively, is provided as described in [12, TS 38.331].

MCCH/MTCH search space

In RAN2 last meeting, it has decided to include MCCH/MTCH search space configuration of MBS broadcast as part of PDCCH-ConfigCommon [2], and thus, the corresponding changes are needed to replace pdcch-Config-MCCH and pdcch-Config-MTCH in 38.213 [1].

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set on the primary cell of the MCG configured by

-	pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or

-	searchSpaceZero in PDCCH-ConfigCommon, when searchSpaceBroadcast is not provided in PDCCH-ConfigCommon, for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0B-PDCCH CSS set configured by searchSpaceBroadcast in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a MCCH-RNTI or a G-RNTI

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type1A-PDCCH CSS set configured by sdt-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI on the primary cell as described in clause 19.1

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or

-	SearchSpace-Multicast in PDCCH-Config-Multicast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI, or

-

< Unchanged parts are omitted >

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or monitors PDCCH candidates for DCI formats with CRC scrambled by a MCCH-RNTI or a G-RNTI and the UE is provided a non-zero value for searchSpaceBroadcast in PDCCH-ConfigCommon for a Type0/0B-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set, or of the Type0/0B-PDCCH set, respectively, based on the search space set associated with the value of searchSpaceID.

Conclusions

In this contribution, we provide our considerations on Rel-17 maintenance issues for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, including broadcast common frequency resource (CFR) and MCCH/MTCH search space, and the corresponding TPs are provided.

Reference

- R1-2200814 draft CR_38.213-MBS

- R1-2200882 LS on MBS issues