3GPP TSG-RAN WG1 Meeting #107bis-e		R1-2201127

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	UE features for NR sidelink enhancement

Agenda Item:	8.16.11

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our view on the Rel-17 NR sidelink enhancement UE features, based on the updated RAN1 UE features lists in [1][2]. The UE features of Rel-17 NR sidelink enhancement are listed in A1 and A2 for NR and LTE, respectively.

# Discussion

  - Partial sensing and random selection

There are two separate Rel-17 SL UE FGs agreed, i.e., 32-4 and 32-4a, for partial sensing and random selection respectively as shown in Annex. Each FG is defined in a “self-contained” manner, i.e., including every necessary component for sidelink transmission.

Agreement - FG 32-4 is updated as follows - FFS whether capability for synchronization as components or prerequisite or another FG - FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a 32. NR_SL_enh; 32-4; Transmitting NR sidelink mode 2 with partial sensing; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 24) UE can perform periodic-based partial sensing and resource allocation operation. 35) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added. Agreement - FG 32-4a is updated as follows - FFS whether capability for synchronization as components or prerequisite or another FG - FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a 32. NR_SL_enh; 32-4a; Transmitting NR sidelink mode 2 with random resource selection; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added.

In the case that a UE reports both FG 32-4 (i.e., partial sensing) and 32-4a (i.e., random selection), and when FG 32-4 indicates “up to 8 SL processes are supported”, while 32-4a indicates “up to 16 SL processes are supported”, it is then not clear how many sidelink processes are supported by the UE. In our view, it should not be understood as the UE supports up to 8+16=24 sidelink processes.

One possible solution is to report the same number of B for each of FG 32-4, 32-4a and 15-3. However, it precludes the possibility that UE supports a smaller number of full/partial sensing sidelink processes, which is desirable for the UE because a larger number of sensing processes requires larger buffer size and higher power consumption. Therefore, it is proposed that the supported sidelink processes for a given UE is the largest number of B among the reported FGs of 32-4, 32-4a and 15-3.

Proposal 1: The total sidelink processes supported by the UE is the largest number of value B among the reported FGs of 32-4, 32-4a and 15-3.

  - Inter-UE coordination

In the previous meeting, it has been agreed to split the scheme 2 FG (i.e., 32-5b) into two separate UE features, one for Tx capability and the other for Rx capability, respectively.

Agreement FG 32-5b is split into multiple FGs; One for Tx capability and the other for Rx capability as follows 32. NR_SL_enh; 32-5b-1; Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. FFS whether/how to split FG 32-5b into multiple FGs. 32. NR_SL_enh; 32-5b-2; Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2..

Similarly, inter-UE coordination scheme 1 FG (i.e., 32-5a) should be further split into two FGs for Tx and Rx capabilities. A UE may only support receiving the inter-UE coordination information for resource selection, but is not capable to perform resource recommendations for the other UEs, especially for the UE not performing full/partial sensing.

Furthermore, there are two separate sub-schemes defined for scheme 1, namely scheme 1 with preferred resource set, scheme 1 with non-preferred resource set. These sub-schemes are targeting different scenarios and device types. A power-limited or low-end SL UE can only support one sub-scheme (e.g., scheme 1 with preferred resource set), while a normal VUE can support both. Therefore, it is natural and reasonable to further split the FG 32-5a (i.e., Inter-UE coordination scheme 1) into two FGs for two sub-schemes respectively.

Proposal 2: Inter-UE coordination scheme 1 FG (i.e., 32-5a) should be further split, based on Tx and RX capabilities, and based on different sub-schemes, i.e., scheme 1 with preferred resource set and scheme 1 with non-preferred resource set, respectively.

Besides, in the previous meeting it has been agreed to introduce the Rx capability of SCI 2-C for inter-UE coordination scheme 1, where SCI 2-C is used for either explicit request or inter-UE coordination information transmissions.

Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A Agreement The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

Thus, the following capability should be added.

| 32. NR_SL_enh | 32-6 | Reception of SCI 2-C | 1) UE can receiver SCI format 2-C for inter-UE coordination Scheme 1. |  | No | Yes |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Introduce a new FG (e.g., 32-6) for indicating support of SCI format 2-C.

Moreover, according to the following agreement in RAN1#107, the support of option 4 (i.e., conflict determination based on relative RSRP measurement) is subject to UE capability.

Working Assumption A resource pool level (pre-)configuration can enable one of the following options: - Option 1: - (omitted) - Option 4: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource. - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s). - Support of Option 4 is subject to UE capability - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

Therefore, the following UE FG 32-7 should be supported.

| 32. NR_SL_enh | 32-7 | Determination of expected conflict in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations |  | No | No |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Introduce a new FG (e.g., 32-7) for indicating support of conflict determination based on RSRP difference of the two overlapping reservations.

On the other hand, regarding the inter-UE coordination scheme 2, one remaining issue is how to report the Tx and Rx capabilities of PSFCH. Firstly, from implementation perspective, the hardware capability of PSFCH is same regardless of the usage. Secondly, it is desirable that the maximum number of simultaneous PSFCH receptions and/or transmissions is shared for both inter-UE coordination and HARQ-ACK reporting, so that in the case that inter-UE coordination feature is not enabled, the capability of PSFCH receptions is not wasted. Therefore, the PSFCH capability is jointly reported for both inter-UE coordination and HARQ-ACK reporting.

Proposal 5: The max number of simultaneous PSFCH receptions and/or transmissions is jointly reported for inter-UE coordination (scheme 2) and HARQ-ACK reporting.

  - LTE UE feature

The Rel-17 NR sidelink enhancement only targets NR specific enhancements. There is no impact to LTE sidelink, nor any LTE/NR sidelink interaction. Consequently, there should not be any impact to a Rel-17 UE supports LTE sidelink only. For a Rel-17 UE supports both LTE sidelink and enhanced NR sidelink, it should report the Rel-17 related UE features in NR sidelink only. Therefore, in our view the Rel-17 NR sidelink enhancement should have no impact to LTE UE feature.

Observation 1: The Rel-17 NR sidelink enhancement WI has no impact to LTE UE feature.

# Conclusion

In the contribution, we provide our view on UE features for NR sidelink enhancement WI, with the following proposals and observation:

Proposal 1: The total sidelink processes supported by the UE is the largest number of value B among the reported FGs of 32-4, 32-4a and 15-3.

Proposal 2: Inter-UE coordination scheme 1 FG (i.e., 32-5a) should be further split, based on Tx and RX capabilities, and based on different sub-schemes, i.e., scheme 1 with preferred resource set and scheme 1 with non-preferred resource set, respectively.

Proposal 3: Introduce a new FG (e.g., 32-6) for indicating support of SCI format 2-C.

Proposal 4: Introduce a new FG (e.g., 32-7) for indicating support of conflict determination based on RSRP difference of the two overlapping reservations.

Proposal 5: The max number of simultaneous PSFCH receptions and/or transmissions is jointly reported for inter-UE coordination (scheme 2) and HARQ-ACK reporting.

Observation 1: The Rel-17 NR sidelink enhancement WI has no impact to LTE UE feature.

# Reference

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, RAN1#107bis-e.

- R1-2112900, “Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e”, RAN1#107-e.

# Annex.

## A1 NR feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. | 32-5b-2, TBD | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

## A2 LTE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. [NR_SL_enh] | 4-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by LTE Uu. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [4-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by LTE Uu. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [4-1], [4-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [4-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |