3GPP TSG RAN WG1 #108-e			                        R1-2201134

e-Meeting, February 21st – March 3rd, 2022

Source: 	ZTE, Sanechips

Title:	Discussion on UE feature for UE power saving enhancements

Agenda item:	8.15.7

Document for:	Discussion and decision

Introduction

The UE feature list for UE power saving enhancement can be found in the appendix according to the agreements in previous meetings [1-4]. In this contribution, more details are discussed.

Discussion

- 29-1 paging enhancement

As to the type of UE FG 29-1, differentiating the feature among multiple aspects, such as frequency range, TDD/FDD, etc., is not beneficial to exploit the best UE power saving benefits. Therefore, per-UE is sufficient for paging enhancement.

The capability type of feature group 29-1 is per UE.

- 29-2 TRS

Agreement The following working assumption is confirmed Working Assumption - Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

##### TS 38.212 ##### 7.3.1.3.8 Format 2_7 ……… The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – bit(s), where …. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise.

The dynamic indication of TRS availability indication can be conveyed by paging DCI and PDCCH-based PEI according to the agreements.

Furthermore, according to the 38.212[5], the DCI format 2-7 (i.e., PDCCH-based PEI) can be used to carry both paging indication and TRS availability information. The TRS availability information is located after the paging indication field. Considering that when UE does not support paging indication, the start position of TRS availability information is not clear. It is expected that PEI based indication for TRS availability is to be supported if FG 29-1 is supported.

Add a note that PEI based indication for TRS availability is expected to be supported if FG 29-1 is supported by the UE for feature group 29-2.

For 29-2, whether or not the TRS is used for idle or inactive UE is based on UE implementation. In addition, if idle/inactive mode UE does not use TRS, the UE can also use SSB for AGC or time/frequency tracking and without affecting the UE experience. Therefore, there is no strong need for the idle and inactive UE to report the capability.

FG 29-2 is ‘optional without capability signalling’.

The capability type of feature group 29-2 is per UE.

The content of column “Consequence if the feature is not supported by the UE”is “Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode”. As discussed above, whether and how to use TRS in idle or inactive mode is UE implementation, so it is expected to update the description to “UE does not support TRS occasions for idle/inactive UEs”.

Update content of column “Consequence if the feature is not supported by the UE” in FG 29-2 to “UE does not support TRS occasions for idle/inactive UEs”.

- 29-3 PDCCH monitoring adaptation

Feature group 29-3c and 29-3d are agreed in previous meeting. The description of 29-3d is “Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching” is ambiguous. An update as “Support of 2-bit indication of SSSG switching between 2 SSSGs with and PDCCH skipping by scheduling DCI and timer based switching” is proposed.

Update FG 29-3d as “Support of 2-bit indication of SSSG switching between 2 SSSGs and PDCCH skipping by scheduling DCI and timer based switching”.

As to FG 29-3, the capability type should be per UE.

Conclusion

In this contribution, we discuss the UE features for power saving enhancements. We have the following proposals.

Proposal 1:	The capability type of feature group 29-1 is per UE.

Proposal 2:	Add a note that PEI based indication for TRS availability is expected to be supported if FG 29-1 is supported by the UE for feature group 29-2.

Proposal 3:	FG 29-2 is ‘optional without capability signalling’.

Proposal 4:	The capability type of feature group 29-2 is per UE.

Proposal 5:	Update content of column “Consequence if the feature is not supported by the UE” in FG 29-2 to “UE does not support TRS occasions for idle/inactive UEs”.

Proposal 6:	Update FG 29-3d as “Support of 2-bit indication of SSSG switching between 2 SSSGs and PDCCH skipping by scheduling DCI and timer based switching”.

Proposal 7:	As to FG 29-3, the capability type should be per UE.

References

- Draft_Minutes_report_RAN1#107

- Draft_Minutes_report_RAN1#107bis

- R1-2112139. Summary on UE features for UE power saving enhancements.

- R1-2200678. Summary on UE features for UE power saving enhancements

- 38212-h00

Appendix

- Agreements in RAN1#106

Agreement

FG 29-2 is kept as “TRS resources for idle/inactive UEs” as follows.

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS receiving for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 29-1 is kept as placeholder as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

- Agreements in RAN1#107

Agreement

- FG 29-1 is kept as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Component 2 in FG 29-2 for support receiving L1 indication for TRS availability is kept

Agreement

- FG 29-3 is split into multiple FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: 2 search space sets group switching

  - [FG 29-3c]: 3 search space sets group switching

  - [FG 29-3d]: 2 search space sets group switching with PDCCH skipping

  - FFS: A UE reporting 29-3a and 29-3b does not implicitly mean the UE supports FG 29-3d

  - FFS: FG for 3 search space sets group switching with PDCCH skipping

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of PDCCH skipping Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 2 search space sets grouping switching Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 3 search space sets grouping switching Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2 search space sets grouping switching with PDCCH skipping Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

- Agreements in RAN1#107bis

Agreement

- FG 29-3c is confirmed as “3 search space sets group switching” as follows

| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 29-3d is confirmed as “2 search space sets group switching with PDCCH skipping” as follows

| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-23b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |