3GPP TSG RAN WG1 #108-e	R1-2201178

e-Meeting, February 21st – March 3rd, 2022

Title: 	Discussion on Rel-17 UE features for NR MBS

Source: 	ZTE

Agenda item:	8.16.12

Document for:	Discussion/Decision

Introduction

After RAN1#107b-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for MBS.

Discussion

Rel-17 is the first release that supports broadcast and multicast for NR. To facilitate the MBS commercial process, it is beneficial to define some basic feature groups for MBS. For now, FG33-1 and FG33-2 can be set as the basic UE feature group for MBS. We can further merge or add more basic functionalities into FG33-1 and FG33-2 if needed.

Proposal 1: FG 33-1 and FG 33-2 are the basic FG for broadcast and multicast, respectively.

Broadcast

In RAN1#107-e meeting, it has been agreed to have a separate FG for dynamic repetition for multicast, and the maximum value of repetition is 8 or 16. Similar mechanism can be reused for broadcast.

Agreement - Only the capability for semi-static repetition is included in FG 33-2 - For semi-static repetition, UE is mandated to support {2, 4, 8} times repetitions - For dynamic repetition, a separate FG is introduced; UE reports one of the maximum values from {8, 16}

Proposal 2: For broadcast, a separate FG is introduced for dynamic repetition; UE reports one of the maximum values from {8, 16}.

In RAN1#107-e meeting, the following agreements have been reached for RB-level rate-matching for broadcast.

Agreement For broadcast RRC_IDLE/INACTIVE UEs, rateMatchPatternToAddModList can be configured in PDSCH-Config-MCCH or PDSCH-Config-MTCH for GC-PDSCH rate matching. - Whether UE can receive the GC-PDSCH with rate matching based on the rateMatchPatternToAddModList is subject to UE capability. - Rel-15/16 UE capability of the supported maximum number of RE mapping patterns per symbol and per slot are kept unchanged to support rate matching for unicast/multicast/broadcast. The RateMatchPattern configured for MBS broadcast is counted into the ones that are configured per serving-cell.

In Rel-15, semi-static RB-level rate-matching is a mandatory FG with capability signalling. It makes sense to include this into FG 33-1 for broadcast.

|  | 5-26 | Semi-static rate-matching resource set configuration for DL | - Bitmap 1/2/3 - controlResourceSet |  | Yes |  | Type 4 | No need | No need |  |  |  |  | Mandatory with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Introduce “Semi-static rate-matching resource set configuration for broadcast” as a component of FG 33-1.

Multicast

FG 33-2a is expected to be the basic FG for multicast feedback. From our perspective, we have the following proposal for FG 33-2a. It is beneficial to merge at least the PTM retransmission with FG 33-2a to avoid the situation when UE only supports UL feedback but doesn’t support retransmission. Furthermore, separate PUCCH-Config configuration for multicast should also be included in FG 33-2a as it is an essential component for UL feedback. There are two types of codebook for ACK/NACK based HARQ-ACK feedback for multicast, i.e., type-1 and type-2 HARQ-ACK codebook. At least one of them should be merged in FG 33-2a. Otherwise, if different UEs supporting different type of codebooks, it would be difficult for network to group them together. From our perspective, type-2 codebook can be merged with FG 33-2a.

Proposal 4: Add the following components for FG 33-2a.

| 33-2a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling At least 33-2c or 33-2d is merged, FFS which one or both 2. Support of PTM retransmission for multicast 3. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations 4. Support of Type-2 HARQ-ACK codebook for multicast |
| --- | --- | --- |

Another issue is how to capture agreements for FG33-3-3. From our perspective, UE can share the same maximum PDSCH receptions in a slot per CC between unicast and multicast. Thus, the FG33-3-3 can be updated as following.

Proposal 5: Update the components description as following.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. -  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). |
| --- | --- | --- |

Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for MBS with the following proposal.

Proposal 1: FG 33-1 and FG 33-2 are the basic FG for broadcast and multicast, respectively.

Proposal 2: For broadcast, a separate FG is introduced for dynamic repetition; UE reports one of the maximum values from {8, 16}.

Proposal 3: Introduce “Semi-static rate-matching resource set configuration for broadcast” as a component of FG 33-1.

Proposal 4: Add the following components for FG 33-2a.

| 33-2a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling At least 33-2c or 33-2d is merged, FFS which one or both 2. Support of PTM retransmission for multicast 3. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations 4. Support of Type-2 HARQ-ACK codebook for multicast |
| --- | --- | --- |

Proposal 5: Update the components description as following.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. -  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). |
| --- | --- | --- |

Reference

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, RAN1#107b-e, Moderator (NTT DOCOMO, INC.).