3GPP TSG RAN WG1 Meeting #108-e  						              R1-2201183

e-Meeting, February 21th  – March 3rd, 2022

Title:			Draft Reply LS to RAN2 on NTN-specific SIB

Reply to:		LS on NTN-specific SIB (R2-2201757)

Release:		Release 17

Work Item:		NR_NTN_solutions-Core

Source:		RAN1

To:		RAN2

Cc:

Contact Person:

#### Name:	Mohamed EL JAAFARI

#### E-mail Address:	mohamed.el-jaafari@thalesaleniaspace.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto: 3GPPLiaison@etsi.org

Attachments:	None

1. 	Overall Description:

RAN1 would like to thank RAN2 for sending the LS on NTN-specific SIB. RAN1 has discussed the question asked by RAN2 and RAN1 answer is provided below.

Regarding the parameters to be indicated within NTN-specific SIB (SIBx), RAN1 discussed the following two options and accorded a preference to option-1:

Option 1:

The four parameters:

- Ephemeris;

- Common TA parameters;

- Validity duration for UL sync information

- Epoch time

are included within ServingCellConfigCommonSIB information element in SIB1.

Other NTN related parameters are broadcast within newly introduced NTN-specific SIB (SIBx)

Option 2:

All parameters (1) to (9) are jointly broadcast within NTN-specific SIB (SIBx):

- In this case, for NTN-specific SIB si-BroadcastStatus shall be always set to broadcasting. And valueTag field should be excluded when determining changes in system information, i.e. changes of Ephemeris and Common TA parameters should neither result in system information change notifications nor in a modification of valueTag in SIB1.

Further, with option 2, Epoch time linked to Ephemeris and Common TA parameters might not be explicitly indicated and can be implicitly known as the end of the SI window during which the SI message is transmitted as per the following RAN1 agreement made at RAN1#107-e:

Agreement - When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. - Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted. - When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

2. 	Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account for future work.

3. 	Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 		16 – 27 May 2022		                 Electronic Meeting

TSG-RAN WG1 Meeting #110 	22 – 26 August 2022 	                         Toulouse