3GPP TSG RAN WG1 Meeting #108-e	R1-2201192

e-Meeting, February 21th – March 3rd, 2022

Source:	ZTE

Title:	Initial views on Rel-18 MIMO evolution

Agenda Item:	8.1.5

Document for:	Discussion and Decision

- Introduction

In RAN#94, a new Rel-18 WID on MIMO evaluation for DL and UL was approved as in [1]. In such new WID, UL-MIMO related enhancement involving 8-Tx UL operation, simultaneous Tx cross multi-panel (STxMP), TA and power control enhancement, unified TCI and UL-DMRS is prioritized as essential feature(s) for 5G-Advanced; meanwhile, DL-MIMO related enhancement involving CSI enhancement for high/medium speed, Coherent-JT and DL-DMRS is also considered as in the Rel-18 MIMO evolution. In this contribution, we individually elaborate our initial views on each objectives of Rel-18 MIMO evolution, including some initial LLS & SLS simulation, technical analysis on performance benefits and implementation challenges, and potential PHY and high-layer impact.

- Enhancement for Rel-18 DL and UL MIMO

  - CSI enhancement for high/medium speed

In [1], the following objective(s) are identified for CSI enhancement for high/medium mobility, and in this subsection our analysis can be found as follows.

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking

Regarding Rel-16/17 Type-II CSI formula, if there is no modification to the spatial and frequency domain basis, the enhancement on the codebook refinement is much relevant to . Therefore, we may need to consider one or more Doppler domain basic vectors, and the co-efficient matrix  can be determined as a function of time. However, before specifying this enhancement, we need to identify typical scenario(s) for high-way or high-speed train in SLS, and then the channel model of spatial consistency should be used rather than traditional drop based.

Observation 1: Regarding potential enhancement for Doppler domain basic for Rel-16/17 Type-II codebook refinement, the channel model of spatial consistency should be used rather than traditional drop based.

Regarding time-domain channel properties measured via CSI-RS for tracking, in the wireless communication systems, as a basic requirement for time/frequency-domain synchronization, UE should always estimate the real-time Doppler shift and compensate it accordingly, via SSB and CSI-RS for tracking. Besides for UE mobility as mentioned in above objectives, the Doppler shift is mainly contributed by a slight deviation of centric frequency between gNB and UE oscillators. Therefore, from UE perspective, it is impossible to identify the respective Doppler shift introduced by UE mobility and imbalance between gNB and UE oscillators. Therefore, if UE always need to compensate the mixed Doppler shift impact for synchronization, the report of Doppler-shift estimate based on either a single CSI-RS (i.e., CSI-RS for tracking) or a unique spatial basic vector becomes meaningless.

Observation 2: From the perspective of UE receiver, Doppler shift is contributed by both UE mobility and deviation of centric frequency between gNB and UE oscillators and can be compensated for synchronization in UE, and therefore the necessity of the report of a single Doppler-shift estimate based on either a single CSI-RS or a unique spatial basic vector (e.g., Type I codebook) is unclear.

Therefore, a potential usage for UE reporting of time-domain channel properties measured via CSI-RS for tracking may be much relevant to the mTRP-HST scenario as discussed in Rel-17, rather than sTRP.

  - Unified TCI for mTRP operation

In [1], the following objective(s) are identified for enhancement on unified TCI framework for multi-TRP operation.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

Although we have several supported schemes for Rel-16/17 specific mTRP-operation, e.g., sDCI-PDSCH, mDCI-PDSCH, URLLC-related repetition for PDCCH/PDSCH/PUCCH/PUSCH, as a basic principle, we should strive to have a unified solution for all above Rel-16/17 mTRP related schemes. In technical, in order to guarantee the same understanding between gNB and UE sides, we need to consider a mechanism of associating a given TCI state with DL/UL data/control channel(s)/RS(s) by an explicit or implicit method, and then once the TCI state is applied, the corresponding channel(s) and RS(s) can be updated accordingly.

Observation 3: In order to accommodate different mTRP-operation use cases (including sDCI/mDCI-PDSCH, PDCCH/PDSCH/PUCCH/PUSCH repetition), strive for a unified solution of associating a given TCI state with PDCCH/PDSCH/PUCCH/PUSCH as well as corresponding RS(s), if needed.

- Once the given TCI state is applied, QCL assumption or spatial filter of above associated channel(s)/RS(s) can be updated immediately.

Straightforwardly, a TCI state can be associated with a TRP ID information (e.g., via MAC-CE activation for unified TCI state(s) as shown in Figure 1), and PDCCH/PDSCH/PUCCH/PUSCH and the resource (set) of DL or UL RS(s) can be explicitly or implicitly associated with a TRP ID information. Notice that, the association for different types of DL/UL data/control channel(s)/RS(s) may need to be discussed separately, and some details can be found in our companion contribution [2]. Then, multiple (i.e. M/N > 1) TCI states can be applied to the target DL or UL channels or signals according to the associated TRP ID information.

Figure 1 MAC-CE based TCI state(s) activation for mTRP operation

  - Orthogonal DMRS for DL and UL MU-MIMO

In [1], the following objective(s) are identified for orthogonal DMRS for DL and UL MU-MIMO.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

Due to the existing legacy UE(s), the backward compatibility should be fully considered for this enhancement on increasing the number of orthogonal DMRS ports, and then we need to modify the DMRS port/OCC mapping based on the legacy DMRS pattern(s). In our initial thoughts, FDM, FD-OCC, and TD-OCC can be considered as candidate solutions, which may suitable for respective deployment scenarios in terms of UE speed (Doppler shift) and delay spread. Our initial LLS simulation results can be found in Figure 2. The details of simulation assumption can be found in Table 2 in the Appendix.

Figure 2 LLS simulation results with (a) UE speed 3km/h and delay spread 30ns, (b) UE speed 3km/h and delay spread 300ns, and (c) UE speed 120km/h and delay spread 30ns

From above evaluation results, we can have the following observations:

- In the scenario of low-speed and low-delay-spread, e.g. as in Figure 2(a), there are similar BLER performance for all of FDM, FD-OCC and TD-OCC.

- Then, with the increase of delay spread, in the scenario of low-speed and high-delay-spread, TD-OCC performs better than FDM and FD-OCC in BLER, i.e. performance gain of 1-1.5dB, as shown in Figure 2(b).

- On the other side, under the consideration of higher UE speed, in the scenario of high-speed and low-delay spread, we have the opposite observation: there is severe performance degradation for TD-OCC compared with FDM and FD-OCC.

Observation 4: In order to accommodate different MU-MIMO scenarios, e.g., low-speed & low-delay-spread, low-speed & high-delay-spread, high-speed & low-delay-spread, comprehensive evaluation on several candidate solutions (e.g., FDM, FD-OCC and TD-OCC) for supporting larger number of orthogonal DMRS ports should be performed.

  - sType-II CSI and SRS enhancement for Coherent JT

In [1], the following objective(s) are identified for Rel-16/17 Type-II CSI and SRS enhancement for coherent JT.

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

Regarding the enhancement on CSI acquisitions for coherent-JT, the deployment scenarios can be divided into the following three categories: a) intra-site, b) inter-site, and c) intra-&inter-site as shown in Figure 3. Then, if RSRP distance between the optimal TRP and the second optimal TRP (e.g., intra-site and/or inter-site) is smaller than a threshold (e.g., 6 dB), coherent joint transmission using the corresponding TRPs is performed; otherwise, sTRP based transmission is used.

Figure 3 Deployment scenarios for coherent JT: (a) intra-site, (b) inter-site, (c) intra-&inter-site

For initial SLS evaluation, we evaluate system performance (considering both intra-site and inter-site cases as shown in Figure 3(c)) for ideal feedback (SVD-based), Type-II CSI enhancement for sTRP, non-coherent JT (NC-JT) and coherent JT (C-JT). The corresponding SLS simulation results can be found in Table-1. It can be observed that C-JT can bring an obvious performance gain compared with sTRP and NC-JT, especially for cell-edge UE.

Table 1 SLS simulation results for coherent-JT

| Case | Ideal feedback (Mbit/s) |  |  |  | eType II feedback(Mbit/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mean | 5% | 50% | 95% | mean | 5% | 50% | 95% |
| sTRP | 191.35 (100%) | 44.38 (100%) | 172.78 (100%) | 358.51 (100%) | 177.38 (100%) | 41.53 (100%) | 158.13 (100%) | 358.51 (100%) |
| NC-JT | 188.48 (-1.5%) | 50.46 (+13.7%) | 169.67 (-1.8%) | 358.51 (0%) | 175.78 (-0.9%) | 46.76 (+12.6%) | 159.08 (+0.6%) | 358.51 (0%) |
| C-JT | 203.60 (+6.4%) | 59.03 (+33.0%) | 184.78 (+6.9%) | 358.51 (0%) | 186.26 (+5.0%) | 52.33 (+26%) | 164.49 (+4.0%) | 358.51 (0%) |

Observation 5: From evaluation results, it can be observed that, compared with sTRP and NC-JT, C-JT can bring performance gains in terms of both cell-edge and mean UPT.

Regarding SRS enhancement to manage inter-TRP cross-SRS interference, there are two candidate sub-objectives of SRS capacity enhancement and interference randomization.

- Regarding SRS capacity enhancement,

  - In coherent JT, as a typical requirement, there are similar receive power(s) (e.g., RSRP gap is less than 6 dB) for each of coordinated TRPs, and then with high probability, the UE may be cell-edge. Consequently, for SRS transmission, for improving coverage or channel estimation accuracy, SRS time-domain repetition may be needed and as a result, costs more SRS overhead for one UE/resource.

  - So, in such case, UE-level TD-OCC can be used on the repetition symbols in order to improve the SRS capacity. That is, different OCC codes can be configured for different UEs performing repetition. Then for a K-symbol repetition, K UEs can be multiplexed in these symbols by using length-K OCC, which achieves same capacity as not using repetition.

- Regarding SRS interference randomization, if there are limited orthogonal resource, the collision or cross-SRS interference for inter-TRP/cell may not be precluded. So, the motivation of interference randomization is to avoid the continuous serious interference for each of measurement occasion, and consequently it can improve the transmission performance of coherent-JT UE. That is, we can consider the randomization for SRS transmission in terms of sequence, frequency and time domains.

Observation 6: Regarding SRS capacity enhancement and interference randomization in coherent JT, the cell-edge UE should be focused as a typical usage.

- For SRS capability enhancement, UE-level TD-OCC can be used on the time-domain repetition symbols in order to improve the SRS capacity

- For interference randomization, as a starting point, we can consider the randomization for SRS transmission in terms of sequence, frequency and time domains.

  - 8-Tx UL operation

In [1], the following objective(s) are identified for enhancement on 8-Tx UL operation.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

Via SLS simulation, we comprehensively evaluate performance for supporting 8 TX UL operation including three cases: a) ‘8-Tx & max RANK-8’, b) ‘8-Tx & max RANK-4’ and c) ‘4-Tx & max RANK-4’. The SLS simulation working assumptions can be found in appendix Table 2.

- Evaluation results with ideal UL precoding (SVD-based) can be found in Figure 4.

  - It can be observe that, with the increase of the number of Tx(s), the UL transmission can perform with higher RANK, and as a consequence the system throughput can be improved significantly (e.g., 13.06% and 27.02% gains over legacy UE with ‘4-Tx & max RANK-4’).

  - Then, when focusing the cases of ‘8-Tx & max RANK-8’ and ‘8-Tx & max RANK-4’, we can find that the enhancement on supporting 8-layer UL transmission can sufficiently obtain the benefits bringing by introducing 8-Tx for cell-centric UE (i.e., 95%-ile UE) (~49.3% gain over legacy UE), and meanwhile the network can obtain an obvious performance gains in terms of average throughput.

- After that, as a straightforward solution, the solution of directly reusing DL Type-I codebook in 8-Tx UL operation are evaluated as shown in Figure 5.

  - Besides for the similar observations for performance comparison among above three cases, we can further identify that we still can have significant performance gain by supporting 8-Tx and up to 8 layers in terms of RANK distribution, average and 95%-ile UPTs, although there is some performance degradation compared with ideal precoding.

Figure 4 Performance evaluation with ideal UL preconding (SVD-based) for ‘8-Tx & max RANK-8’, ‘8-Tx & max RANK-4’ and ‘4-Tx & max RANK-4’: (a) Rank distribution, (b) average UPT, (c) 95%-ile UPT

Figure 5 Performance evaluation reusing DL Type-I codebook for ‘8-Tx & max RANK-8’, ‘8-Tx & max RANK-4’ and ‘4-Tx & max RANK-4’: (a) Rank distribution, (b) average UPT, (c) 95%-ile UPT

Observation 7: Regarding 8 Tx-UL operation, from the evaluation results, it can be observed that:

- Increase number of UL Tx(s) can bring significant performance benefit.

- Increase number of layers from 4 to 8 can bring higher performance gain to maximize benefits introduced by 8-Tx UL operation, especially for 95%-ile UE.

Based on above analysis, we identify the detailed points for further study as follows:

- Support 2 CWs for UL transmission (especially for more than 4-layer UL transmission): with the increase of number of UL layers and UL TXs, 2 CWs using individual MCS, RV and NDI become much essential for uplink transmission. Then, the condition of enabling >1 CWs for UL transmission (e.g., while number of TX(s) and UL layers exceeds a threshold) can be further studied in RAN1.

- Increase of the number of UL DMRS ports per UE for facilitating more than 4 layer transmission.

- SRS design for supporting 6 or 8 Tx (e.g., for CB, NCB, and antenna switching (e.g., 6T6R, 6T8R, 8T8R)).

- Applying to both codebook and non-codebook PUSCH transmission: For codebook based transmission, Type-I like UL codebook, e.g., TPMI refinement, can be considered as a starting point; meanwhile, for non-codebook based transmission, the re-design of DCI SRI codepoint should be considered accordingly.

- Applying to full-coherent and partial coherent UE. But, for non-coherent UE, we fail to identify the performance gain especially for supporting more transmission layers, and so this type of coherence assumption, i.e., non-coherent, should be precluded. After that, for partial coherent, we can further consider how to support full Tx power mode.

  - Simultaneous multi-panel UL transmission

In [1], the following objective(s) are identified for the enhancement on simultaneous multi-panel UL transmission.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

Regarding simultaneous multi-panel UL transmission, our initial SLS simulation results for indoor hotspot can be found in Figure 6. It can be observed that simultaneous multi-panel UL transmission can significantly improve UL capacity (e.g., ~15.7% gain for mean UPT) and reliability via spatial multiplexing/diversity, especially for enabling >2 layer transmission.

Figure 6 UPT performance of simultaneous transmission cross multi-panel (STxMP) in indoor hotspot, and the corresponding PDF as a function of RAN

Observation 8: Regarding simultaneous multi-panel UL transmission, from the evaluation results, it can be observed that the support of simultaneous multi-panel transmission can significantly bring the performance benefits (regardless of mean UPT, 95%-ile and cell-edge UE) via spatial multiplexing and diversity.

Based on above analysis, we identify the detailed points for further study as follows:

- Support both single DCI and multi-DCI based mTRP operation:

  - Regarding single-DCI based mTRP operation, both PUSCH and PUCCH transmission cross multi UE panel should be considered, and then the corresponding DCI signaling enhancement as well as the dedicated transmission scheme(s) should be considered. After that, for PUSCH, we need to further consider panel-specific UL precoding indication and the enhancement on 2 CWs for PUSCH (each of which corresponds to individual UE panel/TRP) with up to four layers. For instance, we may strive for unified signaling design for TD/SD/FD-repetition/multiplexing PUSCH (being compatible for Rel-17 mTRP PUSCH).

    - Notice that, from spec perspective, we can consider this CW-mapping enhancement with 8-Tx UL operation together for saving spec efforts.

  - Regarding multi-DCI based mTRP operation, we need to justify the multiplexing rule and beam collision for PUSCH+PUSCH and PUCCH+PUCCH.

- Unified TCI framework to support of PUCCH/PUSCH in simultaneous multi-panel UL transmission: As we discussed in Section 2.2, the unified solution (e.g., via an implicit or explicit mechanism) should be considered herein.

  - TA and power control enhancement

In [1], the following objective(s) are identified for the enhancement on UL TA and power control.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Regarding two TAs for UL multi-DCI for multi-TRP operation, in our views, the support of two TAs per TRP can clearly enhance the coverage and deployment scenarios for multi-TRP. To be more specific, each of the two TAs corresponds to respective TRP, and consequently, we need to consider the refinement of TA cell-group in such case. For instance, like separate HARQ procedure in mDCI-mTRP operation, each TA can be associated with respective TRP (e.g., CORESETPoolID) cross multi-CC, but how to maintain two individual UL timeline for different UL channel/RS (e.g., frame boundary) and channel/RS cancellation due to TA switching should be further studied. For instance, in the legacy procedure, we only have a single timeline for a serving cell as follows.

Figure 7 Legacy uplink-downlink timing relation in NR

Observation 9: Regarding two TAs for UL multi-DCI for multi-TRP operation, TRP-specific TA should be considered and can apply cross multi-CC, but how to maintain two individual UL timeline for different UL channel/RS (e.g., frame boundary) and channel/RS cancellation due to TA switching should be further studied.

Regarding power control for UL single DCI for multi-TRP operation, once one PUSCH or PUCCH transmission corresponds to multi-TRP via sDCI-mTRP operation, we need to further review the current framework (for instance, current formula for PUSCH power control is provided as follows).

Generally speaking, in NR, the power control is relevant to a channel (so-called as channel-specific power control), and thus the whole Tx power is equally scaled across all non-zero-power PUSCH ports. But, in such case, we may need to consider layer/port-group specific power control for a given PUSCH/PUCCH transmission scheduled by a single DCI in mTRP operation, and therefore we need to support different Tx power for different UL Tx ports.

Straightforwardly, there are two candidate mechanisms: a single Tx power (e.g., channel-specific PPUSCH) but with flexible power scaling scheme (rather than being equally scaled), or two individual Tx powers (e.g., two panel-specific PPUSCH) corresponding to different UE panel/TRP.

- For the former, only a single power control setting is needed, but the power control formula should be updated with the consideration on the impact related to both of two TRPs (e.g., an average pathloss estimate based on two separate pathloss RS(s)).

- For the latter, UE needs to determine two individual Tx powers, e.g., two panel-specific PPUSCH, for each UE panel/TRP, and consequently the panel-specific power control formula(s) for each of power control setting should be introduced. Additionally, the definition of Pc,max may need to be further studied.

Observation 10: In order to support TRP-specific UL power control in sDCI-mTRP operation, the mechanism for determining UL power control should be further studied, e.g., a single power control setting with the flexible power scaling scheme or two individual power control settings but with refinement for the formula of Tx power calculation in TS 38.213.

- Conclusion

In this contribution, we elaborate our initial views on Rel-18 MIMO evolution. Observations are listed as follows.

CSI enhancement for high/medium speed

Observation 1: Regarding potential enhancement for Doppler domain basic for Rel-16/17 Type-II codebook refinement, the channel model of spatial consistency should be used rather than traditional drop based.

Observation 2: From the perspective of UE receiver, Doppler shift is contributed by both UE mobility and deviation of centric frequency between gNB and UE oscillators and can be compensated for synchronization in UE, and therefore the necessity of the report of a single Doppler-shift estimate based on either a single CSI-RS or a unique spatial basic vector (e.g., Type I codebook) is unclear.

Unified TCI for mTRP operation

Observation 3: In order to accommodate different mTRP-operation use cases (including sDCI/mDCI-PDSCH, PDCCH/PDSCH/PUCCH/PUSCH repetition), strive for a unified solution of associating a given TCI state with PDCCH/PDSCH/PUCCH/PUSCH as well as corresponding RS(s), if needed.

- Once the given TCI state is applied, QCL assumption or spatial filter of above associated channel(s)/RS(s) can be updated immediately.

Orthogonal DMRS for DL and UL MU-MIMO

Observation 4: In order to accommodate different MU-MIMO scenarios, e.g., low-speed & low-delay-spread, low-speed & high-delay-spread, high-speed & low-delay-spread, comprehensive evaluation on several candidate solutions (e.g., FDM, FD-OCC and TD-OCC) for supporting larger number of orthogonal DMRS ports should be performed.

Type-II CSI and SRS enhancement for Coherent-JT

Observation 5: From evaluation results, it can be observed that, compared with sTRP and NC-JT, C-JT can bring performance gains in terms of both cell-edge and mean UPT.

Observation 6: Regarding SRS capacity enhancement and interference randomization in coherent JT, the cell-edge UE should be focused as a typical usage.

- For SRS capability enhancement, UE-level TD-OCC can be used on the time-domain repetition symbols in order to improve the SRS capacity

- For interference randomization, as a starting point, we can consider the randomization for SRS transmission in terms of sequence, frequency and time domains.

8-TX UL operation

Observation 7: Regarding 8 Tx-UL operation, from the evaluation results, it can be observed that:

- Increase number of UL Tx(s) can bring significant performance benefit.

- Increase number of layers from 4 to 8 can bring higher performance gain to maximize benefits introduced by 8-Tx UL operation, especially for 95%-ile UE.

Simultaneous multi-panel UL transmission

Observation 8: Regarding simultaneous multi-panel UL transmission, from the evaluation results, it can be observed that the support of simultaneous multi-panel transmission can significantly bring the performance benefits (regardless of mean UPT, 95%-ile and cell-edge UE) via spatial multiplexing and diversity.

TA and power control enhancement

Observation 9: Regarding two TAs for UL multi-DCI for multi-TRP operation, TRP-specific TA should be considered and can apply cross multi-CC, but how to maintain two individual UL timeline for different UL channel/RS (e.g., frame boundary) and channel/RS cancellation due to TA switching should be further studied.

Observation 10: In order to support TRP-specific UL power control in sDCI-mTRP operation, the mechanism for determining UL power control should be further studied, e.g., a single power control setting with the flexible power scaling scheme or two individual power control settings but with refinement for the formula of Tx power calculation in TS 38.213.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- R1-2110955, Further details on Multi-beam and Multi-TRP operation, ZTE

- Appendix

Table 2 LLS simulation parameters for orthogonal DMRS patterns

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5G |
| Channel Model | CDL-B in TR 38.901 |
| MIMO scheme | SU-MIMO |
| Modulation | 64QAM |
| Code Rate | 0.5 |
| Bandwidth | 20 MHz |
| Numerology | 14 OFDM symbol slot, 30kHz SCS |
| MIMO Rank | rank = 1 |
| UE speed | 3km/h 120km/h |
| Precoding granularity | 4RB |
| DMRS | Type 2 DMRS, two symbols on 2,9 |
| DL DMRS channel estimation | 2DMMSE channel estimation |

Table 3 SLS simulation parameters for 8-Tx UL operation

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5GHz |
| Bandwidth | 20MHz |
| SCS | 30kHz |
| Network Layout | UMi in TR 38.901 |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,4,4; 1,1). (dH,dV) = (0.5, 0.8)λ |
| UE antenna configuration | 8Tx: (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,2,2; 1,1). (dH,dV) = (0.5, 0.5)λ 4Tx: (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,2,1; 1,1). (dH,dV) = (0.5, 0.5)λ |
| Scheduling | Subband PF |
| Traffic model | Full buffer |
| ISD | 200 |
| Number of average UE per cell | 10 |
| RB Number | 50 |
| Max UE power | 31dBm |
| UL power control | Open loop TPC |
| power control parameter | P0 = -50, Alpha = 0.8 |
| Modulation | Up to 64QAM |
| Layer mapping | 1st CW@RANK1-4, 2nd CW@RANK5-8 |