3GPP TSG RAN WG1 #108-e                                               R1-2201194

e-Meeting, February 21st -March 3rd, 2022

Source:         ZTE

Title:            Remaining issues on UL-AoA positioning solutions

Agenda item:    8.5.2

Document for:	Discussion and Decision

Introduction

According to the agreed WID [1], a number of agreements have been approved to improve positioning accuracy for UL-AOA method before the Rel-17 was officially done in RAN1. In this contribution, we provide our views on the remaining issues on UL-AoA positioning solutions.

First arrival path of UL measurement result

According to the following agreement made in RAN1#106bis-e, gNB/TRP can also report SRS-RSRP for the first arrival path.

Agreement: - For the first arrival path measurements on SRS for positioning resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference} - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - For the first arrival path measurements on SRS for MIMO resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - Note: The operation of SRS for MIMO is transparent to the UE

There are two remaining issues needed to be further discussed for the first arrival path measurements on SRS,

- Issue#1: Whether gNB/TRP can report multiple path SRS RSRPs for the first arrival path

- Issue#2: Whether gNB/TRP can report one UL-RTOA and one gNB Rx-Tx time difference simultaneously for the first arrival path

For Issue#1, we think the intention is to report up to 8 UL-AOA/ZOA values for first arrival path, where each UL-AoA/ZoA value corresponds to one RSRP. However, according to the discussion in DL-AoD, the path RSRP should be defined in time domain rather than spatial domain, Furthermore, as defined in TS 38.215 for SRS RSRP, if receiver diversity is in use by the gNB, the reported SRS-RSRP value shall not be lower than the corresponding SRS-RSRP of any of the individual receiver branches, which means the path RSRP has already considered all the power contributions from spatial domain.

Proposal 1: There is no need for gNB/TRP to report multiple path SRS RSRPs for the first arrival path since the path SRS RSRP should be defined in time domain rather than spatial domain.

For Issue#2, LMF will take a differential operation between two UL-RTOA values to construct a TDOA value. The TDOA value may not be impacted by the clock misalignment between UE and TRPs because of the differential operation. However, the TDOA value is biased from the actual value due to the synchronization error between the two TRPs. But it’s a different story for Multi-RTT. The RTT value is not affected by either clock misalignment between UE and TRPs or synchronization error between TRPs. Furthermore, the TDOA value can also be obtained by taking differential operation for multiple RTTs from different TRPs. Hence, the gNB Rx-Tx time difference can provide more information than UL-RTOA, and then there is no need to report one UL-RTOA and one gNB Rx-Tx time difference simultaneously for the first arrival path.

Proposal 2: Don’t support gNB/TRP to report one UL-RTOA and one gNB Rx-Tx time difference simultaneously for the first arrival path

Conclusions

In this contribution, we provide our views on the remaining issues on UL-AOA positioning, the following proposals are proposed,

Proposal 1: There is no need for gNB/TRP to report multiple path SRS RSRPs for the first arrival path since the path SRS RSRP should be defined in time domain rather than spatial domain.

Proposal 2: Don’t support gNB/TRP to report one UL-RTOA and one gNB Rx-Tx time difference simultaneously for the first arrival path

References

- RP-210903 Revised WID on NR Positioning Enhancements, Intel Corporation, CATT

- Draft Report of 3GPP TSG RAN WG1 #106bis-e