3GPP TSG RAN WG1 Meeting #108-e	R1-2201222

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.16.4

Source:	MediaTek Inc.

Title:	UE features for NR NTN

Document for:	Discussion and Decision

- Introduction

In RAN#107-e, the RAN1 UE features are provided in [1] for Rel-17 NR NTN. In this contribution, we provide our perspectives on the update RAN1 UE features.

- Discussion

FG 26-1

- Component 2 can be kept without yellow highlight considering saving signalling for the case, where RP is set on the satellite.

- Component 4 can be kept without yellow highlight and with “FFS” removed.

- Component 5 can be kept without yellow highlight considering the agreements.

Agreement(RAN 105e) The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac. How to treat NTA and NTA,offset can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

FG 26-4

- The component text starting with “FFS Report its UE location for any reason at any time” and text crossed and highlighted in yellow can be removed as it is not necessary to include this level of details and UE location can wait until SA3 decision as per agreement in RAN2#116b-e.

Agreements (RAN2 116b-e) - For the TA report triggering event which uses the offset threshold between current information about UE specific TA and the last successfully reported information about UE specific TA, no hysteresis or time to trigger is needed. - UE reports Full TA (i.e., T_TA as defined in the UE’s TA formula). The size of the TA report MAC CE is fixed to two octets. - If SA3 will confirm that NTN-specific user consent will the available in Rel-17, the network could at least ask the UE to report its UE location for any reason at any time. FFS if we define an event-triggered reporting of UE location for TA reporting purposes.

FG 26-9

- Component 2 can delete the yellow highlight “and PDCCH ordered PRACH” considering agreements in RAN1 agreements.

Agreement(RAN1 107e) The K_offset value signaled in system information is always used for PDCCH ordered PRACH timing relationship.

# Conclusion

In this contribution, we discuss the Rel-17 UE features for NR NTN. The following proposals are presented:

Proposal 1: FG 26-1

- Component 2 can be kept without yellow highlight.

- Component 4 can be kept without yellow highlight and with “FFS” removed

- Component 5 can be kept without yellow highlight

Proposal 2: FG 26-4

- Delete the text “FFS Report its UE location for any reason at any time” and text crossed and highlighted in yellow

Proposal 3: FG 26-9

- Delete text in yellow highlight “and PDCCH ordered PRACH” in component 2

# References

- R1-2111809, “Summary of UE features for NR NTN.”

- R1-2112890, “3GPP TSG-RAN WG1 Agreements under 8.4 up to eMeeting RAN1#107-e.”

- R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e.”

# Appendix A

Proposed revision for Rel-17 UE features for NR NTN.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | Support UE reporting of information about the UE specific TA pre-compensation - Report full TA by event triggered used the offset threshold between current information about UE specific TA and the last successfully reported information about UE specific TA. - [FFS: Report its UE location for any reason at any time] [The exact content of UE reporting of information about the UE specific TA pre-compensation e.g., frequency of the reports, granularity of the reported conten, etc.] | [26-1] | Yes | No |  | [Per UE/Per band] | No | No |  |  | Optionalwith capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported | [Per band or per FSPC or per UE] | No | No |  | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Support of type-2 HARQ codebook enhancements [for feedback-disabled HARQ processes] - FFS: UE supports HARQ disabling |  | Yes | No |  | [Per UE/per band] | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ codebook enhancement] | - Enhancement on Type-1 HARQ codebook in NTN - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32 |  | Yes | No |  | [Per UE/per band/Per BC] | [No] | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - Determining the timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |