3GPP TSG RAN WG1 #108-e		R1-2201261

e-Meeting, February 21st – March 3rd, 2022

Source:	OPPO

Title:	Discussion on the LS from RAN2 of MBS SPS

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

A LS from RAN2 in [1] is sent to RAN1 and asking two questions related to MBS SPS, and the LS is copied below for reference.

RAN2 has discussed MBS SPS related issues and made the following agreements: One-to-many mapping between G-CS-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work. Capture CS-RNTI usage in table for MBS in section 7.1 in MBS MAC running CR, i.e. for PTP for PTM retransmission via CS-RNTI and MBS SPS deactivationvia CS-RNTI when MBS SPS is configured. If MBS SPS is configured and CS-RNTI is not configured, the retransmission of SPS via PTP is not supported and MBS SPS deactivation via CS-RNTI is not supported. The sps-ConfigIndex should unique in UE no matter the SPS is for unicast or multicast. Besides, RAN2 has the following understanding: RAN2’s understanding: There can be multiple MBS SPS-configs and multiple G-CS-RNTIs. However, the association between G-CS-RNTIs and MBS SPS-configs will not be specified in RRC signalling. The DCI scrambled with G-CS-RNTI will indicate which MBS SPS-config will be activated for G-CS-RNTI via HARQ process ID field which equals to sps-ConfigIndex in an MBS SPS-config. Based on this, RAN2 understands that one to one mapping or one to multiple mapping from G-CS-RNTI to MBS SPS-config are supported as legacy. However, if multiple G-CS-RNTIs were mapped to same MBS SPS-config, it is not clear how that would work. Thus, RAN2 would like to ask the following question: Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work? In addition, RAN2 discussed the handling of retransmission and wonders if retransmission (i.e. via PTM or PTP) can be changed per TB or per TB per transmission, or configured in RRC signalling and whether a single CS-RNTI is used for PTP retransmissions of all G-CS-RNTIs? Q2: Clarify the handling of PTM retransmissions.

This contribution discusses the questions above from RAN1’s perspective, and the corresponding observation and proposals are provided based on the analysis.

Discussion

  - Association between G-CS-RNTI and SPS-config-multicast

The corresponding RAN1 agreements can be found as follows:

Agreement: Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH. - G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH - FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH. - FFS: Number of G-CS-RNTI. Agreement: If a SPS-config for MBS is configured in CFR, one G-CS-RNTI is associated with the SPS-config. - FFS: Multiple G-CS-RNTIs associated with one SPS-config Agreement: For multicast of RRC_CONNECTED UEs, the G-RNTI(s) is/are configured - Opt.2: per serving cell. - FFS G-CS-RNTI(s) Agreement: The association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, i.e., a value of the HARQ process number field in a DCI format indicates an activation for a SPS GC-PDSCH configuration for multicast with a same value as provided by sps-ConfigIndex in a SPS-Config-Multicast. Agreement For multicast of RRC_CONNECTED UEs, the G-CS-RNTI(s) is/are configured per serving cell.

Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work?

Based on RAN1 agreements in RAN1#106bis-e, the association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, and the HPN field in the DCI indicates an activation for the SPS GC-PDSCH with the same value as provided by sps-ConfigIndex in a SPS-Config-Multicast. In higher layer, multiple G-CS-RNTIs and up to 8 SPS-Config-Multicast are configured independently without any mapping relationship, which means that the association between a G-CS-RNTI and a SPS-Config-Multicast is not specified or configured via any RRC signaling. The association relationship can be discussed as examples in the following types:

- Type-1: One G-CS-RNTI is associated with one SPS-Config-Multicast.

A group-common DCI with CRC scrambled with G-CS-RNTI#01 is used to activate a SPS-Config-Multicast, and the DCI includes HPN field indicate sps-ConfigIndex is #01. After the activation with this association, G-CS-RNTI#01 cannot used to scramble a GC-DCI to activate a SPS-Config-Multicast other than sps-ConfigIndex #01, and sps-ConfigIndex #01 cannot be activated by a GC-DCI scrambled a G-CS-RNTI other than G-CS-RNTI#01.

- Type-2: One G-CS-RNTI is associated with multiple SPS-Config-Multicast.

A group-common DCI with CRC scrambled with G-CS-RNTI#01 is used to activate a SPS-Config-Multicast, and the DCI includes HPN field indicate sps-ConfigIndex is #01. After the activation with this association, another DCI with G-CS-RNTI#01 is used to activation a SPS-Config-Multicast with the HPN indication of sps-ConfigIndex is #02. Both associations, i.e. G-CS-RNTI#01 & sps-ConfigIndex is #01 and G-CS-RNTI#01 & sps-ConfigIndex is #02, are valid and the two SPS-Config-Multicast can be coexistent simultaneously. Furthermore, one G-CS-RNTI can be mapped to multiple MBS sessions, different SPS-Config-Multicast can be activated to transmit different MBS sessions with the same G-CS-RNTI.

- Type-3: Multiple G-CS-RNTI is associated with one SPS-Config-Multicast.

A group-common DCI with CRC scrambled with G-CS-RNTI#01 is used to activate a SPS-Config-Multicast, and the DCI includes HPN field indicate sps-ConfigIndex is #01. After the activation with this association, another DCI with G-CS-RNTI#02 is used to activation a SPS-Config-Multicast with the HPN indication of sps-ConfigIndex is #01. The previously activated sps-ConfigIndex is #01 (associated with G-CS-RNTI#01) is invalid immediately, and the association between sps-ConfigIndex is #01 and G-CS-RNTI#01 is broken. The newly received GC-DCI with G-CS-RNTI#02 & sps-ConfigIndex is #01 by a UE will consider it as a new association rather than a reactivation of G-CS-RNTI#01 & sps-ConfigIndex is #01. Therefore, multiple G-CS-RNTIs cannot associated with one SPS-Config-Multicast at a time because one existing activated SPS will be interrupted by a newly received GC-DCI with new association.

Regarding the maximum number of G-CS-RNTIs that can be configured for a UE, based on RAN1’s agreements, G-CS-RNTI(s) is/are configured per serving cell for multicast of RRC_CONNECTED UEs, and the value range is RNTI-Value which is the same as G-RNTI.

Observation 1: RAN2’s understanding on the association between G-CS-RNTI and SPS-Config-Multicast, as well as RAN1’s agreements, are correct.

Proposal 1: The maximum number of G-CS-RNTI configured for UE is the RNTI-Value which is the same as that of G-RNTI.

Proposal 2: Multiple G-CS-RNTIs cannot be associated with one SPS-Config-Multicast for MBS.

  - PTM/PTP retransmission scheme for MBS SPS

The corresponding RAN1 agreements can be found as follows:

Agreement: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. - FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group Agreement: Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH. - G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH - FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH. - FFS: Number of G-CS-RNTI. Agreement: For PTP retransmission of SPS group-common PDSCH, CS-RNTI is used for CRC scrambling of PDCCH with the NDI bit set to 1.

Q2: Clarify the handling of PTM retransmissions.

Based on RAN1 agreements by now, the retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP, which means the retransmission scheme can be changed per TB. For an example in a activated SPS configuration, one TB1 is initially transmitted through PTM scheme 1, and PTM scheme 1 can be used as the only retransmission scheme for the same group of UEs. For the following up group-common TB2 in the same SPS configuration, PTP can be used as the only retransmission scheme for the same group of UEs, as long as CS-RNTI is configured.

Proposal 3: For MBS SPS, the retransmission scheme (i.e. via PTM scheme 1 or PTP) can be changed per TB for the same group of UEs.

Regarding the relationship between a single CS-RNTI and multiple G-CS-RNTIs, it has been agreed that CS-RNTI is used for CRC scrambling of PDCCH with the NDI bit set to 1 when PTP retransmission is supported of SPS group-common PDSCH. From the perspective of a UE, CS-RNTI is the unique identity which is different from other UEs. It is also agreed in RAN2 that multiple G-CS-RNTIs are supported in MBS SPS, and a UE can support multiple G-CS-RNTIs for receiving multiple MBS SPS configurations. Therefore, from per UE’s perspective for PTP retransmission of MBS SPS, a single CS-RNTI is used associated with all of the G-CS-RNTIs of the SPS configurations of multicast.

Proposal 4: From per UE’s perspective for PTP retransmission of MBS SPS, a single CS-RNTI is used associated with all the G-CS-RNTIs of the SPS configurations of multicast.

Conclusion

In this contribution, some MBS SPS related issues are discussed including the association between G-CS-RNTI and SPS-Config-Multicast, and retransmission schemes. The following observation and proposals are provided:

Observation 1: RAN2’s understanding on the association between G-CS-RNTI and SPS-Config-Multicast, as well as RAN1’s agreements, are correct.

Proposal 1: The maximum number of G-CS-RNTI configured for UE is the RNTI-Value which is the same as that of G-RNTI.

Proposal 2: Multiple G-CS-RNTIs cannot be associated with one SPS-Config-Multicast for MBS.

Proposal 3: For MBS SPS, the retransmission scheme (i.e. via PTM scheme 1 or PTP) can be changed per TB for the same group of UEs.

Proposal 4: From per UE’s perspective for PTP retransmission of MBS SPS, a single CS-RNTI is used associated with all the G-CS-RNTIs of the SPS configurations of multicast.

Reference

- R1-2200888 (R2-2202016), LS on MBS SPS, RAN1#108-e, e-Meeting, February 21st – March 3rd, 2022.