3GPP TSG RAN WG1 #108-e		R1-2201271

e-Meeting, February 21th -March 3th, 2022

Source:	OPPO

Title:	Discussion on remaining issue for timing relationship enhancement

Agenda Item:	8.4.1

Document for:	Discussion and Decision

Introduction

In RAN1 Meeting #107, NR-NTN specs in Rel-17 were almost completed. In this contribution, we discuss the remaining issues for timing relationship enhancement of the latest agreements and email discussions. Some TPs are provided targeting respective clauses of the latest version of Rel-17 specifications.

Text Proposal

Activation/Deactivation MAC CE

Based on the several rounds of discussion about the issue of large TA which results the misalignment of uplink and downlink slot or subframe, the related enhancement of the MAC CE command has been discussed and the following agreement has been made at RAN1#105-e.

RAN1#105-e:

Agreement:

If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH.

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

The discussions relevant to the above agreement were conducted in RAN1#105-e meeting and there was an consensus for the following points:

- For MAC-CE configuring downlink signal, K_mac needs to be additionally considered for the activation time.

- For ‘slot n’ in the legacy spec when defining MAC-CE activation time,  it needs to be clearly defined with downlink timing or uplink timing to avoid further confusion. For example, in the above agreement, the slot n for PUCCH with HARQ-ACK information follows uplink timing, which is why we use ‘uplink slot n’; while the slot n in the equation slot n+3Nslotsubframe,µ+Kmac follows downlink timing, which is there is no ‘uplink’ term in front of slot n.

After our review of the latest CR, we find some remaining issues on the above 2 points.

SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE

Proposal 1: For SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE.

- Adopt following TP#1 for TP 38.214 V17.0.0

## *** < Beginning of TP#1 for TP 38.214 V17.0.0> *** 5.1.4.2 PDSCH resource mapping with RE level granularity *** < Unchanged parts are ommitted> *** For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. - when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#1 for TP 38.214 V17.0.0> ***

TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

Proposal 2: For TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

- Adopt following TP#2 for TP 38.214 V17.0.0 and TP#3 for TS 38.213 V17.0.0.

## *** < Beginning of TP#2 for TP 38.214 V17.0.0> *** 5.1.5 Antenna ports quasi co-location *** < Unchanged parts are ommitted> *** When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'. *** < Unchanged parts are ommitted> *** ## *** < End of TP#2 for TP 38.214 V17.0.0> ***

## *** < Beginning of TP#3 for TS 38.213 V17.0.0> *** 10.1 UE procedure for determining physical downlink control channel assignment *** < Unchanged parts are ommitted> *** For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block - if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command without taking into account timing advance, μ is the SCS configuration for the PUCCH in the slot when the activation command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#3 for TS 38.213 V17.0.0> ***

iang

Aperiodic CSI Trigger State Subselection MAC CE

Proposal 3: For Aperiodic CSI Trigger State Subselection MAC CE.

- Adopt following TP#4 for TP 38.214 V17.0.0.

## *** < Beginning of TP#4 for TP 38.214 V17.0.0> *** 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology *** < Unchanged parts are ommitted> *** A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested. - When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#4 for TP 38.214 V17.0.0> ***

SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Proposal 4: For SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

- Adopt following TP#5 and TP#6 for TP 38.214 V17.0.0.

## *** < Beginning of TP#5 for TP 38.214 V17.0.0> *** 5.2.1.5.2 Semi-persistent CSI/Semi-persistent CSI-RS *** < Unchanged parts are ommitted> *** For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#5 for TP 38.214 V17.0.0> ***

## *** < Beginning of TP#6 for TP 38.214 V17.0.0> *** 5.2.4 CSI reporting using PUCCH *** < Unchanged parts are ommitted> *** A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. The activation command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1. *** < Unchanged parts are ommitted> *** ## *** < End of TP#6 for TP 38.214 V17.0.0> ***

SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

Proposal 5: For SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

- Adopt following TP#7 for TP 38.214 V17.0.0.

## *** < Beginning of TP#7 for TP 38.214 V17.0.0> *** 5.2.1.5.2 Semi-persistent CSI/Semi-persistent CSI-RS *** < Unchanged parts are ommitted> *** For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'. - when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP. - when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#7 for TP 38.214 V17.0.0> ***

SP SRS Activation/Deactivation MAC CE or SP Positioning SRS Activation/Deactivation MAC CE

Proposal 6: For SP SRS Activation/Deactivation MAC CE or SP Positioning SRS Activation/Deactivation MAC CE.

- Adopt following TP#8 for TP 38.214 V17.0.0.

## *** < Beginning of TP#8 for TP 38.214 V17.0.0> *** 6.2.1 UE sounding procedure *** < Unchanged parts are ommitted> *** For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'semi-persistent': - when a UE receives an activation command, as described in clause 6.1.3.17 or 6.1.3.36 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set. *** < Unchanged parts are ommitted> *** - when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. *** < Unchanged parts are ommitted> *** ## *** < End of TP#8 for TP 38.214 V17.0.0> ***

Enhanced SP/AP SRS Spatial Relation Indication MAC CE

Proposal 7: For Enhanced SP/AP SRS Spatial Relation Indication MAC CE.

- Adopt following TP#9 for TP 38.214 V17.0.0.

## *** < Beginning of TP#9 for TP 38.214 V17.0.0> *** 6.2.1 UE sounding procedure For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic': - the UE receives a configuration of SRS resource sets, *** < Unchanged parts are ommitted> *** - when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in uplink slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set. *** < Unchanged parts are ommitted> *** ## *** < End of TP#9 for TP 38.214 V17.0.0> ***

Pathloss Reference RS Update MAC CE

Proposal 8: For Pathloss Reference RS Update MAC CE.

- Adopt following TP#10 for TS 38.213 V17.0.0.

## *** < Beginning of TP#10 for TS 38.213 V17.0.0> *** 7 Uplink Power control *** < Unchanged parts are ommitted> *** A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe, μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE without taking into account timing advance, μ is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#10 for TS 38.213 V17.0.0> ***

BFR MAC CEs

The timing relationship for beam failure recovery has been discussed several rounds and the following agreement has been made at RAN1#107-e.

RAN1#107-e:

Agreement

On beam failure recovery procedure, for PRACH transmission in uplink slot n, UE monitors the corresponding PDCCH starting from downlink slot “n + K_mac + 4” within a corresponding RAR window.

For the similar reason to the MAC CE command, we propose the following TP#12 to clarify the slot which is the uplink or the downlink as the agreement for the beam failure recovery.

Proposal 9: For beam failure recovery procedure.

- Adopt following TP#11 for TS 38.213 V17.0.0.

## *** < Beginning of TP#11 for TS 38.213 V17.0.0> *** 6 Link recovery procedures *** < Unchanged parts are ommitted> *** For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4+2μ∙kmac without taking into account timing advance, where 𝜇 is the SCS configuration for the PRACH transmission and 𝑘mac is a number of slots provided by K-Mac [12, TS 38.331] or 𝑘mac =0 if K-Mac is not provided, within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. *** < Unchanged parts are ommitted> *** ## *** < End of TP#11 for TS 38.213 V17.0.0> ***

K_offset value by a MAC CE command

It was agreed in RAN1#106-e to support the K_offset can be provided and updated by network with MAC CE as follows.

RAN1#106-e:

Agreement:

- The UE-specific K_offset can be provided and updated by network with MAC CE.

- FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration

  - FFS: Details on whether and how the two solutions work together

In our understanding, the UE-specific K_offset is used to the uplink slot, so we propose to clarify some details related to the current spec.

Proposal 10: For K_offset value by a MAC CE command.

- Adopt following TP#12 for TS 38.213 V17.0.0.

## *** < Beginning of TP#12 for TS 38.213 V17.0.0> *** 9 UE procedure for reporting control information *** < Unchanged parts are ommitted> *** For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. *** < Unchanged parts are ommitted> *** ## *** < End of TP#12 for TS 38.213 V17.0.0> ***

Conclusion

In this contribution, we have discussed the timing relationship enhancement. The following proposals were made:

Proposal 1: For SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE.

- Adopt following TP#1 for TP 38.214 V17.0.0

Proposal 2: For TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

- Adopt following TP#2 for TP 38.214 V17.0.0 and TP#3 for TS 38.213 V17.0.0.

Proposal 3: For Aperiodic CSI Trigger State Subselection MAC CE.

- Adopt following TP#4 for TP 38.214 V17.0.0.

Proposal 4: For SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

- Adopt following TP#5 and TP#6 for TP 38.214 V17.0.0.

Proposal 5: For SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

- Adopt following TP#7 for TP 38.214 V17.0.0.

Proposal 6: For SP SRS Activation/Deactivation MAC CE or SP Positioning SRS Activation/Deactivation MAC CE.

- Adopt following TP#8 for TP 38.214 V17.0.0.

Proposal 7: For Enhanced SP/AP SRS Spatial Relation Indication MAC CE.

- Adopt following TP#9 for TP 38.214 V17.0.0.

Proposal 8: For Pathloss Reference RS Update MAC CE.

- Adopt following TP#10 for TS 38.213 V17.0.0.

Proposal 9: For beam failure recovery procedure.

- Adopt following TP#11 for TS 38.213 V17.0.0.

Proposal 10: For K_offset value by a MAC CE command.

- Adopt following TP#12 for TS 38.213 V17.0.0.

Reference

[1]        R1-2111314, “Discussion on timing relationship enhancement”, OPPO, RAN1#107-e.

[2]        Chair's Notes RAN1#107-e v18