3GPP TSG RAN WG1 #108-e		R1-2201283

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.8.1.1

Source: 	OPPO

Title:	Enhancements on PUSCH repetition type A

Document for:	Discussion

Introduction

In RAN1#107bis-e meeting, Enhancements on PUSCH repetition type A was discussed, and the following agreements were made [1].

Conclusion:

No consensus to introduce pusch-AggregationFactor-r17.

Agreement

- Remove the notes from “Per (UE, cell, TRP, …)” and “Comment” columns of the existing AvailableSlotCounting in the consolidated RRC parameter list.

- If separate FGs are defined for DG-PUSCH and CG-PUSCH, add another AvailableSlotCounting to the consolidated RRC parameter list, with the following contents.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in ConfiguredGrantConf | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. |

Conclusion:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

Conclusion

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

In this contribution, the remaining issues on Enhancements on PUSCH repetition type A are discussed.

Enhancements for PUSCH Repetition Type A

The slot indicated by K2 offset for the available slot counting

During the email discussion in RAN1#107bis, there is no consensus on whether the slot indicated by K2 offset is by default an available slot or not. It should have the clarification on this point, because different understanding on this point would cause mis-aligned assumptions between the UE and the network.

In terms of relation between K2 timeline and available slot determination, following assumptions were discussed during the last meeting.

For DG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following

- Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

- Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot indicated by K2 offset.

- Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot indicated by K2 offset and does not perform the PUSCH transmission in later slots, either.

- Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

For DG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following.

- Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

- Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following

- Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

- Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

- Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and does not perform the PUSCH transmission in later slots, either.

- Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following

- Option 1: A UE does not assume that the slot determined in 38.321 Section 5.8.2 is the slot which is not counted in K available slot(s).

- Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

In our view, when AvailableSlotCounting is enable, the counting of PUSCH repetitions is based on the available slot. Thus, if the K2 timeline slot is not an available slot, the K2 timeline slot is not the start slot counted in K available slot(s), i.e. UE could determine the first available slot behand K2 timeline as the first slot of counted in K available slot(s). And it is same understanding for K=1 and K>1.

When AvailableSlotCounting is disabled, the counting of PUSCH repetitions is based on the physical slot. Thus, the K2 timeline slot is always the slot counted in K continuous physical slots. When K=1 and the K2 timeline slot is overlapped with DL symbol or SSB symbol, it means no PUSCH transmission, i.e. the scheduling is useless.

Observation 1: When AvailableSlotCounting is enable, the K2 timeline slot is not the start slot counted in K available slot(s) when the slot is not an available slot.

Observation 2: When AvailableSlotCounting is disable, the K2 timeline slot is always the start slot counted in K continuous physical slot(s).

Proposal 1: For DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enables, and for K=1. We both prefer option 2-2.

Proposal 2: For DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enables, and for K>1. We both prefer option 2.

In last meeting, there is also no consensus on the issue whether available slot counting is applicable to PUSCH repetition Type A with K=1 or not. Regarding the clarification for the relation between K2 timeline and available slot determination, we support the available slot counting is applicable to PUSCH repetition Type A with K=1.

Proposal 3: For DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enable, a slot for PUSCH repetition Type A with K=1 is determined by available slots.

Correction on available slot counting for FDD

In last meeting, TP on available slot counting for FDD were discussed, and there is no consensus on this issue. Two alternatives TP were provided to down selection in this meeting.

- TP#A - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2, irrespective of whether AvailableSlotCounting is enabled or not. - Except for the case of reduced capability half-duplex UEwith AvailableSlotCounting enabled, in which case a slot is not counted in the number of slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] - 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - Irrespective of whether AvailableSlotCounting is enabled or not, the UE shall repeat the TB across the consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - Except for the case of reduced capability half-duplex UE with AvailableSlotCounting enabled, in which case a slot is not counted in the number of slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- TP#B‘ - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - A reduced capability half-duplex UE with AvailableSlotCounting enabled determines slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] - 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - The UE shall repeat the TB across the consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the slots applying the same symbol allocation in each slot. A slot is not counted in the number of slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

In our view, TP#B with a separate bullet seems clearer.

Proposal 4: Slightly prefer TP#B.

Conclusions

In this contribution, whether the slot indicated by K2 offset for the available slot counting is an available slot or not, and the correction on available slot counting for FDD are discussed. As summary, we have the following proposals:

Observation 1: When AvailableSlotCounting is enable, the K2 timeline slot is not the start slot counted in K available slot(s) when the slot is not an available slot.

Observation 2: When AvailableSlotCounting is disable, the K2 timeline slot is always the start slot counted in K continuous physical slot(s).

Proposal 1: For DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enables, and for K=1. We both prefer option 2-2.

Proposal 2: For DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enables, and for K>1. We both prefer option 2.

Proposal 3: For DG-PUSCH and CG-PUSCH, when AvailableSlotCounting is enable, a slot for PUSCH repetition Type A with K=1 is determined by available slots.

Proposal 4: Slightly prefer TP#B.

Reference

- Chair's Notes RAN1#107bis-e v13