3GPP TSG RAN WG1 #108-e							R1-2201288

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.15.7

Source: 	OPPO

Title:	Rel-17 UE Power Saving features

Document for:	Discussion

Introduction

In RAN1#107bis-e, the UE feature of UE power saving enhancements for NR was discussed. Including Feature Groups 29-1 Paging Enhancement, 29-2 TRS resources for IDLE/Inactive UEs and 29-3 PDCCH monitoring adaptation within an active BWP [1]. In this contribution, we share our views on NR Rel-17 UE Power Saving features.

UE Features of UE Power Saving Enhancements for NR

- UE feature for paging enhancement (UE feature 29-1)

FG 29-1 is captured as below.

| Features | Index | Feature group | Components |  |  |  | Consequence if the feature is not supported by the UE | Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |

During last meeting, whether UE subgroup indication should be separated was discussed, and RAN1 send the LS to RAN2 informing the RAN1 understanding of the consequence if UE separately or jointly supports component 1 and/or 2.

From UE capability’s perspective, in our view, both paging subgrouping and PEI are paging enhancements in different aspects. Although support for both features may bring the most significant UE power saving gain, for UEs supporting PEI only, unnecessary UE paging reception may also be reduced compared to legacy UEs. If we decouple the UE capabilities for paging subgrouping and PEI, UE may choose whether to support PEI only or support both features, which would be more flexible for UE implementation. Based on this, we prefer to introduce separate UE capabilities for PEI and paging subgrouping. At the same time, some companies indicate even if the two components are included in a single FG, but a UE can be configured with PEI without subgrouping, RAN1 still faces the same issue that subgroup index is undefined in current RAN1 specification.

Regarding the “Components” contents of FG 29-1, one company also suggest a modifying as below. Whether UE support paging subgrouping or not, it need have the capability to receive DCI format 2_7. The modifying is OK for us.

Proposal 1: Prefer component 2 is separated from 29-1.

Regarding the type of the UE feature, in terms of power saving, per band is a little more complicated, we think per-UE is sufficient.

Proposal 2: For the UE feature 29-1, the capability type is per UE.

- TRS resources for idle/inactive UEs (UE feature 29-2)

FG 29-2 is captured as below.

| Features | Index | Feature group | Components |  |  |  | Consequence if the feature is not supported by the UE | Type |  |  |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | UE cannot receive TRS resources for idle/inactive mode | Per UE | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional without capability signalling |

Regarding the type of the UE feature, per-UE is sufficient.

Proposal 3: For the UE feature 29-2, the capability type is per UE.

If the UE supports 29-2 but not 29-1, then UE can only read TRS availability from paging DCI. If UE supports 29-2 and 29-1 then UE can used PEI based indication for TRS availability.

Proposal 4: Support the note “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1”.

- PDCCH monitoring adaptation within an active BWP (UE feature 29-3)

In RAN1#107bis-e, the following UE feature 29-3 was agreed.

Agreement

- FG 29-3c is confirmed as “3 search space sets group switching” as follows

| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 29-3d is confirmed as “2 search space sets group switching with PDCCH skipping” as follows

| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-23b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the type of the UE feature, per-UE is sufficient.

Proposal 5: For the UE feature 29-3, the capability type is per UE.

Conclusions

As summary, we have the following proposals:

FG29-1:

Proposal 1: Prefer component 2 is separated from 29-1.

Proposal 2: For the UE feature 29-1, the capability type is per UE.

FG29-2:

Proposal 3: For the UE feature 29-2, the capability type is per UE.

Proposal 4: Support the note “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1”.

FG29-3:

Proposal 5: For the UE feature 29-3, the capability type is per UE.

Reference

- R1-2200678 Summary on UE features for UE power saving enhancements	Moderator