3GPP TSG RAN WG1 #108-e	R1-2201293

e-Meeting, February 21th – March 3th, 2022

Source:	OPPO

Title:	Discussion on Scheduling and HARQ enhancement

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In RAN1 107 e-meeting, several options for intra prioritization were discussed. At the last round, the following 2 options are to be discussed further:

Option 2: The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels.

  - Any HP channel with a corresponding DCI that overrides or overlaps with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all HP DCIs corresponding to these HP channels must arrive Tproc,2+d1 before the earliest symbol of the LP channel that would be cancelled by the any of these HP channels.

  - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channel

Option 5: The UE makes a determination about canceling the LP channel at the cancellation deadline. This determination is based on the multiplexing/overriding of the HP channels that are determined up to the cancellation deadline.

  - Multiplexing/overriding of the HP channels are performed based on their associated timelines defined in R15.

  - Each and every dynamically scheduled HP channel as well as the HP channels that are the result of the HP channel multiplexing/overriding and are overlapping with a LP channel should satisfy the cancellation timeline, i.e., the gap between the ending symbol of the HP DCI to the starting symbol of that HP channel should be at least Tproc,2+d1.

  - The UE cancels the LP channel starting from the first symbol that overlaps with the HP channel at the latest, i.e., the current specification wording is kept.

  - Once a LP channel is determined to be cancelled at the cancellation deadline, a UE is not expected to revert its decision.

In this contribution, the above 2 options will be analyzed.

Discussion

Option 2 brings forward multiplexing timeline in some cases. As shown in Figure 1[1], Case a is allowed due to the second HP DCI is received before cancellation deadline. Case b is not allowed due to the second HP DCI is received after cancellation deadline. Option 2 restricts scheduling for HP channel. In some cases, especially for the same numerology and processing capability for HP and LP channel, the restriction is very limited, e.g. 1-2 symbols. However, in some cases, especially for different processing capabilities and different numerologies for HP and LP channel, the restriction become serious.

Option 5 keeps R15 multiplexing timeline but still introduces intermediate multiplexing, which increases implementation complexity and introduces large specification work. Before we go to option 5, we’d like to clarify that the multiplexing channel determination in section 9.2.5 in 38.213 is still performed completely for intermediate multiplexing due to overlapping situation for resultant multiplexing channel maybe different from original to-be-multiplexed channel.

Case a                                                                                      Case b

Figure 1 Example for intra UE prioritization

In short, Opiton 2 can avoid unnecessary cancellation of LP channel but timeline for HP channel multiplexing is further restricted. Option 5 ensures flexible scheduling for URLLC but may lead unnecessary cancellation of LP channels and intermediate multiplexing at the deadline of cancellation. Intermediate multiplexing is a new concept, which leads additional implementation and specification work. At this stage, we prefer to a simple solution, i.e. option 2. And we could compromise to option 5 if the whole solution is very clear.

Proposal 1: Option 2 is preferred and Option 5 can be acceptable.

Conclusions

In this contribution, the following proposal is made:

Proposal 1: Option 2 is preferred and Option 5 can be acceptable.

Reference

[1] R1-2112889	Summary of email discussion [107-e-NR-L1enh-URLLC-05] on UCI multiplexing and prioritization in Rel-16	Moderator (Apple)