3GPP TSG RAN WG1 #108-e                                                                                             R1-2201308

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft reply LS on interruption for PUCCH SCell activation in invalid TA case

Response to:	R1-2200871 (R4-2120420)

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	CATT [RAN1]

To:	RAN WG4

Cc:	RAN WG2

Contact person:

Name:	Yanping Xing

E-mail Address:	xingyanping@catt.cn

Attachments:

1	Overall description

RAN1 would like to thank RAN4 for the questions in LS in R4-2120420 on interruption for PUCCH SCell activation in invalid TA case.

Please find RAN1 answers to the questions below.

According to TS38.213 Clause 8.1, for single cell operation and intra-band CA, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when the gap between PRACH transmission and PUSCH/PUCCH/SRS transmission is less than N symbols.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

For inter-band CA, the UE capability of parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination is defined by parallelTxPRACH-SRS-PUCCH-PUSCH regardless of whether the SCS of PRACH and the SCS of SRS/PUCCH/PUSCH are the same or different.

2	Actions

To RAN4

ACTION: 	RAN1 respectfully asks RAN4 to take the above answers into account.

3	Dates of next TSG RAN WG1 meetings

RAN1#109-e	2022-05-16 – 2022-05-27	Online

RAN1#110	2022-08-22 – 2022-08-26	Toulouse, France