3GPP TSG RAN WG1 #108-e		                                                                 R1-2201318

e-Meeting, February 21st – March 3rd, 2022

Title:	[Draft] Reply LS on the reporting of the Tx TEG association information

Response to:	R2-2201776

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:

To:	RAN WG2

Cc:	RAN WG4, RAN WG3

Attachment:	-

Contact Person:

Name:	Xiaotao REN

E-mail Address:	renxiaotao@catt.cn

Overall description

RAN1 thanks RAN2 for the reply LS on the reporting of the Tx TEG association information, and would provide responses to the following questions from RAN2.

Firstly, RAN2 discussed the parameters on Mitigation of UE Rx/Tx timing delays and Mitigation of TRP Rx/Tx timing delays indicated in RAN1 LS (R1-2112976) and found the issues listed below which will need to be confirmed by RAN1: - These two duplicated parameters can be deleted in the RAN1 parameter table: - numOfUERxTEG-PerPRSResource: duplicated with MeasPRSwithDiffRxTEGs_Request_RSTD - numOfTRPRxTEG-PerPRSResource_RTOA: duplicated with MeasPosSRSwithDiffRxTEGs_Request, - The value range of maxNumOfUE-RxTEG is supposed to be 32, rather than as 8 in the table.

Response to RAN2:

Yes. The issues listed above are confirmed. RAN1 had corrected these issues in the updated RRC parameters table.

Secondly, RAN2 does not conclude that if the srs-PosResourceSetId is required or not in UE TxTEG association report.

Response to RAN2:

From RAN1’s perspective, there is no need to include the positioning SRS resource set ID in ueTxTEG association report, since srs-PosResourceId is global and uniquely represents the transmission occasions of SRS resource for positioning.

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#109-e			16-27 May 2022   			E-Meeting