3GPP TSG RAN WG1 #108-e                                              R1-2201342

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.14.1

Source:	CATT

Title:	Remaining issues on time and frequency synchronization enhancement for IoT over NTN

Document for:	Discussion and Decision

Introduction

In the RAN1#107e meeting, we made a good progress and there were some agreements and conclusions reached on UL time and frequency synchronization for IoT NTN. Some agreements and FFS points are listed as the follows[1]:

Agreement

For UL Segmented transmission during RRC_CONNECTED:

- If a segment duration is configured, the UE is expected to adjust the value for pre-compensation for a segment.

- FFS: UL transmission segment duration for NPDCCH ordered NPRACH/NPUSCH for NB-IoT and PDCCH ordered PRACH/PUSCH/PUCCH for eMTC is configurable by dedicated RRC Signalling

- For UE pre-compensation per segment, further discuss how the following options apply from one segment to the next segment, and potential down-selection among the options:

  - Option 1: Skip / drop / insert samples

  - Option 2: puncture OFDM symbols

  - Option 3: Blanking subframes/slots where UE skip a slot or a subframe

  - FFS whether this can be left to UE implementation or if specification impact is needed

Agreement

For eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration.

Agreement

The following agreements from NR NTN are re-used for IoT NTN

- The granularity of Common TA is set to be 1.Ts

Conclusion

The following conclusion from NR NTN is re-used for IoT NTN

- Conclusion: Do not define a TA margin.

Agreement

For DL synchronization enhancements:

- Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used, otherwise for bands where RAN4 can introduce a 200 kHz channel raster there is no signalling of the part-of ARFCN indication on MIB.

For NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC:

Agreement

UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples / Puncture OFDM symbols

- UE may blank subframes / slots where UE skip a slot or a subframe

The total transmission time is not changed

UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot

The method used for the UE pre-compensation is known to the eNB by a single UE capability

- UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing)

FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

For NPRACH for NB-IoT and PRACH for eMTC:

Agreement

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples

- UE may blank subframe / repetition unit where UE drops a subframe / repetition unit

The total transmission time is not changed

FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit

FFS Specification impact

Agreement

For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble.

- The total transmission time is not changed

- FFS Details of method(s) to drop / insert samples

UL segmented transmission configuration:

Agreement

UL transmission segment duration with one value X per NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC may be indicated on SIB.

- For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH/PUCCH

- The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively.

Agreement

At least UL transmission segment duration with one value X for NPRACH for NB-IoT and PRACH for eMTC may be indicated on SIB

- For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPRACH/PRACH

- FFS One value X, one or more values Xi

Agreement

UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature.

Agreement

For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format.

- Format 0 and format 1: 3-bit field, K=6 candidate values 2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ)

- Format 2:  3-bit field, K=5 candidate values 1.6.(TCP+TSEQ), 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ)

Agreement

Support network re-configuration of UL transmission segment by dedicated RRC Signalling

Agreement

For IoT NTN, indicate two LSBs of the ARFCN in the MIB.

GNSS validity:

Agreement

The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

- X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

Agreement

Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity:

- For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated

- The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

Long UL Transmission

Agreement

For eMTC PUCCH, a 3-bit field to indicate K=7 values for the uplink transmission segment duration:

- 2 4 8 16 32 64 128 subframes

Agreement

For eMTC PUCCH/PUSCH with frequency hopping enabled, the UE can adjust the uplink transmit timing and transmit frequency when hopping to a new narrowband if the frequency hopping interval is less than or equal to the configured transmission segment duration.

Validity timer for UL synchronization:

Agreement

The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time.

Agreement

A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB.

Agreement

Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information.

Agreement

Validity timer duration is configured per cell and indicated to the UE in X bits with:

- Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240}

- Unit is second

- FFS Additional values for GEO

Synchronization aspects common to IoT NTN and NR NTN

Working assumption:

Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are indicated with the following range, granularity and bits allocation:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommon | 0 ...8316827 (i.e: 0… 270.73 ms) | 32.55208 ×10-3 μs | 23 bits |
| TACommonDrift | - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s) | 0.2×10-3μs/s | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 μs/s2) | 0.2×10-4μs/s2 | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

Agreement

Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network:

- Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network:

  - Position and velocity state vector ephemeris format is 17 bytes payload.

    - The field size for position (m) is 78 bits

      - Position range is driven by GEO : +/- 42 200 km

      - The quantization step is 1.3m for position

    - The field size for velocity (m/s) is 54 bits

      - Velocity range is driven by LEO@600 km: +/- 8000 m/s

      - The quantization step is 0.06 m/s for Velocity

  - Orbital parameter ephemeris format 18 byte payload

    - Semi-major axis α (m) is 33 bits

      - Range: [6500, 43000]km

    - Eccentricity e is 19 bits

      - Range: ≤ 0.015

    - Argument of periapsis ω (rad) is 24 bits

      - Range: [0, 2π]

    - Longitude of ascending node (Ω rad) is 21 bits

      - Range: [0, 2π]

    - Inclination i (rad) is 20 bits

      - Range: [- π/2 , + π/2]

    - Mean anomaly M (rad) at epoch time to is 24 bits

      - Range: [0, 2π]

Agreement

Using indicated Higher-layer Common TA parameters, if configured, the UE can determine the one-way propagation time ( Delaycommon) used for NTA, common  calculation as follows:

Delaycommont= DCommon tepoch+ DCommonDrift× t-tepoch+DCommonDriftVariation× t-tepoch2

Where:

- DCommon = TACommon 2, DCommonDrift=TACommonDrift2 and DCommonDriftVariation=TACommonDriftVariation2

- TACommon, TACommonDrift and TACommonDriftVariation are Common TA parameter defined in RAN1 Meeting #106-bis-e

- Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. DL and UL are frame aligned at the reference point with an offset given by NTA,offset.

- NTA, common is derived by the UE based on Delaycommont to pre-compensate the two-way transmission delay between the uplink time reference point and the satellite.

Up to now,we have achieved consensus on most IoT NTN issues of time and frequency synchronization. In the contribution, we further analyze the remaining issues and propose potential solutions for these issues.

Discussion

  - Long UL Transmission enhancements

Indication of UL transmission segment duration

In the last meeting, we have an agreement on configuration of UL transmission segment indicated on SIB for initial access. The duration of UL transmission segment is related to the characteristics of satellite orbiting. Satellite orbiting causes the propagation delay drift on the service link, but delay drift rate is related to the elevation angle. If allowing UE to report its elevation angle, it may cause additional power consumption. Another aspect is that UE would consistently monitor network configuration of segment duration, it will increase big complexity. Hence, UE specific and dynamic segment is not necessary since sporadic short transmission is mainly considered in Rel-17. Based on this context, we believe UE specific RRC signaling is not needed.

- UL transmission segment duration can be provided to UE by SIB message, and dedicated RRC signaling is not necessary.

New UL gaps

The maximum number of NPRACH repetitions in NB-IoT is 128. For frame structure type1, after transmissions of 4∙64(TCP+TSEQ)time units for preamble formats 0 and 1, or 16∙6(TCP+TSEQ) time units for preamble format 2, a gap of 40∙30720Ts time units shall be inserted. After transmitting (N)PUSCH with 256∙30720Tstime units, for NB-IoT and eMTC, a gap of time units shall be inserted, and next NPUSCH transmission is postponed. Since there is a UCG of 40 ms every 256 ms, it can also be used to update ephemeris.

For long (N)PRACH/(N)PUSCH transmission, the transmission is one by one contiguous transmission. Applying different TA may cause signal overlapping between previous segment and next segment. Hence, different solution should be considered to resolve this issue. PRACH in the eMTC applies ZC sequence and ZC sequence has good correlation property, so the signal overlapping may not be one big issue. Since NPRACH in the NB-IoT doesn’t use ZC sequence, so the impact of overlapping on NPRACH and (N)PUSCH should be evaluated further by the simulation, which is related to the timing error. One simple method is to drop a few of tail samples of last symbol of one PUSCH slot, or puncture the last symbol of one slot, which may cause some performance loss. Another method is to insert new small UL gaps to delay (N)PRACH or (N)PUSCH transmission for avoiding the effect of overlapping. The length of new UL gaps is related to the segment interval. The new UL gaps may be inserted by GP duration for (N)PRACH. The new UL gaps will cause slot misalignment for (N)PUSCH, if the length of new UL gaps is not the integer of a slot. If enforcing slot alignment, the new UL gaps will cost too much time resource. For small TA change, TA can be updated with implementation way, instead one gap insertion. For larger TA variation, small gap configuration should be considered, for example, reserving last symbol for one slot for TA gap. For IoT scenarios, UE specific segment duration may not be needed for simplicity, so one configuration for UL gap can be used for all users of one cell. In this sense, puncturing the last symbols of one segment is preferred.

- In order to avoid signal overlapping in segment compensation, the gap can be configured via last symbol puncturing of one segment.

Conclusion

In this contribution, we analyzed potential issues of UL timing and frequency synchronization impact due to longer propogation delay and satellite moving in NTN-IoT. We have some following observations and proposals for enhancement on time and frequency synchronization in NTN-IoT:

- UL transmission segment duration can be provided to UE by SIB message, and dedicated RRC signaling is not necessary.

- In order to avoid signal overlapping in segment compensation, the gap can be configured via last symbol puncturing of one segment.

References

- Chair's Notes RAN1#107e.

- 3GPP TR36.373 V17.0.0, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)

- TS 36.133, Requirements for support of radio resource management, 3GPP, 17.1.0, Mar 2021.

- R1-2112803	Summary #4 of AI 8.15.1 	Enhancements to time and frequency synchronization	Moderator (MediaTek)

- 3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

- 3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".