3GPP TSG RAN WG1 #108-e		                                                                             R1-2201346

e-Meeting, February 21st – March 3rd, 2022

Source:	CATT

Title:	Discussion on UE features of enhanced IIoT and URLLC

Agenda Item:	8.15.3

Document for:	Discussion and Decision

Introduction

In RAN1#107bis-e, updated Rel-17 UE feature lists in R1-2200780 [1] for NR is endorsed.

In this contribution, we discuss remaining issues of UE features for Rel-17 IIoT/URLLC.

Discussion

- FG 25-3b

| 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The prerequisite feature group for FG25-3b is TBD, which should be support of sub-slot based PUCCH repetition in our view. Accordingly, it is proposed that the prerequisite feature group for FG25-3b is 25-3.

Proposal 1: The prerequisite feature group for FG25-3b is 25-3.

- FG25-16

| 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

There is an FFS for FG25-16 on whether to separate capability for different UCI type. We do not think separate capability for different UCI type is needed. Therefore, it is proposed to delete the FFS.

In addition, for components 4 and 5, there is no need to support separate beta_offset values since PUSCH conveys UL-SCH only, i.e. no UCI of same priority is multiplexed in the PUSCH. For components 6 and 7, it is proposed to add support of separate beta_offset values.

Proposal 2: Update components for FG25-16 as follows:

1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs.

2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).]

3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.]

4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination.

5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination.

6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. Support separate beta_offset values for this priority combination.

7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. Support separate beta_offset values for this priority combination.

FFS whether to separate capability for different UCI type

- FG25-18

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The following conclusions were made in RAN1#107-e. So the FG25-18 is proposed to be updated according to the conclusions.

Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.

Proposal 3: Adopt the following update to FG25-18.

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells in an [at least] for inter-band CA band combination. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion

In this contribution, we discussed remaining issues of UE features for Rel-17 IIoT/URLLC with the following proposals.

Proposal 1: The prerequisite feature group for FG25-3b is 25-3.

Proposal 2: Update components for FG25-16 as follows:

1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs.

2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).]

3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.]

4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination.

5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination.

6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. Support separate beta_offset values for this priority combination.

7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. Support separate beta_offset values for this priority combination.

FFS whether to separate capability for different UCI type

Proposal 3: Adopt the following update to FG25-18.

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells in an [at least] for inter-band CA band combination. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#107bis-e

- R1-2200249, [draft] Summary on UE features for enhanced IIoT and URLLC, Moderator (NTT DOCOMO, INC.), RAN1#107bis-e