3GPP TSG RAN WG1 #108-e                                                                 R1-2201354

e-Meeting, February 21st – March 3rd, 2022

Source:	CATT

Title:	Remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation

Agenda Item:	8.2.5

Document for:	Discussion and Decision

- Introduction

The work item on supporting NR from 52.6 GHz to 71 GHz was updated and approved in RAN 90 meeting [1], and objects of WI are listed as in the following.

- Physical layer aspects including [RAN1]: - In addition to 120 KHz SCS, specify new SCS, 480 KHz and 960 KHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480 KHz to 960 KHz - Time line related aspects adapted to 480 KHz and 960 KHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively. - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120 KHz SCS for SSB and 120 KHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240 KHz, 480 KHz, 960 KHz) for SSB, and additional SCS(480 KHz, 960 KHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480 KHz, 960 KHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify timing associated with beam-based operation to new SCS (i.e., 480 KHz and/or 960 KHz), study, and specify if needed, potential enhancement for shared spectrum operation - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum - Evaluate, and if needed, specify the PTRS enhancement for 120 KHz SCS, 480 KHz SCS and/or 960 KHz SCS, as well as DMRS enhancement for 480 KHz SCS and/or 960 KHz SCS.

In this document, the remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation are discussed, which include activation of SPS PDSCH/CG PUSCH, some clarifications about scheduled PUSCH/PDSCH and 2 TB disabling/enabling related issues .

- Discussion on remain issues on PDSCH/PUSCH enhancements

- Issue 1) Activation/retransmission of DL SPS (or UL CG) by using DCI format 1_1

Agreement (RAN1#107bis-e) - A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

Agreement: For generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - HARQ-ACK bit corresponding to SPS PDSCH release or SCell dormancy indication without scheduled PDSCH, belongs to the first sub-codebook (which is defined in the previous agreement made in RAN1#105-e)

- In R15/R16, one DCI can activate at most one SPS-configure and only one SLIV is expected. When the SPS PDSCH without PDCCH is not decoded correctly, gNB may perform re-transmission by DCI format 1_0/1_1/1_2 scrambled by CS-RNTI and the NDI=1. Since multiple PDSCHs can be scheduled by a DCI format 1_1, the PDCCH indicating a TDRA row index may include more than one SLIV(s), for simple operation, it is also recommend that allowing only single SLIV-based for SPS PDSCH re-transmission. And for UL grant Type 2 PUSCH retransmission, only single SLIV-based is allowed.

- Proposal 1: When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV.

- Proposal 2: When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV.

When the DCI format 1_1 is used to perform SPS PDSCH activation and re-transmission, the scheme of only one SLIV requires that at least one single SLIV item is configured in TDRA parameter and only the single SLIV entry can be used for indication of SPS activation. In other words, if all entries are multi-SLIV(s) in the TDRA configuration, SPS PDSCH activation/re-transmission is not allowed to be triggered by DCI format 1_1, therefore only DCI format 1_0/1_2 can be used to activate/re-transmit the SPS PDSCH. It is too restrictive for TDRA configuration and SPS PDSCH signal transmission. One possible scheme to this issue is that when DCI format 1_1 is scrambled by CS-RNTI, pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is not used. That is if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 scrambled by C-RNTI or MCS-RNTI only.

- Proposal 3: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 scrambled by C-RNTI or MCS-RNTI only.

- Issue 2) Clarification on whether “scheduled PXSCH” in previous agreements implies valid PXSCH or not:

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Conclusion: (RAN1#105-e) For a DCI that can schedule multiple PUSCHs, - CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2. Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. Agreement: (RAN1#106bis-e) For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

Based on the above agreement, the UE/gNB will have different behaviors according to the number of PDSCH/PUSCH scheduled. To align the understanding between the gNB and UE, the following clarifications were raised in the previous meeting:

1) For NDI/RV, are NDI/RV fields for invalid PXSCHs present in multi-PXSCH scheduling DCI?

2) For RV field, is the bit-width between 1 bit and 2 bits determined based on the number of configured SLIVs or valid SLIVs?

3) For CSI-request, is the number M determined based on the number of configured SLIVs or valid SLIVs?

4) For CBGTI field, is the presence of CBGTI field determined based on the number of configured SLIVs or valid SLIVs?

For issue 1, theoretically, the NDI/RV fields for invalid PXSCHs should not be present in DCI, but the benefit in this case is not clear, since it cannot make total DCI size smaller due to the fact that DCI size shall keep constant for UE performs PDCCH detecting. The possible benefit is re-defining the purpose of NDI/RV bits designed for invalid PXSCHs. If this optimization is allowed, it may cause more discussion on this issue. Considering the current phase is maintenance and there is limited time for meeting discussion, we suggest that NDI/RV fields for both valid and invalid PXSCHs are present in multi-PXSCH scheduling DCI.

Proposal 4: NDI/RV fields for both valid and invalid PXSCHs are present in multi-PXSCH scheduling DCI.

For issue 2, as in the agreement for multi-PDSCHs scheduling, if only one PDSCH is scheduled, bit width of RV is 2 bits, and if more than one PDSCH is scheduled, bit width of RV is 1 bit for each PDSCH scheduled. If the bit-width of RV between 1 bit and 2 bits is determined based on the valid SLIVs, the opportunity that using 2 bits indicate RV may increase, and potential soft combining gain can be achieved. But this will cause more complexity on both UE and gNB side.

For issue 3, the aperiodic CSI feedback will be carried on the M-th PUSCH scheduled, the M is computed according to number of PUSCHs scheduled. If the number is determined based on configured SLIVs, the M-th PUSCH scheduled may be dropped when the PUSCH is overlapping with downlink symbols. This will cause the problem that the aperiodic CSI feedback cannot be transmitted to the gNB. In our view, this problem can be resolved by gNB implementation and no optimize by specification is needed.

For issue 4, when CBG-based (re)transmission is configured for single PUSCH scheduling, UE shall interpret the bit fields of DCI as different based on number of scheduled PUSCH. If the number of scheduled PUSCH(s) is based on valid SLIVs, more CPUs resources will be needed since UE needs to consider more factors (e.g., slot format information, overlapping with CG PUSCH). All these will impact the capability of PDCCH detection.

Proposal 5: For cases of RV/ M on CSI-request / CBGTI, the parameters are determined based on configured SLIV(s).

- Issue 3) Type-2 HARQ-ACK codebook generation

- Based on enabled or configured TBs for bundling grouping

Agreement (RAN1#107bis-e) For multi-PDSCH scheduling with a single DCI and for type-2 HARQ-ACK codebook generation, - Time domain bundling and spatial bundling can be independently configured.

- It was agreed that second TB can be supported when the multiple PDSCHs are scheduled by DCI format 1_1. When maxNrofCodeWordsScheduledByDCI is configured to 2, how to generate the HARQ-ACK bits needs further discussion. This is especially true for case of time domain bundling is configured but spatial bundling is not configured

It is assumed that the maximum number of PDSCH can be scheduled is 8, and numberOfHARQ-BundlingGroups is 2. The total 8 bits of HARQ-ACK are expected to feedback for a DCI format 1_1.

The possible scheduling scenario is 8 PDSCHs are scheduled and one TB is disabled as indicated by MCS and RV indication combination (e.g. MCS=26, and all 8 bits of RV is “1”), it is shown as the following figure.

Figure 1 multiple PDSCHs scheduling and only one TB is enable

In figure 1, TB (1, 1) represents first TB of PDSCH-1 scheduled by DCI format 1_1, and TB (1, 2) represents second TB of PDSCH-1 scheduled by DCI format 1_1, and so on. There are two possible schemes to generate HARQ-ACK.

- Option 1: Based on scheduled /enabled TB(s)

In this scheme, the HARQ-ACK bits of 8 TBs scheduled join the time domain bundling procedure, and the TBs that are disabled by DCI indication will not join time domain bundling procedure. For the case as shown in Table 1, each scheduled/enabled TB can occupy 1 bit to feedback HARQ-ACK.

- Option 2: Based on maximum number configured TB(s)

In this scheme, the HARQ-ACK bits of 16 TBs scheduled join the time domain bundling procedure, including disabled/enabled TBs indicated by DCI. For the case as shown in Table 1, two TBs can occupy 1 bit to feedback HARQ-ACK.

For both option 1 and option 2, the total 8 bits HARQ-ACK are generated for a multi-PDSCH scheduling, but for option2, the last 4 bits is invalid and filled with NACK, gNB can know the NACK information when gNB transmits scheduling information, and not much value can be obtained. The time domain bundling result of the above two options is shown in the following listed table:

Table 1 scenario of 8 consecutive PDSCHs scheduled and 2nd TB is disabled

| Options | Result of HARQ-ACK bits information |
| --- | --- |
| Based on scheduled /enabled TB(s) | {TB(1,1)}, {TB(2,1)}, {TB(3,1)}, {TB(4,1)}, {TB(5,1)}, {TB(6,1)}, { TB(7,1)},{TB(8,1)}, |
| Based on configured TB(s) | {TB(1,1) TB(2,1)}, { TB(3,1) TB(4,1)},{TB(5,1) TB(6,1)},{TB(7,1) TB(8,1)} {X, X}, {X, X}, {X, X}, {X, X}, |

Proposal 6: For case of time domain bundling is configured, only TBs that are scheduled /enabled by DCI join time domain bundling procedure to generate Type-2 HARQ-ACK codebook.

Issue 4) Valid SLIVs vs Configured SLIVs for bundling grouping

Agreement For multi-PDSCH scheduling with a single DCI - Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell. - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell. - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell. - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method. - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured) - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook. - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook, - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

In the last RAN1#107b-e meeting, RAN1 has discussed how to make a bundling group and no agreement was reached about this. The solutions listed as follows:

- Based on configured SLIVs in a TDRA row

For this scheme, the SLIVs indicated by DCI are grouped without taking into account the SLIV’s validity. Therefore, a bundling group may contain invalid SLIVs only, and this will result that the HARQ-ACK information for the bundling group is always set to NACK. As gNB can know the NACK information when gNB transmits scheduling information, there is no more information that can be obtained.

- Based on valid SLIVs in a TDRA row

For this scheme, UE will use valid SLIVs (e.g. the SLIV including all down symbols) only to make a bundling group. Since every bundling group consist of valid SLIVs, all HARQ-ACK information for the bundling groups is meaningful. It will improve the efficiency of HARQ-ACK feedback.

Proposal 7: The PDSCHs corresponding to valid SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups

- Conclusion

This contribution discussed the remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation. These include activation SPS PDSCH/CG PUSCH, clarifications on some scheduling related issues and 2 TB disabling/enabling. The following proposals are given:

- Proposal 1: When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV.

- Proposal 2: When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV.

- Proposal 3: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 scrambled by C-RNTI or MCS-RNTI only.

Proposal 4: NDI/RV fields for both valid and invalid PXSCHs are present in multi-PXSCH scheduling DCI.

Proposal 5: For cases of RV/ M on CSI-request / CBGTI, the parameters are determined based on configured SLIV(s).

Proposal 6: For case of time domain bundling is configured, only TBs that are scheduled /enabled by DCI join time domain bundling procedure to generate Type-2 HARQ-ACK codebook.

Proposal 7: The PDSCHs corresponding to valid SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups

- Reference

RP-202925, “Revised WID:  Extending current NR operation to 71 GH”, CMCC

3GPP TSG RAN WG1 Meeting RAN1 #106-e chairman notes, August  16th – August  27th, 2021

Chair's Notes RAN1#106b-e, e-Meeting, October 11th – 19th, 2021

Chair's Notes RAN1#107-e, e-Meeting, November 11th – 19th , 2021

- [5]      3GPP TSG RAN WG1 Meeting RAN1 #107b-e chairman notes, January 17th-25th, 2022