3GPP TSG RAN WG1 #108	R1-2201410

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.16.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for enhanced IIoT and and URLLC

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined for selected topics [1, 2]. In this contribution we present our views on the UE features for enhanced IIoT and and URLLC.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107-e are available in the Annex.

- 25-1:

  - Per UE

- 25-2:

  - Per UE

- 25-3:

  - Per UE

- 25-3a:

  - The pre-requisite feature group from Cov. Enh. WI of the dynamic PUCCH repetition indication should be 30-5.

  - Per UE

- 25-3b:

  - Add 25-3 as pre-requisite.

  - Per UE

- 25-4/5:

  - Confirm the FG components (i.e. remove yellow highlight)

  - Per UE

- 25-6:

  - Remove the yellow marked ’type 3 or’ from the FG components, as Type 3 CB is a Rel-16 feature already.

  - Per UE

- 25-7:

  - Per UE

- 25-9:

  - No need for separate capabilities for different numerologies

Per UE

- 25-10:

  - No need for separate capabilities for different numerologies

  - Per UE

- 25-11:

  - Per UE

- 25-12:

  - Confirm the FG components (i.e. remove yellow highlight)

- 25-14:

  - Per UE

- 25-15:

  - Per UE

  - Confirm the capability indication for component 2, as RAN1 confirmed the earlier working assumption on the signaling values in RAN1#107bis-e (i.e. remove ’FFS’ and the yellow highlight from the Notes column)

- 25-16:

  - No need for separate capabilities per UCI type

  - Per UE

- 25-18:

  - Change to the FG description as discussed already during RAN1#107bis-e to:

Support simultaneous PUCCH/ and PUSCH transmissions of different priorities on different cells [at least] for inter-band CA.

- 25-19:

  - Per UE, FR1-only, with TDD/FDD differentiation. There is no need for per FS signaling due to dependency on 2-53. As already the case for Rel-16 MIMO, the UE will only support the feature for the bands and band combinations where the pre-requisites are fulfilled. Hence, this one can be reported with coarser granularity than its pre-requisites.

- 25-19a:

  - Remove 13-1 from the pre-requisite FG. First, the PRS reception for RTT is more limited (e.g. only a single PRS resource needed) than usual PRS reception for positioning. Moreover, this will remove the need to signal 13-1 to the gNB.

  - Per UE, FR1-only, with TDD/FDD differentiation. There is no need for per FS signaling due to dependency on 2-53. As already the case for Rel-16 MIMO, the UE will only support the feature for the bands and band combinations where the pre-requisites are fulfilled. Hence, this one can be reported with coarser granularity than its pre-requisites.

- 25-20:

  - Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for enhanced IIoT and and URLLC, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107bis-e, Jan. 2022

- R1-2200780, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), Jan. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. The values higher than 1 can be applied to sub-slot based configuration only. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A | Note: this feature applies to cells in the same TAG only | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A | Note: this feature applies to cells in the same TAG only | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a, 25-12 | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | FFS: Candidate value set for component 2: d3 = {0, 1, …, 2μ-1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure |  | Yes | N/A |  | Per UE | no | no | N/A |  | Optional with capability signaling |