3GPP TSG RAN WG1 #108	R1-2201415

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.16.8

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR coverage enhancement

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined for selected topics [1, 2]. In this contribution we present our views on the UE features for NR coverage enhancement.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107bis are available in the Annex.

- 30-1:

  - Move candidate values to notes column and add a proper description of the feature. No need to list the legacy values, hence only list values for K>16.

  - Add FG11-6 (PUSCH repetition Type A) as pre-requisite

  - Per UE

- 30-1a:

  - Similarly to FG30-1, move values to notes column and restrict range to K>16, with proper description of the FG itself.

  - Per UE

- 30-2:

  - Confirm FG description

  - Add 30-1 as pre-requisite

  - Per UE

- 30-2a:

  - Confirm FG description

  - Replace pre-requisite FG5-16 with FG30-1a

  - Do not merge with 30-2

  - Per UE

- 30-3:

  - Confirm FG description

  - OK to split in 2 FGs for DG ad CG, for consistency with other FGs for coverage enhancements

  - Per UE

- 30-4/a/b/c/d:

  - Per band

- 30-4g:

  - Description of "Feature group" should be (changes are highlighted in red):

    - "Restart DM-RS bundling after the dynamic events that violate power consistency and phase continuity"

  - Description of "Components" should be (changes are highlighted in red):

    - "Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity, except events which are triggered by DCI or MAC CE but regarded as semi-static events"

  - Description of "Consequence if the feature is not supported by the UE" should be (changes are highlighted in red):

    - "UE does not support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity"

- 30-4h:

  - It is not consistent if 30-4h is the only place where it is indicated that 30-4a/b/c/d are limited to back-to-back cases. The simpler solution seems to be to update the description of 30-4/a/b/c/d to indicate this restriction explicitly, in which case the yellow highlighted text in 30-4h can be confirmed as is.

- 30-5:

  - Confirm FG description

  - Per UE

- 30-6:

  - Confirm FG description

  - Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR coverage enhancement, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107bis-e, Jan. 2022

- R1-2200780, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), Jan. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A | FFS whether to Add a note in FG 30-4c: “Note: If a UE reports support of FG [30-4a], 30-4c, [30-3] and/or 30-3a, the UE supports DMRS bundling for the repetitions of TBoMS” | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | 4-23 and/or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |