3GPP TSG RAN WG1 #108	R1- 2201421

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.16.14

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for IOT over NTN

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined for selected topics [1, 2], however IOT over NTN has not been part of the scope of RAN1#107-e. In this contribution we present our views on the UE features for IOT over NTN.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107-e are available in the Annex.

- General:

  - Keep the current structure, with essential FGs combined in a single one.

  - ”Need for the eNB to know if the feature is supported” should be ”yes”

  - Notes for all FGs can be removed, they are not essential information for specifications

  - Component 2: keep ”receive” instead of ”indicate”

- 2-1:

  - Remove components 12 and 13 as they are duplicated in FG 2-2.

  - Confirm component 16 on K_mac support (see related agreements below).

Agreement:

For NB-IoT, if the UE has initiated an NPUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the NPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN).

Agreement:

For eMTC, if the UE has initiated an PUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the MPDCCH from DL subframe n+4+K_mac (where K_mac is defined as in NR-NTN).

- 2-2:

  - Per UE, for UEs supporting IOT over NTN

  - In principle a single FG for NB-IOT and eMTC is enough.

- 2-3:

  - Per UE, for UEs supporting IOT over NTN

  - In principle a single FG for NB-IOT and eMTC is enough, but it should be optional for eMTC UE, as it may support full-duplex as well.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for IOT over NTN, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107bis-e, Jan. 2022

- R1-212900, ”Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e”, Moderators (AT&T, NTT DOCOMO, INC.), Jan. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE report the validity duration of GNSS] 2. Receive serving satellite ephemeris in either state vector format or orbital element format 4. UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE applies common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops ] 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] [16. UE receives cell specific K_mac] 17. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segment duration configuration | Support segment duration configuration by the network |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | - UE applies UE specific K_offset in timing relationship enhancements | 2-1 [, 2-3] | Yes | N/A | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | - [Support reporting of information about the UE specific TA pre-compensation] | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |