3GPP TSG RAN WG1#108-e                                              	            R1- 2201430

e-Meeting, February 21st –March 3rd, 2022

Agenda Item:	8.1.3

Source:	Lenovo, Motorola Mobility

Title:	Enhancements on SRS

Document for:	Discussion

Introduction

Basic SRS function is specified in Rel-15 and enhanced in Rel-16 to support high efficiency operation in FR2. The SRS will be further enhanced with more flexible UL and DCL channel sounding procedure as listed in the approved WID[1]:

- Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

The following agreements are achieved in RAN1#104e[4]:

Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in1PFmSRS, BSRS contiguous RBs in one OFDM symbol, where mSRS, BSRS indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the 1PFmSRS, BSRS RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17. Agreement - For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration. - For 1T6R, K=6, N_max = [4], and each resource has 1 port. - For 1T8R, K=8, N_max = [4], and each resource has 1 port. - For 2T6R, K=3, N_max = [3], and each resource has 2 ports. - For 2T8R, K=4, N_max = [4], and each resource has 2 ports. - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports. - FFS the number of supported candidate values of N for each xTyR. - FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources - FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS - Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability. Agreement Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples - CAT A: Time-domain parameters - A-1: Indication of available slot position, i.e., the t values - A-2: Indication of slot offset - A-3: Indication of SRS symbol-level offset - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting - CAT B: Frequency-domain parameters - B-1: Indication of a group of CCs for SRS transmission - B-2: Indication of frequency domain resource in a BWP for SRS transmission - B-3: Indication of whether DL/UL BWP is applied for SRS transmission - CAT C: Power control parameters - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ - FFS impact on power control, impact from triggering a group of CCs for SRS - C-2: Indication of open loop power control parameter e.g., p0. - CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming - CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states - Other examples are not precluded Agreement A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t. - In DCI format 0_1/0_2 without data and without CSI request, - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 1-2: Re-purpose unused DCI field to indicate t - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB - FFS design details with other potential field(s) - FFS: whether t can be slot offset - In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 2-1: t is indicated by adding a new configurable DCI field - Alt 2-2: t is indicated without adding DCI payload - Note: The size of DCI payload does not change dynamically - Note: RAN1 should strive for unified solution for different DCI formats. - FFS: The number of RRC configured t values per SRS resource set and DCI bit field size. Agreement Confirm the following working assumption with modifications An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI

The following agreements are achieved in RAN1#104bis-e[5]:

Agreement For increased repetition in Rel-17, support the following Nsymbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values - Nsymbol = 8, R = {1, 2, 4, 8} - Nsymbol = 12, R = {1, 2, [3], 4, 6, 12} - FFS the following configurations - N_symbol = 10, R = {1, 2, 5, 10} - N_symbol = 14, R = {1, 2, 7, 14} - FFS options to reduce SRS BW for R>1 Agreement On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values - 1T6R: N_max = 3 - 1T8R: N_max = 4 - 2T6R: N_max = 3 - 2T8R: N_max = 4 - [4T8R: N_max = 2] - The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed R1-2103878 FL summary #2 on SRS enhancements Moderator (ZTE) Agreement For RB-level partial frequency sounding (RPFS) in Rel-17 - The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1} - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern - Support to determine PF and Noffset at least via RRC configuration per SRS resource. - FFS whether to introduce DCI and/or MAC CE in addition Working Assumption For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured Agreement On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e - Alt 1: All the non-zero integer values <= N_max are supported for N - Alt 2: Support N=N_max only - Alt 3: Support specific N values <= N_max - FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot - FFS: whether different alternatives may be selected for different xTyR configuration Agreement Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs Agreement - Up to 4 “t” values can be configured per SRS resource set. R1-2103928 FL summary #3 on SRS enhancements Moderator (ZTE) Agreement - For RPFS in Rel-17, support PF = {2, 4}. - FFS 3, 8, 12, 16 or fractional numbers - Support at least one of the following alternatives (to be decided in RAN1#105-e) - Alt 1:1PFmSRS, BSRS 1PFmSRS, BSRS is an integer value - Alt 2:1PFmSRS, BSRS 1PFmSRS, BSRS is an integer value with minimum value 4 - Alt 3: 1PFmSRS, BSRS is a multiple of 4 - Alt 4: Round 1PFmSRS, BSRS1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2 Agreement On aperiodic SRS configuration for antenna switching with 4T8R, support N_max = 2 Agreement For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e) - Alt 1: Generate length-12PFmSRS, BSRSComb12PFmSRS, BSRSComb ZC sequence - Alt 2: Truncate from legacy length-12mSRS, BSRSComb12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS Agreement For antenna switching, support one of the following - Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set - Note: the two SP-SRS resource sets are not activated at the same time. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed - FFS whether configurations on SRS repetitions have impact - FFS relevant UE capability design

The following agreements are achieved in RAN1#106-e[6]:

Agreement Confirm the following WA. For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured Agreement Support start RB location (Noffset) hopping in different SRS frequency hopping periods for RPFS and at least periodic/semi-persistent SRS, where Noffset Noffset is the start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs. - For a given SRS transmission occasion, Noffset=kF+khopping mod PFPFmSRS, BSRS , where khopping is same for all SRS occasions within a legacy FH period but changes across legacy FH periods, kF and PF are at least configured by RRC signaling (kF = {0, 1, …, PF-1}). - Support at least one pattern for khopping in time domain, FFS detailed pattern - Note: the legacy FH period is the period to sound the full SRS hopping bandwidth across the different subbands of mSRS, BSRS RBs each. - This start RB location hopping is enabled or disabled by RRC signaling. - FFS whether MAC CE or DCI can be additionally used - When this start RB location hopping is disabled,khopping khopping is fixed to be 0 for all SRS symbols - This start RB location hopping is UE optional. - FFS whether start RB location hopping is also applicable on SRS occasion(s) within one FH period (e.g., when R>1) and/or on aperiodic SRS, if so, how Agreement For aperiodic xTyR antenna switching SRS, where xTyR is from {1T6R, 1T8R, 2T6R, 2T8R, 4T8R}, support all the non-zero integer values N<=N_max except N=1 for 1T8R - For each xTyR configuration, UE does not expect multiple SRS resource sets are configured or triggered in one slot - UE does not expect that the OFDM symbols contained in one SRS resource set exceed UE capability on which OFDM symbols can be used for SRS taking guard period into account Agreement Support Opt. 2: Reference slot is the slot indicated by the legacy triggering offset. - If DCI is transmitted in slot n, and k is the legacy triggering offset, reference slot is slot n+k. - Note: the legacy triggering offset can be 0, if slotOffset is absent. Decision: As per email posted on Aug 20th, Conclusion MAC CE for t value update in Rel-17 is not supported. R1-2108373 FL summary #2 on SRS enhancements Moderator (ZTE) From GTW session: Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS Agreement Support 4T6R SRS antenna switching in Rel-17. Agreement For RPFS SRS sequence generation, support - Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence. Agreement For SRS increased repetitions in Rel-17, support the following configurations, and no other values are supported. - (N_symbol, R) = {(8, 1), (8, 2), (8, 4), (8, 8), (12, 1), (12, 2), (12, 3), (12, 4), (12, 6), (12, 12), (10, 1), (10, 2), (10, 5), (10,10), (14, 1), (14, 2), (14, 7), (14, 14)} - Note: N_symbol SRS symbols are adjacent in a slot. Decision: As per email posted on Aug 20th, Agreement - On the presence of guard symbols in Rel-17 for SRS antenna switching, down-select one of the following - Alt 1-0: Guard symbols are always-on, which is same as Rel-15 - Alt 1-1: Guard symbols are configurable subject to UE capability - On whether to introduce guard symbols between SRS resource sets for antenna switching, down-select one of the following - Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set - Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots - Note: Rel-15 guard period symbols are supported if none of the above enhancements is agreed Agreement For Comb-8 SRS in Rel-17, down-select one of the following in RAN1#106bis-e - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

The following agreements are achieved in RAN1#106bis-e[7]:

Agreement For two SRS resource sets of an xTyR antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a minimum gap period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot - The value of Y is same as the inter-resource GP defined in Rel-15 - FFS: Whether or not the minimum GP exists can be RRC configurable subject to UE capability - Whether this inter-set GP is needed for 4T6R can be discussed later per the decision on 4T6R configuration. - FFS: How/Whether to handle the case where the interval between SRS resource sets is larger than Y Agreement For the detailed pattern of khoppingwhen start RB location hopping across legacy FH periods is enabled, support the following - For PF = 2, khopping = {0, 1} - For PF = 4, khopping = {0, 2, 1, 3} - Note: khopping={x0,…,xPF-1} khopping={x0,⋯xPF-1} meanskhopping=xn mod PFkhopping=xn mod PF for the (n+1)-th legacy FH period, where n = {0, 1, 2, 3, …} Agreement Bit width of SOI depends on the maximum number of “t” values configured for any of the aperiodic SRS resource sets (FFS: across all CCs or across a CC/BWP) - The SOI field is 0 bit if the maximum number of ‘t’ values is one - If at least one resource set has “t” configured - For the resource sets with “t” value configured, each of them is configured with K values of “t”, where 1<=K<=4 - t=0 applies for the resource set(s) without “t” configured in RRC - If none of the resource sets is configured with “t” values, follow Rel-15 approach to determine slot offset R1-2110475 FL summary #2 on SRS enhancements Moderator (ZTE) Agreement For comb-8 SRS in Rel-17, the maximum number of CSs is 6. - FFS: Whether a maximum number of 12 CSs is supported Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional R1-2110522 FL summary #3 on SRS enhancements Moderator (ZTE) Agreement On SRS configuration for 4T6R, select at least one from the following three alternatives in RAN1#107e - Alt 1: 4 + 2 - Alt 2: 2+2+2 - Alt 2-1: - No guard symbols exist between the 1st and the 2nd transmission. Y guard symbol(s) exist between 2nd and 3rd transmission, where Y is same as the value defined in the current specification for different SCSs - Alt 2-2: - For SCS=15, 30 and 60KHz: No guard symbols exist - For SCS=120 KHz: No guard symbols exist between the 1st and the 2nd transmission, and 1 guard symbol exists between the 2nd and 3rd transmission - Clarification on the notation: x1+…+xK means totally K resources are needed, where the k-th resource contains xK ports, 1<=k<=K

The following agreements were achieved in RAN1#107[8]:

Agreement When ca-SlotOffset is configured, reference slot to use the Rel-17 mechanism for determining the SRS offset is slot n⋅2μSRS2μPDCCH+k+Nslot,offset, PDCCHCA2μoffset,PDCCH-Nslot,offset, SRSCA2μoffset,SRS⋅2μSRS, otherwise reference slot isn⋅2μSRS2μPDCCH+k, where Nslot,offset, PDCCHCA, 2μoffset,PDCCH, Nslot,offset, SRSCA and 2μoffset,SRS are determined by ca-SlotOffset configurations of the PDCCH carrier and SRS carrier. Agreement For a CC with t value configured, SOI bit width depends on the maximum number of t values configured for all the resource sets across all configured BWPs in a CC for SRS transmission. - For the CCs without any t value configured, follow Rel-15/16 mechanism to determine the SRS slot offset, where SOI bit width is 0 Working assumption To support 4 ports with Max CS = 6, - Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively. - Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively. - Note: n_CS and k_TC are the configured CS and comb offset values. - Note: This working assumption can be revisited if Max CS = 12 is agreed. Agreement For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS if there are multiple frequency hopping period in the slot. Conclusion In Rel-17, SRS 4T6R is not supported. Conclusion No consensus to have further restriction on the number of RBs for RPFS in Rel-17. - No introduction of new sequence length Conclusion There is no consensus in RAN1 to support Max CS = 12 for comb-8 in Rel-17.

In this paper, we provide our views on the remining issues on enhancement on SRS.

Discussion

Flexible SRS triggering

Three different types of SRS can be configured for one UE. Periodic SRS is configured and triggered by RRC signal, semi-persistent SRS is configured by RRC but activated/deactivated by MAC CE. Multiple aperiodic SRS resource sets are configured by RRC and each SRS resource set is associated with one or more aperiodic SRS triggering states, i.e., the non-zero SRS request field contained in the DCI, by RRC configuration. One or more aperiodic SRS resource set can be triggered by a DCI containing non-zero SRS request field. However, the association between aperiodic SRS triggering states and aperiodic SRS resource sets is fixed and configured by RRC signaling according to the higher layer parameter aperiodicSRS-ResourceTrigger or aperiodicSRS-ResourceTriggerList configured for each aperiodic SRS resource set, which limits the flexibility of aperiodic SRS triggering.

Aperiodic CSI-RS is supported for DL channel measurement and beam management, and the aperiodic CSI-RS is triggered by aperiodic CSI trigger states, i.e., non-zero CSI request field contained in a DCI. However, the association between aperiodic CSI trigger states and aperiodic CSI-RS resource(s) can updated by MAC CE for flexible aperiodic CSI-RS triggering. The same mechanism can be introduced for aperiodic SRS.

Proposal 1: The association between aperiodic SRS resource set(s) and aperiodic SRS triggering states can be updated by MAC CE.

Another issue is how to handle the collision happens between multiple SRS resource sets triggered by a same DCI as well as that are triggered by different DCIs.

Figure 1 Collision for multiple aperiodic SRS resource sets triggered by a same DCI

As illustrated in Figure 1, a UE receives a DCI containing an SRS request field with value ’10’ and a value t of ‘2’. Three aperiodic SRS resource sets including SRS resource set#1, SRS resource set#2 and SRS resource set#3 are associated with the SRS request codepoint ‘10’. The slot offset are configured as k=0, 1, 2 for each SRS resource set. It means that SRS resource set#1 should be transmitted in the 3rd available slot counted from slot n, SRS resource set#2 should be transmitted in the 3rd available slot counted from slot n+1 and SRS resource set#1 should be transmitted in the 3rd available slot counted from slot n+2. If the slot n+1, n+6 and n+6 are not available slot, SRS resource set#1 and SRS resource set#2 are transmitted in slot n+4 and SRS resource set#3 is transmitted in slot n+5. If the time location for the SRS resources in SRS resource set#1 and the SRS resources in SRS resource set#2 are overlapped in a same OFDM symbol, a dropping rule is required to the UE to select one SRS resource to transmit.

In this situation, one simple solution is to define the priority for different SRS resources according to the slot offset configured for each SRS resource sets. For example, the SRS resource with larger slot offset has higher priority. By adopting this principle, SRS resource within SRS resource set 2 has higher priority than the SRS resources with SRS resource set 1 since it has larger slot offset. Then the SRS resources within SRS resource set 1 that overlap with the SRS resources within SRS resource set 2 shall be dropped.

Proposal 2: For the collision between the SRS resources triggered by a same DCI, the SRS resource configured with larger slot offset has higher priority.

Another example is provided in Figure 2, where SRS resource set#1, SRS resource set#2 and SRS resource set#3 are triggered by DCI2, and SRS resource set#4 and SRS resource set#5 are triggered by DCI1. SRS resource set#5 and SRS resource set#3 are determined to be transmitted in the same slot n+5. If the SRS resources in SRS resource set#5 and SRS resources in SRS resource set#3 are determined to be transmitted on a same symbol, a dropping rule is required.

Figure 2 Collision for multiple aperiodic SRS resource sets triggered by different DCIs

The receiving timing of different triggering DCIs can be considered in this situation. For example, the SRS resource triggered by the latest received DCI may have higher priority. By adopting this solution, SRS resources within SRS resource set 3 have higher priority than the SRS resources within SRS resource set 5. Therefore, the SRS resource within SRS resource set 5 that are overlapped with SRS resources within SRS resource set 3 shall be dropped.

Proposal 3: For the collision between the SRS resources triggered by different DCIs, the SRS resources triggered by the latest DCI have higher priority.

Enhancements on antenna switching

SRS for antenna switching is supported in NR Rel-15/16 for DL CSI acquisition based on the UL-DL channel reciprocity for the UE with capability 't1r2', 't2r4', 't1r4', 't1r1', 't2r2' and 't4r4'. Up to 8 RX for a UE is possible and up to 8 layers SU-MIMO PDSCH transmission can be beneficial by using 8 SRS for antenna switching. It is reasonable to support SRS antenna switching with up to 8 antennas to enable scheduling higher downlink layer per UE.

SRS switching with 8 Rx can be used for the full DL CSI acquisition for potential high rank DL transmission. For example, up to 8 layers PDSCH transmission can be achieved if the channel is good enough for the UE with 8 Rx antennas. However, higher rank transmission is not always possible for a UE. For the cell-edge UE, rank 2 transmission may be enough and RI restriction is also supported for DL interference coordination by the gNB. So partial antenna channel sounding is reasonable and also be beneficial for quick channel sounding, especially for lower rank transmission.

Generally, two type of SRS, e.g. one for periodic/semi-persistent and another for aperiodic, can be configured for a UE for antenna switching, and the gNB can get the full channel matrix based on the periodic/semi-persistent SRS and can pick some antennas with strong channel vectors for partial antenna sounding using aperiodic SRS. For this purpose, the gNB will assume the SRS port of the SRS resource within the SRS resource set for periodic/semi-persistent is 1-to-1 mapped to the SRS resource within the SRS resource set(s) for aperiodic according to the SRS resource ID. And the UE will transmit the aperiodic SRS resource and the associated periodic/semi-persistent SRS resource using the same antenna port(s). Based on the above analysis, we have the following proposal

Proposal 4: Support partial antenna sounding at least for UE with 6Rx and 8Rx.

In LTE Rel-16, SRS for antenna switching transmitted in the additional SRS symbols is specified, and the guard period for antenna switching and frequency hopping is configurable for the UE according to UE capability introduced by RAN4 in R1-1913452. It means that the guard period for antenna switching is not always required for some numerology, e.g. 15 kHz and 30 kHz.

In additional, according to RAN4 LS in R4-2202413, the transient period between SRS resources is 15us. It means that the UE cannot complete the antenna switching within a CP even for SCS=15kHz. And the guard period for antenna switching is always needed for the SRS resources used for antenna switching.

RAN4 also raised another question “Should there be a guard period defined in RAN1 specs or a switching time defined in ran4 specs in between SRSs and other signals?”. When the antennas used for the PUSCH/PUCCH transmission are different from the antennas used for the followed SRS, a guard period is preferred used for antenna switching. However, as pointed out by RAN4, an always-on guard period between PUSCH/PUCCH and SRS for antenna switching is unsuitable since a same antenna may be used for the transmission of PUSCH/PUCCH and SRS. Therefore, this issue can be up to gNB implementation. For example, when different antennas are used for PUSCH/PUCCH and the followed SRS resource transmission, a guard period can be reserved by gNB scheduling for antenna switching.

Proposal 5: The guard period between PUSCH/PUCCH and SRS resources belong to different sets can be up to gNB implementation.

For 4T6R UE, 1 or 2 SRS resource sets can be configured for the UE with different values for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has 2 SRS resource transmitted in different symbols, one SRS resource in a set consisting of 4 SRS ports and the other SRS resource in a set consisting of 2 SRS ports, and the SRS ports of the second resource in the set is associated with different UE antenna ports than the SRS ports of the first resource in the same set.

For 4T6R without guard period, 1 or 2 SRS resource sets can be configured for the UE with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has 3 SRS resource transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of each resource is associated with a different UE antenna port pair.

Enhancements on coverage and capacity

Several enhanced schemes were agreed in RAN1#104e to improve the coverage and capacity of SRS. For example, the repetition number is increased to 14 to improve the sounding coverage, and Comb 8 is introduced to improve the SRS capacity. Those two features can be supported by changing the Rel-15/16 RRC configuration. Another important feature is partial frequency sounding.

Larger comb size

To support the maximum number of CS 6 for Comb 8, how to obtain orthogonal SRS resources for different SRS port when 4 ports are configured should be resolved since orthogonal sequences for 4 ports can not be obtained according to TS38.211. Further, the following working assumption was made in RAN1#107 to support of 4 ports SRS transmission as follows

Working assumption

To support 4 ports with Max CS = 6,

- Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively.

- Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively.

- Note: n_CS and k_TC are the configured CS and comb offset values.

- Note: This working assumption can be revisited if Max CS = 12 is agreed.

The working assumption is aligned with Rel-15 principle, for example, for nSRSCS=0,1,2, different SRS ports are multiplexed with CDM manner, i.e., different SRS ports are assigned with different SRS sequences in a same RE set. While for nSRSCS=3,4,5 SRS port 1000 and 1002 are multiplexed in a same RE set with different SRS sequences and SRS port 1001 and 1003 are multiplexed in another RE sets with different SRS sequences. To minimize the spec impact, we propose to confirm the working assumption.

Proposal 6: Confirm the working assumption to support 4 SRS ports with Max CS=6:

To support 4 ports with Max CS = 6,

- Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively.

- Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively.

- Note: n_CS and k_TC are the configured CS and comb offset values.

- Note: This working assumption can be revisited if Max CS = 12 is agreed.

Partial frequency sounding

It has been agreed that the start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1}. Considering that partial frequency sounding mainly targets the SRS capacity enhancement for multi-user scenario with lower mobility and small delay spread, and the multiplexing pattern of different SRS resources of different users is semi-statically changed,  updating/indicating PF and Koffset with MAC-CE can make a good trade-off for flexibility and signalling overhead.

Proposal 7: PF and koffset for a SRS resource can be indicated by MAC CE or DCI in addition RRC signaling.

Conclusion

Proposal 1: The association between aperiodic SRS resource set(s) and aperiodic SRS triggering states can be updated by MAC CE.

Proposal 2: For the collision between the SRS resources triggered by a same DCI, the SRS resource configured with larger slot offset have higher priority.

Proposal 3: For the collision between the SRS resources triggered by different DCIs, the SRS resource triggered by the latest DCI have higher priority.

Proposal 4: Support partial antenna sounding at least for UE with 6Rx and 8Rx.

Proposal 5: The guard period between PUSCH/PUCCH and SRS resources belong to different sets can be up to gNB implementation.

Proposal 6: Confirm the working assumption to support 4 SRS ports with Max CS=6:

To support 4 ports with Max CS = 6,

- Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively.

- Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively.

- Note: n_CS and k_TC are the configured CS and comb offset values.

- Note: This working assumption can be revisited if Max CS = 12 is agreed.

Proposal 7: PF and koffset for a SRS resource can be indicated by MAC CE or DCI in addition RRC signaling.

References

- RP-193133 WID proposal FeMIMO.

- Chainman Notes of RAN1#102e

- Chainman Notes of RAN1#103e

- Chainman Notes of RAN1#104e

- Chainman Notes of RAN1#104bis-e

- Chainman Notes of RAN1#105-e

- Chainman Notes of RAN1#106-e

- Chainman Notes of RAN1#106bis-e

- Chainman Notes of RAN1#107-e