3GPP TSG RAN WG1 Meeting #108-e	R1-2201432

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	FUTUREWEI

Title:	Discussion on CORESET#0 impact of CBW narrower than 40MHz of n79

Document for:	Discussion and decision

# Introduction

In order to allow a RedCap UE to operate in each FR1 band, the minimum bandwidth for a channel in each band must be less than the maximum bandwidth of a RedCap UE. In Rel-16, one band whose minimum bandwidth exceeds the maximum bandwidth of a RedCap UE is n79. For Rel-17, RAN4 is modifying the minimum bandwidth for n79 and introducing channel bandwidths of 10 and 20 MHz to accommodate RedCap UEs [4]. With this modification, RAN4 recognized there might be impact to the procedures to obtain CORESET#0 after the SSB is detected. RAN4 sent an LS [1] to RAN1 asking how to resolve a potential backward compatibility issue [3].

In TS 38.213, upon detection of SS/PBCH block, UE determines CORESET#0 from MIB by looking up the table for controlResourceSetZero. The table is determined according to subcarrier spacing of SSB, subcarrier spacing of PDCCH, and minimum channel bandwidth of the frequency band where UE located. For example, a new UE supporting 10 MHz the minimum channel bandwidth for n79 will have a different table (table 13-4) for CORESET#0 configuration with a legacy UE still supporting 40 MHz (table 13-6).

Another way to express this is: in band n79, a 10 MHz minimum BW Rel-17 uses one set of tables to determine the configuration for CORESET#0 while a 40 MHz minimum BW legacy UE uses a different set of tables. When a Rel-17 UE and legacy UE access the same SSB, they should use the same CORESET#0, but it is unclear how a Rel-17 UE determines the CORESET#0 configuration.

The LS also included several alternatives to consider. This contribution examines the alternatives listed in the LS and provides observations and proposals.

# Discussion

## Background

Prior to Rel-17, the procedure for a UE to access channels in band n79 was to first assume the minimum bandwidth for a channel in that band was 40 MHz and the SCS for the SSB was 30 kHz. The candidate SSB locations are 16×1.44 MHz (23.04 MHz) apart. In band n79, there are 26 possible SSB locations or equivalently sync raster locations. The Global Synchronisation Channel Number (GSCN) is used to map a sync raster location to a frequency value.

Once a UE detects a valid SSB, it examines the value of subCarrierSpacingCommon in the MIB in order to select Table 13-5 or Table 13-6 in 38.213 [5]. With Table 13-5, a 15 kHz SCS is used for CORESET#0 while 30 kHz SCS is used with Table 13-6. After which, the UE uses pdcch-ConfigSIB1 in the MIB to determine the attributes of CORESET#0, such as the bandwidth of CORESET#0. The field pdcch-ConfigSIB1 is effectively an index within the selected table. There are up to 16 possible configurations that can be signaled by pdcch-ConfigSIB1. With either table, the bandwidth of CORESET#0 is either 10 or 20 MHz, depending on the index used. The 40 MHz minimum channel bandwidth means that either a 552.5 kHz or 905 kHz guard band is used for 15 kHz and 30 kHz SCS, respectively [6]. The guard bands are used to set the transmission bandwidth.

## Analysis

With a 10 MHz minimum bandwidth and 30 kHz SCS for the SSB, different tables in 38.213 are used: Table 13-3 for 15 kHz SCS CORESET#0 and Table 13-4 for a 30 kHz SCS CORESET#0. The MIB fields subCarrierSpacingCommon and pdcch-ConfigSIB1 are used in the same manner as described above. For a 10 MHz CORESET#0, the guard bands are 312.5 / 665 kHz for 15 / 30 kHz SCS, respectively, while 452.5 / 805 kHz guardband are used for a 20 MHz CORESET#0.

To see the similarities and differences in the CORESET#0 attributes, the elements of Tables 13-3, 13-4, 13-5, and 13-6 are displayed side-by-side in Table 1.

Table 1. Comparison of Tables 13-3, 13-4, 13-5, and 13-6. Gray shading shows elements for a 10 MHz BW CORESET#0 while orange shading is used for 20 MHz BW CORESET#0. The index in the first column corresponds to the field pdcch-ConfigSIB1.

| Index | 15 kHz SCS CORESET#0 |  |  |  |  |  | 30 kHz SCS CORESET#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Table 13-3 |  |  | Table 13-5 |  |  | Table 13-4 |  |  | Table 13-6 |  |  |
|  | RBs | Sym. | Offset | RBs | Sym. | Offset | RBs | Sym. | Offset | RBs | Sym. | Offset |
| 0 | 48 | 1 | 2 | 48 | 1 | 4 | 24 | 2 | 0 | 24 | 2 | 0 |
| 1 | 48 | 1 | 6 | 48 | 2 | 4 | 24 | 2 | 1 | 24 | 2 | 4 |
| 2 | 48 | 2 | 2 | 48 | 3 | 4 | 24 | 2 | 2 | 24 | 3 | 0 |
| 3 | 48 | 2 | 6 | 96 | 1 | 0 | 24 | 2 | 3 | 24 | 3 | 4 |
| 4 | 48 | 3 | 2 | 96 | 1 | 56 | 24 | 2 | 4 | 48 | 1 | 0 |
| 5 | 48 | 3 | 6 | 96 | 2 | 0 | 24 | 3 | 0 | 48 | 1 | 28 |
| 6 | 96 | 1 | 28 | 96 | 2 | 56 | 24 | 3 | 1 | 48 | 2 | 0 |
| 7 | 96 | 2 | 28 | 96 | 3 | 0 | 24 | 3 | 2 | 48 | 2 | 28 |
| 8 | 96 | 3 | 28 | 96 | 3 | 56 | 24 | 3 | 3 | 48 | 3 | 0 |
| 9 |  |  |  |  |  |  | 24 | 3 | 4 | 48 | 3 | 28 |
| 10 |  |  |  |  |  |  | 48 | 1 | 12 |  |  |  |
| 11 |  |  |  |  |  |  | 48 | 1 | 14 |  |  |  |
| 12 |  |  |  |  |  |  | 48 | 1 | 16 |  |  |  |
| 13 |  |  |  |  |  |  | 48 | 2 | 12 |  |  |  |
| 14 |  |  |  |  |  |  | 48 | 2 | 14 |  |  |  |
| 15 |  |  |  |  |  |  | 48 | 2 | 16 |  |  |  |

Fig. 1 shows the locations of the SSB with respect to CORESET#0. Note the number of symbols in a CORESET are not displayed.

CORESET

2

4

CORESET

6

28

1

2

3

0

56

4

0

12

14

16

0

28

a) 10 MHz CORESET#0 BW, 15 kHz SSB

b) 20 MHz BW, 15 kHz SSB

d) 20 MHz BW, 30 kHz SSB

c) 10 MHz CORESET#0 BW, 30 kHz SSB

CORESET

CORESET

Fig. 1. Location of SSB in relation to CORESET#0 for Tables 13-3, 13-4, 13-5, and 13-6. (a), (c) Gray shading is for 10 MHz CORESET#0 BW while (b), (d) orange shading is for 20 MHz CORESET#0 BW. Solid green is the SSB location for 10 MHz minimum BW (Tables 13-3, 13-5), diagonal stripe is the SSB location for 40 MHz minimum BW (Tables 13-4 and 13-6); vertical stripes is the SSB location whose location is the same for 10 and 40 MHz minimum BW). The numbers within the SSB represent the offset in Table 1.

For a 15 kHz SCS CORESET#0, the locations for the SSBs of 10 and 40 MHz minimum bandwidth are always different. For a 30 kHz SCS CORESET#0, there are two locations that are the same for a 10 MHz CORESET#0 BW while there are no locations that are the same for a 20 MHz CORESET#0 BW.

## Discussion

As the LS indicated, with the changing of the minimum bandwidth to 10 MHz, there is an impact to legacy devices that support 40 MHz minimum bandwidth. With the smaller minimum bandwidth and changed sync raster spacing, Rel-17 devices can monitor SSB locations that are multiples of 1.44 MHz apart in band n79, rather than once every 16×1.44 MHz for legacy devices. There are 410 SSB locations for a 10 MHz minimum bandwidth in n79. Specifically, legacy devices are unable to detect 15 sync raster locations in a sequence of 16 consecutive sync raster locations in band n79. This means that Rel-17 devices can use Tables 13-3 / 13-4 for those 15 sync raster locations. However, for the one sync raster location common to legacy and Rel-17 devices, there is an ambiguity in what table should be used. Specifically, for the same value of pdcch-ConfigSIB1, a Rel-17 device generally uses a different set of CORESET#0 resources compared to legacy devices, as shown in Table 1. This ambiguity affects how devices establish CORESET#0 on those channels.

RAN4 listed some alternatives to address this ambiguity [1].

- Alt-1: Add narrower channel bandwidth to n79, and indicate different table to legacy UE and new UE for CORESET#0 configuration.

- Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table 13.6 to legacy UE and new UE for CORESET#0 configuration.

- Alt-1b: Add narrower channel bandwidth to n79, and indicate:

  - A UE synchronizing on a GSCN with step 4 shall look at the table 13.6.

    - Rationale: This enables new and legacy to connect in band n79 using the same CORESET#0 configuration.

  - A UE synchronizing on a GSCN with step 1 but not step 4 shall look at the table 13.4.

    - Rationale: Table 3.4 offers more flexibility on CORESET#0 configuration

- Alt-2: Add narrower channel bandwidth to new band nX instead of n79.

In our opinion, we found the descriptions of these alternatives listed in LS needed additional information. We referred to the summary document [3] and one contribution [2] for more details.

The motivation for Alt-1 is: Rel-17 devices use Tables 13-3 and 13-4 in band n79 while legacy devices use Tables 13-5 and 13-6. When a Rel-17 UE and legacy UE access the same sync raster location (SSB), the network configures the MIB so that 30 kHz SCS is used for the 10 MHz BW CORESET#0. Furthermore, the field pdcch-ConfigSIB1 is set to 0 since that is the one entry common to Table 13-4 and Table 13-6. While there is no specification impact, there may be deployment impact because the network has no flexibility to configure CORESET#0; it has only one possible value to signal in the field pdcch-ConfigSIB1.

Observation 1. Alt-1 has no impact to specifications but limits network deployment for the SCS used for CORESET#0 and the location of SSB in relation to CORESET#0 by allowing one value for pdcch-ConfigSIB1

The basis for Alt-1a can be seen from Fig. 1(c). The shading illustrates the two instances where the location of the SSB relative to 10 MHz BW CORESET#0 are the same (offset=0 and 4). Table 1 shows there are 4 configurations (# symbols, offsets) for a 10 MHz CORESET#0 BW with 30 kHz SCS that correspond to the two instances. They are for a 2-symbol CORESET with 0 RB offset for the SSB (2,0); (2,4), (3,0), and (3,4). The motivation for Alt-1a is: Rel-17 devices use Tables 13-3 and 13-4 in band n79 while legacy devices use Tables 13-5 and 13-6. When a Rel-17 device locates a sync raster location (SSB) that a legacy device can use, the Rel-17 device uses Table 13-6. To facilitate implementation for Rel-17 devices, only the (2,0), (2,4), (3,0) and (3,4) 10 MHz CORESET#0 BW configuration would be allowed. A Rel-17 device is able to support those configurations since it supports Table 13-4. A pseudo-code description for a Rel-17 device is (where the GSCN represents the frequency location of the SSB)

If band == n79:

if detected GSCN mod 16 == 0:

use Table 13-6 and restrict pdcch-ConfigSIB1 to 0, 1, 2, 3

else:

use Table 13-3/13-4

else:

use Table 13-3/13-4

In [4] (relevant table presented in the Appendix), a legacy UE monitors SSB locations corresponding to GSCN values of 8480 + 16n, n=0, …, 25 while a Rel-17 UE monitors SSB locations corresponding to GSCN values of 8475 + n, n=0, …, 409. The pseudocode rule identifies SSB locations whose GSCN that satisfy 8480 + 16n.

This alternative allows more possible configurations than Alt-1 for a 10 MHz CORESET#0 BW with 30 kHz SCS. There are standards impact to change the mapping rule for Rel-17 UEs as well as network restriction of what configurations can be used when a Rel-17 UE monitors the same SSB as a legacy UE. There also must be a change to indicate the minimum bandwidth to use when a Rel-17 device uses Table 13-6.

Observation 2. Alt-1a has a small impact to specifications of adding band-specific information about the mapping rules but allows four values for pdcch-ConfigSIB1 in Table 13-6.

The motivation for Alt-1b is that a Rel-17 device uses Tables 13-3 and 13-4 in band n79 except when a Rel-17 UE and legacy UE access the same SSB where a Rel-17 device would use Tables 13-5 and 13-6. For this alternative, the Rel-17 device must support the CORESET#0 configurations within Tables 13-5 and 13-6. Note that there also must be a change to indicate the minimum bandwidth to use when a Rel-17 device uses Tables 13-5 and 13-6.

For the LS, the description for “step 4” is unclear. In the set of GSCN for legacy UEs in band n79, valid locations satisfy GSCNmod16=0. A pseudo-code description for a Rel-17 device is

If band == n79:

if detected GSCN mod 16 == 0:

use Table 13-5/13-6

else:

use Table 13-3/13-4

else:

use Table 13-3/13-4

This alternative allows more flexibility in band n79 but adds a band-specific rule in RAN1 for n79.

Observation 3. With Alt-1b, when a legacy UE and a Rel-17 UE can monitor the same SSB in band n79, a Rel-17 UE uses Tables 13-5 and 13-6; otherwise, a Rel-17 UE uses Tables 13-3 and 13-4. It offers full network flexibility but a Rel-17 UE must support CORESET configurations listed in Tables 13-5 and 13-6.

With Alt-2, a new band that overlaps with n79 is introduced by RAN4 in Rel-18. According to the discussions in [3], RAN4 will revert the agreements for n79. This implies a RedCap UE would not be able to access band n79.

Observation 4. With Alt-2, a RedCap UE would not be able to access band n79.

A summary of the observations and impacts is presented in Table 2.

Table 2. Summary of observations of alternatives

| Alt | RAN1 spec impact | RAN4 spec impact | Network flexibility | Rel-17 UE impact |
| --- | --- | --- | --- | --- |
| 1 |  |  | 1 possible configuration (10 MHz BW CORESET#0 with 30 kHz SCS) can be used when legacy UE and Rel-17 UE monitor same SSB | None |
| 1a | Y |  | 4 possible configurations (10 MHz BW CORESET#0 with 30 kHz SCS) can be used when legacy UE and Rel-17 UE monitor same SSB | Use first 4 rows of Table 13-6 when legacy UE and Rel-17 UE monitor same SSB; otherwise use Tables 13-3, 13-4 |
| 1b | Y |  | Full flexibility (17 possible configurations (9 with 10MHz BW CORESET#0, 30kHz SCS and 8 with 10MHz BW CORESET#0, 15kHz SCS) | Use Tables 13-5 and 13-6 when legacy UE and Rel-17 UE monitor same SSB; otherwise use Tables 13-3, 13-4 |
| 2 |  | Y | No RedCap UE able to operate in n79 | No RedCap UE able to operate in n79 |

To assess how to deal with the NBC issue, it is better to have the same understanding of the alternatives and the impacts before making any agreements. In addition, we are open to discuss alternatives not considered by RAN4.

Proposal: Consider exceptional handling for band n79 for better support of RedCap UEs within Rel-17

# Conclusion

This contribution examined the RAN4 LS regarding the non-backward compatible issues for n79. The following observations were made

Observation 1. Alt-1 has no impact to specifications but limits network deployment for the SCS used for CORESET#0 and the location of SSB in relation to CORESET#0 by allowing one value for pdcch-ConfigSIB1

Observation 2. Alt-1a has a small impact to specifications of adding band-specific information about the mapping rules but allows four values for pdcch-ConfigSIB1 in Table 13-6.

Observation 3. With Alt-1b, when a legacy UE and a Rel-17 UE can monitor the same SSB in band n79, a Rel-17 UE uses Tables 13-5 and 13-6; otherwise, a Rel-17 UE uses Tables 13-3 and 13-4. It offers full network flexibility but a Rel-17 UE must support CORESET configurations listed in Tables 13-5 and 13-6.

Observation 4. With Alt-2, a RedCap UE would not be able to access band n79.

Based on these observations, we propose

Proposal: Consider exceptional handling for band n79 for better support of RedCap UEs within Rel-17

# References

R1-2200907 (R4-2202286), “LS on CORESET#0 impact of CBW narrower than 40MHz of n79”, Samsung, RAN1#108, Feb. 21-Mar. 3, 2022

R4-2200922, “On new channel bandwidths narrower than 40 MHz of n79”, Samsung, RAN4#101bis, Jan. 17-25, 2022

R4-2202309. “Email discussion summary for [101-bis-e][111] NR_bands_R17_BWs”, Ericsson, RAN4#101bis, Jan. 17-25, 2022.

R4-2118155. “Big CR to TS 38.101-1: Adding channel BW support in existing NR bands”, Ericsson, RAN4#101, Nov. 1-12, 2021.

38.213

38.104

# Appendix

Draft CR [4] showing the sync raster for n79.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| … |  |  |  |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 88807 |
|  |  |  | 8475 – <1> – 88848 |
| n85 | 15 kHz | Case A | 1826 – <1> – 1858 |
| n90 | 15 kHz | Case A | 6246 – <1> – 6717 |
|  | 30 kHz | Case C | 6252 – <1> – 6714 |
| n91 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n92 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n93 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n94 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n964 | 30 kHz | Case C | 9531 – <1> – 10363 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. NOTE 2: The applicable SS raster entries are GSCN = {6432, 6443, 6457, 6468, 6479, 6493, 6507, 6518, 6532, 6543}. NOTE 3: The following GSCN are allowed for operation in band n46: GSCN = {8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9288, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9472, 9485, 9499, 9513}. NOTE 4: The following GSCN are allowed for operation in band n96: GSCN = {9548, 9562, 9576, 9590, 9603, 9617,9631, 9645, 9659, 9673, 9687, 9701, 9714, 9728, 9742, 9756, 9770, 9784, 9798, 9812, 9826, 9840, 9853, 9867, 9881, 9895, 9909, 9923, 9937, 9951, 9964, 9978, 9992, 10006, 10020, 10034, 10048, 10062, 10076, 10090, 10103, 10117, 10131, 10145, 10159, 10173, 10187, 10201, 10214, 10228, 10242, 10256, 10270, 10284, 10298, 10312, 10325, 10339, 10353}. NOTE 5: The applicable SS raster entries are GSCN = {5032, 5043, 5054} NOTE 6: The applicable SS raster entries are GSCN = {4707, 4715, 4718, 4729, 4732, 4743, 4747, 4754, 4761, 4768, 4772, 4782, 4786, 4793} NOTE 7: The SS raster entries apply for channel bandwidths larger than or equal to 40 MHz NOTE 8: The SS raster entries apply for channel bandwidths smaller than 40 MHz |  |  |  |