3GPP TSG RAN WG1 #108-e		R1-2201456

e-Meeting, February 21st – March 3rd, 2022

Title: 	Enhancements to the IAB resource multiplexing

Source: 	ZTE, Sanechips

Agenda item:	8.10.1

Document for:	Discussion and decision

Introduction

This contribution discusses some remaining issue on enhancements to the IAB resource multiplexing in Rel-17 IAB.

Discussions

H/S/NA configuration

The following working assumption has been made in RAN1#106bis-e on H/S/NA configuration for DU cell [1].

Working Assumption(RAN1#106bis-e) If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

Strong concerns have been raised by several companies on the above working assumption, i.e., the work assumption on how to determine which configuration will be applied if both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot. The Alt. 1 in the working assumption can be considered as a dynamic H/S/NA configuration selection which would lead to the following problems:

- It is difficult to make a consistent understanding on the selected H/S/NA configuration on the IAB-node DU among the IAB-node, its parent node(s), and its donor CU. This may cause the parent node to be unable to make appropriate decisions on the available resources of IAB-MT.

- Semi-static H/S/NA configurations would be coordinated for the purpose of CLI and/or resource multiplexing in DC. It is impossible/difficult to make the peer node/neighbor node know the dynamic selected HSNA configuration at the IAB node.

It was agreed that CU can indicate the DU resource configuration (UL/DL/FL, H/S/NA) of the peer IAB-node or donor DU that can be used for resource coordination in case of DC, and/or for interference management. If an IAB node dynamically selects H/S/NA configuration when both Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration are provided for a given resource, it is difficult for peer IAB-node or donor DU to know which H/S/NA configuration has been selected/applied at the IAB-node DU. Dynamic H/S/NA configuration selection will not make the H/S/NA configuration indication/interaction to play a good role in resource coordination in case of DC, and/or interference management.

Therefore, we suggest that Rel-17 frequency domain H/S/NA configuration, if provided, always overrides Rel-16 time domain H/S/NA configuration.

Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration may lead to inconsistent understanding of H/S/NA types applied on an IAB-node DU among the IAB-node, its parent node(s), and its donor CU.

Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration will not play a good role in resource coordination in case of DC, and interference management.

If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, Rel-17 frequency domain H/S/NA configuration is applied.

RB set group

The following agreement has been made in RAN1#107-e[2] on availability indication for the soft resources of the RB sets.

Agreement For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell: - AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group - One RB set group consists of one or multiple RB sets

According to the above agreement, each resourceAvailabilty includes availability indication for one RB set group. One RB set group consists of one or multiple RB sets. However, what signaling is used to provide RB set group configuration needs to further study. In our view,  it is a better way that RB set group configuration are provided by RRC signaling because usually the change of RB set group configuration may cause the change of high layer parameter AvailabiltyCombination since the two are closely related.

RB set group configuration should be provided by RRC signaling per IAB-DU cell.

Considering that the number of RB sets within a carrier is not too larger, it is not necessary to always provide RB set group configuration. Therefore, we suggest that if RB set group configuration is not provided, resourceAvailablity indicates availability of soft resources in all RB sets for an IAB-DU cell.

If RB set group is not provided, resourceAvailablity indicates availability of soft resources in all RB sets for an IAB-DU cell.

Conclusion

According to the discussion above, we provide the following observations and proposals:

Observation 1: Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration may lead to inconsistent understanding of H/S/NA types applied on an IAB-node DU among the IAB-node, its parent node(s), and its donor CU.

Observation 2: Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration will not play a good role in resource coordination in case of DC, and interference management.

Proposal 1:	If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, Rel-17 frequency domain H/S/NA configuration is applied.

Proposal 2:	RB set group configuration should be provided by RRC signaling per IAB-DU cell.

Proposal 3:	If RB set group is not provided, resourceAvailablity indicates availability of soft resources in all RB sets for an IAB-DU cell.

References

Chair's Notes RAN1#106bis-e

Chair's Notes RAN1#107-e