3GPP TSG RAN WG1 #108-e		R1-2201458

e-Meeting, February 21st – March 3rd, 2022

Title: 	Remaining issue on higher layers parameter list for Rel-17 IAB

Source: 	ZTE, Sanechips

Agenda item:	8.10.3

Document for:	Discussion and decision

Introduction

This contribution discusses some remaining issue on higher layers parameter list for Rel-17 IAB.

Discussions

The following agreement has been made in RAN1#107-e on RB sets configuration [1].

Agreement The maximum number of non-overlapping RB sets configurable per DU cell is M - where, M is to be selected from one of values from 4, 8, 16 - DU frequency configuration information should be provided to the parent node.

It is reasonable that parent node can be made aware of IAB-node DU cell H/S/NA resource configuration in frequency-domain to have a good understanding of H/S/NA resource type of an IAB-node DU cell and then make appropriated assumption about the resource availability at IAB-node MT. However, according to current specification, the parent node does not know carrier frequency information of IAB-node DU cell (i.e. its child node). Therefore, the CU also needs to indicate carrier frequency information of the IAB-node DU cell to its parent node(s).

The 2nd sub-bullet in the above agreement is to provide the frequency configuration information (such as, FreInfo, CarrierList) of an IAB-node DU cell to its parent node by CU. However, higher layers parameter corresponding to the 2nd sub-bullet in the above agreement is not included in higher layers parameter list for Rel-17 IAB[2], which may cause the parent node to not correctly understand the H/S/NA configuration of the IAB-node DU cell.

The higher layers parameter corresponding to “DU frequency configuration information should be provided to the parent node” in the RAN1#107-e agreement should be included in higher layers parameter list for Rel-17 IAB, as shown in the following table.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_IAB_enh-Core | Resource multiplexing | Child gNB-DU Cell Frequency Configuration (final name in specification to be determined by RAN2/3) | New | IAB-donor CU indicates, to an IAB-node, DL/UL Frequency information (9.3.1.17 NR Frequency Info in TS 38.473) and DL/UL Carrier list (9.3.1.137 NR Carrier List in TS 38.473) of its child-node cells |  | IAB node specific |  | F1AP signaling. RAN1 #107-e Agreement The maximum number of non-overlapping RB sets configurable per DU cell is M - where, M is to be selected from one of values from 4, 8, 16 - DU frequency configuration information should be provided to the parent node. |

Conclusion

According to the discussion above, we provide the following proposals:

Proposal 1: The higher layers parameter corresponding to “DU frequency configuration information should be provided to the parent node” in the RAN1#107-e agreement should be included in higher layers parameter list for Rel-17 IAB, as shown in the following table.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_IAB_enh-Core | Resource multiplexing | Child gNB-DU Cell Frequency Configuration (final name in specification to be determined by RAN2/3) | New | IAB-donor CU indicates, to an IAB-node, DL/UL Frequency information (9.3.1.17 NR Frequency Info in TS 38.473) and DL/UL Carrier list (9.3.1.137 NR Carrier List in TS 38.473) of its child-node cells |  | IAB node specific |  | F1AP signaling. RAN1 #107-e Agreement The maximum number of non-overlapping RB sets configurable per DU cell is M - where, M is to be selected from one of values from 4, 8, 16 - DU frequency configuration information should be provided to the parent node. |

References

Chair's Notes RAN1#107-e

R1-2112976, Consolidated higher layers parameter list for Rel-17 NR