3GPP TSG RAN WG1 #108-e			R1-2201462

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	7.2.11

Source: 	NTT DOCOMO, INC.

Title: 	Correction on UE features list for NR positioning and eMIMO

Document for:	Discussion and Decision

- Introduction

RAN1 received a LS from RAN2 in R1-2200856 [1], and the LS pointed a mistake in RAN1 UE features list on NR positioning as below.

RAN2 discussed the updates indicated in RAN1 LS, and realized that it should be a typo in RAN1 LS on following updates since there is no value 96 in TS37.355 for component 1 in FG13-2b, FG13-3b, and FG13-4b. However value 96 is missing for component 3. Therefore RAN2 decided to update component 3 in FG13-2b, FG13-3b, and FG13-4b. - RAN1 agreed to add the value 96 for component 1 in FG13-2b, FG13-3b, and FG13-4b in RAN1 NR UE features list, to align with the TS 37.355.

In addition, RAN1 received a LS from RAN4 in R1-2200872, and the LS asked RAN1 to clarify UE capabilities about single DCI multi-TRP schemes as below.

In Rel-16 RAN4 has defined performance requirements for multi-DCI and single DCI transmission schemes for multi-TRxP transmission in FR1. These requirements are applicable to UEs supporting the following UE capability. Test Case/ Transmission scheme; UE Capability. Multi-DCI non-overlapping; multiDCI-MultiTRP-r16, maxSimultaneousResourceSetsPerCC ≥ 2. Single DCI SDM; singleDCI-SDM-scheme-r16, maxSimultaneousResourceSetsPerCC ≥ 2. Single DCI Inter-Slot TDM; supportInter-slotTDM-r16, maxNumberTCI-states-r16=2, maxSimultaneousResourceSetsPerCC ≥ 2. Single DCI FDM Scheme A; supportFDM-SchemeA-r16, maxSimultaneousResourceSetsPerCC ≥ 2. The requirements are defined with 2 active TCI states. The multi-DCI transmission scheme requirements are based on UE capability multiDCI-MultiTRP-r16 also includes maxNumberCORESETPerPoolIndex-r16, which is sufficient to indicate support of 2 active TCI states. For single DCI Inter-Slot TDM the capability of maxNumberTCI-states-r16=2 in supportInter-slotTDM-r16 covers support for 2 active TCI states. For single DCI FDM and SDM transmission schemes, the UE capability doesn’t include any additional UE capability for support of more than 1 active TCI state or any pre-requisite to support maxNumberActiveTCI-PerBWP other than n1. RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

This contribution proposes an update for UE features list on NR positioning and eMIMO.

- Discussion on UE features for Positioning

At the RAN1#106-e meeting, following proposal was made in [3].

Another problem is on the misalignment between TS 37.355 and the UE capability spreadsheet/TR 38.822 on the number of PRS resources that UE can processing for FR1 in a band combination involving both FR1 and FR2. The value range of the field fr1-r16 in maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16 of TS 37.355 [4], includes the n96: DL-PRS-ResourcesBandCombination-r16 ::= SEQUENCE { bandList-r16 SEQUENCE (SIZE (1..maxSimultaneousBands-r16)) OF FreqBandIndicatorNR-r16, maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16 CHOICE { fr1-Only-r16 ENUMERATED {n6, n24, n64, n128, n192, n256, n512, n1024, n2048}, fr2-Only-r16 ENUMERATED {n24, n64, n96, n128, n192, n256, n512, n1024, n2048}, fr1-FR2Mix-r16 SEQUENCE { fr1-r16 ENUMERATED {n6, n24, n64, n96, n128, n192, n256, n512, n1024, n2048}, fr2-r16 ENUMERATED {n24, n64, n96, n128, n192, n256, n512, n1024, n2048}, ... }, ... }, ... } However, the value n96 does not exist in TR 38.822. We do not think changing TS 37.355 is needed, and thus propose that the existing UE capability value be updated.

The proposal in [3] clearly intends the component 3 of FG13-2b/3b/4b regarding “Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation”.

However, there was a mistake that RAN1 made following agreement for the component 1 of FG13-2b/3b/4b as it also does not have value 96 [4].

Agreement: - Add the value 96 for component 1 in FG13-2b, FG13-3b, and FG13-4b in RAN1 NR UE features list, to align with the TS 37.355

Therefore, RAN1 should update the above agreement and corresponding part of UE features list on NR positioning as below.

Proposal 1:

Following RAN1 agreement made at RAN1#106-e is revised as below

- Add the value 96 for component 3 in FG13-2b, FG13-3b, and FG13-4b in RAN1 NR UE features list, to align with the TS 37.355

| 13. NR Positioning | 13-2b | DL PRS Resources for DL AoD on a band combination | - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 | No | N/A |  | Per BC | N/A | N/A | N/A | Need for location server to know if the feature is supported. the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-3b | DL PRS Resources for DL-TDOA on a band combination | - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 | No | N/A |  | Per BC | N/A | N/A | N/A | Need for location server to know if the feature is supported. the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 13. NR Positioning | 13-4b | DL PRS Resources for Multi-RTT on a band combination | - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 | No | N/A |  | Per BC | N/A | N/A | N/A | Need for location server to know if the feature is supported. the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

- Discussion on UE features for eMIMO

As clarified in the RAN4 LS [2], the requirements for multi-DCI and single DCI transmission schemes for multi-TRxP transmission in FR1 are defined with 2 active TCI states.

Following is the current descriptions in TS38.306 regarding UE capabilities for the corresponding single DCI multi-TRP schemes, and whether the support of 2 active TCI states is necessary or not seems not clear for these capabilities.

supportFDM-SchemeA-r16 Indicates whether UE supports single DCI based FDMSchemeA. supportInter-slotTDM-r16 Indicates whether UE supports single-DCI based inter-slot TDM. This capability signalling includes the following: - supportRepNumPDSCH-TDRA-r16 indicates support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - maxTBS-Size-r16 indicates maximum TBS size. - maxNumberTCI-states-r16 indicates the maximum number of TCI states. supportTDM-SchemeA-r16 Indicates whether UE supports single DCI based TDMSchemeA. The capability signalling includes the maximum TBS size. singleDCI-SDM-scheme-r16 Indicates whether the UE supports single DCI based spatial division multiplexing scheme.

RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

In our view, since the support of 2 active TCI states is practically mandatory for UE to support above single DCI multi-TRP schemes, it is sufficient to clarify it for above UE capabilities.

Therefore, we propose to add a note “UE supporting this feature should support 2 active TCI states per BWP per CC” for above capabilities and ask RAN2 to add the note in TS38.306.

Proposal 2:

Following note is added for FG16-2b-1, 16-2b-2, 16-2b-4 and 16-2b-5.

- UE supporting this feature should support 2 active TCI states per BWP per CC

| 16-2b-1 | Single-DCI based SDM scheme | - Support of single-DCI based SDM scheme |  | Yes | N/A |  | Per FS | N/A | N/A |  | UE supporting this feature should support 2 active TCI states per BWP per CC | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2b-2 | Single-DCI based FDMSchemeA | Support of single-DCI based FDMSchemeA |  | Yes | N/A |  | Per band | No | No |  | UE supporting this feature should support 2 active TCI states per BWP per CC | Optional with capability signaling |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of single-DCI based TDMSchemeA - Supported maximum TBS size for TDMSchemeA |  | Yes | N/A |  | Per band | No | No |  | Component 2 candidate values {3, 5, 10, 20, no restriction} KByte UE supporting this feature should support 2 active TCI states per BWP per CC | Optional with capability signaling |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of single-DCI based inter-slot TDM - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size - Maximum number of TCI states |  | Yes | N/A |  | Per band | No | No |  | Component 2 candidate values: {{2,3,4,5,6,7,8,16}} Component 3 candidate values {{3, 5, 10, 20, no restriction} KByte } Component 4 candidate values: {1,2} UE supporting this feature should support 2 active TCI states per BWP per CC | Optional with capability signaling |

- Conclusion

Based on the discussion, following proposals were made.

Proposal 1:

Following RAN1 agreement made at RAN1#106-e is revised as below

- Add the value 96 for component 3 in FG13-2b, FG13-3b, and FG13-4b in RAN1 NR UE features list, to align with the TS 37.355

Proposal 2:

Following note is added for FG16-2b-1, 16-2b-2, 16-2b-4 and 16-2b-5.

- UE supporting this feature should support 2 active TCI states per BWP per CC

# Reference

[1]	R1-2200856	Response LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#105-e	RAN2, Intel

[2]	R1-2200872	LS UE capability for supporting single DCI transmission schemes for multi-TRP	RAN4, Apple

[3]	R1-2106502	Discussion on positioning UE features	Huawei, HiSilicon

[4]	R1-2108424	Summary on [106-e-NR-UEFeature-Positioning-01]	Moderator (NTT DOCOMO, INC.)