3GPP TSG RAN WG1 #108	R1-2201468

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC

Title:	Remaining issues on SRS enhancement

Agenda Item:	8.1.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues for SRS enhancements.

- Flexible aperiodic SRS triggering

In an agreement made at RAN1#104-e, there was an FFS point on rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI.

Agreement (#104-e) Confirm the following working assumption with modifications An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI

This has been discussed but no conclusion has been reached. Therefore, in the latest specifications, UE behavior when such collision happens is not described, which results in that all the collided SRS transmissions may not be performed by considering such a case as error. In our view, at least one of the SRS transmissions collided should be performed as configured; otherwise, the configuration itself can be inefficient.

Then the question is how to determine which SRS transmission to be transmitted among the collided transmissions. The following is the latest status in RAN1; which we think seems reasonable.

FL Proposal 2-1: Introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs. - Adopt the following priority rules (with priority level from high to low) - Usage > CC ID > Set ID - For usages, priority order is AS > CB > NCB > BM - For CC ID/set ID, lower ID has higher priority than higher ID - The new dropping rule is a UE optional feature - UE will take collision as error case if UE does not support this feature

Meanwhile, we understand some companies do not prefer the priority rule above since the order of usage would be difference case-by-case. For this issue, we are open to discuss on whether to consider usage here.

Proposal 1:

- Support to introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CC

  - The priority rule should consider at least CC ID and Set ID

  - Usage configured for each SRS resource set can also be considered

- SRS switching for DL CSI acquisition with up to 8 Rx antenna ports

For guard symbols needed for SRS antenna switching, below were agreed in the previous e-meetings:

Agreement For two SRS resource sets of an xTyR antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a minimum gap period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot - The value of Y is same as the inter-resource GP defined in Rel-15 - FFS: Whether or not the minimum GP exists can be RRC configurable subject to UE capability - Whether this inter-set GP is needed for 4T6R can be discussed later per the decision on 4T6R configuration. - FFS: How/Whether to handle the case where the interval between SRS resource sets is larger than Y

In the upper agreement, Y-symbol guard period, which is the same as the inter-resource guard symbols defined in Rel-15, was agreed to be inserted even between two SRS resource configured in separate SRS resource sets when UE is capable of transmitting SRS in all symbols in a slot. Two FFSs remain; one is whether or not the minimum guard period exists can be RRC configurable subject to UE capability. We believe it would be better to support zero-symbol gap SRS resource configuration. This can be useful for those capable UEs which can handle zero-symbol gap antenna switching. UE can report whether it can support zero-symbol gap antenna switching as part of its capability reporting. NW can then configure zero-symbol gap SRS resources only to those capable UEs, which improves resource utilization efficiency.

Proposal 2:

- Support zero-symbol gap for antenna switching, subject to UE capability

The other FFS in the upper agreement is whether/how to handle the case where interval between SRS resource sets is larger than Y. For this issue, RAN4 has sent an LS [1] to this RAN1 e-meeting, in which the following information is shared:

- RAN4 thinks not sending PUSCH and PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient. - RAN4 clarifies that the transient period between SRS resources is 15us.

As RAN4 indicates, we believe not sending PUSCH/PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient when such resources are more than the ones needed for compensating transient period for SRS antenna switching (15us) in time domain. It should be clarified in the specification that the resources between SRS transmissions for antenna switching, at least other than Y-symbol guard period, would be available for other UL transmissions. Thus, we propose the following TP to 38.214.

Proposal 3:

- Adopt the following TP#1:

TP#1

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition [Unchanged Parts Omitted] The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. If the number of symbols between the SRS resources is larger than Y, the UE can be scheduled to transmit any other signals or channels in the symbols except for Y symbols before the latter SRS resource. [Unchanged Parts Omitted]

Another issue is whether to support updating Tx/Rx antennas for antenna switching in dynamic signaling. In our understanding, this functionality should be discussed after determining whether to support downgrading configuration for SRS antenna switching, which has been discussed a little bit in UE feature session but not yet concluded. If any downgrading configuration is not supported, then this discussion is not necessary at all since only a type of configuration is allowed for each antenna architecture. After supporting downgrading configurations, how to improve the flexibility can be discussed and then supported.

We actually find such downgrading configuration for SRS antenna switching beneficial from NW operation flexibility perspective, while how to report the capability can be discussed further in UE feature session. If supported, we are open to discuss on whether/how to improve the approach to update Tx/Rx antennas.

Observation 1:

- Before discussing whether to support updating Tx/Rx antennas for antenna switching in dynamic signaling, whether to support downgrading configuration for antenna switching has to be concluded

Proposal 4:

- Support downgrading configurations for SRS antenna switching with each antenna architecture introduced in Rel-17 (i.e. with 6 and 8 for Rx)

  - How to report its support from UE can be discussed further in UE feature session.

- Coverage and capacity enhancements

An agreement in RAN1#104-e captured an FFS which has not been resolved yet, that is, whether RPFS is applicable to FH case and non-FH case. During the email discussion after RAN1#107-e, we see companies’ views a little bit divergent on whether it is applicable to non-FH case, while it seems common understanding that it is applicable at least to FH case.

Agreement (#104-e) For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17.

In our view, RAN1 should support RPFS in Rel-17 regardless of FH case or non-FH case. One of the significant benefits gained by RPFS would be coverage enhancement of an SRS transmission by limiting the bandwidth, that is clearly valid in both FH case and non-FH case. If RPFS is limited only for FH case, the operation with RPFS will be restrictive, which results in that RPFS may not be used practically. Given that, we strongly hope to support RPFS regardless of FH or non-FH case.

Proposal 5:

- Support RPFS for both FH case and non-FH case

In addition to above, there would be another issue related to RPFS SRS sequence length. The following has been agreed so far on RPFS SRS sequence:

Agreement (#104-e) For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17. Agreement (#106-e) For RPFS SRS sequence generation, support - Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence. Conclusion (#107-e) No consensus to have further restriction on the number of RBs for RPFS in Rel-17. - No introduction of new sequence length

These three agreements and conclusion imply that 1) RPFS SRS sequence is generated by dividing legacy SRS sequence by partial frequency sounding factor PF, which could be either of {1, 2, 4}, with no restriction, while no new sequence length is specified by this feature on top of Rel-15/16. For SRS sequence in Rel-15/16, ZC sequence is used for the one whose length is equal to or larger than 36, and CG sequence is defined for several shorter sequences (i.e., 6, 12, 18, 24 and 30).

However, depending on the exact value of mSRS, Comb and PF, the length of RPFS SRS sequence could be other than the ones specified already. In the latest specification, UE behavior when such parameters are configured is not clear. We believe this issue needs to be resolved explicitly.

One potential approach, which is our best preference, is to make sure that only the combination(s) of mSRS, Comb and PF that result in the existing sequence length can be configured by NW. This rule can avoid any error case where UE cannot identify the sequence length for SRS. Although it limits the flexibility of the corresponding configurations, we believe this is the simplest solution preferred at this late stage.

Another approach can be to revise the equation for SRS sequence length specified in 38.211 so that any combination of mSRS, Comb and PF will result in one of the existing sequence lengths. This may make the equation much more complex, while the configuration flexibility will be kept at the same level.

In summary, we believe either of the following TPs has to be adopted to comply with the agreements so far, where the first one is our best preference.

Proposal 6:

- Adopt either of the following TPs

TP#2-1 (to 38.214)

### 6.2.1 UE sounding procedure [Unchanged Part Omitted] - Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0. The UE only expects to be configured with partial frequency sounding factor which generates one of sequence lengths defined in Clause 5.2.2 of [4, TS 38.211]. [Unchanged Part Omitted]

TP#2-1 (to 38.211)

##### 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF Msc,bSRS=min6mSRS,bNscRBKTCPF 6, 30 if mSRS,bNscRBKTCPF <36mSRS,bNscRBKTCPF otherwise where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. [Unchanged Part Omitted]

- Summary

In this contribution, we have discussed possible approaches for enhancing SRS transmission in NR Rel. 17. Based on the discussion, we have made following proposals.

Proposal 1:

- Support to introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CC

  - The priority rule should consider at least CC ID and Set ID

  - Usage configured for each SRS resource set can also be considered

Proposal 2:

- Support zero-symbol gap for antenna switching, subject to UE capability

Proposal 3:

- Adopt the following TP#1:

TP#1

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition [Unchanged Parts Omitted] The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. If the number of symbols between the SRS resources is larger than Y, the UE can be scheduled to transmit any other signals or channels in the symbols except for Y symbols. [Unchanged Parts Omitted]

Observation 1:

- Before discussing whether to support updating Tx/Rx antennas for antenna switching in dynamic signaling, whether to support downgrading configuration for antenna switching has to be concluded

Proposal 4:

- Support downgrading configurations for SRS antenna switching with each antenna architecture introduced in Rel-17 (i.e. with 6 and 8 for Rx)

  - How to report its support from UE can be discussed further in UE feature session.

Proposal 5:

- Support RPFS for both FH case and non-FH case

Proposal 6:

- Adopt either of the following TPs

TP#2-1 (to 38.214)

### 6.2.1 UE sounding procedure [Unchanged Part Omitted] - Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0. The UE only expects to be configured with partial frequency sounding factor which generates one of sequence lengths defined in Clause 5.2.2 of [4, TS 38.211]. [Unchanged Part Omitted]

TP#2-1 (to 38.211)

##### 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF Msc,bSRS=min6mSRS,bNscRBKTCPF 6, 30 if mSRS,bNscRBKTCPF <36mSRS,bNscRBKTCPF otherwise where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. [Unchanged Part Omitted]

References

- R1-2200895, “LS on Rel-17 FeMIMO SRS related impact”, 3GPP RAN WG1, February 2022