3GPP TSG RAN WG1 #108-e			R1-2201487

e e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on enhancements on PUSCH repetition type A

Agenda Item:	8.8.1.1

Document for: 	Discussion

- Introduction

In this contribution, we discuss remaining issues on enhancements on PUSCH repetition type A.

- Discussion on enhancements on PUSCH repetition type A

2.1 Slot indicated by K2 offset for the available slot counting

At the RAN1#107bis-e meeting, the slot indicated by K2 offset for the available slot counting was discussed and following proposal was introduced [1].

FL proposal:

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following

  - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot indicated by K2 offset.

    - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot indicated by K2 offset and does not perform the PUSCH transmission in later slots, either.

    - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following

  - Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following

  - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

  - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

    - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and does not perform the PUSCH transmission in later slots, either.

    - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following

  - Option 1: A UE does not assume that the slot determined in 38.321 Section 5.8.2 is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

FL proposal:

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following

  - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot indicated by K2 offset.

    - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot indicated by K2 offset and does not perform the PUSCH transmission in later slots, either.

    - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For DG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following

  - Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select on from the following

  - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

  - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

    - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and does not perform the PUSCH transmission in later slots, either.

    - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, select on from the following

  - Option 1: A UE does not assume that the slot determined in 38.321 Section 5.8.2 is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

The discussion point is whether PUSCH symbols can overlap with DL/SSB symbol in the slot indicated by K2 offset or not for DG-PUSCH, as well as whether PUSCH symbols can overlap with DL/SSB symbol in the slot determined in 38.321 Section 5.8.2 or not for CG-PUSCH. In our understanding, there is no restriction of indicating the PUSCH resource for DL symbol or SSB symbol, although we may not assume this kind of scheduling in the operation. Therefore, the introduction of new restriction is not necessary for the feature, and Option2 is considered for DG/CG-PUSCH with K=1 and K>1. In the case of K=1, we assume the determination of available slot and PUSCH dropping rules, so that the PUSCH transmission is expected in the next available slot as in Option 2-2.

Proposal 1: For the slot indicated by K2 offset for the available slot counting, Option 2 and Option 2-2 are considered for DG/CG PUSCH with K>1 and K=1, respectively.

- Conclusion

In this contribution, we discussed on the enhancements on PUSCH repetition type A. Based on the discussion we made following proposal.

Proposal 1: For the slot indicated by K2 offset for the available slot counting, Option 2 and Option 2-2 are considered for DG/CG PUSCH with K>1 and K=1, respectively.

References

- R1-2200789, Moderator (Sharp), “FL Summary #4 on Enhancements on PUSCH repetition type A”, 3GPP, RAN1#107bis-e, January 2022