3GPP TSG RAN WG1 #108-e			R1-2201500

e-Meeting, February 21th – March 3th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on efficient activation/deactivation mechanism for SCells

Agenda Item:	8.13.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#107-e meeting, the techniques for efficient SCell activation/deactivation were discussed and RAN1 made following agreements [1].

Agreement The max number of NZP CSI-RS resource set configurations for temporary RS per serving cell is the same as current maxNrofNZP-CSI-RS-ResourceSetsPerConfig. Agreement For efficient SCell activation with assistance of temporary RS, a SSB P-TRS of the to-be-activated SCell is to be configured as a QCL source for the temporary RS in case of known SCell same as existing specification. - Note: a SSB of the to-be-activated SCell is a QCL source for the P-TRS per existing specification - Note: It is RAN1 understanding that Scell activation latency can be reduced compared to Rel-16 even when P-TRS is configured as QCL source for the temporary RS in case of known SCell Below Working Assumption does not need to be confirmed. Working Assumption For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell - FFS: QCL type - FFS: the case of unknown SCell - FFS: other QCL source, e.g. the SSB/P-TRS of another active cell Agreement(for reference during the discussion) - For triggering temporary RS, down-select based on the following alternatives, or let RAN2 be aware the status of this discussion - Alt 1: Bitmap approach in MAC-CE similar to SCell activation - Every Z-bit block in the bitmap corresponds to a SCell, Z>=0 - A Z-bit block indicates the temporary RS [configuration index], and a value zero indicated by the bit block means no RS resource transmitted. - The to-be-activated SCell is indicated via the C values in the legacy SCell activation/de-activation MAC CE or in the new MAC-CE - Alt 2: Reuse A-TRS triggering framework - A trigger state is indicated by the MAC-CE explicitly - The association between a trigger state and aperiodic temporary RS for one or multiple SCells is configured by RRC according Rel-16 A-TRS triggering framework - SCell ID is configured as a part of the temporary RS configuration. Some SCell IDs derived from the trigger state triggered by the new MAC-CE may not refer to to-be-activated SCells that are indicated by the new MAC-CE or the legacy SCell activation/de-activation MAC-CE - FFS: The value zero of the MAC-CE indication means no temporary RS is triggered by the MAC-CE for all to-be-activated SCells - Note: The down-selection targets at a RAN1 consensus on MAC-CE functionality and the list of RRC parameters for this feature. Any MAC-CE signaling design above are reference concept, its final MAC-CE signaling design is up to RAN2. Agreement For both Alt 1 and Alt 2 of temporary RS triggering, - For Alt 1, the gap between temporary RS bursts is explicitly configured. - A set of possible gap lengths from which the triggering MAC-CE can indicate one from RAN1 perspective. Up to RAN2 to decide details. - For Alt 2, a gap length is configured by RRC for each temporary RS having two bursts. For different temporary RS, the value of the gap length can be different based on RRC configuration. - the number of bursts is up to 2. It can be either explicitly configured, or implicitly indicated by the gap configuration ((Up to RAN2 to decide one) Agreement For Alt 2 of temporary RS triggering, to avoid potential impact on the existing CSI-AperiodicTriggerStateList, a separate trigger-state list is used. - Note: it does not imply that Alt 2 has been selected by RAN2. Agreement For the RRC and MAC-CE designs of temporary RS triggering (both Alt1 and Alt2), from functionality perspective, the max number of to-be-activated SCells should be 15, irrespective of triggered number of temporary RS bursts per cell.  Note: UE capability for the max number of to-be-activated SCells with 2-burst temporary RS is not precluded.

In this contribution, we discuss efficient activation/deactivation mechanism for SCells.

- Discussion

According to the WID, the objective of the WI in RAN1 is to support efficient SCell activation/deactivation based on RAN1 leading mechanism. In Rel-16, the dormant BWP was introduced, and L1-based indication for the switching to/from the dormant BWP was supported, i.e., the indication in DCI format 0_1/1_1/2_6. On the other hand, SCell activation/deactivation is still based on MAC CE indication or timer-based mechanism, and also SCell activation delay could be long due to e.g., long SSB periodicity. Thus, fast SCell activation should be considered. In addition, even if the dormant behaviour is introduced, SCell would enter deactive state after long duration with no traffic, and fast SCell activation from deactive state is still beneficial.

  - Fast SCell activation

According to TS38.133, upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as an example is shown in Fig.1. THARQ is the duration between DL data transmission and ACK. Tactivation_time is SCell activation delay for time/frequency tracking, AGC setting, MAC CE processing and so on, which depends on the SCell measurement cycle and whether the SCell is known or not. TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources.

Figure 1: Example of SCell activation delay

At the last meeting, we discussed whether SSB of one of the active cells can be indicated as a QCL source for temporary RS for the case of unknown SCell. As it was mentioned by some companies, RAN4 had already informed RAN1 of the following agreements in the LSs to support the case of intra-band continuous CA.

R1-2102300: - SCell is unknown and belongs to FR1 - When SCell is contiguous to an active serving cell in the same band (Intra-band continuous CA) - 1 burst (2-slot with four CSI-RS resources) is necessary for AGC when the power difference in serving cell and to be activated Scell is smaller than or equal to 6dB - No cell detection provided the conditions specified for intra-band contiguous CA case in TS38.133 section 8.3.2 are satisfied; - 1 burst (2-slot with four CSI-RS resources) is necessary for time-frequency tracking

R1-2106427: - SCell is unknown and belongs to FR1 - When SCell to be activated is non-contiguous to an active serving cell in the same band, or - When SCell to be activated and active serving cell are in the different band - It is not a target scenario for temporary RS based SCell activation latency optimization. - The agreement above applies based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798 - SCell to be activated belongs to FR2 - If the SCell being activated is unknown and there is no active serving cell on that FR2 band, - It is not a target scenario for temporary RS based SCell activation latency optimization. - The agreement above applies based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798

Proposal 1: At least in the case of intra-band continuous CA, SSB of one of the active cells can be indicated as a QCL source for temporary RS for the case of unknown SCell.

- Conclusion

In this contribution, efficient activation/deactivation mechanism for SCells was discussed. Based on the discussion, the following proposals were made:

Proposal 1: At least in the case of intra-band continuous CA, SSB of one of the active cells can be indicated as a QCL source for temporary RS for the case of unknown SCell.

References

- 3GPP RAN1#107-e, Chairman’s note, November 11th – 19th, 2021.