3GPP TSG RAN WG1 #108-e			R1-2201515

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.6

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for REDCAP

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.6 regarding UE features for RedCap and captures the following email discussion.

[108-e-R17-UE-features-RedCap-01] Email discussion on UE features for RedCap – Shinya (DOCOMO) - 1st check point: February 25 - Final check point: March 3

In the updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e [1], there are following feature groups for RedCap.

- 28-1	RedCap UE

- 28-2	Number of UE Rx branches and DL MIMO layers for RedCap UE

- 28-3	Half-duplex FDD operation type A for RedCap UE

The issues to be discussed are tagged and colour coded with High priority, Medium priority, or Low priority, considering RAN2 impact especially for capability signaling design.

- 28-1: RedCap UE

In [1], FG 28-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH FFS whether to add any other basic features for RedCap UE |  | Yes |  | Network assumes the UE is not a RedCap UE | Per UE | No | No |  | RedCap UEs do not support carrier aggregation or dual connectivity. | Optional with capability signaling RedCap UE must indicate this FG is supported |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [2] | Huawei, HiSilicon | RAN1 introduces NCD-SSB for serving cell purpose. Although NCD-SSB is introduced for performing RRM/RLM/BFD measurement in an active BWP, RAN4 has reached conclusion that it is feasible for a RedCap UE to perform L1/L3 measurement based on CSI-RS and/or FG 6-1a by reporting optional capabilities, if the RRC-configured active DL BWP has no SSB in connected mode [1]. The previous RAN1 agreements state that there is an operation without need of NCD-SSB based on support of CSI-RS and/or FG 6-1a. Since the basic FG 6-1 also addresses CORESET#0, and it is not preferable to use an existing FG with different interpretation, for RedCap UEs. Also, IODT is an aspect that needs to be considered, therefore a signaling bit might be desirable. We suggest to introduce a new FG as mandatory for RedCap UE to report whether it can expect NCD-SSB in the active BWP, e.g. to perform RRM/RLM/beam measurement of the serving cell. Also, it is required for indication of support if a UE does not report either CSI-RS based measurements nor FG 6-1a, as per the RAN1 agreements. Proposal 1: For RedCap, define a new FG to report whether a UE supports NCD-SSB based measurements in the active BWP or not, if neither FG 6-1a nor CSI-RS based measurement is supported. |
| --- | --- | --- |
| [3] | FUTUREWEI | Observation 1. - FG 28-1 must include a component for separate initial DL BWP - FG 28-1 must include a component for separate initial UL BWP - FG 28-1 must include a component to disable frequency hopping for PUCCH formats 0 and 1 during initial access - Add a note to FG 9-1 stating that if a RedCap UE supports 2-step RACH, it supports early indication The motivations and justifications for these observations are presented below. FG 28-1 The basic feature group 28-1 already includes a BW requirement and support of early indication via 4-step RACH. For FG 28-1, there is an FFS to capture additional components, such as support of separate initial DL BWP and separate initial UL BWP [2]. Based on the agreements for reduced bandwidth operation in RAN1#107 [7], both separate initial DL and UL BWPs are needed when the channel bandwidth for non-RedCap UEs exceeds the maximum bandwidth for RedCap UEs. The need for the separate initial DL BWP is based on several reasons including when the center of the MIB-configured initial BWP is not aligned to the center of the separate initial UL BWP in TDD deployments. One component to add to FG 28-1 is the disabling of PUCCH frequency hopping for formats 0 and 1 during initial access to address PUSCH fragmentation. A RedCap UE receives configuration signaling to disable frequency hopping in SIB1. While further details about resources used for hopping remain, the point is a RedCap UE must support disabling PUCCH frequency hopping. Early indication for 2-step RACH During RAN1#107, there was a discussion about components related to early indication. It was agreed to add early indication for 4-step RACH as a component for FG 28-1. It was agreed to support early indication for 2-step RACH. However, since 2-step RACH is an optional feature for RedCap UEs, it cannot be a component in FG 28-1. In one option, it was proposed to create a separate FG for early indication for 2-step RACH in [2]. Another option is to update the description of FG 9-1 “Basic channel structure and procedure of 2-step RACH” using a modification similar to the agreement for DL 256QAM. For the former option, a new FG is needed with a prerequisite of FG 9-1. For the latter option, a note is needed in the description of FG 9-1, such as ‘For RedCap UEs, early indication is supported if a RedCap UE supports 2-step RACH’. Of the two options, adding a note is preferable to reduce the signaling overhead, but RAN2 will make the determination. The key point is that the agreement in RAN1#105 states that FG 6-1 is used as a starting point, and it is not precluded to make changes to the components. Although the RAN1#107 agreements state that FG 6-1 is mandatory, several companies suggested that a modification of FG 6-1 may be needed for RedCap UEs. For example, a RedCap UE supporting FG 6-1 needs either (a) a NCD-SSB if the RRC-configured active DL BWP does not include CORESET#0/SIB and CD-SSB; or (b) CD-SSB and CORESET#0/SIB in the RRC-configured active DL BWP. For (a), the network should provide appropriate search spaces in order to receive SIB or use dedicated signaling. We are open to consider modifying FG 28-1 with appropriate components if modification to the existing FG 6-1 for RedCap UEs is not trivial. For FG 6-1a, based on the FG description and RAN1#107 agreement, the network does not have to provide any SSB. However, the UE cannot rely on the CSI-RS for RRM measurements according to the RAN4 LS [11]. We are open to consider modifications to FG 6-1a or even adding an FG for RedCap UE for non-trivial modifications. |
| [4] | vivo, Guangdong Genius | Based on the progress achieved for maximum UE bandwidth reduction in AI 8.6.1, it was agreed that for an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, a RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB [2]. Therefore, we think following can be considered to be added in the FG 28-1 as basic RedCap UE features. Proposal 1: Following “Basic BWP operation for RedCap UE” should be added as RedCap UE basic UE features, including following components: 3. Basic BWP operation with restriction for RedCap UE - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - BW of a UE-specific RRC configured BWP includes SSB for PCell - BW of a UE-specific RRC configured BWP may not include BW of MIB-configured CORESET#0 For issue#2 of the indication type for FG 28-1, it can be found in TS 38.101 Table 5.3.5-1 that many bands could be deployed with channel bandwidth of up to 20MHz, for example band n2/n5/n8, etc. In these bands, since the channel bandwidth deployed by the NW is not larger than the maximum bandwidth supported by RedCap UEs, there is no need for a UE capable of up to 20MHz BW to report it is a RedCap UE, which will have lower performance. Therefore, it is reasonable to indicate the RedCap type per-band. If FG 28-1 is specified per band, there is no need of FDD/TDD differentiation and no need of FR1/FR2 differentiation. For confirming the WA that a RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities, based on the reply LS [3] from RAN4, see following, it is observed that for a RedCap UE operation without SSB in a UE-specific BWP, CSI-RS cannot be used as a standalone method and FG6-1a is the precondition. Proposal 2: An optional, new FG with FG6-1a as prerequisite can be introduced for the operation without SSB in a UE-specific BWP for RedCap UEs when the WA on supporting relevant operation based on for CSI-RS is confirmed. Proposal 3: The indication type for 28-1 should be per band. - There is no need for FDD/TDD differentiation and no need of FR1/FR2 differentiation For a separate initial UL BWP, it should include the configuration(s) needed for RedCap UE to perform random access, e.g., the RedCap-specific PRACH configuration, Msg.3 and PUCCH for Msg.4 acknowledgment etc. For a separate initial DL BWP if configured, it should at least include the CSS/CORESET for random access, and it is configurable to include the SSB, CORESET/CSS for paging, CORESET/CSS for SIB1/OSI and MIB-configured CORESET#0. Besides, according to the agreements, a separate initial UL and DL BWP can be used during the initial access before UE indicating its capability, further discussion is needed on whether the two FGs should be optional without capability or optional with capability. Proposal 4: Introduce following new FGs related to the separate initial UL BWP and the separate initial DL BWP for RedCap UEs: - FG 28-y: A separate initial UL BWP - It includes the configuration(s) needed for RedCap UE to perform random access; - FG 28-z: A separate initial DL BWP - It includes CSS/CORESET for random access. Proposal 5: For the two new FGs of the separate initial UL and DL BWP, further discussion is needed on whether the two FGs are optional without capability signaling or optional with capability signaling. |
| [5] | ZTE, Sanechips | RedCap UE has the ability to share the initial UL BWP with the legacy NR UE, also has the ability to be configured with a separate initial UL BWP, which can be used during initial access. Obviously, it is kind of new capability different with NR UE. And it is also agreed that the separate initial DL BWP can be used for RAR. Considering the separate initial DL/UL BWP can be used during initial access before UE capability report, the separate initial DL/UL BWP should be set as mandatory feature for RedCap UE or basic component in FG28-1. Proposal 2: For RedCap UE, separate DL/UL initial BWP used in initial access is a basic feature group or a basic component in FG28-1. RedCap is expected to support all the FR1/FR2 bands. Therefore, it is not needed to differentiate FR1/FR2. Proposal 3: For FG 28-1, RedCap UE should be ‘Per UE’ reported. - FDD/TDD differentiation can be set as ‘NO’ - FR1/FR2 differentiation can be set as ‘NO’ |
| [6] | OPPO | We had concluded that the FR1 RedCap UE support 20MHz, and FR2 RedCap UE support 100MHz. No additional bandwidths are further introduced. That FR differentiation can be directly determined from RedCap UE. The 28-1 the indication is now per UE. - Proposal 1: Maximum Supported bandwidth of RedCap UE is indicated by FG28-1. |
| [7] | CATT | In FG 28-1, it is FFS whether to add any other basic features for RedCap UE. From view of UE characteristics, it is nature to take ‘reduced number of Rx branches’ as one typical property of RedCap UE, thus adding ‘reduced number of Rx branches (and corresponding number of DL MIMO layers)’ in FG 28-1 seems reasonable. Nevertheless, number of Rx branches is likely to be per band basis, e.g. 1Rx or 2Rx in TDD band and 1Rx in FDD band. This may result in ambiguity in this ‘per UE’ feature. Therefore, we can live with the current FG 28-1, without adding any other features for RedCap UEs. Proposal 1: Prefer not to add any other basic features in FG 28-1. |
| [8] | Nokia, Nokia Shanghai Bell | - 28-1: - Per UE - FR1/2 differentiatoin - No need for xDD differentiation Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list. |
| [9] | NTT DOCOMO | - Based on the agreements in RAN1#107-e [2], active DL BWP without CD-SSB or NCD-SSB is optional capability. We are fine with either defining a new FG for RedCap or reusing existing FG 6-1a (BWP operation without restriction on BW of BWP(s)). - Report type of this FG should be per UE with FR1/FR2 differentiation or per band for more flexibility to indicate the support of this BWP operation for different FR. - Regarding FFS whether to add any other basic features for RedCap UE, - Following capabilities can be added as components in FG 28-1 based on the agreements in RAN1#107-e [2]: - Separate initial DL BWP for RedCap UEs - Separate initial UL BWP for RedCap UEs - Active DL BWP with CD-SSB or NCD-SSB - RAN2 made following agreement highlighted in yellow in RAN2#116bis-e [3]: Agreements via email - via offline 105 second round: - Capture “Support of RedCap early indication based on Msg1, MsgA and Msg3 for RACH” in the field description of capability bit “support of RedCap”; 2. Capture the limitation on BW, Rx and MIMO in 4.2.xx RedCap Parameters of TS38.306 running CR as: - The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2; UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs; - 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported. UE features and corresponding capabilities related to more than 2 UE Rx branches and more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches and more than 2 UL MIMO layers are not supported by RedCap UEs; - Therefore, following capabilities can be added as components in FG 28-1 - Early indication of RedCap UE in Msg.3 for 4-step RACH - Early indication of RedCap UE in Msg.A preamble part for 2-step RACH - Conditioned on support of 2-step RACH - Early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH - Conditioned on support of 2-step RACH - Report type of FG 28-1 should be per UE without FDD/TDD or FR1/FR2 differentiation - We don’t think a UE operates as RedCap UE in a band but as non-RedCap UE in another band. UE can report its supported bands where it can operate as RedCap UE. Also, RAN2 made following WA in RAN2#116bis-e [3] Working Assumption: - The capability “support of RedCap” is per UE capability. Take a final agreement in the next meeting based on possible further feedback from RAN1 - Note of FG 28-1 can be kept |
| [10] | NEC | Regarding FG 6-1, basic BWP operation for RedCap UE supports a case where BW of a UE-specific RRC configured BWP does not include BW of CORESET#0 and SSB for PCell/PSCell (i.e. CD-SSB) but includes NCD-SSB and a common CORESET (if configured). To accommodate the case for RedCap UE, FG 6-1 would need updates. Proposal: - FG 6-1 is proposed to be updated as follows: 6-1; Basic BWP operation with restriction; 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; for RedCap, BW of a UE-specific RRC configured BWP includes BW of CORESET#0 and SSB for PCell, or alternatively a common CORESET (if configured) and NCD-SSB. 6-1a; BWP operation without restriction on BW of BWP(s); BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell. |
| [11] | Ericsson | Proposal 1 Discuss how to capture the RAN1 agreements regarding operation in an RRC-configured active DL BWP in connected mode (e.g., in the definitions of FGs 6-1 and 6-1a, or in the new FGs 28-x). In our view, early indication of RedCap UE in Msg3 for 4-step RACH will also be a mandatory component for RedCap UEs, but this can be decided in RAN2. FR1/FR2 differentiation would be needed if it is desired to indicate that a UE supporting both FR1 and FR2 is a RedCap UE in one of the frequency ranges but not in the other frequency range. It is currently not clear to us whether there might be a need for such a hybrid UE. For FG 28-1 (‘RedCap UE’), discuss whether there is any need of FR1/FR2 differentiation. For FG 28-1 (‘RedCap UE’), for the fields “Consequences if not supported”, “Type”, “Need of FDD/TDD differentiation” and “Note”, the highlighted text can be used as is. |
| [12] | Intel Corporation | For some of the FGs #28-x identified so far, the FG type can be confirmed as “per band”, with exception of FG #28-1 which should be per UE. For FG #28 -1, the type should be “per UE” since the overall cost/complexity reduction benefit for a UE that may behave as non-RedCap UE in some bands while as a RedCap UE in some others is rather limited. More importantly, given the single carrier operation for RedCap UEs, what matters is if the UE reports itself a RedCap UE for the carrier in the band in which the UE may be currently camping on. Thus, a “per UE” capability indication for FG 28-1 is sufficient. Proposal 1: - Confirm the type for FG 28-1 as “per UE”. - Confirm the type for FG 28-3 as “per band”. For FG 28-1, support of early identification during Msg1 transmission has been captured as a component. Further, if 2-step RACH is optionally supported by RedCap UE, RedCap early identification during MsgA preamble transmission should also be added as a component that is relevant only to RedCap UEs for existing 2-step RACH feature. Alternatively, a separate feature FG 28-y may be introduced that is mandatory for RedCap UE (i.e., supporting FG 28-1) supporting 2-step RACH. However, updating FG 28-1 as suggested below in Proposal 2 would be most appropriate and consistent with the handling for 4-step RACH. Also, support of separate initial DL and UL BWPs should also be included as a component of FG 28-1 as this is a key feature that define RedCap UEs and should be supported by all RedCap UEs. Proposal 2: - Capture the following as a component (#4) for FG 28-1: - “4. For 2-step RACH, support of early identification of RedCap UE during MsgA preamble transmission.” - “5. Support of separate initial DL and UL BWPs configured by SIB1.” - Add a note to FG 28-1: “Component 4 is applicable only if the UE also supports FG 9-1”. Next, on xDD and FRx differentiations, while FG #28-3 is applicable to FDD spectra and FR1 bands only, there is no need for FDD/TDD differentiation for FG #28-1 (for similar reasons why “per band” signaling is not necessary for FG #28-1). Neither is FRx differentiation necessary for FG 28-1. Proposal 3: - xDD and FRx differentiations are not necessary for FGs 28-1 and 28-3. - FG 28-3 is only applicable in FDD and FR1 bands. In addition to the above, several other UE features from Rel-15/16 are expected to need adjustments for RedCap UEs as compared to non-RedCap UEs. Most of these have already been resolved in earlier meetings with FG 6-1 being one key FG awaiting further resolution. - FG #6-1 - For RedCap UEs, operation without CORESET #0 within the active DL BWP should be mandatory and only expectation on SSB in RRC-configured DL BWP may be considered as a modification to FG 6-1. This would require updates to at least FG #6-1. - Alternatively, a new mandatory FG for RedCap UEs such that the RedCap UE expects SSB in RRC-configured DL BWP should be introduced with FGs 6-1 and 28-1 as pre-requisites. Proposal 4: - FG # 6-1 should be adapted for RedCap UEs such that operation without CORESET #0 within the active DL BWP is mandated for RedCap UEs. - Add a note to FG 6-1: “For a UE supporting FG 28-1, component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)”” - Alternatively, a new mandatory FG for RedCap UEs such that the RedCap UE expects SSB in RRC-configured DL BWP should be introduced with FGs 6-1 and 28-1 as pre-requisites. |
| [13] | Apple | One FFS aspect for FG 28-1 is whether to add other UE features [2]. In RAN1 107 e-meeting, it was agreed that SSB, either CD-SSB or NCD-SSB, can be expected for basic Redcap UE but not CORESET#0 [1]. Therefore, the following features can be added into basic feature group FG 28-1 for Redcap: - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - BW of a UE-specific RRC-configured BWP includes BW of SSB - BW of UE-specific RRC-configured BWP may not include BW of MIB-configured CORESET#0. We therefore proposed the following: Proposal 1: Consider adding the following basic FGs into FG 28-1: - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - BW of a UE-specific RRC-configured BWP includes BW of SSB - BW of UE-specific RRC-configured BWP may not include BW of MIB-configured CORESET#0. In addition, we propose to introduce an optional UE feature FG 28-x to indicate the support of DL BWP without SSB in RRC_CONNECTED state, analogous to legacy FG 6-1A. Proposal 2: Introduce a new FG 28-x that is used to indicate the support of a UE-specific DL BWP without SSB when UE is in RRC_CONNECTED state. |
| [14] | CMCC | According to the following agreements made in RAN1#106b-e and 107e, network can configure separate initial UL/DL BWP for RedCap UEs by SIB before UE capability signaling, and when bandwidth of legacy initial UL/DL BWP is larger than 20MHz, network has to configure a separate initial UL BWP, and for initial DL BWP, it can also configure a separate initial DL BWP or reuse CORESET#0. So RedCap should mandatory support separate initial DL/DL BWP configuration, this should be a basic feature for RedCap UE. Proposal 1: Following features are added as components in FG 28-1 - Separate initial DL BWP for RedCap UEs - Separate initial UL BWP for RedCap UEs According to the agreements of RAN1#107e, there is still one FFS for whether to introduce other early indication schemes into FG28-1. Related features includes[4], - Whether to add the capability of early indication of RedCap UE in Msg.A preamble part for 2-step RACH. - Whether to add the capability of early indication of RedCap UE in Msg.3 for 4-step RACH as a component in FG 28-1 or leave to RAN2. - Whether to add the capability of early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH as a component in FG 28-1 or leave to RAN2. Among the above three features, the first one and the third one are related to 2-step RACH, which is optionally supported according to following agreements made in RAN1#105e, so we don’t think these two features should be added to FG28-1, and they can be separate feature groups. While both early indication of RedCap UE in Msg.3 for 4-step RACH and early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH are RAN2 issues, the discussion of these two capability is up to RAN2. Proposal 2: The capability of early indication of RedCap UE in Msg.A preamble part for 2-step RACH is not added to FG28-1. Proposal 3: Leave the feature discussion of the following two capabilities to RAN2, - The capability of early indication of RedCap UE in Msg.3 for 4-step RACH . - The capability of early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH. For the first kind of capability, there are two options to define the UE feature, - Alt.1: No separate new FG is introduced, while add a note in FG6-1(Basic BWP operation with restriction) that “For RedCap UEs, BW of a UE-specific RRC configured BWP may or may not include BW of CD-SSB and the entire CORESET#0, if not, BW of the UE-specific RRC configured BWP includes NCD-SSB. - Alt.2: A new FG 28-x is introduced, with the following component, - BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB. Comparing these two options, we prefer Alt.2. Alt.1 introduces one case that BW of a UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0 into the traditional FG6-1, this is different from what legacy FG6-1 mentions. While Alt.2 has no impact on legacy FG. The prerequisite feature group of this new feature can be FG 6-1, and since this is the default capability, RedCap UEs have to report this capability. Proposal 4: Define a new FG28-x1 for the capability of UE expect NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. Proposal 5: The new FG28-x1 includes the following component, and the prerequisite feature group of this is FG 6-1, - BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB. According to analysis in section 2.1, 2-step RACH is optionally supported according to agreements made in RAN1#105e, so the capability of early indication related to 2-step RACH is proposed to be a separate FG, with component that RedCap dedicated PRACH resource or PRACH preamble determination for early indication. If the corresponding capabilities are not supported, UE will rely on re-tuning to CD-SSB for such measurement,which may be already supported as FG6-1a. So we propose to introduce a new FG 28-x2, to support this optional capability, which include the following components, - BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0. - support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. - support measurement operation based on FG6-1a. And the prerequisite feature group of this new feature can be FG 6-1a. Proposal 6: Define a new feature FG28-x2 for the capability of supporting operation without NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. Proposal 7: The new FG28-x2 includes the following component, and the prerequisite feature group of this is FG 6-1a, - BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0. - Support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - Support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. - Support measurement operation based on FG6-1a. Proposal 8: Define a new FG28-x3 to include capability of early indication of RedCap UE in Msg.A preamble part for 2-step RACH Proposal 9: The new FG28-x3 includes the following component, and the prerequisite feature group is FG9-1, - RedCap dedicated PRACH resource or PRACH preamble determination for early indication. |
| [15], [19] | Xiaomi | In our opinion, defining the capability “support of Redcap” per band is preferable. In some frequency bands, the system bandwidth is up to or less than 20MHz, then for some high-end RedCap devices e.g. supporting 2 Rx, FD-FDD and 256QAM in DL could show the same capability with normal devices in these bands. Then RedCap devices could work as a non-RedCap in these frequency bands. From this point, we think it is beneficial for both network and devices. In this case, the network system could incorporate the RedCap devices without upgrading the network system and extend the RedCap’s service coverage, which could accelerate the commercialization of RedCap. Proposal 1: Define capability “support of RedCap” per band During RAN1 #107-e meeting, it was agreed to include early indication in Msg.1 as the basic component of FG 28-1 and leave FFS whether include early indication in Msg.3 in FG 28-1. Given the agreement and working assumption achieved in RAN2 #116bis-e meeting, early indication Msg.3 should be captured into FG 28-1 as well Agreements online: 1.In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data (no other precondition) 2.Also when msg1 early identification is configured, new dedicated LCID is used for CCCH identification Working assumption: 1.Msg3 early identification is mandatorily supported by RedCap UE. Proposal 2: Capture the following initial UL/DL BWP related description into the component of FG 28-1 - Up to 1 separate initial UL BWP - Up to [1] separate initial DL BWP - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - Early indication in Msg.3 in 4-step RACH |
| [16] | Samsung | RAN 1 agreed to support random access procedure in separate initial BWP, where RedCap UE does not expect it to contain SSB/CORESET #0/SIB. Since this feature applies during initial access, it should be part of basic feature of RedCap for FG 28-1. Proposal #1: Update the component of FG 28-1 as following: 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1, and Msg. 3 for 4-step RACH and Msg.A for 2-step RACH 4. Random access procedure in initial DL BWP with and without SSB/CORESET #0/SIB FFS whether to add any other basic features for RedCap UE Proposal #2: Add a note for FG 28-1: the support of early indication based on Msg A depends on the support of FG 9-1 RAN 2 agreed on “The capability “support of RedCap” is per UE capability.” as working assumption. From RAN 1 pespective, we think this should be a per UE feature since this is related to hardware of a user device. Proposal #3: FG 28-1 is per UE. |
| [17] | MediaTek | - Confirm RAN2’s working assumption that type of FG 28-1 is “Per UE.” - Add that RedCap UEs are not allowed to support maximum UE bandwidth larger than 20MHz/100MHz in FR1/FR2 to the Note. Proposal 1: For FG28-1, confirm the type is “Per UE.” Proposal 2: Add that RedCap UEs are not allowed to support maximum UE bandwidth larger than 20MHz/100MHz in FR1/FR2, respectively, to the Note of FG 28-1. |
| [18] | Qualcomm | According to the WID for R17 RedCap UE, system should support deployment of RedCap UE in all FR1/FR2 bands for FDD and TDD. On the other hand, RedCap UE’s L1 features, such as BW reduction, RX branch number reduction and duplex mode, are band-specific. Given the potential UE testing differentiation among licensed, unlicensed and NTN bands, the specification and capability signalling for R17 RedCap UE FG should be per band by default. Based on the replies of RAN2 and RAN4 [3-4] to the RAN1 LS [1], any RRC_CONNECTED RedCap UE can perform L1/L3 measurements based on NCD-SSB transmitted in its DL BWP configured by RRC. Therefore, the basic UE FG definition for RedCap should include the capability of L1/L3 measurements based on NCD-SSB. Proposal 1: The basic RedCap UE FG should include a component of performing L1 and L3 measurements based on NCD-SSB transmitted in the RRC-configured DL BWP of RedCap UE. Given the “per band” differentiation for the components in FG 28-1, 28-2 and 28-3, it is not necessary to further differentiate the frequency range or duplex mode of these FGs. As a result, a RedCap UE can be deployed on paired spectrum or unpaired spectrum in FR1, and a RedCap UE is not mandated to support deployment in both FR1 and FR2. The deployment of RedCap UE should support 1 RX branch and single DL MIMO layer. If some use cases do not require a small form factor but need a higher peak data rate on DL, RedCap UEs with 2 RX branches can be optionally supported. If a RedCap UE is deployed on paired spectrum in FR1, it can support either FD-FDD or Type-A HD-FDD. Therefore, we have the following proposals on L1 UE features of R17 RedCap devices: Proposal 2: Support the following definition for FG 28-1 of R17 RedCap UE: |

Discussion

[GTW1] High priority question 2-1:

- Companies are encouraged to provide views on whether to add an FG for RRC-configured DL BWP without CD-SSB or NCD-SSB

| Company | Comment |
| --- | --- |
| vivo | Not necessary. We think the existing FG6-1a can be reused. |
| FUTUREWEI | We are open to consider discussing such a FG if it is necessary |
| Ericsson | It can either be a new FG or a RedCap-specific modification of the FG 6-1a definition. If the functionality relies on sufficient measurement gap configuration, that may need to be clarified in the Note field. |
| Nordic | Reuse existing FG6-1a |
| Intel | Same view as Vivo and Nordic – at this point, we do not see a reason for separate FG for FG 6-1a. |
| Qualcomm | If FG 6-1a of legacy UE implies legacy UE can measure CD-SSB outside RRC-configured DL BWP without measurement gap, FG 6-1a does not apply to R17 RedCap UE due to BW reduction. |
| FL1 | This question was discussed in the GTW session on Feb 21 but no consensus was achieved. As discussed in the GTW session, we will wait for the progress on RedCap UE behavior in RRC-configured DL BWP without CD-SSB or NCD-SSB in AI8.6.1.1 |

High priority question 2-2:

- Companies are encouraged to provide views on whether to add an FG for CSI-RS based L1/L3 measurement

| Company | Comment |
| --- | --- |
| Moderator | This issue will be discussed after the corresponding working assumption is confirmed in AI8.6.1.1 |

[GTW1] High priority question 2-3:

- Companies are encouraged to provide views on whether to add following capabilities as components in FG 28-1

  - a) Up to 1 separate initial UL BWP for RedCap UEs

    - It includes the configuration(s) needed for RedCap UE to perform random access

  - b) Up to 1 separate initial DL BWP for RedCap UEs

    - It includes CSS/CORESET for random access

    - [For separate initial DL BWP used for paging, CD-SSB is included]

    - For separate initial DL BWP only used for RACH, SSB may or may not be included

  - c) RRC-configured DL BWP with CD-SSB or NCD-SSB

  - d) NCD-SSB based measurements in RRC-configured DL BWP

  - e) Disable frequency hopping for PUCCH formats 0 and 1 during initial access

  - f) Early indication of RedCap UE in Msg.3 for 4-step RACH

  - g) Early indication of RedCap UE in Msg.A preamble part for 2-step RACH

    - Conditioned on support of 2-step RACH

  - h) Early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH

    - Conditioned on support of 2-step RACH

| Company | Comment |
| --- | --- |
| vivo | - a) and b) can be made as separate FGs - e) and f) can be included as component of FG28-1 - g), h) should not be included as components of FG 28-1, as clearly 2-step RACH should not be the basic feature for RedCap UE - FFS for c) and d), depending on whether non-RedCap UEs are allowed to implement NCD-SSB related features, and if so, c) and d) can be made as separate FG (not coupled with FG-28-1. But even if c) and d) are separate FG, they may still be mandatory for RedCap UEs. |
| FUTUREWEI | We would like to add a), b), e) as components of FG28-1 For f), RAN2 should make that determination For g) and h) – we should avoid placing an optional element (2-step RACH early indication) in the basic FG. We would prefer to see changes made to FG9-1 (similar to 256QAM and MIMO). If companies do not agree to that, then a new FG is added. |
| Ericsson | - FG 28-1 should include at least points a) – e) as components. In any case, it is important that all of a) – e) are mandatory for RedCap UEs. - For point e), the wording “during initial access” can be changed to “before dedicated PUCCH resource configuration” to be more precise. - Point f) should be included in FG 28-1, but the decision can possibly be made in RAN2 instead of RAN1. - Points g) – h) can be decided in RAN2. |
| Nordic | a-f are essential parts of RedCap feature, and should be part of FG28-1 g-h should be optional In addition - and b) “Support 1 separate initial UL/DL BWP for RedCap UEs” |
| Intel | We support adding all these as components of FG 28-1. Separate FGs are not warranted at this point: - Decision on applicability of NCD-SSB to non-RedCap UEs is out of scope of current discussions and can be addressed separately. - We do not see any issues with including g) and h) as components of basic FG for RedCap with the conditioning on support of 2-step RACH. |
| Qualcomm | - From RAN1 perspective, FG 28-1 should include at least points a) – e) - f) can be left to RAN2 discussion - g) and h) are not needed for FG 28-1 - FG 28-1 should be defined as “per band.” |
| FL1 | Following was agreed at the GTW session on Feb 21. Agreement - Following capabilities are added as components in FG 28-1 - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - b) Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included For the following capabilities, we will wait for the progress on RedCap UE behavior in AI8.6.1.1 - Enabling/disabling frequency hopping for PUCCH formats 0 and 1 during initial access / before dedicated PUCCH configuration - c) RRC-configured DL BWP with CD-SSB or NCD-SSB - d) [NCD-SSB based measurements in RRC-configured DL BWP] For the following capabilities, companies are invited to provide view whether they can be left to RAN2 or not - f) Early indication of RedCap UE in Msg.3 for 4-step RACH - g) Early indication of RedCap UE in Msg.A preamble part for 2-step RACH - Conditioned on support of 2-step RACH - h) Early indication of RedCap UE in Msg.A PUSCH part for 2-step RACH - Conditioned on support of 2-step RACH |
| Panasonic | We are ok with above handling i.e. frequency hopping and c) and d) are to wait AI8.6.1.1 and f)g)h) are to be left to RAN2. |
| CATT | We are fine to leave (f)~(h) to RAN2. |
| NTT DOCOMO | Based on the RAN2 agreement, (f), (g) and (h) can be included as components in FG 28-1. But if it is controversial, we are fine to leave to RAN2. |
| ZTE, Sanechips | We are OK to leave them to RAN2. |
| NEC | We are fine to leave capabilities f), g) and h) to RAN2. |
| CMCC | We are fine to leave f), g), h) to RAN2. |
| Vodafone | Same view as NTT Docomo on components (f)~(h). It may be useful for IODT that components (c)~(d) have a separate FG but fine to wait for 8.6.1.1 |
| Xiaomi | We are OK with the to handle frequency hopping and c) and d) based on the progress in AI8.6.1.1 For f) and h), we are OK to leave to RAN2 For g）， we think it could be handled in RAN1. During last meeting, RAN1 agreed to include The capability of early indication of RedCap UE in Msg.1 for 4-step RACH is added as a component in FG 28-1. The situation of Early indication via Msg.A PRACH in 2-step RACH is similar to that of early indication via Msg.1 in 4-step RACH, it can also be discussed in RAN1 as well. |
| Ericsson | We are fine to leave (f)~(h) to RAN2. |
| FUTUREWEI | OK to leave (f), (g), (h) to RAN2. |
| Qualcomm | We think it is fine to leave (f)-(h) to L2 UE capability discussion of RAN2. |
| Sierra Wireless | Fine to leave decision on UE capabilities f), g) and h) to RAN2. |
| Intel | Fine to leave f), g), h) to RAN2. |
| FL2 | Regarding f) to h), most companies are fine to leave to RAN2. Regarding FH, following was agreed in AI 8.6.1.1. Agreement Disabling of frequency hopping for common PUCCH resources for RedCap UEs is only supported for separate (not shared) initial UL BWP. Following proposal is made based on the above. [FL2] Low priority proposal 2-3a: - Component in FG 28-1 is updated as follows - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Add a note in FG 28-1: It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A |
| Vivo | Fine with the proposals assuming we will continue discussing c) and d) |
| Huawei, HiSilicon | Generally OK with 2-3a while it seems also natural to include mapping sides is configurable, if later it is agreed. |
| CATT | Support |
| Samsung | We are fine to FL’s proposal. |
| NEC | We are fine with proposal 2-3a. |
| Panasonic | Support |
| CMCC | We are fine with the proposal. |
| MediaTek | Fine with the proposal |
| OPPO | We are fine with the proposal. |
| FUTUREWEI | OK with proposal |
| Intel | Support. |
| Xiaomi | OK with proposal |
| Qualcomm | Support |
| Nokia, NSB | Support |
| NTT DOCOMO | We support the proposal. |
| LGE | We are okay with the proposal. |
| Ericsson | We are fine with the proposal. |
| FL3 | All companies are fine with the proposal following proposal is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector asap. Low priority proposal 2-3a: - Component in FG 28-1 is updated as follows - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Add a note in FG 28-1: It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A |
| Moderator | Following was agreed via email endorsement Agreement - Component in FG 28-1 is updated as follows - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Add a note in FG 28-1: It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A |

[FL1] Medium priority question 2-4:

- Companies are encouraged to provide views on whether the type of FG 28-1 should be per UE or per band

  - Per UE: Ericsson, Huawei, HiSilicon, ZTE, Sanechips, Nokia, NSB, OPPO, Intel, Samsung, DOCOMO

  - Per band: vivo, Guangdong Genius, Apple, Qualcomm

| Company | Comment |
| --- | --- |
| vivo | Per band. The same argument as in previous meetings still valid As explained in our contribution R1-2111054, in TS 38.101 Table 5.3.5-1 many low frequency bands are defined with channel bandwidth of up to 20MHz, for example band n2/n5/n8, …. In these bands, since the channel bandwidth deployed by the NW is not larger than the maximum bandwidth supported by RedCap UEs, there is no need for a UE capable of up to 20MHz BW to report it is a RedCap UE, if the RedCap UE implements 2Rx and therefore can perform the same as non-RedCap UEs. |
| Qualcomm | We support “Per band” definition. Based on the conclusion of RAN#93 for the operating bands of R17 RedCap UE, FG 28-1 should be defined as “per band” to avoid the negative impacts on UE testing. |
| Panasonic | Per UE is our view. If it is per band, it is not so clear what is the inter-frequency measurement performance assumption on the number of antennas. |
| CATT | Per UE, as also the RAN2 WA. |
| MediaTek | Per UE, as agreed in RAN2 as a working assumption. |
| NTT DOCOMO | Our preference is per UE same as RAN2 WA. |
| Nokia, NSB | Per UE |
| ZTE, Sanechips | Per UE is preferred based on RAN2 WA. |
| NEC | Per UE. |
| CMCC | Per UE. Except for channel bandwidth, RedCap UEs may also have reduced capabilities for Rx, modulation order. So for bands with small channel bandwidth, when UEs reports RedCap capability, gNB can decide to schedule them accordingly. For example, if one RedCap UE has same Rx, modulation order as non-RedCap UE, only reduced channel bandwidth, then it is reasonable for gNB to schedule it same as non-RedCap UEs. |
| vivo | Regarding CMCC’s comment. Let’s take an example to illustrate the problem with per UE granularity. Let’s say we have two bands, band X with channel BW up to 20MHz, and band B with channel BW up to 40MHz. And there are two UEs - UE A supports 2Rx and 20MHz for band X (same as non-RedCap UEs for band X), 2Rx and 20MHz for band Y (lower capability than non-RedCap UEs for band Y which shall support 2Rx and 40MHz) - UE B supports 1Rx and 20MHz for band X (lower capability than non-RedCap for band X which shall support 2Rx and 20MHz), 1Rx and 20MHz for band Y (lower capability than non-RedCap UEs for band Y which shall support 2Rx and 40MHz) Based on per UE granularity, both UE A and UE B shall declare itself as RedCap UEs regardless of frequency band. For an NW operating on band X with 20MHz, it may intent to configure cell barring to deny UE B or configure early indication to identify UE B during initial access, however, UE A will also be rejected or identified as RedCap UEs unnecessarily. UE A has the same capability as non-RedCap UE in bandX, there is no reason to be treated differently. With per-band granularity, UE A can declare itself as non-RedCap UE on band X, thus no issue exists. Regarding Panasonic’s comment, UE A will report its capability as non-RedCap for band X, and RedCap for band Y, therefore inter-frequency measurement on band Y shall follow the RedCap performance requirement. |
| Xiaomi | We support “per band” definition |
| Ericsson | Per UE |
| Huawei, HiSilicon | Per UE. Although early identification is up to gNB, which may or may not be enabled, mimicking-nonRedCap UEs for RedCap UE is prohibited from the beginning of this SI/WI. Therefore, even for the bands/carriers with small spectrum, declaration of RedCap UE is necessary for gNB to know. One potential issue without doing that is for some requirements that a normal UE meets but RedCap may not be required, allowing access of RedCap UEs as a non-RedCap UE may cause performance impact. Additionally, if network can know a UE is a new RedCap UE, separate initial BWP can be used to benefit the network from offloading, when the total number of accessing UEs increases for small carrier. |
| Sierra Wireless | Per UE. For simplicity. |
| Intel | Per UE is sufficient. |
| Vivo | @Huawei, thanks for the comment. In the example we gave in the previous comment, UE A supports everything the same as non-Redcap UE does in band X, including the corresponding performance requirement, in this case we see no reason UE A has to be treated as RedCap UE on band X. A UE can declare itself as non-RedCap UE on a given band only when it can meet all the non-RedCap UE requirement, there is no intention to mimick nonRedCap UEs, but rather to avoid a non-RedCap capable UE be falsely treated as RedCap on a certain band due to the coarse capability reporting granularity, i.e. per UE. |
| FL2 | Summary of companies view - Per UE: Ericsson, Huawei, HiSilicon, ZTE, Sanechips, Nokia, NSB, OPPO, Intel, Samsung, DOCOMO, Pana, CATT, MTK, NEC, CMCC, Sierra Wireless - Per band: vivo, Apple, Qualcomm, Xiaomi, Given majority companies prefer per UE, following proposal is made together with question 2-5 [FL2] Medium priority proposal 2-4: - Type of FG 28-1 is per UE without xDD/FRx differentiation Companies supporting this proposal are invited to address the concern from vivo |
| vivo | We cannot accept the proposal. Clear technical reason has been shown by vivo and Qualcomm, but no real technical argument was seen about how to address the issues with per UE granularity. |
| Huawei, HiSilicon | Agree with FL. @Vivo, thanks for following up. I think RAN1 is intending to make some different performance requirements, for example related to the support of CSI-RS and the support of 6-1a may also be different implication. If in the end all requirements are the same it would be possible to treat a RedCap UE as non-RedCap UE, which I agree. On the other hand, it is still benefical if network knows it is RedCap UE to enable e.g. separate initial BWP such that there is offloading from coreset0, which is particularly useful for small carrier where all UEs are congested in coreset0 today. I’m not sure I understand there is an issue for per UE report – we assume that network can know all requirements/capability after connected mode and if all other performance is the same in the example you mentioned, then there is no performance difference afterwards regardless the network treat the UE as RedCap or else, i.e. a 20Mhz UE in 20Mhz carrier with same performance requirement as non-RedCap UE, then except for initial access where a RedCap UE can use separate initial BWP, it does not make different how the scheduler treat the UE as RedCap or not after CONNECTED.. |
| CATT | Support. In short, FGs for RedCap UE are different from normal UE. Other than BW and Rx, they do not support quite some FGs, including CA, DC, 256QAM, IAB and maybe more. A gNB should aware of these. |
| vivo | @Huawei/CATT, support of CSI-RS and 6-1a, and CA/DC/256QAM/IAB are nothing to do with FG29-1, they have the corresponding capability, lets focus on 29-1 for this discussion. The issue with per UE granularity, as explained before, mainly exists before RRC connection. A UE with full capability and meet all the requirement as a non-redcap UE does on a given band, would be barred unnecessarily, if cell barring is enable by the NW for RedCap UEs, or would be treated differently during initial access resulting worse initial access performance (e.g. delay). I tend to agree that after initial access, there is no much concern as all the related UE capability are clearly know by the NW. Another problem is the IODT testing availability, we doubt IODT testing will be available at the same time for all the band that a RedCap device may potentially support, per UE granularity will unnecessarily complicate the IODT testing and delay the time to market of the product. |
| Samsung | Support the proposal. @vivo, in that case, gNB can choose not to differentiate RedCap with 2Rx and non-RedCap for all the operation. We don’t see any problem for gNB learns about the information of UE capability. On the other hand, with this RedCap information, it might be helpful for handover or cell re-section, etc |
| NEC | We are fine with proposal 2-4. It is our understanding network should be able to identify RedCap UE for access control in any band. It would not prevent network to handle a RedCap UE as if a non-RedCap UE. |
| Panasonic | Support |
| Huawei, HiSilicon | @vivo Thanks for following up. Agree with SS, NEC and Pana. One more I can think about is even with the same 1Rx assumption the fact is that RedCap UE in wearable may have 3dB antenna efficiency loss, which affects initial access. |
| CMCC | Support the proposal. We share similar view as Huawei, CATT, Samsung, NEC. And for the barring problem, if gNB knows there are two kinds of RedCap UEs with different Rx numbers to be served in the networks and it doesn’t want those with 1Rx to access, can the following be a solution: gNB doesn’t bar the RedCap UEs from the beginning, but apply access control for 1Rx UE later? |
| MediaTek | We support proposal 2-4. @vivo, in your previous example where UE A has 2Rx and UE B has 1Rx, you mentioned: “For an NW operating on band X with 20MHz, it may intent to configure cell barring to deny UE B or configure early indication to identify UE B during initial access, however, UE A will also be rejected or identified as RedCap UEs unnecessarily.” As far as we know, RAN2 has designed IE/parameters for gNB to bar UE with 1Rx but not UE with 2Rx if it chooses to. Therefore, UE A will not be rejected by the cell even if it indicates to NW as a “RedCap UE.” |
| vivo | Thanks Samsung, NEC, Huawei, CMCC, MediaTek for the technical comments. We agree there are potential solution to alleviate the problem, however, in order to avoid impact these “full capability UEs”, network lose some means to handle other “real” RedCap UEs. For example, based on CMCC’s solution, NW has to disable cell barring and early indication mechanism. Based on Samsung and MediaTek’s solution, NW will not be able to bar other RedCap UEs with 2Rx. It seems no ideal solution. And again, per UE granularity also have the issue with IODT availability. |
| OPPO | Regarding the UE capability of RedCap, we originally think the essential capability like 20 MHz, 1RX, would not be much different among different band. To reduced capability fragmentation, we would like to make it per UE. We are open for discussion if the Per Band definition is needed at all. For the IDOT cases, is that per band requirement have to let UE full fill every band it to be as RedCap one? We are not sure there is the problem. Hope companies can clarify in further discussion. |
| Intel | Support the FL proposal. In our view, the potential benefit in serving a RedCap UE as a non-RedCap one comes at the cost of complicating the handling of such UEs when it comes to handovers and can complicate NW RRM unnecessarily. |
| Qualcomm | Agree with the comments of Vivo. We cannot accept this proposal. FG 28-1 definition should be “per band” instead of “per UE” |
| Nokia, NSB | Support the proposal |
| NTT DOCOMO | We support the proposal based on RAN2 WA. But we are open to discuss per band granularity if RedCap UE can operate properly in the reported band as RedCap UE. |
| LGE | We support the proposal. We share the view with previous companies in favor of “per UE” and in addition to that we would like to point out that the performance difference b/w 1Rx and 2Rx may not be clearly seen for some famous wearable devices, e.g., due to form factor limitations. |
| Ericsson | We support the proposal. It is clear from the WID that a RedCap UE should be explicitly identifiable to networks, so we do not think it is in the WI scope to define a RedCap UE that masquerades as a non-RedCap UE. |
| Sierra Wireless | We support the FL proposal. |
| FL3 | Companies not supporting the proposal are not convinced with the argument from supporting companies. This issue is discussed directly over the reflector. |
| Moderator | No conclusion is made in this meeting. |

[FL1] Medium priority question 2-5:

- Companies are encouraged to provide views whether xDD/FRx differentiation is necessary for FG 28-1

  - FDD/TDD differentiation:

    - Necessary:

    - Not necessary: Ericsson, Huawei, HiSilicon, vivo, Guangdong Genius, Nokia, NSB, Intel, Samsung, DOCOMO

  - FR1/FR2 differentiation:

    - Necessary: Nokia, NSB

    - Not necessary: Huawei, HiSilicon, vivo, Guangdong Genius, Nokia, NSB, Samsung, DOCOMO

    - FFS: Ericsson

| Company | Comment |
| --- | --- |
| vivo | Both of them are not necessary if FG28-1 is made per band. |
| Qualcomm | Agree with Vivo |
| Panasonic | Not necessary on both points in our view for the unification of UE features. |
| CATT | Not necessary for both. |
| MediaTek | Not necessary for both. To our understanding, a RedCap UE can indicate to NW the supported bands and band combinations using existing capabilities. |
| NTT DOCOMO | Both of them are not necessary. |
| Nokia, NSB | OK with no differentiation |
| ZTE, Sanechips | Both of them are not necessary. |
| NEC | Both FDD/TDD and FR1/FR2 differentiation are not necessary. |
| Vodafone | Fine with no differentiation |
| Xiaomi | Better to wait for the conclusion of question 2-4 |
| Ericsson | We are happy to go with the majority view that xDD/FRx differentiation is not necessary. |
| Sierra Wireless | No differentiation needed |
| Intel | Not necessary. |
| FL2 | This issue is discussed together with proposal 2-4 |

Low priority question 2-6:

- Companies are encouraged to provide views on whether/how to revise FG 6-1/6-1a/6-2/6-3/6-4, e.g.,

  - Component 4 in FG 6-1: BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell; for RedCap, BW of a UE-specific RRC configured BWP includes BW of CORESET#0 and SSB for Pcell, or alternatively a common CORESET (if configured) and NCD-SSB

  - Add a note in FG 6-1/6-1a/6-2/6-3/6-4: For RedCap UE, CORESET#0 here means CORESET#0 or CORESET of CSS

| Company | Comment |
| --- | --- |
| FL2 | This issue can be discussed after some progress on RedCap UE behavior in RRC-configured DL BWP without CD-SSB or NCD-SSB is made in AI8.6.1.1 |

Low priority question 2-7:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 28-1 which do not have capability hortsing impacts

| Company | Comment |
| --- | --- |

- 28-2: Number of UE Rx branches and DL MIMO layers for RedCap UE

In [1], FG 28-2 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the Gnb to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-2 | Number of UE Rx branches and DL MIMO layers for RedCap UE | 1. For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. 2. For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. | 28-1 | Yes |  | Impact on UE complexity | Per band | No | No |  | For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. Detailed signalling is up to RAN2. | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [3] | FUTUREWEI | FG 28-2 With the note in the agreement [2] Note: If RAN2 decides to reuse the existing hortsing (FG2-3) with modification for RedCap, then FG 28-2 is not needed from RAN1 perspective. If RAN2 decides to keep FG28-2, a RedCap UE must indicate FG28-2 from RAN1 perspective. No further discussion for FG 28-2 is expected except whether to remove the feature based on RAN2 decision. |
| --- | --- | --- |
| [4] | vivo, Guangdong Genius | For FG 28-2, as captured and clarified in the note that the “Detailed signalling is up to RAN2”, we can keep FG 28-2. Regarding to the type, RAN2 agreed following in RAN2#116bis-e meeting [4]: - RAN2 confirms that for RedCap Ues, “maxNumberMIMO-LayersPDSCH ” is still per FSPC although per band is enough. Therefore, the Type for FG 28-2 should be per FSPC. Proposal 6: The Type of FG28-2 should be per FSPC. |
| [6] | OPPO | UE have to report one of the supported maximum Rx branches. However, the signalling about 28-2 can be implicitly decided by number of MIMO layer. In the WI phase, we already conclude that the number of RX antenna for RedCap UE is implicitly indicated by the corresponding capability parameter of MIMO layer. The MIMO layer parameter is maxNumberMIMO-LayersPDSCH in FG2-3. In the current listed report of UE capability, FG28-2 is included with yellow mark. From our understanding, it seems to be clarified if it means setting in the hortsing bits. We think it would be clarified further in the Note of FG28-2 that the value of RX is implicitly determined by FG2-3 and FG28-1. Without that clarification, we consider the current list would means there are dedicated indication of capability about FG28-2 will be set. However, if the UE set number of RX is 2, but the FG2-3 indicate 1, what the capability will be for that UE. - Proposal 2: For Maximum Supported number of RX branches of RedCap UE, it is implicitly determined by FG2-3 and FG28-1. |
| [7] | CATT | For FG 28-2, since it is per band basis, we prefer not to merge it into 28-1. For FG 28-3, it is only for FDD band, which appears only in FR1, and should be optional for RedCap UE. Hence, we think the current form of FG 28-2 and FG 28-3 can be kept unchanged. Proposal 2: Keep the current form of FG 28-2 and FG 28-3. |
| [8] | Nokia, Nokia Shanghai Bell | - 28-2: - No need for separate indication. Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list. |
| [9] | NTT DOCOMO | - An reply LS to RAN2 [4] was agreed in RAN1#107-e to inform that “From RAN1 perspective, it would be enough to indicate the maximum number of PDSCH MIMO layers per band for RedCap Ues, but RAN1 notes that the type of FG2-3 (maxNumberMIMO-LayersPDSCH) is currently per FSPC and that it is up to RAN2 whether to signal per band or per FSPC”. - RAN2 made following agreement highlighted in yellow in RAN2#116bis-e that maxNumberMIMO-LayersPDSCH is still per FSPC for RedCap Ues [3]. Therefore, no further discussion is necessary for FG 28-2 Agreements via email – from offline 105: - ANR feature is optional for RedCap UE; - CHO related capabilities are applicable for RedCap Ues (understanding that CHO is already defined as an optional feature). “FFS on CHO” can be removed. - RAN2 confirms RAN1 agreements, i.e. introduce explicit bit to indicate the support of RedCap; To be captured in Mega CR; - RAN2 confirms RAN1 agreements, i.e. introduce capability bit on Half-duplex FDD operation type A for RedCap Ues; To be captured in Mega CR. - RAN2 confirms that for RedCap Ues, “maxNumberMIMO-LayersPDSCH ” is still per FSPC although per band is enough. - Clarify in the field description of horts and am-WithShortSN that, RedCap UE should always report “1” in TS 38.306 section 4.2.4 and 4.2.5. |
| [11] | Ericsson | Note that according to the above RAN4 agreements, for an FR2 RedCap UE there is no one-to-one mapping between the number of antenna elements and the number of DL MIMO layers. This means that the indication of the supported number of DL MIMO layers cannot be used to deduce whether the number of antenna elements is reduced or not. Assuming that it is desired that gNB knows whether the UE implements a reduced number of antenna elements or not, it may be necessary to specify a separate FG for reduced number of antenna elements. However, that may not be needed if it is decided that a RedCap UE has a reduced number of antenna elements if and only if uses the new power class. Then the gNB can deduce from the reported UE power class that the UE has a reduced number of antenna elements. Discuss how to report that an FR2 RedCap UE has a reduced number of antenna elements (e.g., using a new FG, or using implicit indication through the UE power class indication). In our understanding, it is likely that it will be desired to indicate support of different number of Rx branches for different bands. FG 28-2 (‘Number of UE Rx branches and DL MIMO layers for RedCap UE’’) is per band (or per FSPC if it is desired to align with legacy signaling solution). |
| [15] | Xiaomi | Considering the achieved agreement, there is no need for FG 28-2. And how to update the capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework could leave to RAN2 Proposal 3： - Remove FG 28-2 - Leave RAN2 to discuss how to update the capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework |
| [16] | Samsung | RAN 2 already made the following agreement on the number of Rx and MIMO layer. There is no need to keep a separate FG 28-2, and reuse existing FG 2-3 maxNumberMIMO-LayersPDSCH is enough. Moreover, RAN 2 confirmed that, maxNumberMIMO-LayersPDSCH is still per FSPC. Agreements via email – from offline 105: - ANR feature is optional for RedCap UE; - CHO related capabilities are applicable for RedCap Ues (understanding that CHO is already defined as an optional feature). “FFS on CHO” can be removed. - RAN2 confirms RAN1 agreements, i.e. introduce explicit bit to indicate the support of RedCap; To be captured in Mega CR; - RAN2 confirms RAN1 agreements, i.e. introduce capability bit on Half-duplex FDD operation type A for RedCap Ues; To be captured in Mega CR. - RAN2 confirms that for RedCap Ues, “maxNumberMIMO-LayersPDSCH ” is still per FSPC although per band is enough. - Clarify in the field description of horts and am-WithShortSN that, RedCap UE should always report “1” in TS 38.306 section 4.2.4 and 4.2.5. Proposal #4: Delete FG 28-2. |
| [18] | Qualcomm | Proposal 3: Support the following definition for FG 28-2 of R17 RedCap UE: |

Discussion

[GTW1] High priority proposal 3-1:

- FG 28-2 is removed

| Company | Comment |
| --- | --- |
| vivo | If 28-2 is removed. It should be clarified in existing FG “maxNumberMIMO-LayersPDSCH” that the UE Rx number equals to the maxNumberMIMO-LayersPDSCH. |
| FUTUREWEI | Our understanding is that the non-agreed or yellow FGs will be removed automatically as we finalize the list for RAN2 |
| Ericsson | For FR1, there is a one-to-one mapping between the (minimum) number of Rx branches and the supported number of DL MIMO layers, so it may not be necessary to signal both. However, for FR2, RAN4#101bis-e agreed to implement the objective as a reduced number of antenna elements and a new UE power class and also agreed that there is no one-to-one mapping between the number of antenna elements and the number of DL MIMO layers. This means that the indication of the supported number of DL MIMO layers cannot be used to deduce whether the number of antenna elements is reduced or not. Assuming that it is desired that gNB knows whether the UE implements a reduced number of antenna elements or not, it may be necessary to specify a separate FG for reduced number of antenna elements. However, that may not be needed if it is decided that a RedCap UE has a reduced number of antenna elements if and only if uses the new power class. Then the gNB can deduce from the reported UE power class that the UE has a reduced number of antenna elements. |
| Intel | Considering the update from RAN4, FG 28-2 may need to be kept at least for FR2. |
| FL1 | This proposal was discussed in GTW session on Feb 21 but no consensus was achieved. As commented by Ericsson in the GTW session, RAN4 made following agreement in RAN4#101bis-e Issue 5-1: New power class for RedCap UE [Agreement:] - For power class for FWA device - Reuse the PC5 power class - For power class for wearable UE and additional industry sensor use case - FFS whether to define the new power class for wearable UE and additional industry sensor separately, or define one power class for both - To limit the number of additional RF requirements, focus on the following three requirements - Minimum EIRP - Minimum EIS - Spherical coverage - For all the devices above, 2-layer DL MIMO is not mandated and FFS whether to define 2-layer MIMO performance requirements for them. Issue 5-2-1-1: Reducing the# of Rx branch [Agreement:] Not reduce the number of Rx branch for FR2 Issue 5-3-1-1: Reducing the# of Rx branch [Agreement:] Agree on Option 1. - Option 1: No Issue 5-3-1-2: For Min EIRP and array arrangement for wearable use case RedCap UE (fine tuning needed) [Agreement:] For Min EIRP and array arrangement for wearable use case RedCap UE, agree on - 20log(2) = 6 dB lower than FR2 PC3, reduce to half array size of PC3 with array arrangement of (4x1 single panel or 2x1 dual panel, dual pol),. --- Ericsson view from their contribution [11] Note that according to the above RAN4 agreements, for an FR2 RedCap UE there is no one-to-one mapping between the number of antenna elements and the number of DL MIMO layers. This means that the indication of the supported number of DL MIMO layers cannot be used to deduce whether the number of antenna elements is reduced or not. Assuming that it is desired that gNB knows whether the UE implements a reduced number of antenna elements or not, it may be necessary to specify a separate FG for reduced number of antenna elements. However, that may not be needed if it is decided that a RedCap UE has a reduced number of antenna elements if and only if uses the new power class. Then the gNB can deduce from the reported UE power class that the UE has a reduced number of antenna elements. Companies are invited to provide view whether we can remove FG 28-2 or define a separate FG for reduced number of antenna elements for FR2 taking the above into account. |
| Vivo | After further checking the RAN4 status, we see the points made by Ericsson is valid for FR2, therefore are fine with keeping 28-2 to indicate the Rx structure for FR2 redcap Ues. The detailed values should be defined by RAN4. Alternatively, we can leave the whole thing to RAN4 UE feature list. |
| Panasonic | As it is RAN4 expertise and our view is this is transparent to RAN1 specification, it should be discussed and defined as RAN4 related UE feature. |
| CATT | Fine to keep FG 28-2. Open to see if it is apply to FR2 only, or both FR1 and FR2. |
| MediaTek | Ericsson has made a point. We support to keep 28-2 at least for FR2. |
| NTT DOCOMO | We think Ericsson’s point is valid and are fine to keep FG28-2 for FR2 or leave to RAN4 to define corresponding FG. |
| Nokia, NSB | We tend to agree with Panasonic that it is then RAN4 who should define a corresponding FG or decide if the association with the power class is enough. |
| ZTE, Sanechips | For FR1, 28-2 is not needed. For FR2, we are OK to leave to RAN4. |
| NEC | We are fine to leave this to RAN4 as need of FG 28-2 would depend on RAN4 discussion on FR2. |
| Vodafone | Fine to keep FG 28-2 |
| Xiaomi | Considering the Ericsson’s comment and RAN4’s agreement, we are OK to keep FG28-2 |
| Ericsson | It would be good to keep FG 28-2 in this draft feature list for a while longer until it is clear whether a corresponding RAN4 FG will be introduced, which will hopefully be clear later in this meeting. With some luck, perhaps we can then remove FG 28-2 without problem, otherwise we’ll need to update the fields “Feature group”, “Components” and “Note” to reflect the situation. |
| FUTUREWEI | The RAN4 decision to not reduce the number of RX branches for FR2 should be informed directly to RAN2 by RAN4, and then RAN2/RAN4 will handle it. There is no point to keep 28-2 as some sort of “reminder”. Based on RAN4’s input, RAN2 can then modify their running CR (1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported for FR1. For FR2 the number of RX branches is 2 and 1 or 2 DL MIMO layers are supported.) and any other modification (if needed, seems not), and RAN4 can propose themselves any new FG they need. We have enough to do in RAN1 without trying to do the other WG’s work for them. |
| Qualcomm | We share the same view as Ericsson |
| Intel | We can accept keeping FG 28-2 for now. |
| FL2 | Given many companies are fine to keep FG 28-2 for now considering the RAN4 agreement. Some companies think it should be discussed/defined in RAN4. Therefore, moderator suggest following proposal and to conclude RAN1 discussion [FL2] High priority proposal 3-1a: - Add a note in FG 28-2: It is up to RAN4 whether/how to report the number of UE Rx branches for FR2 |
| vivo | Is it the correct understanding that FG28-2 will be updated to FR2 only? |
| CATT | OK. We should also make it clear that, current component 1 and 2 in FG 28-2 are applied to FR1 only. |
| Samsung | We suggest to delete FG28-2 for FR1. And leave it up to RAN 4 for FR 2. |
| Panasonic | Support |
| MediaTek | In principle, we agree it can be left to RAN4. If it is up to RAN4, do we still need to keep FG28-2 in RAN1’s list? The entire row will be yellow and RAN2/RAN4 may or may not look into its details. We think it is better to clearly state RAN1’s agreements/conclusions to RAN2/RAN4 in the LS rather than in (the note) of FG28-2. |
| OPPO | We agree this can leave to RAN4, then we should remove or put the whole 28-2 as yellow or remove it temporarily |
| FUTUREWEI | For 28-2, it should be removed and the issue left to RAN4 |
| Xiaomi | Generally, we are OK with the intention of the FL’s proposal. Since RAN4 agreed the cost reduction on the Rx is achieved by antenna element reduction rather than the Rx branch reduction, then we suggest the following update for FL’s proposal High priority proposal 3-1a: - Add a note in FG 28-2: It is up to RAN4 whether/how to report the number of UE Rx branches Rx structure for FR2 |
| Qualcomm | Support the FL2 proposal |
| Nokia, NSB | Support |
| NTT DOCOMO | We support the proposal. |
| LGE | We support the proposal. |
| Ericsson | To our understanding, RAN4 will prepare an LS to RAN2 to communicate the RAN4 agreements regarding the FR2 case (involving reduced number of antenna elements and new power class rather than reduced number of Rx branches), so it may be safe to remove FG 28-2. |
| FL3 | As suggested by some companies, previous proposal was unclear whether to update/keep FG 28-2. Therefore, the proposal is updated to delete FG 28-2 and to leave RAN4 for any RAN4 specific issues. This issue is discussed directly over the reflector [FL3] High priority proposal 3-1a: - Delete FG 28-2 - (As conclusion) It is up to RAN4 whether/how to report number of UE Rx branches UE Rx structure for FR2 |
| Moderator | Following was agreed via email endorsement Agreement - Delete FG 28-2 - (As conclusion) It is up to RAN4 whether/how to report UE Rx structure for FR2 |

- 28-3: Half-duplex FDD operation for RedCap UE

In [1], FG 28-3 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | UE is assumed to support FD-FDD in FDD bands | Per band | FDD only | FR1 only |  |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [4] | vivo, Guangdong Genius | For 28-3 of Half-duplex FDD operation type A for RedCap UE, the type should be per band, as the implementation could be different across wide range of different FDD bands. Proposal 7: The Type of FG28-3 should be per band. |
| --- | --- | --- |
| [5] | ZTE, Sanechips | Obviously, HD-FDD is only supported in FDD bands. Moreover, as we know, FR2 bands do not support FDD operation. Therefore, FG 28-3 is only used in FR1. Regarding the feature type, if it is set as per band, it means the RedCap UE may not support HD-FDD in some FDD bands in FR1. From our understanding, if 28-1 is Per UE, the FDD RedCap UE have the ability to support all the FDD bands, we do not see the necessity to set the HD-FDD as per band. Therefore, the following is proposed. Proposal 4: FG 28-3, HD-FDD should be ‘Per UE’ reported. - FDD/TDD differentiation can be set as ‘FDD only’ - FR1/FR2 differentiation can be set as ‘FR1 only’ |
| [6] | OPPO | - In the updated UE capability, this capability FG28-3is per band reported. It is allowed to let device to be a RedCap HD-FDD UE in one band, but not in another band. - In the note of FG28-3, there could be further clarification that the values should be “HD” or “FD”. Basically, this should be binary. Proposal 3: For FG 28-3, clarify it is for Type A HD-FDD and values should be “HD/FD” in the note. |
| [7] | CATT | For FG 28-2, since it is per band basis, we prefer not to merge it into 28-1. For FG 28-3, it is only for FDD band, which appears only in FR1, and should be optional for RedCap UE. Hence, we think the current form of FG 28-2 and FG 28-3 can be kept unchanged. Proposal 2: Keep the current form of FG 28-2 and FG 28-3. |
| [8] | Nokia, Nokia Shanghai Bell | - 28-3: - Per UE - FR1/2 differentiation - No need for xDD differentiation Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list. |
| [9] | NTT DOCOMO | - Report type of FG 28-3 should be per band for more flexibility to indicate the support of either HD-FDD or FD-FDD for different FDD bands. - FG 28-3 is applicable only to FR1 FDD bands |
| [11] | Ericsson | In our understanding, it might be desired to indicate support of different duplex modes (HD-FDD/FD-FDD) for different FDD bands. For FG 28-3 (‘Half-duplex FDD operation for RedCap UE’), discuss whether the capability should be per UE or per band. |
| [12] | Intel Corporation | Proposal 1: - Confirm the type for FG 28-1 as “per UE”. - Confirm the type for FG 28-3 as “per band”. Next, on xDD and FRx differentiations, while FG #28-3 is applicable to FDD spectra and FR1 bands only, there is no need for FDD/TDD differentiation for FG #28-1 (for similar reasons why “per band” signaling is not necessary for FG #28-1). Neither is FRx differentiation necessary for FG 28-1. Proposal 3: - xDD and FRx differentiations are not necessary for FGs 28-1 and 28-3. - FG 28-3 is only applicable in FDD and FR1 bands. |
| [15] | Xiaomi | The main motivation of HD-FDD is to further reduce the RedCap’s cost. If removing the duplexer only in some bands and keeping the duplexer in the other bands, the overall cost reduction is marginal. In addition, it also requires the RedCap devices maintain two sets of protocol for HD-FDD and FD-FDD, respectively, which goes against the original intention of supporting HD-FDD. From this point the “Type” column should be “per UE” rather than “per band”. Proposal 4: Update FG 28-3 as follows - Change the “Type “from “per band” to “per UE” |
| [16] | Samsung | From implementation point of view, half-duplex FDD operation is determinated based on hardware. It is not a band related feature. Therefore, they type of feature 28-3 should be Per UE. Besides, this feature is FDD specific features and can be applied for FR1 only. # Proposal #5: Feature 28-3 is defined per UE other than per band, and confirm FG28-3 is “FDD only” and “FR 1 only” |
| [18] | Qualcomm | Proposal 4: Support the following definition for FG 28-3 of R17 RedCap UE: |

Discussion

[FL1] Medium priority question 4-1:

- Companies are encouraged to provide views on whether the type of FG 28-3 should be per band or per UE

  - Per band: Ericsson, vivo, Guangdong Genius, Spreadtrum, DOCOMO, Qualcomm

  - Per UE: Huawei, HiSilicon, Nokia, NSB, OPPO, Xiaomi, Samsung

    - FDD/TDD differentiation:

      - Necessary:

      - Not necessary: Huawei, HiSilicon, Nokia, NSB, Samsung (FDD only)

    - FR1/FR2 differentiation:

      - Necessary: Nokia, NSB

      - Not necessary: Huawei, HiSilicon, Samsung (FR1 only)

| Company | Comment |
| --- | --- |
| vivo | Per band thus no need for FDD/TDD or FR1/FR2 differentiation. Actually not sure why TDD and FR2 are relevant, since this is a feature about FDD bands. |
| Qualcomm | Agree with Vivo |
| Panasonic | Per band is our view. |
| CATT | Not sure ‘per band’ is really the valid implementation, i.e. a RedCap UE implements both duplexer and switch? ‘Per UE’ is more reasonable. |
| MediaTek | This should be “per band” as it is related to RF capability. |
| NTT DOCOMO | It should be per band to indicate the support of either HD-FDD or FD-FDD for different FDD bands flexibly. |
| Nokia, NSB | We have a preference for per UE indication, but we are open to per band indication in this case if needed. |
| ZTE, Sanechips | Per UE is more preferred. We are not sure whether a HD-FDD UE can operate as FD-FDD UE in some bands. |
| Xiaomi | Our preference is “per UE” If removing the duplexer only in some bands and keeping the duplexer in the other bands, the overall cost reduction is marginal. In addition, it also requires the RedCap devices maintain two sets of protocol for HD-FDD and FD-FDD, respectively, which goes against the original intention of supporting HD-FDD |
| Ericsson | The proposal in our contribution was just to discuss whether FG 28-3 should be per band or per UE, therefore Ericsson has been removed from the list of company names above. If some clear benefits are identified with “per band” then we are willing to consider that, otherwise we prefer to go with “per UE” for simplicity. |
| Huawei, HiSilicon | In the study phase it is clear that the complexity/cost is reduced only if the duplexer is removed. And it is understood that HD-FDD can negatively impact network performance/efficiency or increase the scheduling complexity due to collision handling. We fail to see benefits/motivation for per-band reporting on gNB/network, UE or signaling overhead. |
| Sierra Wireless | Per Ue, for simplicity. |
| Intel | Per band provides more flexibility to the UE, but can also accept per UE. FRx/xDD differentiation not needed. |
| vivo | To answer the question above on the necessity of per band granularity. A RedCap UE may support both low-band FDD (e.g. 700MHz) and mid-band FDD (e.g. 2.3GHz), the RF chain for low-band and mid-band could be separated according to the current UE implementation. Therefore, UE may choose to support HD-FDD in the low band but FD-FDD in the mid-band, or vice versa, since the complexity/cost due to the duplexer could be different for low and mid bands. Per-band granularity can provide better flexibility for implementation. |
| FL2 | Summary of companies view - Per band: vivo, Spreadtrum, DOCOMO, Qualcomm, Pana, MTK, [E///], [Nokia, NSB], Intel - Per UE (for FR1 FDD only): Huawei, HiSilicon, Nokia, NSB, OPPO, Xiaomi, Samsung, E///, CATT, ZTE, Xiaomi, Sierra Wireless, [Intel] Given more companies prefer per UE, following proposal is made. Companies are invited to check whether you can live with other option taking the above comments into account [FL2] Medium priority proposal 4-1: - Type of FG 28-3 is per UE - The column of “Need of FDD/TDD differentiation” is confirmed as “FDD only” - The column of “Need of FR1/FR2 differentiation” is confirmed as “FR1 only” |
| vivo | No. We think there are sufficient reasons provided and good support for per band, and no clear objections. |
| Huawei, HiSilicon | Agree with FL. @vivo, thanks for sharing more views. We agree that the RF chain for low-band and mid-band could be separated according to the current UE implementation. However, it is our understanding that the cost ratio is not much different (or meaningfully significant) in different bands within the same FR according to the cost estimate in study phase. From use case perspective it is also natural that a wearable device is a wearable device in all bands. |
| CATT | Support. We also recall that, one important reason to support HD-FDD when SI is converted to WI, is that the duplexer can be shared/reused among different bands, hence the cost saving can be increased from ‘negligible’ to ‘considerable’. Another reason is that, implementing both HD-FDD and FD-FDD seems very complicated. The UE will have to implement both collision handling rules for HD-FDD and FD-FDD, which are quite different. |
| vivo | Respond to Huawei/CATT, We do not see there was conclusion that duplexer can be shared among low and mid-bands, on the contrary, the following sentence in the HD-FDD section of the TR indicates that the cost saving are accumulative across bands, which is because different hardware components are used to support different bands (at least when the bands have large frequency separation) Furthermore, all sourcing companies indicated that the RF cost savings (but not the baseband cost savings) accumulate across supported bands. |
| Samsung | Support |
| Huawei, HiSilicon | @vivo Thanks for further promotion. Yet again as agreeing with you, we said RF can be different across bands thus similar it is understood for the duplexer. What we pointed out is the gain is no much difference across different bands, i.e., ratio is the same/similar for a given band within the same FR (but different FR has different ratios, with a same saving for any given band as all companies agreed in the BB:RF split ratio), which holds different view from you previously commented: “complexity/cost due to the duplexer could be different for low and mid band”. Indeed it is true that the cost would be accumulative if the UE chose to implement more and more bands, hence, it seems to be more consistent for a UE to remove the duplexer for all bands in order to maximize the cost saving gain. We don’t see the target use case/services for a UE tends to save its cost on one band but not on another band. |
| vivo | Thanks Huawei for the technical discussion. We might hold different views on the cost saving aspect, that’s is fine. But there could be other reasons why UE vendor decides to implement HD-FDD on band A but not on band B, for example, market/operators on band B does not accept HD-FDD UEs due to the service requirement or any other reason, while operator running band A may accept. Then UE vendor has to consider such implementation to save the cost to the maximum extend. Furthermore, the LTE HD-FDD feature was also signaled per band, please refer to the following from 36.331. SupportedBandEUTRA ::= SEQUENCE { bandEUTRA FreqBandIndicator, halfDuplex BOOLEAN } halfDuplex If halfDuplex is set to true, only half duplex operation is supported for the band, otherwise full duplex operation is supported.; -. |
| OPPO | We support the proposal. For the per band difference their might be some difference between bands. But we see the closing band will have very similar requirement regarding the ul/dl swiching, emission and so on. Then, same duplex will be assumed for them. Every for much different bands, using one duplex in one and another duplex in another would be unlikely. |
| Intel | We can accept the proposal from FL. |
| Xiaomi | We support FL’s proposal We are not convinced by the motivation to support HD-FDD per band. While, on the hand, we are concerned about the complexity of supporting both FD-FDD and HD-FDD. |
| Qualcomm | We don’t accept “per UE” definition for FG 28-3. We support “per band” |
| Nokia, NSB | Support |
| NTT DOCOMO | Our preference for the type of FG 28-3 is per band. If a RedCap UE supports both HD and FD operation, then it should be allowed to operate as FD. If this FG is defined as per UE, such operation would not be achieved. |
| LGE | We support the FL’s proposal. |
| Sierra Wireless | Support |
| FL3 | Summary of companies view - Per band: vivo, Spreadtrum, DOCOMO, Qualcomm, Pana, MTK, [E///], - Per UE (for FR1 FDD only): Huawei, HiSilicon, Nokia, NSB, OPPO, Xiaomi, Samsung, E///, CATT, ZTE, Xiaomi, Sierra Wireless, Intel Thank you Intel and Nokia/NSB for your flexibility. Some companies not supporting the proposal are not convinced with the argument from supporting companies. This issue is discussed directly over the reflector. |
| Moderator | No conclusion is made in this meeting. |

[FL2] Low priority question 4-2:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 28-3 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We tend to consider ‘Type A’ is not needed as there is no Type B HD in NR and the current Type A is already different from that in LTE. Having this term does not seems to help any differentiation but rather cause confusion, especially for NR standalone which is targeted for RedCap where there is no LTE at all. |
| CATT | We think Huawei’s has the point. Even in current Rel-17 38.213 spec, when capturing HD-FDD rules for RedCap UE, the editor does not use the terminology like ‘Type A half-duplex’, but just say ‘HD-UE’. |
| Ericsson | We are fine with removing the mentioning of ‘Type A’. |

- Other FGs

This section discusses other FGs which are not included in [1].

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [2] | Huawei, HiSilicon | RAN1 introduces NCD-SSB for serving cell purpose. Although NCD-SSB is introduced for performing RRM/RLM/BFD measurement in an active BWP, RAN4 has reached conclusion that it is feasible for a RedCap UE to perform L1/L3 measurement based on CSI-RS and/or FG 6-1a by reporting optional capabilities, if the RRC-configured active DL BWP has no SSB in connected mode [1]. The previous RAN1 agreements state that there is an operation without need of NCD-SSB based on support of CSI-RS and/or FG 6-1a. Since the basic FG 6-1 also addresses CORESET#0, and it is not preferable to use an existing FG with different interpretation, for RedCap UEs. Also, IODT is an aspect that needs to be considered, therefore a signaling bit might be desirable. We suggest to introduce a new FG as mandatory for RedCap UE to report whether it can expect NCD-SSB in the active BWP, e.g. to perform RRM/RLM/beam measurement of the serving cell. Also, it is required for indication of support if a UE does not report either CSI-RS based measurements nor FG 6-1a, as per the RAN1 agreements. Proposal 1: For RedCap, define a new FG to report whether a UE supports NCD-SSB based measurements in the active BWP or not, if neither FG 6-1a nor CSI-RS based measurement is supported. |
| --- | --- | --- |
| [3] | FUTUREWEI | The key point is that the agreement in RAN1#105 states that FG 6-1 is used as a starting point, and it is not precluded to make changes to the components. Although the RAN1#107 agreements state that FG 6-1 is mandatory, several companies suggested that a modification of FG 6-1 may be needed for RedCap UEs. For example, a RedCap UE supporting FG 6-1 needs either (a) a NCD-SSB if the RRC-configured active DL BWP does not include CORESET#0/SIB and CD-SSB; or (b) CD-SSB and CORESET#0/SIB in the RRC-configured active DL BWP. For (a), the network should provide appropriate search spaces in order to receive SIB or use dedicated signaling. We are open to consider modifying FG 28-1 with appropriate components if modification to the existing FG 6-1 for RedCap UEs is not trivial. For FG 6-1a, based on the FG description and RAN1#107 agreement, the network does not have to provide any SSB. However, the UE cannot rely on the CSI-RS for RRM measurements according to the RAN4 LS [11]. We are open to consider modifications to FG 6-1a or even adding an FG for RedCap UE for non-trivial modifications. From the WID [9], the following capabilities are not applicable for RedCap UEs: - Carrier aggregation - Dual connectivity - UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 - More than 2 UE Rx branches or more than 2 DL MIMO layers RAN#93-e had the following agreements [10]: - In Rel-17, there will be no work on any RedCap specific specification update for any of the following: - RedCap UEs also supporting V2X/PC5 on n47 - RedCap UEs operating in unlicensed bands - RedCap UEs supporting SUL - The specification will not contain any explicit restriction to prevent implementation of RedCap UEs with these features. - Note: The consequence of this agreement would be: - If any spec change/addition is found necessary in order to enable one of the options above, then it will not happen in Rel-17. The RAN2 agreements in [4] indicate that the following capabilities are not applicable for RedCap UEs: - More than [4 or 8] DRBs - NE-DC and (NG)EN-DC - DAPS and CPC related capabilities - IAB related capabilities RAN2 may or may not reply to the LS sent in RAN#107 [6]. If RAN2 needs RAN1’s views of non-supported Rel-15/16 features for RedCap UE, there are contributions such as [12] and discussions in [5] that can be used as starting points to generate lists of not-supported features or features needing possible modification. The guidelines listed in Section 2.2 are also applicable for Rel-17 features. Although these features are still being discussed in the meetings, a general assessment of the applicability of the features is presented in Table 2. In RAN1#107, companies expressed views regarding these feature groups. A summary of the views in [5] is also presented. Table 2. List of Rel-17 features. The column “# FG” is the number of features for the work item. Prefix; Name; # FGs; Comment; Observations from FL summary [5]. 23; Further enhanced MIMO; 53; MIMO features with more than 2 Rx branches or with than 2 Tx branches should not supported (agreement); One company suggested no support of MIMO features with more than 2 Rx branches or with more than 2 Tx branches. 24; Supporting NR from 52.6GHz to 71 GHz; 22; RedCap UE supports a maximum of 100 MHz. The minimum bandwidth for B52 is 100 MHz (120 kHz SCS). If RedCap UE supports B52, then some features may be not applicable due to BW considerations or from RAN1 agreements - 24-1c (Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2) - 24-4 (480 kHz SCS support) - 24-5 (960 kHz SCS support) - 24-8 / 24-9 (support 32 HARQ processes for 480 / 960 kHz SCS) - 24-10 (Additional beam switching time delay); One company suggested no support. 25; Enhanced Industrial Internet of Things (IoT) and URLLC; 22; FG 25-18 (Parallel PUCCH and PUSCH transmission across CCs in inter-band CA) cannot be supported due to WID [9]; One company seemed to suggest no support. 26; Solutions for NR to support non-terrestrial networks (NTN); 9; Two companies seemed to suggest no support. One company wanted optional. 27; NR Positioning Enhancements; 36; Three companies seemed to suggest no support. Two companies wanted optional. 29; UE Power Saving Enhancements; 6. 30; coverage enhancement; 17. 31; Enhancements to Integrated Access and Backhaul; 9; RAN2 agreement that RedCap devices do not support IAB can apply [4]; Six companies seemed to suggest no support. 32; Sidelink enhancement; 6; RAN decision that SL features and RedCap devices can apply [10]; One company seemed to suggest no support. 33; NR Multicast and Broadcast Services; 21. 34; NR Dynamic spectrum sharing (DSS); 2; Do not support FGs that indicate cross-carrier scheduling due to WID objective of no CA [9]; Four companies seemed to suggest no support. 35; LTE_NR_DC_enh2; 1; Do not support FGs that indicate dual connectivity due to WID objective of no DC [9]; Four companies seemed to suggest no support. 36; DL 1024QAM for NR FR1; 2; Prerequisite that a UE supports 256QAM; Six companies seemed to suggest no support. As a baseline for discussing which Rel-17 features are not supported by RedCap UEs, we should apply the same agreements for discussions about Rel-15 and Rel-16 features. Proposal: apply the following agreements for evaluating Rel-15 and Rel-16 features when considering Rel-17 features - A RedCap UE does not support capabilities related to more than 2 Tx branches or more than 2 Rx branches (from RAN1) - NE-DC and (NG)EN-DC capabilities are not applicable for RedCap UEs (from RAN2) - IAB related capabilities are not applicable for RedCap UEs (from RAN2) |
| [4] | vivo, Guangdong Genius | For confirming the WA that a RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities, based on the reply LS [3] from RAN4, see following, it is observed that for a RedCap UE operation without SSB in a UE-specific BWP, CSI-RS cannot be used as a standalone method and FG6-1a is the precondition. Proposal 2: An optional, new FG with FG6-1a as prerequisite can be introduced for the operation without SSB in a UE-specific BWP for RedCap UEs when the WA on supporting relevant operation based on for CSI-RS is confirmed. For a separate initial UL BWP, it should include the configuration(s) needed for RedCap UE to perform random access, e.g., the RedCap-specific PRACH configuration, Msg.3 and PUCCH for Msg.4 acknowledgment etc. For a separate initial DL BWP if configured, it should at least include the CSS/CORESET for random access, and it is configurable to include the SSB, CORESET/CSS for paging, CORESET/CSS for SIB1/OSI and MIB-configured CORESET#0. Besides, according to the agreements, a separate initial UL and DL BWP can be used during the initial access before UE indicating its capability, further discussion is needed on whether the two FGs should be optional without capability or optional with capability. Proposal 4: Introduce following new FGs related to the separate initial UL BWP and the separate initial DL BWP for RedCap UEs: - FG 28-y: A separate initial UL BWP - It includes the configuration(s) needed for RedCap UE to perform random access; - FG 28-z: A separate initial DL BWP - It includes CSS/CORESET for random access. Proposal 5: For the two new FGs of the separate initial UL and DL BWP, further discussion is needed on whether the two FGs are optional without capability signaling or optional with capability signaling. |
| [5] | ZTE, Sanechips | Given the Rel-17 RedCap WID description, the maximum UL peak data rate for werables is 50Mbps. Obviously, support of more than 1Tx is not necessary, since the peak data rate can be up to 90Mbps for 64QAM and 1 layer with SCS=15KHz according to the calculation. More Tx antennas bring more complexity and cost, which is not aligned with the motivation of the WI. Moreover, for 1Rx RedCap UE, supporting more than 1 UL port seems to be impossible due to the physical circuit. More specifically, the FGs exceeding 1 Rx UE capability includes 2-3, 2-13, 2-14, 2-16b. Therefore, we have the following proposal: Proposal 1: UE capabilities exceeding 1Tx capability is not supported. |
| [8] | Nokia, Nokia Shanghai Bell | One controversial discussion in RAN1#107-e was whether or not RedCap UEs could possibly support 2TX or not. This aspect is not clearly stated in the WID, which focuses on RX aspects. However, since 2RX UEs are optionally supported by RedCap UEs, one can infer that 2-port uplink is in principle possible as well. Proposal: Support of 2-port uplink is optional for RedCap UEs. |
| [9] | NTT DOCOMO | It was discussed in the last RAN1 meeting [5] whether there are any Rel-15/16 UE features which should not be applicable for RedCap UEs other than the identified capabilities, i.e., - Carrier aggregation - Dual connectivity - UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 - More than 2 UE Rx branches or more than 2 DL MIMO layers - More than [4 or 8] DRBs - NE-DC and (NG)EN-DC - DAPS and CPAC related capabilities - IAB related capabilities and following agreements were made: Agreement: The following Rel-15/16 capabilities (FGs) for L1 UE features in TR 38.822 V16.1.0 are related to more than 2 UE Rx branches or more than 2 DL MIMO layers and should therefore not be applicable to RedCap UEs. - 4-12 - 16-3a-3 - 16-3b-2 Agreement: - A RedCap UE does not support capabilities related to more than 2 UE Tx branches or more than 2 UL MIMO layers. Agreement: - RAN1 does not provide a complete list of Rel-15/16 capabilities (FGs) for L1 UE features in TR 38.822 V16.1.0 that should not be applicable to RedCap UEs because they are related to IAB. Agreement: Capture the following earlier RAN1 agreements regarding RF/RRM FGs 1-4 and 1-5 in the LS reply to RAN2: - RedCap UE supports FG1-4 (256QAM for PDSCH) as optional with capability signalling both for FR1 and FR2 - Add a note in FG 1-4 (256QAM for PDSCH) that “For RedCap UEs, the 256QAM MCS table for PDSCH and CQI table 2 are only supported if the UE supports 256QAM for PDSCH” - Add a note in FG 1-5 (256QAM for PUSCH) that “For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH”. Also, it was discussed in RAN#93-e meeting whether or not RedCap UEs can support V2X, operate in unlicensed bands, and support SUL. It was concluded that there will be no work on any RedCap specific specification update for these features but the specification will not contain any explicit restriction to prevent implementation of RedCap UEs with these features [6]. Therefore, V2X, NR-U, and SUL related L1 UE features are applicable for RedCap UEs without any changes. For mandatory features for non-RedCap UEs that should not be applicable for RedCap UEs, following FGs were discussed but no consensus was achieved. - 2-16b: Support 1+2 DMRS (uplink) - This FG is applicable for the RedCap UE supporting more than one port uplink. Therefore, this FG can be supported by RedCap UEs as optional with capability signaling - 2-55: SRS Tx switch - Components 2 and 3 are not applicable for RedCap UEs For mandatory features for non-RedCap UEs that should be optional for RedCap UEs, following FGs were discussed but no consensus was achieved. - 1-7: CSI-RS based RLM - We don’t see any reason for optional support of this FG. Keep mandatory support as legacy. - 2-4a: Additional active TCI state for PDCCH - We don’t see any reason for optional support of this FG. Keep mandatory support as legacy. - 2-61: Additional active spatial relation for PUCCH - We don’t see any reason for optional support of this FG. Keep mandatory support as legacy. For mandatory features for non-RedCap UEs that should be supported for RedCap UEs but with different value, following FGs were discussed but no consensus was achieved. - 2-33: CSI-RS and CSI-IM reception for CSI feedback - Components 4 and 6 are not applicable for RedCap UEs - 2-35: CSI report framework - Component 9 is not applicable for RedCap UEs - 2-51: TRS (CSI-RS for tracking) - Component 4 is not applicable for RedCap Ues - 6-1: Basic BWP operation with restriction - We don’t see any reason for supporting different value. Keep mandatory support as legacy. For optional features for non-RedCap UE that should not be applicable for RedCap UEs, following FGs were discussed but no consensus was achieved. - 2-13: Codebook based PUSCH MIMO transmission - This FG is applicable for the RedCap UE supporting more than one port uplink. - 2-14: non-codebook based PUSCH transmission - This FG is applicable for the RedCap UE supporting more than one port uplink. - 4-27: More than one group of overlapping channels for control multiplexing - This FG should not be applicable for RedCap Ues - 5-5b: UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS - Component 2 does not have dedicated capability bit, and some clarification note would be enough for RedCap UEs not to support max BW of 136 PRBs - 10-20: Support search space set configuration with freqMonitorLocation-r16 - This FG is not applicable for RedCap UEs since only one RB set is defined for RedCap UEs (maximum 20 MHz BW for FR1 unlicensed bands) - 10-20a: Support coreset configuration with rb-Offset - We don’t see any reason for not supporting this FG. Keep optional support as legacy. - 10-29: Support available RB set indicator field in DCI 2_0 - This FG is not applicable for RedCap UEs since only one RB set is defined for RedCap UEs (maximum 20 MHz BW for FR1 unlicensed bands) - 15-18: Support of rank 2 transmission - This FG is applicable for the RedCap UE supporting more than one port Tx. - 15-19: Support of rank 2 reception - This FG is applicable for the RedCap UE supporting more than one port Rx. For optional features for non-RedCap UE that should be mandatorily supported for RedCap UEs, following FGs were discussed but no consensus was achieved. - 5-17a: PDSCH repetitions over multiple slots - Keep optional support as legacy - 6-1a: BWP operation without restriction on BW of BWP(s) - As mentioned in Section 2, we are fine with either defining a new FG for active DL BWP without CD-SSB or NCD-SSB for RedCap UEs or reusing existing FG 6-1a as optional capability |
| [10] | NEC | Regarding FG 6-1, basic BWP operation for RedCap UE supports a case where BW of a UE-specific RRC configured BWP does not include BW of CORESET#0 and SSB for PCell/PSCell (i.e. CD-SSB) but includes NCD-SSB and a common CORESET (if configured). To accommodate the case for RedCap UE, FG 6-1 would need updates. Proposal: - FG 6-1 is proposed to be updated as follows: 6-1; Basic BWP operation with restriction; 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; for RedCap, BW of a UE-specific RRC configured BWP includes BW of CORESET#0 and SSB for PCell, or alternatively a common CORESET (if configured) and NCD-SSB. 6-1a; BWP operation without restriction on BW of BWP(s); BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell. |
| [11] | Ericsson | - Proposal 1 Discuss how to capture the RAN1 agreements regarding operation in an RRC-configured active DL BWP in connected mode (e.g., in the definitions of FGs 6-1 and 6-1a, or in the new FGs 28-x). |
| [12] | Intel Corporation | On support of 2 Tx capabilities, it was discussed during RAN1 #107-e meeting without a conclusion as to whether RedCap UEs may support 2Tx as optional feature. To maintain some of the key targets for RedCap UEs other than complexity (viz., small form factor, low power consumption, lower QoS requirements), it may not be practical to have support of 2Tx features. Thus, precluding support of 2Tx for Rel-17 RedCap is slightly preferred. In addition to the above, several other UE features from Rel-15/16 are expected to need adjustments for RedCap UEs as compared to non-RedCap UEs. Most of these have already been resolved in earlier meetings with FG 6-1 being one key FG awaiting further resolution. - FG #6-1 - For RedCap UEs, operation without CORESET #0 within the active DL BWP should be mandatory and only expectation on SSB in RRC-configured DL BWP may be considered as a modification to FG 6-1. This would require updates to at least FG #6-1. - Alternatively, a new mandatory FG for RedCap UEs such that the RedCap UE expects SSB in RRC-configured DL BWP should be introduced with FGs 6-1 and 28-1 as pre-requisites. Proposal 4: - FG # 6-1 should be adapted for RedCap UEs such that operation without CORESET #0 within the active DL BWP is mandated for RedCap UEs. - Add a note to FG 6-1: “For a UE supporting FG 28-1, component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)”” - Alternatively, a new mandatory FG for RedCap UEs such that the RedCap UE expects SSB in RRC-configured DL BWP should be introduced with FGs 6-1 and 28-1 as pre-requisites. Proposal 5: - Applicability of some of the Rel-17 features for RedCap UEs are as follows: - UE power saving enh - Can be optionally supported - Coverage enh - Can be optionally supported - SDT - Can be optionally supported - MBS - Can be optionally supported - IIoT/URLLC further enhancements - Can be optionally supported - NTN - Can be optionally supported - feMIMO - At least some Multi-TRP-related features related to high reliability may be optionally supported - Above-52GHz - Not supported by RedCap UEs - ePositioning - Not supported by RedCap UEs - eIAB - Not supported by RedCap UEs - SL/V2X enh - Not supported by RedCap UEs - DSS enh - Not supported by RedCap UEs - NR DC/CA further enhancements - Not supported by RedCap UEs - DL 1024QAM - Not supported by RedCap UEs |
| [13] | Apple | In addition, we propose to introduce an optional UE feature FG 28-x to indicate the support of DL BWP without SSB in RRC_CONNECTED state, analogous to legacy FG 6-1A. Proposal 2: Introduce a new FG 28-x that is used to indicate the support of a UE-specific DL BWP without SSB when UE is in RRC_CONNECTED state. |
| [14] | CMCC | For the first kind of capability, there are two options to define the UE feature, - Alt.1: No separate new FG is introduced, while add a note in FG6-1(Basic BWP operation with restriction) that “For RedCap UEs, BW of a UE-specific RRC configured BWP may or may not include BW of CD-SSB and the entire CORESET#0, if not, BW of the UE-specific RRC configured BWP includes NCD-SSB. - Alt.2: A new FG 28-x is introduced, with the following component, - BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB. Comparing these two options, we prefer Alt.2. Alt.1 introduces one case that BW of a UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0 into the traditional FG6-1, this is different from what legacy FG6-1 mentions. While Alt.2 has no impact on legacy FG. The prerequisite feature group of this new feature can be FG 6-1, and since this is the default capability, RedCap UEs have to report this capability. Proposal 4: Define a new FG28-x1 for the capability of UE expect NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. Proposal 5: The new FG28-x1 includes the following component, and the prerequisite feature group of this is FG 6-1, - BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB. If the corresponding capabilities are not supported, UE will rely on re-tuning to CD-SSB for such measurement,which may be already supported as FG6-1a. So we propose to introduce a new FG 28-x2, to support this optional capability, which include the following components, - BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0. - support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. - support measurement operation based on FG6-1a. And the prerequisite feature group of this new feature can be FG 6-1a. Proposal 6: Define a new feature FG28-x2 for the capability of supporting operation without NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. Proposal 7: The new FG28-x2 includes the following component, and the prerequisite feature group of this is FG 6-1a, - BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0. - Support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - Support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. - Support measurement operation based on FG6-1a. |
| [17] | MediaTek | As we describe in our companion contribution [3], the existing FG 1-4 is not applicable to RedCap UE because there is no RF retuning is assumed for non-RedCap UE to support the case when a configured CSI-RS resource is a different BWP than the associated SSB. Hence, we propose the following two new UE feature groups to explicitly distinguish the case that requires RF retuning from the case that doesn’t. Proposal 3: Add two new UE feature groups, FG 28-4 and FG 28-5, for RedCap UE to indicate its support for CSI-RS based RRM measurement with associated SSB in a same and a different BWP, respectively. - FG 28-4: CSI-RS based RRM measurement with associated SS-block within maximum RedCap UE bandwidth - FG 28-5: CSI-RS based RRM measurement with associated SS-block exceeding maximum RedCap UE bandwidth Index; Feature group; Component; Prerequisite FG; Type; Note; Mandatory / Optional. 28-4; CSI-RS based RRM measurement with associated SS-block within maximum RedCap UE bandwidth; Perform the following measurement with CSI-RS resource with associated SSB when the bandwidth of the span of the configured CSI-RS resource and the associated SSB is within UE maximum bandwidth 1) CSI-RSRP measurement 2) CSI-RSRQ measurement; [Per band]; Optional with capability signaling. 28-5; CSI-RS based RRM measurement with associated SS-block exceeding maximum RedCap UE bandwidth; Perform the following measurement with CSI-RS resource with associated SSB when the bandwidth of the span of the configured CSI-RS resource and the associated SSB exceeds UE maximum bandwidth 1) CSI-RSRP measurement 2) CSI-RSRQ measurement; FG 6-1a; [Per band]; Optional with capability signaling. In our view, the following mandatory with capability signaling features for non-Red UEs should be optional for RedCap UEs to reduce RedCap UE’s complexity. Table 1: Mandatory features for non-RedCap that are optional for RedCap Index; Feature group; Components; Note; Mandatory / Optional. 1-7; CSI-RS based RLM; CSI-RS based RLM; Applicable to UE supporting more than 4 layers; Mandatory with capability signalling. 2-4a; Additional active TCI state for PDCCH; Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH; Only applicable if Component-1 of 2-4 is set to 1; Mandatory with capability signalling. 2-16b; Support 1+2 DMRS (uplink); Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port; Mandatory with capability signalling. 2-55; SRS Tx switch; 1) Support SRS Tx port switch, 2) Report whether the uplink TX switching impact to downlink receiving in a band, 3) Report whether the UL Tx is switched together with UL Tx in another band; Component-2 is per band pair per band combination Component-3 is per band pair per band combination The band pair in Component-2 and Component-3 can be an LTE band and an NR band 2T4R is 2 pairs of antennas "R" refers to a subset/set of receive antennas for PDSCH; "T" refers to the SRS antennas used for DL CSI acquisition; Mandatory with capability signalling Component-1 is a list of TRx pairs, candidates are {"Not supported", "1T2R", "1T4R", "2T4R", "1T4R/2T4R", "1T=1R", "2T=2R", "4T=4R"} Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no}. Proposal 4: Mandatory FGs 1-7, 2-4a, 2-16b, 2-55, and 2-61 should be optional for RedCap. For FG 2-33, 2-35, and 2-51, components for “across all CCs” in the following FGs should not be applicable to RedCap UEs because they are related to CA aspects. Table 2: Mandatory UE features for non-RedCap UEs that are mandatory for RedCap UEs but with different components and/or values Index; Feature group; Components; Note; Mandatory / Optional. 2-33; CSI-RS and CSI-IM reception for CSI feedback; 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC; All the candidate values are the range of capability signalling which doesn't determine whether UE is mandatory to support all the signalling values.; Mandatory with capability signalling Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 }. 2-35; CSI report framework; 1) Maximum number of periodic CSI report setting per BWP for CSI report 2) Maximum number of periodic CSI report setting per BWP for beam report 3) Maximum number of aperiodic CSI report setting per BWP for CSI report 4) Maximum number of aperiodic CSI report setting per BWP for beam report 5) Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC, 6) Maximum number of semi-persistent CSI report setting per BWP for CSI report 7) Maximum number of semi-persistent CSI report setting per BWP for beam report 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type.; Other MIMO capabilities than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update The CSI report in component 4 and 5 includes the beam report and CSI report Each component is independent CSI report setting are counted in the CC indicated by the parameter carrier in CSI-ResourceConfig.; Mandatory with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-1a candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-2a candidate values {1, 2, 3, 4} Component-2b candidate values {3, 7, 15, 31, 63, 128} Component-3 candidate values: {0, 1, 2, 3, 4} Component-3a candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32}. 2-51; TRS (CSI-RS for tracking); 1) TRS burst length (X), 2) Max # of TRS resource sets (per CC) UE is able to track simultaneously 3) Max # of TRS resource sets configured to UE per CC 4) Max # of TRS resource sets configured to UE across CCs; Mandatory with capability signalling Component-1: Candidate values {1, "both 1 and 2"}. UE is mandated to report "both 1 and 2" Component-2: Candidate value set: {1 to 8} Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. Component-4: Candidate value set: {1 to 256} UE is mandated to report at least 16 for FR1 and 32 for FR2. Optional with capability signalling. Proposal 5: Components 4) and 6) of mandatory FG 2-33 are not applicable to RedCap. Proposal 6: Component 9) of mandatory FG 2-35 are not applicable to RedCap. Proposal 7: Component 4) of mandatory FG 2-51 are not applicable to RedCap. The following optional FGs should not be applicable to RedCap UEs because they require more than two DL MIMO layers. Table 3: Optional features FG 1-4 and FG 1-5 related to CSI-RS L3 measurement are not applicable to RedCap UE. Index; Feature group; Components; Prerequisite; Mandatory / Optional. 1-4; CSI-RS based RRM measurement with associated SS-block; 1) CSI-RSRP measurement 2) CSI-RSRQ measurement; 1-1, CSI-RS; Optional with capability signaling. 1-5; CSI-RS based RRM measurement without associated SS-block; 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed; 1-1, CSI-RS; Optional with capability signaling. Proposal 8: Optional FGs 1-4 and 1-5 related to CSI-RS L3 measurement are not applicable to RedCap. Proposal 9: The following optional feature groups related UE processing time capability 2 are not applicable to RedCap: - FGs 5-5a, 5-5b, 5-5c, 5-13, 5-13a-f - Uplink CBG with UE processing capability 2: FGs 22-3a, 22-3b, 22-3c, 22-3d, 22-3e, 22-3f, 22-g, 22-h Proposal 10: The following optional feature groups related to uplink CBG-based retransmission are not applicable to RedCap: - FG 5-25: CBG-based re-transmission for UL using CBGTI - FG 11-12: CBG-based re-transmission for UL using CBGTI with only in-order CBG-based re-transmission(s) for cancelled initial PUSCH transmission - Uplink CBG with UE processing capability 2: FGs 22-3a, 22-3b, 22-3c, 22-3d, 22-3e, 22-3f, 22-g, 22-h - Uplink CBG with UE processing capability 1: FGs 22-4a, 22-4b, 22-4c, 22-4d Proposal 11: The following optional feature groups related to URLLC are not applicable to RedCap: - FG 11-2x: sub-slot based PDCCH monitoring - FG 11-3x: support more than one PUCCH for HARQ-ACK in a slot - FG 11-4x: support HARQ-ACK codebooks with different priorities at a UE - FG 11-7x: UL cancellation Proposal 12: Optional RF/RRM FG1-1 (60kHz of subcarrier spacing for FR1) is not applicable for RedCap. Proposal 13: Optional FG6-4 (BWP adaptation with different numerologies) is not applicable for RedCap. We don’t see a need to mandate RedCap UEs to support optional features. Besides, we think RAN1 should prioritize on identifying features that are not applicable to RedCap to reduce RedCap UE implementation complexity. Proposal 14: Optional UE features for non-RedCap UEs are not mandated for RedCap UEs. Among other Rel-17 work items, we prefer only features related power saving and coverage enhancements are considered as applicable to RedCap. The reason is not to complicate other Rel-17 work item’s advanced features by RedCap UE’s stringent design constraints and causing unnecessary optional components. We hence propose that feature groups in the following Rel-17 work items are not applicable to RedCap. For feature groups in other work items, we can discuss at a per-feature-group level. Proposal 15: Rel-17 FGs 31-x in NR_IAB_enh, 34-x in NR_DSS, and 35-x in LTE_NR_DC_enh2 are not applicable to RedCap. Proposal 16: Rel-17 FGs 26-x in NR_NTN_solutions are not applicable to RedCap. Proposal 17: Rel-17 FGs 25-x in NR_IIOT_URLLC_enh are not applicable to RedCap. Proposal 18: Rel-17 FGs 27-x in NR_pos_enh are not applicable to RedCap. |

Discussion

[FL1] Medium priority question 6-1:

- Companies are encouraged to provide views on which Rel-15 UE features (FGs 1-x to 8-x) are not applicable to RedCap UEs

| Company | Comment |
| --- | --- |
| FUTUREWEI | In the discussions for the LS reply, companies had different views for a number of FGs and it was difficult to achieve consensus on specific FGs |
| Moderator | As explained in the GTW session on Feb 21, repeated discussion as previous LS (R1-2112754), which is summarized in R1-2112503, is not necessary. Companies are encouraged to provide views on which Rel-15 UE features are not applicable to RedCap UEs on top of the discussion in previous RAN1 meetings. |
| MediaTek | We think FG 1-4 (CSI-RS based RRM with associated SSB) and 1-5 (CSI-RS based RRM without associated SSB) are not applicable to RedCap. Based on RAN4’s reply LS, CSI-RS is not used as a standalone mechanism for RRM measurements. Besides, in legacy, for non-RedCap UE to support FG 1-4, no RF retuning is required as SSB presence is guaranteed within UE’s CBW which is not greater than the UE’s maximum BW. Due to its reduced maximum BW, a RedCap UE must perform RF retuning if the frequency span of the CSI-RS and its associated SSB is larger than RedCap UE’s maximum BW. Proposal 1: FG 1-4 and FG 1-5 are not applicable to RedCap. Proposal 2: If “CSI-RS based RRM measurement with associated SSB” is supported by RedCap, add the following two new UE feature groups as counterparts for FG 1-4. Index; Feature group; Component; Prerequisite FG; Type; Note; Mandatory / Optional. 28-x1; CSI-RS based RRM measurement with associated SS-block within maximum RedCap UE bandwidth; Perform the following measurement with CSI-RS resource with associated SSB when the bandwidth of the span of the configured CSI-RS resource and the associated SSB is within UE maximum bandwidth 1) CSI-RSRP measurement 2) CSI-RSRQ measurement; FG 28-1; [Per band]; Optional with capability signaling. 28-x2; CSI-RS based RRM measurement with associated SS-block exceeding maximum RedCap UE bandwidth; Perform the following measurement with CSI-RS resource with associated SSB when the bandwidth of the span of the configured CSI-RS resource and the associated SSB exceeds UE maximum bandwidth 1) CSI-RSRP measurement 2) CSI-RSRQ measurement; FG 28-1 and 6-1a; [Per band]; Optional with capability signaling. Proposal 3: Mandatory FGs 1-7, 2-4a, 2-16b, 2-55, and 2-61 should be optional for RedCap. Proposal 4: Components 4) and 6) of mandatory FG 2-33 are not applicable to RedCap. Proposal 5: Component 9) of mandatory FG 2-35 are not applicable to RedCap. Proposal 6: Component 4) of mandatory FG 2-51 are not applicable to RedCap. |
| Nokia, NSB | In our view no further restrictions are needed from what has been indicated to RAN2 already in RAN1 LS. |
| Ericsson | Beside what has already been communicated in the earlier LS (R1-2112754), in general we see no strong motivation to forbid RedCap UEs to support any other Rel-15/16 NR features as optional features. |
| Qualcomm | Agree with Ericsson |
| Sierra Wireless | No other feature restrictions are necessary in addition to the ones communicated earlier in LS (R1-2112754). |
| FL2 | Given most companies don’t see the necessity for further discussing this issue, no further input is necessary. The applicability of FGs 1-4/1-5, which is pointed out by MTK, will be discussed after some progress is made in AI 8.6.1.1 |

[FL1] Medium priority question 6-2:

- Companies are encouraged to provide views on which Rel-16 UE features (FGs 9-x to 22-x) are not applicable to RedCap UEs

| Company | Comment |
| --- | --- |
| FUTUREWEI | In the discussions for the LS reply, companies had different views for a number of FGs and it was difficult to achieve consensus on specific FGs |
| Moderator | As explained in the GTW session on Feb 21, repeated discussion as previous LS (R1-2112754), which is summarized in R1-2112503, is not necessary. Companies are encouraged to provide views on which Rel-16 UE features are not applicable to RedCap UEs on top of the discussion in previous RAN1 meetings. |
| Nokia, NSB | In our view no further restrictions are needed from what has been indicated to RAN2 already in RAN1 LS. |
| Ericsson | Beside what has already been communicated in the earlier LS (R1-2112754), in general we see no strong motivation to forbid RedCap UEs to support any other Rel-15/16 NR features as optional features. |
| Qualcomm | Agree with Ericsson |
| Sierra Wireless | No other feature restrictions are necessary in addition to the ones communicated earlier in LS (R1-2112754). |
| FL2 | Given all companies don’t see the necessity for further discussing this issue, no further input is necessary. |

[FL1] Medium priority question 6-3:

- Companies are encouraged to provide views on which Rel-17 UE features (FGs 23-x to 36-x) are not applicable to RedCap UEs

| Company | Comment |
| --- | --- |
| FUTUREWEI | There are some views on FGs not applicable to RedCap UE For FG 31 – Do not support FG that are IAB- related (RAN2 indicated capabilities are not applicable for RedCap UEs include IAB related capabilities) For FG 34 - Do not support FGs that indicate cross-carrier scheduling due to WID objective of no CA For FG 35 - Do not support FGs that indicate dual connectivity due to WID objective of no DC We are also OK to disallow FG 36 (1024QAM) if it is the majority view, as we expect the market will go that way anyway |
| Moderator | As explained in the GTW session on Feb 21, detailed discussion per FG would not be necessary but some high level (e.g., WI level) assumption can be discussed. Companies are encouraged to provide views on which Rel-17 FGs for a specific WI are not applicable to RedCap UEs. |
| MediaTek | We think the following R17 features are not applicable to RedCap to align with WID description. - FGs 31-x in NR_IAB_enh, - FGs 34-x in NR_DSS - FGs 35-x in LTE_NR_DC_enh2 - FGs 36-x in NR_DL1024QAM_FR1 - FGs 24-x in NR_ext_to_71GHz - FGs 25-x in NR_IIOT_URLLC_enh - FGs 26-x in NR_NTN_solutions - FGs 27-x in NR_pos_enh |
| NTT DOCOMO | We share the same view as FUTUREWEI that FG31 (eIAB), FG34 and FG35 (CA/DC enhancement) is not applicable for RedCap UE. |
| Nokia, NSB | In our view no further restrictions are needed from what has been indicated to RAN2 already in RAN1 LS. Enhancements to features that are not supported by RedCap UEs are naturally not supported either. |
| Intel | The following R17 features should not be applicable to RedCap UEs: - Above-52GHz - ePositioning - eIAB - SL/V2X enh - DSS enh - NR DC/CA further enhancements - DL 1024QAM |
| FL2 | Given companies have different view on which Rel-17 UE features (FGs 23-x to 36-x) are not applicable to RedCap UEs, no further input is required. If companies see strong concern not having further decision, please explain why such FGs should be excluded from RedCap UE. |
| Intel | Given we have already agreed to exclude certain features for RedCap from Rel-15/16, it seems rather inconsistent if other related (e.g., NR DC/CA further enhancements, DSS enhancements that deal with CA/DC, eIAB, SL/V2X enh, etc.) and other complex features with heavy demands from UE from Rel-17 (e.g., A52GHz, ePositioning, DL 1024QAM) are considered available to RedCap UEs for optional support. |
| NTT DOCOMO | At the previous RAN1 meeting, it has been already agreed that RedCap UE does not support carrier aggregation and IAB related capabilities in Rel-15/16. Similarly for Rel-17, carrier aggregation and IAB related capabilities are naturally not applicable for RedCap UE. |

- Conclusions

Following agreements were made in this meeting.

Agreement

- Following capabilities are added as components in FG 28-1

  - a) Separate initial UL BWP for RedCap UEs

    - It includes the configuration(s) needed for RedCap UE to perform random access

  - b) Separate initial DL BWP for RedCap UEs

    - It includes CSS/CORESET for random access

    - FFS: For separate initial DL BWP used for paging, CD-SSB is included

    - For separate initial DL BWP only used for RACH, SSB may or may not be included

Agreement

- Component in FG 28-1 is updated as follows

  - a) Separate initial UL BWP for RedCap UEs

    - It includes the configuration(s) needed for RedCap UE to perform random access

    - Enabling/disabling of frequency hopping for common PUCCH resources

- Add a note in FG 28-1: It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A

Agreement

- Delete FG 28-2

- (As conclusion) It is up to RAN4 whether/how to report UE Rx structure for FR2

References

[1]	R1-2200780	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2200919	Rel-17 UE features for RedCap	Huawei, HiSilicon

[3]	R1-2200986	Discussions of UE features for RedCap	FUTUREWEI

[4]	R1-2201124	Discussion on UE feature for NR REDCAP	vivo, Guangdong Genius

[5]	R1-2201139	Discussion on RedCap UE features	ZTE, Sanechips

[6]	R1-2201290	Rel-17 RedCap UE features	OPPO

[7]	R1-2201348	Discussion on Rel-17 UE features for RedCap	CATT

[8]	R1-2201413	On UE features for REDCAP	Nokia, Nokia Shanghai Bell

[9]	R1-2201506	Discussion on UE features for RedCap	NTT DOCOMO, INC.

[10]	R1-2201606	Discussion on UE feature for RedCap	NEC

[11]	R1-2201672	UE features for RedCap	Ericsson

[12]	R1-2201731	On UE features for Rel-17 RedCap	Intel Corporation

[13]	R1-2201796	UE features for RedCap	Apple

[14]	R1-2201884	Discussion on UE features for RedCap	CMCC

[15]	R1-2201954	Discussion on UE features on RedCap	Xiaomi

[16]	R1-2202043	UE feature for RedCap	Samsung

[17]	R1-2202062	UE features for REDCAP	MediaTek Inc.

[18]	R1-2202170	UE features for RedCap	Qualcomm Incorporated

[19]	R1-2201957	Discussion on the remaining issues of RAN2-led features for RedCap	Xiaomi