3GPP TSG RAN WG1 #108-e			R1-2201521

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.15

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for LTE based 5G terrestrial broadcast

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.15 regarding UE features for LTE based 5G terrestrial broadcast and captures the following email discussion.

[108-e-R17-UE-features-LTE-Bcast-01] Email discussion on UE features for LTE based 5G terrestrial broadcast – Shinya (DOCOMO) - 1st check point: February 25 - Final check point: March 3

In the updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e [1], there is following feature group for LTE based 5G terrestrial broadcast.

- 3-1		Support of new channel bandwidth for PMCH

- 3-1: Support of new channel bandwidth for PMCH

In [1], FG 3-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3. LTE_terr_bcast_bands_part1 | 3-1 | Support of new channel bandwidth for PMCH | [TBD: whether separate components are needed for different bandwidths] | Support of dedicated MBMS cells | Yes |  | UE cannot receive MBMS in the corresponding MBSFN area | Per band | N/A | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [2] | Huawei, HiSilicon | The remaining issue is whether separate components are needed for different bandwidths. As agreed the supported system bandwidth indicated in MIB is set to 5 MHz or 3 MHz, for which PMCH can be configured with 6/7/8 MHz. From the perspective of UE implementation capability, a single UE capability supporting flexible bandwidth of 6/7/8 MHz is sufficient. The component of the feature could be written to support the feature of flexible bandwidth for PMCH with system bandwidth limited to 5 or 3 MHz. However, there is worry from other company that UE that is targeting a single region shall implement the bandwidths for the other regions with the corresponding potential different RF requirements if a common capability is defined. With this regard taken into account, separate UE capabilities corresponding to different bandwidth is also acceptable. Proposal: - The component of the feature can be written as: - Support flexible PMCH bandwidth (6/7/8MHz) allocation for the system bandwidth indicated in MIB set to 5 MHz or 3 MHz. - Alternatively, define separate UE capabilities corresponding to different bandwidth (6/7/8 MHz). |
| --- | --- | --- |
| [3] | ZTE | This issue has been extensively discussed in RAN1#107-e meeting [2]. If separate components are defined for different bandwidths, it may end up with that different UEs support different bandwidths, which may complicate the network deployment. However, if no separate component for different bandwidths, UEs may have to implement some other bandwidths even if one certain bandwidth may not be used in a certain region. It seems the following two approaches are possible. We propose to further discuss the following two approaches. Proposal 1: RAN1 further discusses the following two approaches. Approach 1) - RAN1 defines separate UE capabilities for 6M, 7M and 8M - RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region Approach 2’) - RAN1 doesn't define separate UE capabilities for 6M, 7M and 8M - RAN1 informs RAN4 of the RAN1's UE capability design. RAN4 can then proceed with their work, including potential refinement of related UE capability design, if needed. |
| [4] | Nokia, NSB | Regarding TBD on listing separate components for different bandwidths, it should be noted that components cannot be individually supported/not supported. Moreover, from RAN1 point of view there is no reason for separate indication of bandwidths, and hence this FG should be a simple supported/not supported indication for all bandwidths. |
| [5] | DOCOMO | - 3-1: Support of new channel bandwidth for PMCH - Whether separate components/FGs for different bandwidths are necessary or not has been discussed in previous RAN1 meetings [2]. It was pointed that RAN4 will define the band plan for broadcast UHF in Rel-18, and hence it is unclear whether single FG is sufficient or not when Rel-17 capability signalling is designed and finalized. - Given the strong view on the separate indication for the support of different PMCH bandwidths, we can live with having separate FGs i.e., one FG for 6 MHz PMCH bandwidth, one FG for 7 MHz PMCH bandwidth and one FG for 8 MHz PMCH bandwidth. As we (and some other companies) commented at the last meeting, support or not support for each component within a FG cannot be individually indicated according to the definition of “FG” provided by RAN2. Therefore, if we go to support separate indication for the support of different PMCH bandwidths, separate FGs are necessary and original 3-1 as general support of new PMCH bandwidth may not be necessary in such case. - There was an alternative compromised proposal to capture the note for FG 3-1 “UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region”. We are also fine with this alternative. Proposal 1: For FG 3-1, one of following alternatives is adopted. - Alt.1: having separate FGs i.e., one FG for 6 MHz PMCH bandwidth, one FG for 7 MHz PMCH band-width and one FG for 8 MHz PMCH bandwidth, instead of having separate components within FG 3-1 - Alt.2: adding a note for FG 3-1 “UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region” |
| [6] | Qualcomm | The support of different channel bandwidths is due to the different channelization for broadcast UHF spectrum in different regions. At this stage, it is unclear how RAN4 will decide on the band plan for broadcast UHF, but it is possible that different regions reuse the same band number, but use different channel bandwidth. In this case, it would be necessary for a UE to be able to support only a subset of the bandwidths. Otherwise, there will be additional UE complexity (due to having to implement bandwidths that the UE may not use), and potential lack of IODT opportunities (e.g. if some of the regions do not deploy 5G broadcast). Thus, we make the following proposal: Proposal 1: FG 3-1 has separate components for different bandwidths, and their support is separately indicated. |

Discussion

[FL1] Proposal 2-1:

- For different bandwidths for PMCH, select one of the following approaches

  - Approach 1:

    - RAN1 defines separate UE capabilities for 6MHz, 7MHz and 8MHz

    - RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region

  - Approach 2:

    - RAN1 doesn’t define separate UE capabilities for 6MHz, 7MHz and 8MHz

    - RAN1 informs RAN4 of the RAN1’s UE capability design. RAN4 can then proceed with their work, including potential refinement of related UE capability design, if needed.

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Approach 1: HW/HiSi, ZTE, DOCOMO, QC(?) - Approach 2: HW/HiSi, ZTE, Nokia/NSB, DOCOMO This issue was discussed in RAN1#107-e but no consensus was achieved. Approach 1 was acceptable for most companies and hence, companies are invited to provide view whether you can live with Approach 1. |
| NTT DOCOMO | We are fine with Approach 1. |
| QC | We are fine with Approach 1 with the following minor modification: - RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region (for those regions and bands where regulatory requirements allow multiple bandwidths among 6/7/8MHz) |
| Huawei, HiSilicon | Ok with approach 1 and fine with QC’s revision. |
| ZTE | As summarized by the moderator, we can live with either Approach1 or Approach2. Qualcomm’s update for Approach1 is also fine for us. |
| FL2 | All companies are fine with Approach 1. Let’s final check whether Approach 1 with update from QC can be agreed or not. |
| Nokia, NSB | We thought we had clearly indicated we are not fine with Approach 1, also in the email reflector. Approach 1 is trying to address potential implications of future RAN4 decisions, and it has implications on roaming and system access, none of which is in scope of RAN1. We are fine to indicate to RAN2 and RAN4 that they are free to define more detailed capabilities if they see a need for it, but from RAN1 point of view a single FG is enough. |
| Moderator | Following was agreed via email endorsement Agreement - For different bandwidths for PMCH, - RAN1 defines separate UE capabilities for 6MHz, 7MHz and 8MHz - RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region (for those regions and bands where regulatory requirements allow multiple bandwidths among 6/7/8MHz) - Add a note (in the “Notes” column) saying: NOTE: This capability is sent to a unicast eNB, and indicates whether the UE in RRC_CONNECTED supports MBMS reception via MBSFN from MBMS-dedicated cells in an MBSFN area with 6/7/8MHz. |

- Conclusions

Following agreement was made in this meeting.

Agreement

- For different bandwidths for PMCH,

  - RAN1 defines separate UE capabilities for 6MHz, 7MHz and 8MHz

  - RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region (for those regions and bands where regulatory requirements allow multiple bandwidths among 6/7/8MHz)

  - Add a note (in the “Notes” column) saying:  NOTE: This capability is sent to a unicast eNB, and indicates whether the UE in RRC_CONNECTED supports MBMS reception via MBSFN from MBMS-dedicated cells in an MBSFN area with 6/7/8MHz.

References

[1]	R1-2112900	Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2200979	Rel-17 UE features for LTE based 5G terrestrial broadcast	Huawei, HiSilicon

[3]	R1-2201180	Discussion on Rel-17 UE features for LTE based 5G terrestrial broadcast	ZTE

[4]	R1-2201422	On UE features for LTE based 5G terrestrial broadcast	Nokia, Nokia Shanghai Bell

[5]	R1-2201512	Discussion on UE features for LTE based 5G terrestrial broadcast	NTT DOCOMO, INC.

[6]	R1-2202179	UE features for LTE-based 5G terrestrial broadcast	Qualcomm Incorporated