3GPP TSG RAN WG1 #108-e			R1- 2201530

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.11.1.1

Source:	InterDigital, Inc.

Title:	Remaining issues on resource allocation for power saving

Document for:	Discussion and Decision

# Introduction

In RAN Plenary #94, the following remaining opening issues of sidelink resource allocation for power saving have been confirmed as starting point to finalize the Rel-17 eSL [1]:

- Finalization of pre-emption/re-evaluation checking for aperiodic transmission - Finalization of selection/report of candidate resources in which at least its subset is within RX UE's active time - Finalization of SL CBR measurement in partial sensing - CPS monitoring window for aperiodic transmission when UE performs at least CPS in a Tx pool - T1 of RSW when UE performs only CPS in a Tx pool with periodic reservation for another TB disabled - Sensing and SL CBR measurement during its SL DRX inactive time - Re-evaluation and pre-emption checking after random selection - Resource pool segregation for periodically occurring resources - Random resource selection in pools with mixed RA schemes - Conditions in which CPS can be disabled in resource (re)selection

In RAN1#107b-e meeting, the following agreements and working assumption were made [2]

Agreement

When UE is configured to perform partial sensing by a UE higher layer (including when SL DRX is configured), SL RSSI is measured in slots where the UE performs partial sensing and PSCCH/PSSCH reception over the SL CBR measurement window defined in Rel-16. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured.

- If the number of SL RSSI measurement slots is below a (pre-)configured threshold, a (pre-)configured SL CBR value is used.

Agreement

When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n,

- The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots.

  - tyiSL is the first candidate slot after slot n+T3.

- UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool

  - It is up to UE implementation

- UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL.

  - FFS: When the minimum M slots for CPS cannot be guaranteed,

- All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

- For minimum size M of the CPS monitoring window [n+TA, n+TB]:

  - By default, M is 31 unless (pre-)configured with another value

  - The range of (pre-)configured M is from 0 (working assumption) to 30

Agreement

When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- T1 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4.

  - No update to specification is necessary due to this agreement

- Note: The selected Y’ slots do not overlap with the sensing window

Agreement

Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer.

- When it is enabled,

  - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion.

  - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS.

- Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time.

- Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17.

- FFS the case when full sensing is configured in the UE by a higher layer

In this contribution, we continue to discuss the remaining details of sidelink resource allocation for power saving.

# Remaining open issues

## Resource allocation for transmission to Rx UE in SL DRX

In RAN1 #107e meeting, it was agreed that PHY layer needs to apply a restriction on selecting candidate resources to ensure that at least a subset of candidate resources reported to MAC layer is located within an indicated active time of the RX UE in SL DRX. Specifically, RAN1 agreed that PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE as in the following agreement:

Agreement When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following is agreed: When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. - PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE

One remaining essential issue is how PHY layer can select candidate resources so that at least a subset of the candidate resources is within the indicated active time of the RX UE. It was argued that UE implementation can guarantee such restriction. However, in our view, there may be no remaining candidate resource within the active time of the Rx UE after the resource exclusion procedure. Specifically, according to the legacy resource exclusion procedure, the UE may first select Y or Y’ candidate slots, which can be both inside and outside of active time of the Rx UE, for set SA initialization. The UE then performs resource exclusion procedure and guarantees that the number of remaining candidate resources is at least X.Mtotal. As the result of the resource exclusion procedure, there may be no remaining candidate resource within the active time of the Rx UE. Instead, all the remaining candidate resources are in the inactive time of the Rx UE. Therefore, the previous RAN1 agreement, which guarantees that at least a subset of the candidate resources is within the indicated active time, is not respected.

In the last meeting, to address the above issue, the following proposal was discussed intensively but no consensus was reached due to lack of study.

Proposal 2-1 (IV): When SL DRX active time of RX UE is provided by the higher layer for candidate resource selection for resource (re)selection, - The reported subset of the candidate resources within the provided SL DRX active time of RX UE shall satisfy a threshold. - Option 1: The same higher layer parameter (sl-TxPercentageList) is reused for the ratio / threshold That is, X.Ntotal number of candidate single-slot resources remaining within the SL DRX active time of the initialized set SA in Step 4) is to be met by using the RSRP threshold increment only for the SL DRX active time in Step 7, where the Ntotal is the total number of candidate single-slot resources of the set SA within the SL DRX active time. - The UE shall satisfy this new threshold in addition to the remaining minimum X.Mtotal number of candidate single-slot resources threshold for the whole set SA in Step 7.

In our view, to guarantee at least a subset of candidate resources is within the active time of the Rx UE, the reported subset of the candidate resources within the active time of RX UE shall satisfy a threshold, which can be at least one resource. When the remaining candidate resources within the active time of the Rx UE is smaller than the threshold, the UE may need to increase the RSRP threshold (i.e., 3dB increment) to increase the number of candidate resources in set SA until it meets the threshold.

In the legacy NR V2X R16 resource exclusion procedure, PHY layer needs to report at least X.Mtotal candidate resources for MAC layer to perform random resource selection within the reported set SA. The value of X can be from 20% to 50%, which is (pre-)configured based on priority of the TB. This solution has been proved to reduce the resource collision among the UEs selecting transmission resources at similar time. Since the UE also needs to perform random resource selection at least for the initial transmission in the subset of candidate resources within the active time of the Rx UE, a similar principle can be reused. Specifically, the UE should report at least X.Ntotal candidate resources in the active time of the RX UE, in which Ntotal is the total number of candidate resources of the set SA within the SL DRX active time of the Rx UE.

Proposal 1: When SL DRX active time of RX UE is provided by the higher layer for resource (re)selection, at least X.Ntotal candidate single-slot resources remaining within the SL DRX active time of the set SA is to be met by using the RSRP threshold increment only for the SL DRX active time, where the Ntotal is the total number of candidate single-slot resources of the set SA within the SL DRX active time and X is the value of the sl-TxPercentageList parameter indicated from higher layer.

- The UE shall satisfy this new threshold in addition to the remaining minimum X.Mtotal number of candidate single-slot resources threshold for the whole set SA.

In the following, we further discuss how to select Y or Y’ candidate slots. If the UE selects Y or Y’ candidate slots outside of the Rx UE’s active time, regardless of the resource exclusion procedure, the UE cannot find any candidate resource in the active time of the Rx UE. In addition, if the number of candidate slots within the active time of the Rx UE is too small, the number of candidate resources for random selection at MAC layer will be limited, which may result in more collision. Therefore, to guarantee a proper selection of Y or Y’ candidate slots, it is necessary to restrict at least N candidate slots within the active time of the Rx UE. The number of N can be (pre-)configured by the network per resource pool.

Proposal 2: When SL DRX active time of RX UE is provided by the higher layer for resource (re)selection,

- At least the first N slots of Y or Y’ candidate slots are to be selected within the provided SL DRX active time when partial sensing is (pre-)configured in the UE by higher layer, where N is (pre-)configurable from a range of 1 to 10.

## Resource re-evaluation and pre-emption

In the last meeting, we made the following agreements regarding resource re-evaluation and pre-emption checking for aperiodic transmission:

Agreement When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, - The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots. - tyiSL is the first candidate slot after slot n+T3. - UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool - It is up to UE implementation - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - FFS: When the minimum M slots for CPS cannot be guaranteed, - All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures.

In the following, we address the remaining FFS issue for re-evaluation and pre-emption checking when minimum M slots for CPS cannot be guaranteed. Re-evaluation is used to determine whether the pre-selected resource is still available and reselect another resource if the pre-selected resource is not in the set of candidate resources. Similarly, pre-emption checking is used to determine whether a reserved resource is still available. Therefore, if the minimum M slots for CPS cannot be guaranteed, the UE needs to perform sensing as many slot as possible to maximize the sensing results. Specifically, the UE needs to perform contiguous sensing from the initial resource selection slot to the first candidate slot tyiSL after n+T3.

Proposal 3: When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, if the minimum M slots for CPS cannot be guaranteed, UE performs CPS sensing from the first slot of resource (re)selection trigger slot until before tyiSL-(Tproc,0SL+Tproc,1SL), where tyiSL is the first candidate slots after slot n+T3.

## Random resource allocation in a resource pool with mixed RA schemes

- In the last RAN1 meetings, the following proposal regarding collision mitigation of random resource allocation in a resource pool of mixed RA schemes between random and partial/full sensing was intensively discussed.

Proposal 3-1 (IX): For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, · Option 1: a priority threshold value can be (pre-)configured for the resource, at or below the threshold value which random resource selection is allowed. - Note, lower value means higher priority - FFS: Remaining details for the RRC parameter (e.g., possible priority threshold values, and whether the priority threshold values can be based on different measured CBR) - FFS: whether to support a priority threshold value (pre-)configured for a subset of resources in a resource pool

- In our view, if Option 1 is not supported, a UE can perform random selection in the resource pool for any TB. Such behavior may demotivate the UE to perform sensing since sensing may not be able to avoid collision with random resource selection. Specifically, persistent collisions may happen between reservation of low priority transmission using random selection and periodic reservation of high priority transmission using sensing-based selection. It is because when performing pre-emption checking, the UE performing sensing will not reselect the collided resources as it expects that the low priority transmission will perform the reselection. However, the low priority transmission will not reselect the resource since it does not perform pre-emption. Such collision between the transmission with low priority using random selection and transmission with high priority using sensing-based selection may happen at any transmission and any time. Such collision may degrade the performance of the system and degrade the reliability of the high priority transmissions, which is not desirable.

- One remaining issue is whether to support a priority threshold value (pre-)configured for a subset of resources in a resource pool. In our view, it is not necessary since this design can be obtained in a different way. Specifically, the network can configure the UE with another resource pool containing the subset of resources in an original resource pool. The UE may then be (pre-)configured a priority threshold in the new resource pool containing the subset of the resources.

- Proposal 4: For collision mitigation of random resource selection in a resource pool of mixed RA schemes between random and partial/full sensing, a priority threshold value is (pre-)configured for the resource pool, at or below the threshold value which random resource selection is allowed.

- The (pre-)configured priority threshold can be any of the 8 priority values.

## Remaining details for PBPS

In the last RAN1 meetings, it was agreed that a minimum set of candidate slots for set SA initialization are (pre-)configured from the range of values, in which Ymin and Y’min are used for periodic and aperiodic traffic, respectively. In LTE, the minimum set of candidate slots is (pre-)configured in a resource pool regardless of the QoS of the TB since each TB can have only one or two transmissions. However, in NR, the number of retransmissions for one TB can vary significantly. Specifically, the (pre-)configuration allows one TB to have from zero to 32 retransmissions based on the priority of the TB. A higher number of retransmissions may require more candidate slots Y to reduce collision for each transmission. Hence, Ymin and Y’min and should be (pre-)configured per priority per resource pool.

- Proposal 5: The minimum set of candidate slots, Ymin and Y’min, for set SA initialization are (pre-)configured per priority.

# Conclusion

In this contribution, the following conclusions were made for resource allocation for power saving:

Proposal 1: When SL DRX active time of RX UE is provided by the higher layer for resource (re)selection, at least X.Ntotal candidate single-slot resources remaining within the SL DRX active time of the set SA is to be met by using the RSRP threshold increment only for the SL DRX active time, where the Ntotal is the total number of candidate single-slot resources of the set SA within the SL DRX active time and X is the value of the sl-TxPercentageList parameter indicated from higher layer.

- The UE shall satisfy this new threshold in addition to the remaining minimum X.Mtotal number of candidate single-slot resources threshold for the whole set SA.

Proposal 2: When SL DRX active time of RX UE is provided by the higher layer for resource (re)selection,

- At least the first N slots of Y or Y’ candidate slots are to be selected within the provided SL DRX active time when partial sensing is (pre-)configured in the UE by higher layer, where N is (pre-)configurable from a range of 1 to 10.

Proposal 3: When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, if the minimum M slots for CPS cannot be guaranteed, UE performs CPS sensing from the first slot of resource (re)selection trigger slot until before tyiSL-(Tproc,0SL+Tproc,1SL), where tyiSL is the first candidate slots after slot n+T3.

- Proposal 4: For collision mitigation of random resource selection in a resource pool of mixed RA schemes between random and partial/full sensing, a priority threshold value is (pre-)configured for the resource pool, at or below the threshold value which random resource selection is allowed.

- The (pre-)configured priority threshold can be any of the 8 priority values.

- Proposal 5: The minimum set of candidate slots, Ymin and Y’min, for set SA initialization are (pre-)configured per priority.

# References

- RP-212880, “Status Report to TSG”, LG Electronics.

- RAN1#107b-e Chairman’s Notes, January 2022.